Technical Specification: High-Density Passive Multimode Optical TAP (OM4, 70/30)

| Specification Parameter | Technical Standard & Requirement |
| --- | --- |
| System Classification | Inline Passive Optical Test Access Point (TAP) Module & Chassis System |
| Optical Medium | 50/125 μm Laser-Optimised Multimode Fibre (MMF), OM4 (backward compatible with OM3) |
| Operating Wavelengths | Dual-window operation: 850 nm and 1300 nm (optimised for 850 nm VCSEL sources) |
| Optical Connectors | Duplex LC with UPC (Ultra Physical Contact) polish across all ports |
| Split Ratio | 70:30 (70% optical power preserved for live network; 30% diverted to monitor path) |
| Port Configuration (per TAP Link) | Total 4 x Duplex LC Ports: • 2 x Network Ports: Network A (In/Out) & Network B (In/Out) for inline live traffic • 2 x Monitor Ports: Tap A (Rx capture) & Tap B (Tx capture) dedicated capture paths |
| Network Insertion Loss (70%) | ≤2.0 dB to 2.2 dB (typical, including connector mating loss) |
| Monitor Insertion Loss (30%) | ≤5.8 dB to 6.2 dB (typical, including connector mating loss) |
| Return Loss (Optical Reflectance) | ≥25 dB (MMF UPC) |
| Supported Data Rates | Protocol and bit-rate agnostic: 1 Gbps, 10 Gbps, 25 Gbps, 40 Gbps, 100 Gbps, and 400 Gbps |
| Chassis Form Factor | 1U 19-inch standard rack-mountable chassis, modular high-density cassette design (supporting 8, 16, or up to 24 TAP modules per 1U) |
| Power Requirements | 100% Passive (0 Watts); no power supply, no active cooling, zero MTBF hardware failure point |
| Environmental Parameters | • Operating Temperature: -20∘C to +70∘C • Operating Relative Humidity: 5% to 95% non-condensing |
| Compliance & Standards | IEEE 802.3ae (10G), IEEE 802.3ba (40G/100G), ANSI T11.2 (Fibre Channel), RoHS, REACH |