Bid Specification:

| RFB No: | RFB 3053-2024 |
| --- | --- |
| Description | Request for the Building Contractor to Upgrade the Training Centres, Including the Offices at Centurion and Erasmuskloof |
| Publication Date | 21 July 2025 |
| Compulsory Briefing Session | Compulsory Briefing Session will be held as follows: Date: 31 July 2025 Time: 11H00am Place: We will start at Appollo Building Erasmuskloof and proceed to Centurion security search point (vulibag) |
| Closing Date for questions / queries | 07 August 2025 at 16:00pm |
| Bid Response Submission Address | Tender Office, Pongola in Apollo 459 Tsitsa Street, Erasmuskloof, Pretoria, 0048 |
| RFB Closing Details and Time | Date: 15 August 2025 Time: 11:00am (South African Time) |
| RFB Validity Period | 200 Days from the closing date |

# Contents

1.	Introduction	4

1.1	Purpose	4

1.2	Background	4

2.	Scope of Bid	5

2.1	Project Scope of Work	5

2.2	Measurements, Quantities & Bill of Quantities (BoQ) – Summary	7

2.2.1	Erasmuskloof Training Room Dimensions and Specifications	7

2.2.2	Centurion Training Rooms Dimensions and Specifications	16

2.2.3	Centurion Training Ablution Facilities Dimensions and Specifications	25

2.3	General Notes on Compliance	33

2.4	Delivery address	33

2.5	Customer Infrastructure and environment requirements	33

2.6	Service Elements	34

2.6.1	Full-Service Agreement	34

3.	Bid Evaluation Stages	35

3.1	Administrative requirements (Stage 1)	35

3.1.1	Attendance of briefing session	35

3.1.2	Registered Supplier	35

3.2	Technical returnable documents	36

3.2.1	Instruction and evaluation criteria	36

3.2.2	Technical mandatory requirements (Stage 2)	36

3.3	Special Conditions of Contract Verification (Stage 3)	37

3.3.1	Special Conditions of Contract	37

3.3.2	Contracting Conditions	37

3.3.3	Delivery Address	37

Physical Address	37

3.3.4	Services and Performance Metrics	38

3.3.5	Supplier Performance Reporting	42

3.3.6	Certification, Expertise and Qualification	42

3.3.7	Logistical Conditions	42

3.3.8	Regulatory, Quality and Standards	42

3.3.9	Security screening and clearance requirements	42

3.3.10	Confidentiality and non -disclosure conditions	43

3.3.11	Guarantee and warranties	44

3.3.12	Intellectual Property Rights	44

3.3.13	Counter Conditions	45

3.3.14	Fronting	45

3.3.15	Business Continuity and Disaster Recovery Plans	45

3.3.16	Supplier Due Diligence	45

3.3.17	Preference Goal Requirements conditions	45

3.3.18	Declaration of compliance and acceptance SCC	46

3.4	Price and Preference Points Evaluation (Stage 4)	46

3.4.1	Costing and Preference Evaluation	46

3.4.2	Costing and Pricing Conditions	47

3.4.3	Bid Pricing Schedule	47

3.4.4	Declaration of Acceptance	47

4.	Preference Requirements	47

Annex A: Bidder substantiating evidence	51

5.	Technical Mandatory Requirement Evidence	51

5.1	Bidder Experience and Capability Requirements	51

5.2	CIDB rating	51

The Bidder must complete and sign ANNEX D as evidence that the Bidder, and/or its Subcontractor is registered with the CIDB with a minimum rating of GB4, or higher	51

6.	Preference Points Preferential Goals Evidence	51

Annex B: CIDB REGISTRATION REQUIREMENT	52

# Tables

Table 1: Erasmuskloof Training Room Dimensions and Specifications	8

Table 2: Centurion Training Room Dimensions and Specifications	16

Table 3: Centurion Training Ablutions (H1.1 & H1.3) Dimensions and Specifications	26

Table 4: Centurion Training Ablutions (HH1.1 & HH1.2) Dimensions and Specifications	29

Table 4: Bid Evaluation Stages	35

Table 5: Technical Mandatory Requirements	36

Table 6: References	51

# Introduction

## Purpose

The purpose of this RFB is to invite Service Providers (hereinafter referred to as “bidders”) to submit bids for the appointment of the service provider for the upgrading of the training rooms for a period of two (2) years.

## Background

The current condition of the training centres and the Auditorium at the Erasmuskloof and Centurion buildings has significantly deteriorated, impacting their effectiveness and usability. To maintain high standards and ensure these spaces meet the needs of our clients and high-profile stakeholders, a comprehensive upgrade is required.

Both Erasmuskloof and Centurion are integral to SITA operations, serving as key venues for training SITAzens and, clients. However, the facilities have suffered from wear and tear over the years. Carpets, blinds, and ceiling boards are in a dilapidated state while the paint and lighting are outdated and require modernization. The current environment does not provide an atmosphere conducive to effective learning.

# Scope of Bid

The objective of this project is to upgrade existing training facilities at both the Erasmuskloof and Centurion buildings. This includes interior finishes, basic electrical and data infrastructure, lighting, and ventilation, ensuring that the facilities meet current standards and enhance the learning environment.

All work must conform to the applicable National Building Regulations, Occupational Health and Safety Act, and the SANS 10400 series.

## Project Scope of Work -

- Removal and Replacement of Lighting Systems

  - Remove outdated or faulty lighting systems and fittings and replace them with new energy-efficient LED (including dimmable fittings where specified). All electrical work must adhere to SANS 10142 (where applicable) and relevant occupational health and safety requirements.

  - Ensure compliance with SANS 10400–O (Lighting and Ventilation), particularly regarding minimum illumination levels and safe electrical installations.

- Suspended Ceilings and Grids

  - Uninstall and remove all old or damaged suspended ceilings (including grids).

  - Where necessary, temporarily remove and reinstall Wi-Fi devices, projectors, fire detectors, and alarms to accommodate the new ceilings and lighting layouts.

  - Install new ceiling systems in compliance with SANS 10400, ensuring safe access to ceiling voids for maintenance (e.g., for Wi-Fi routers, fire detectors, and alarms).

- Repositioning of Equipment and Fittings

  - Where necessary, temporarily remove and reinstall Wi-Fi devices, projectors, fire detectors, and alarms to accommodate the new ceilings and lighting layouts.

  - Verify all reinstalled equipment meets statutory fire-protection requirements in SANS 10400–T (Fire Protection).

- Painting of Walls and Doors

  - Prepare and repaint walls and doors using durable, washable paint.

  - All paint applications must align with manufacturer and environmental regulations for indoor air quality and occupant safety.

- Blinds

  - Remove existing blinds were indicated and install new similar blinds suited for training and office environments/rooms (e.g., vertical or roller blinds).

  - Comply with safe installation methods to avoid damage or obstructions near fire escapes and ventilation sources.

- Carpet Tiles

  - Remove existing carpets and install new carpet tiles suitable for high-traffic training areas.

  - Adhere to SANS 10400-O guidelines to avoid slip hazards and ensure accessibility for persons with disabilities (e.g., level transitions between different flooring surfaces).

- Glazing and Shopfronts

  - All new glass (shopfronts, doors, internal glazing) must be safety glass or laminated glass, per SANS 10400-N and AAAMSA specifications.

  - All aluminium shopfronts are to be weatherproofed and sealed with durable gaskets and sealants.

  - Edges and beading to be sealed with weatherproof materials that withstand temperature and moisture variations.

- Upgrade of Ablution Facilities

  - Supply and install new white ceiling tiles.

  - Supply and install wall-mounted toilet pans for wall cistern with its proper toilet seat covers, replace wall plumping pipes up to the connection joints at the back-service entrance (provision of up to 1m piping).

  - Supply and install new urinals with its proper wall cistern and automatic flushing system.

  - Supplying and installing urinal automatic sanitizer dispenser

  - Supply and install wall mounted dispensers (i.e., for soap, hand towel, seat sanitizer, jumbo roll toilet paper).

  - Supply and install new wall tiles, including toilet cubicle walls.

  - Supply and lay new floor matt tiles.

  - Supply and install new vanity slab, also boxing the vanity to cover waste pipes

  - Supply and install new washbasins with waste outlets and water taps.

  - Painting toilet cubicle doors.

  - Supply and install bathroom explanatory/safety signage - Male and Female

  - Supply and install electric hand dryer machine.

- General Requirements

  - Health and Safety: The appointed contractor must enforce strict housekeeping, signage, and personal protective equipment (PPE) use. A safety file and method statements must be prepared prior to commencement of work, in line with the OHS Act. This safety file must be kept on site.

  - Quality Assurance & Inspections: All materials (carpet tiles, paint, blinds, ceiling tiles) must be SABS-approved and installed according to manufacturer guidelines. Frequent inspections by the site supervisor or project manager must confirm compliance with local building standards.

  - Environmental Considerations: Comply with the Occupational Health and Safety Act (Act No. 85 of 1993) throughout the project, including scaffolding, elevated work, and waste disposal. Maintain a clean, hazard-free work area throughout the project. Waste disposal, dust control, and noise management plans must be in place.

  - Accessibility: Where applicable, door widths, thresholds, and any approach/ramps must comply with SANS 10400 Part S (Facilities for Persons with Disabilities).

- Project Coordination

  - Coordinate with on-site facility managers to minimize disruptions.

  - Any work outside standard business hours must be pre-approved to maintain occupant safety and comply with local noise or access ordinances.

### Measurements, Quantities & Bill of Quantities (BoQ) – Summary

The tables below detail a summary of the floor areas, ceiling areas, wall surface areas, door counts, and blinds to be replaced in the training facilities only. All measurements are approximate and must be verified on site by qualified personnel before finalizing construction plans. Where relevant, references to double doors and single doors have been retained for accuracy but must be reassessed for compliance with SANS 10400–T (fire exits and egress).

### Erasmuskloof Training Room Dimensions and Specifications

The scope of work for all training room listed on the table below include:

- Removal of exit burglar bars

- Supply and new shopfront as per drawings

- Uninstall old carpet tiles and install new carpet tiles

- Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system

- Replacement of Wooden Skirting

- Vanishing Wooden Skirting

- Preparation and Painting of Walls

- Supply and Install Blinds

- Preparation and painting of single doors

- Preparation and painting of double doors

- Remove bugler bars on the opening of the shopfront and replace with shopfronts.

- Supply and Install a Canopy over Tugela & Incus Data Training Rooms.

Table 1: Erasmuskloof Training Room Dimensions and Specifications

| Training Room | Floor Area (m²) | Floor-to-Ceiling Height (mm) | Door Type & Dimensions (mm) | Aluminium Shopfront | PDF Reference | Notes & Regulatory References |
| --- | --- | --- | --- | --- | --- | --- |
| Palala | 61.21 | 2840 | Single door 813×2032 | Type-1 Approx. 3520×1800 glass frontage (verify on site) | Erasmuskloof Training Rooms 01 | - Remove existing burglar bars - Supply and new shopfront as per drawings Glazing: laminated double safety glass (SANS 10400 Part N). Install as per AAAMSA specs. - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting - Repaint walls |
| Molopo | 61.21 | 2840 | Single door 1610×2032 | Type-2 Approx. 3520×1800 glass frontage (verify on site) | Erasmuskloof Training Rooms 02 | - Remove existing burglar bars - Supply and new shopfront as per drawings Glazing: laminated double safety glass (SANS 10400 Part N). Install as per AAAMSA specs. - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting - Repaint walls |
| S14 | 17.58 | 2640 | Single door 813×2032 | Type-2 Approx. 3520×1800 glass frontage (verify on site) | Erasmuskloof Training Rooms 02 | - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting - Repaint walls |
| Letaba | 61.21 | 2840 | Double door 1610x2032 | Type-2 Approx. 3520×1800 glass frontage (verify on site) | Erasmuskloof Training Rooms 03 | - Remove existing burglar bars - Supply and new shopfront as per drawings Glazing: laminated double safety glass (SANS 10400 Part N). Install as per AAAMSA specs. - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting - Repaint walls |
| Komati | 61.21 | 2840 | Double door 1610x2032 | Type-2 Approx. 3520×1800 glass frontage (verify on site) | Erasmuskloof Training Rooms 04 | - Remove existing burglar bars - Supply and new shopfront as per drawings Glazing: laminated double safety glass (SANS 10400 Part N). Install as per AAAMSA specs. - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting - Repaint walls |
| S7, S8, S11, S12 | 61,07 ea | 2640 ea | 4x Single Door 813x2032 (S7, S8, S11, S12) | N/A | Erasmuskloof Training Rooms 04. | - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting Repaint walls |
| Illovo | 61.21 | 2840 | Double door 1610×2032 | Type-1 Approx. 3520×1800 glass frontage (verify on site) | Erasmuskloof Training Rooms 05 | - Remove existing burglar bars - Supply and new shopfront as per drawings Glazing: laminated double safety glass (SANS 10400 Part N). Install as per AAAMSA specs. - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting Repaint walls |
| S5 & S6 | 61.07 ea | 2839 | 2x Single door 813×2032 (S5, S6) | N/A | Erasmuskloof Training Rooms 05 | - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting Repaint walls |
| Gamtoos | 61.21 | 2840 | Double door 1626×2032 | Type-1 Approx. 3520×1800 glass frontage (verify on site) | Erasmuskloof Training Rooms 06 | - Remove existing burglar bars - Supply and new shopfront as per drawings Glazing: laminated double safety glass (SANS 10400 Part N). Install as per AAAMSA specs. - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting Repaint walls |
| S3 & S4 | 61.07 ea | 2839 | 2x Single door 813×2032 (S3, S4) | N/A | Erasmuskloof Training Rooms 06 | - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting Repaint walls |
| Caledon | 61.21 | 2840 | Double door 1626×2032 | Type-1 Approx. 3520×1800 glass frontage (verify on site) | Erasmuskloof Training Rooms 07 | - Remove existing burglar bars - Supply and new shopfront as per drawings Glazing: laminated double safety glass (SANS 10400 Part N). Install as per AAAMSA specs. - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting Repaint walls |
| S1 & S2 | 61.07 ea | 2839 | 2x Single door 813×2032 (S1, S2) | N/A | Erasmuskloof Training Rooms 07 | - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting Repaint walls |
| Incus Data | 61.21 | 2840 | Single door 1610×2032 | Type-1 Approx. 3520×1800 glass frontage (verify on site) | Erasmuskloof Training Rooms 08 | - Remove existing burglar bars - Supply and new shopfront as per drawings Glazing: laminated double safety glass (SANS 10400 Part N). Install as per AAAMSA specs. - Replace single door with new solid wood and paint the door. - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Replace damaged wooden floor skirting - Vanish new wooden floor skirting - Repaint walls |
| Tugela | 61.21 | 2840 | Double door 1610×2032 | N/A | Erasmuskloof Training Rooms 08 | - Replace double door with new solid wood and paint the door. - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Replace damaged wooden floor skirting - Vanish new wooden floor skirting - Repaint walls |
| Capony over Tugela & Incus Data Training Rooms | N/A | N/A | N/A | N/A | Erasmuskloof Training Rooms 09 | The canopy to be installed must be: - 0.6mm thick Galvanized IBR Roof sheeting on galvanized steel structure with supporting steel columns. |
| Umgeni | 121.75 | 2840 | Double door 1610×2032 | Type-2 Approx. 3520×1800 glass frontage (verify on site) | Erasmuskloof Training Rooms 10 | - Remove existing burglar bars - Supply and new shopfront as per drawings Glazing: laminated double safety glass (SANS 10400 Part N). Install as per AAAMSA specs. - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting Repaint walls |
| Gamka | 121.75 | 2840 | 2x Double doors 1610×2032 | N/A | Erasmuskloof Training Rooms 11 | - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting Repaint walls |
| S9 & S10 | 61.07 ea | 2839 | 2x Single door 813×2032 (S1, S2) | N/A | Erasmuskloof Training Rooms 11 | - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting Repaint walls |
| Hennops | 56.70 | 2840 | Double door 1610×2032 | Type-1 Approx. 3520×1800 glass frontage (verify on site) | Erasmuskloof Training Rooms 08 | - Remove existing burglar bars - Supply and new shopfront as per drawings Glazing: laminated double safety glass (SANS 10400 Part N). Install as per AAAMSA specs. - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting Repaint walls |
| S Room | 61.07 | 2839 | Single door 813×2032 | N/A | Erasmuskloof Training Rooms 11 | - Uninstall and remove all old/damaged suspended ceilings (including grids) and replace with new ceiling system. - Uninstall existing 5ft light fitting and reinstall after new ceiling system is installed. - Repaint door - Vanish wooden floor skirting Repaint walls |

Note: All measurements are subject to on-site verification. Upgrades to lighting, ceilings, carpets, and blinds in these rooms must align with SANS 10400 guidelines for flooring, acoustic comfort, fire safety, and occupant circulation. Materials chosen should meet or exceed national standards (e.g., SABS-approved for fire resistance)

### Centurion Training Rooms Dimensions and Specifications

The scope of works for all training rooms including surrounding offices is as follows:

- Uninstall existing carpet tiles and install new carpet tiles.

- Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system.

- Replacement of Wooden Skirting.

- Vanishing wooden skirting.

- Preparation (e.g. filling of cracks) and Painting of Walls.

- Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds).

- Preparation and painting of single doors and frame.

- Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED.

- Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb.

- Remove Old aircon unit and Install New complete 36000 BTU Cassette HVAC units with copper pipes.

- Remove Old aircon unit and Install New 60000 BTU Cassette HVAC unit with copper pipes.

Table 2: Centurion Training Room Dimensions and Specifications

| Training Room | Floor Area (m²) | Floor-to-Ceiling Height (mm) | Door Type & Dimensions (mm) | Type of Lighting to be Installed | PDF Reference | Air-Condition to be Replaced | Notes & Regulatory References |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Persal HH1.11 | 104.74 | 2825 | Single door 813×2032 | Knightsbridge Surface Fitting with CAT2 Louvre Surface-Mounted Low Height Fitting - DN570C LED20S/840 with applicable Dimmable LED Light Bulbs | Centurion Training Rooms_01 | 2x Cassette, 36 000 btu | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single door and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. - Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb. - Remove Old aircon unit and Install New complete 36000 BTU Cassette HVAC units with copper pipes. |
| Persal HH1.12 | 104.74 | 2825 | Single door 813×2032 | Knightsbridge Surface Fitting with CAT2 Louvre Surface-Mounted Low Height Fitting - DN570C LED20S/840 with applicable Dimmable LED Light Bulbs | Centurion Training Rooms_01 | 2x Cassette, 36 000 btu | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single doors and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. - Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb. - Remove Old aircon unit and Install New complete 36000 BTU Cassette HVAC units with copper pipes. |
| Persal Storeroom | 7.5 | 2285 | Single door 813×2032 | Knightsbridge Surface Fitting with CAT2 Louvre | Centurion Training Rooms_01 | N/A | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single door and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. |
| Funda Nathi | 97.87 | 2825 | Single door 813×2032 | Knightsbridge Surface Fitting with CAT2 Louvre Surface-Mounted Low Height Fitting - DN570C LED20S/840 with applicable Dimmable LED Light Bulbs | Centurion Training Rooms_02 | 1x Cassette, 60 000 btu | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single door and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. - Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb. - Remove Old aircon unit and Install New complete 60000 BTU Cassette HVAC units with copper pipes. |
| Tsebong H1.20 | 74.66 | 2825 | Single door 813×2032 | Knightsbridge Surface Fitting with CAT2 Louvre Surface-Mounted Low Height Fitting - DN570C LED20S/840 with applicable Dimmable LED Light | Centurion Training Rooms_03 | 2x Cassette, 36 000 btu | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single doors and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. - Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb. - Remove Old aircon unit and Install New complete 36000 BTU Cassette HVAC units with copper pipes. |
| Offices H1-14 & H1-16 | 20.48 ea | 20.48 ea | 2x Single door 813×2032 ea | Knightsbridge Surface Fitting with CAT2 Louvre | Centurion Training Rooms_03 | N/A | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single doors and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. - Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb. |
| Sangweni H1.9 | 92.19 | 2825 | Single door 813×2032 | Knightsbridge Surface Fitting with CAT2 Louvre Surface-Mounted Low Height Fitting - DN570C LED20S/840 with applicable Dimmable LED Light | Centurion Training Rooms_03 | 2x Cassette, 36 000 btu | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single doors and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. - Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb. - Remove Old aircon unit and Install New complete 36000 BTU Cassette HVAC units with copper pipes. |
| Sangweni Storeroom | 7.5 | 2285 | Single door 813×2032 | Knightsbridge Surface Fitting with CAT2 Louvre | Centurion Training Rooms_01 | N/A | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single doors and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. - Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb. |
| Offices H1-21 & H1-22 | 20.48 ea | 20.48 ea | 2x Single door 813×2032 ea | Knightsbridge Surface Fitting with CAT2 Louvre | Centurion Training Rooms_04 | N/A | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single doors and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. - Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb. |
| Twitter H1.12; Storeroom H1.15; Filing Room H1.17 | 92.19 | 2825 | 3x Single doors 813×2032 | Knightsbridge Surface Fitting with CAT2 Louvre | Centurion Training Rooms_05 | N/A | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single doors and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. - Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb. |
| Thutong HH1.3 | 97.87 | 2825 | Single door 813×2032 | Knightsbridge Surface Fitting with CAT2 Louvre Surface-Mounted Low Height Fitting - DN570C LED20S/840 with applicable Dimmable LED Light | Centurion Training Rooms_06 |  | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single doors and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. - Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb. |
| Ulwazi HH1.4 | 100.31 | 2825 | Single door 813×2032 | Knightsbridge Surface Fitting with CAT2 Louvre Surface-Mounted Low Height Fitting - DN570C LED20S/840 with applicable Dimmable LED Light | Centurion Training Rooms_07 | N/A | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single doors and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. - Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb. |
| Timaka HH1.16 | 100.31 | 2825 | Single door 813×2032 | Knightsbridge Surface Fitting with CAT2 Louvre Surface-Mounted Low Height Fitting - DN570C LED20S/840 with applicable Dimmable LED Light | Centurion Training Rooms_08 | N/A | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single doors and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. - Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb. |
| Zamani HH1.20 | 98.24 | 2825 | Single door 813×2032 | Knightsbridge Surface Fitting with CAT2 Louvre Surface-Mounted Low Height Fitting - DN570C LED20S/840 with applicable Dimmable LED Light | Centurion Training Rooms_09 | 1x Cassette, 60 000 btu | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single doors and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. - Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb. - Remove Old aircon unit and Install New complete 60000 BTU Cassette HVAC units with copper pipes. |
| Offices H1-18 & H1-17 Storeroom | 20.48 ea | 20.48 ea | 3x Single door 813×2032 ea | Knightsbridge Surface Fitting with CAT2 Louvre | Centurion Training Rooms_09 | N/A | - Uninstall existing carpet tiles and install new carpet tiles. - Uninstall and remove all old/damaged suspended ceilings (including grids) and install new ceiling system. - Vanishing wooden skirting. - Preparation and Painting of Walls. - Remove existing blinds and install new similar blinds suited for training rooms and office environments/rooms (e.g., vertical or roller blinds). - Preparation and painting of single doors and frame. - Supply and Install Light fitting: Knightsbridge Surface Fitting with CAT2 Louvre + 2x 5ft LED. - Supply and Install Light fitting: Surface-Mounted Low Height Fitting - DN570C LED20S/840 + Applicable Dimmable LED Light Bulb. |

### Centurion Training Ablution Facilities Dimensions and Specifications

See attached drawings for illustration

The scope of works for refurbishing all training ablution facilities, Blocks H1.1, H1.3, HH1.1 & HH1.2, is as follows:

- Taking out and removing old dilapidated suspended ceilings, including grids and prepare to receive new installations

- Removal of old tiles, disconnect and remove old pans, wall cisterns and toilet seats including urinals, flush masters, washbasins, counter tops and mirrors

- Patching and preparing the area to receive new installations

- Supply and install new white ceiling tiles (600x600mm) and grids drop of 600mm from the concrete roof

- Supply and install wall-mounted white floor toilet pans for wall cistern with its proper toilet seat covers, replace wall plumping pipes up to the connection joints at the back-service entrance (provision of up to 1m piping).

- Supply and install urinals with its proper wall cistern and automatic flushing system automatically after usage.

- Supplying and installing urinal automatic sanitizer dispenser

- Supply and install wall mounted Rolled White Hand Towel Dispenser

- Supply and install soap dispenser with automatic sensor

- Supply and install satin wall seat sanitizer dispenser

- Supply and install stainless steel jumbo roll round toiler dispenser

- Supply and install new (600mmx600mmx10mm) thick polished grey porcelain wall tile, laid with a 2mm wide grout joint and inside toilet cubicle walls

- Supply and lay new charcoal (600mmx600mmx10mm) thick porcelain floor matt tile, laid with a 2mm wide grout joint

- Supply and install new vanity slab made of Caesarstone countertop, also boxing the vanity to cover waste pipes– white shimmer and a mirror measurement must be verified on site

- Supply and install new duravit dura-square washbasins with waste outlets and water taps (cobra tap mixers).

- Provide and applying chalk brush enamel paint to toilet cubicle doors (standard door size)

- Supply and install bathroom explanatory/safety signage - Male and Female

- Supply and install 2300W, 230V electric hand dryer machine- stainless steel

#### The Bill of Quantities (BoQ) for ablution facilities, H1.1 & H1.3

Table 3: Centurion Training Ablutions (H1.1 & H1.3) Dimensions and Specifications

| Area | Type of Works | Goods Name | Goods/Service description | Unit of Measure | Total Qty |
| --- | --- | --- | --- | --- | --- |
| Male Ablution H1.1 | Refurbish | Ceiling Tiles for both Male and Female Toilets | Supplying and installing new white ceiling tiles (600x600mm) and grids drop of 600mm from the concrete roof | m² | 100,00 |
|  |  | LED Down Lighters & Fittings | Supply and install new LED down lighters with fittings and applicable LED bulb, cool-white | Each | 5,00 |
|  |  | Toilet Pans for Wall Cistern | Supply and install wall-mounted white floor toilet pans for wall cistern with its proper toilet seat covers, replace wall plumping pipes up to the connection joints at the back-service entrance (provision of up to 1m piping). | Each | 2,00 |
|  |  | Urinals | Supply and install urinals with its proper wall cistern and automatic flushing system automatically after usage. | Each | 1,00 |
|  |  | Urinal Automatic Sanitizer Dispenser | Supply and install urinal automatic sanitizer dispenser | Each | 1,00 |
|  |  | Wall Seat sanitizer Dispenser | Supply and install satin wall seat sanitizer dispenser | Each | 1,00 |
|  |  | Stainless Steel Jumbo Toilet Roll Dispenser | Supplying and installing stainless steel jumbo roll round toiler dispenser | Each | 1,00 |
|  |  | Wall Tile for both Male and Female Toilets | Supplying and laying new (600mmx600mmx10mm) thick polished grey porcelain wall tile, laid with a 2mm wide grout joint and inside 2x toilet cubicle walls, and paint 2x doors with chalk brush enamel paint colour. | m² | 100,00 |
|  |  | Floor Matt Tile for both Male and Female Toilets | Supplying and laying new charcoal (600mmx600mmx10mm) thick porcelain floor matt tile, laid with a 2mm wide grout joint | m² | 100,00 |
|  |  | Vanity Slab Countertop also Boxing the vanity covering the waste pipes | Supply and install new vanity slab made of Caesarstone countertop – white shimmer and a mirror - see exact sizes on specification | Each | 1,00 |
|  |  | Washbasins | Supplying and fitting new duravit dura-square washbasins with waste outlets and water taps (cobra tap mixers). | Each | 2,00 |
|  |  | Chalk Brush Enamel Paint to Toilet Cubicle Doors | Supply and apply chalk brush enamel paint to toilet cubicle doors (standard door size) | Each | 2,00 |
|  |  | Explanatory/Safety Signage | Supply and install bathroom explanatory/safety signage -26x male, 26x female and 12x for paraplegic toilets | Each | 1,00 |
|  |  | Electric Hand Dryer Machine | Supply and install 2300W, 220V electric hand dryer machine- stainless steel | Each | 1,00 |
|  | Replacement | Replace Main Entrance Door | Removal of existing door; Supply and install new solid door, 813x2032mm; and paint the new door with chalk brush enamel paint to toilet cubicle doors (standard door size | Each | 1,00 |
| Female Ablution H1.3 | Refurbish | LED Down Lighters & Fittings | Supplying and installing new LED down lighters and fittings | Each | 15,00 |
|  |  | Toilet Pans for Wall Cistern | Supply and Install wall-mounted white floor toilet pans for wall cistern with its proper toilet seat covers, replace wall plumping pipes up to the connection joints at the back-service entrance (provision of up to 1m piping). | Each | 4,00 |
|  |  | Wall Seat sanitizer Dispenser | Supplying and installing satin wall seat sanitizer dispenser | Each | 4,00 |
|  |  | Stainless Steel Jumbo Toilet Roll Dispenser | Supplying and installing stainless steel jumbo roll round toiler dispenser | Each | 4,00 |
|  |  | Vanity Slab Counter Top also Boxing the vanity covering the waste pipes | Supplying and installing new vanity slab made of Caesarstone countertop, also boxing the vanity to cover waste pipes– white shimmer and a mirror measurement must be verified on site. | Each | 1,00 |
|  |  | Washbasins | Supplying and fitting new duravit dura-square washbasins with waste outlets and water taps (cobra tap mixers). | Each | 4,00 |
|  |  | Chalk Brush Enamel Paint to Toilet Cubicle Doors | Provide and applying chalk brush enamel paint to toilet cubicle doors (standard door size) | Each | 4,00 |
|  |  | Explanatory/Safety Signage | Supply and install bathroom explanatory/safety signage -26x male, 26x female and 12x for paraplegic toilets | Each | 1,00 |
|  |  | Electric Hand Dryer Machine | Supply and install 2300W, 220V electric hand dryer machine- stainless steel | Each | 1,00 |

#### The Bill of Quantities (BoQ) for ablution facilities, HH1.1 & HH1.2

Table 4: Centurion Training Ablutions (HH1.1 & HH1.2) Dimensions and Specifications

| Area | Type of Works | Goods Name | Goods/Service description | Unit of Measure | Total Qty |
| --- | --- | --- | --- | --- | --- |
| Male Ablution HH1.1 | Refurbish | Ceiling Tiles for both Male and Female Toilets | Supply and install new white ceiling tiles (600x600mm) and grids drop of 600mm from the concrete roof | m² | 100,00 |
|  |  | LED Down Lighters & Fittings | Supply and install new LED down lighters with fittings and applicable LED bulb, cool-white | Each | 6,00 |
|  |  | Toilet Pans for Wall Cistern | Supply and install white floor toilet pans for wall cistern with its proper toilet seat covers, replace wall plumping pipes up to the connection joints at the back-service entrance (provision of up to 1m piping). | Each | 2,00 |
|  |  | Urinals | Supply and install urinals with its proper wall cistern and automatic flushing system automatically after usage. | Each | 1,00 |
|  |  | Urinal Automatic Sanitizer Dispenser | Supply and install urinal automatic sanitizer dispenser | Each | 1,00 |
|  |  | Wall Seat sanitizer Dispenser | Supply and install satin wall seat sanitizer dispenser | Each | 1,00 |
|  |  | Stainless Steel Jumbo Toilet Roll Dispenser | Supplying and installing stainless steel jumbo roll round toiler dispenser | Each | 1,00 |
|  |  | Wall Tile for both Male and Female Toilets | Supplying and laying new (600mmx600mmx10mm) thick polished grey porcelain wall tile, laid with a 2mm wide grout joint and inside 2x toilet cubicle walls, and paint 2x doors with chalk brush enamel paint colour. | m² | 100,00 |
|  |  | Floor Matt Tile for both Male and Female Toilets | Supplying and laying new charcoal (600mmx600mmx10mm) thick porcelain floor matt tile, laid with a 2mm wide grout joint | m² | 100,00 |
|  |  | Vanity Slab Countertop also Boxing the vanity covering the waste pipes | Supplying and installing new vanity slab made of Caesarstone countertop – white shimmer and a mirror - see exact sizes on specification | Each | 1,00 |
|  |  | Washbasins | Supplying and fitting new duravit dura-square washbasins with waste outlets and water taps (cobra tap mixers). | Each | 2,00 |
|  |  | Chalk Brush Enamel Paint to Toilet Cubicle Doors | Supply and apply chalk brush enamel paint to toilet cubicle doors (standard door size) | Each | 2,00 |
|  |  | Window Stays | Replace broken window stay with new one | Each | 2,00 |
|  |  | Explanatory/Safety Signage | Supply and install bathroom explanatory/safety signage -26x male, 26x female and 12x for paraplegic toilets | Each | 1,00 |
|  |  | Electric Hand Dryer Machine | Supply and install 2300W, 220V electric hand dryer machine- stainless steel | Each | 1,00 |
| Female Ablution HH1.3 | Refurbish | LED Down Lighters & Fittings | Supply and install new LED down lighters and fittings | Each | 15,00 |
|  |  | Toilet Pans for Wall Cistern | Supply and install white floor toilet pans for wall cistern with its proper toilet seat covers, replace wall plumping pipes up to the connection joints at the back-service entrance (provision of up to 1m piping). | Each | 4,00 |
|  |  | Wall Seat sanitizer Dispenser | Supply and install satin wall seat sanitizer dispenser | Each | 4,00 |
|  |  | Stainless Steel Jumbo Toilet Roll Dispenser | Supply and install stainless steel jumbo roll round toiler dispenser | Each | 4,00 |
|  |  | Vanity Slab Countertop also Boxing the vanity covering the waste pipes | Supply and install new vanity slab made of Caesarstone countertop – white shimmer and a mirror - see exact sizes on specification | Each | 1,00 |
|  |  | Washbasins | Supplying and fitting new duravit dura-square washbasins with waste outlets and water taps (cobra tap mixers). | Each | 4,00 |
|  |  | Chalk Brush Enamel Paint to Toilet Cubicle Doors | Provide and applying chalk brush enamel paint to toilet cubicle doors (standard door size) | Each | 4,00 |
|  |  | Window Stays | Replace broken window stay with new one | Each | 1,00 |
|  |  | Explanatory/Safety Signage | Supply and install bathroom explanatory/safety signage -26x male, 26x female and 12x for paraplegic toilets | Each | 1,00 |
|  |  | Electric Hand Dryer Machine | Supply and install 2300W, 220V electric hand dryer machine- stainless steel | Each | 1,00 |

### General Notes on Compliance

The works shall be carried out in compliance with:

- South African National Building Regulations and Building Standards Act.

- Occupational Health and Safety Act (OHS Act).

- All construction work must meet the deemed-to-satisfy rules of SANS 10400 (Parts A through X, where applicable), including:

  - Part A (General Principles)

  - Part C (Dimensions), ensuring minimum room heights exceed 2.4 m

  - Part M (Roof), if roof changes or penetrations are performed

  - Part N (Glazing), requiring safety glazing where necessary, in accordance with AAAMSA (Association of Architectural Aluminium Manufacturers of South Africa) Regulations.

  - Part O (Lighting and Ventilation), ensuring adequate ventilation and natural/artificial lighting levels

  - SANS 10142: Part 1: Low voltage installations.

  - Part S (Facilities for Persons with Disabilities), if applicable for access and egress

- Any additional local bylaws, codes, and regulations as may be required.

## Delivery address

The address where the required works must be delivered is:

| Building | Physical Address |
| --- | --- |
| SITA Erasmuskloof Head Office | 459 Tsitsa Street Erasmuskloof Pretoria |
| SITA Centurion | John Vorster Drive, Centurion, Pretoria |

## Customer Infrastructure and environment requirements

- Bidders must consider and adhere to the following infrastructure and environmental requirements:

- Site Conditions:

  - Review and adhere to provided specifications.

  - Ensure understanding of limitations, where applicable.

  - Follow specified access and logistics guidelines, including parking and traffic management.

  - All working areas must be kept clear from obstructive material and clean, and always.  The activities should not hamper or endanger any SITA employees, Clients and Visitors.

  - The service provider must always adhere to SA National Building

- Environmental Conditions:

  - Account for typical weather conditions and their impact on the construction schedule.

- Health and Safety Requirements:

  - Implement on-site safety protocols and adhere to health and safety regulations.

  - Provide and use the required personal protective equipment (PPE).

- Building Usage and Occupancy:

  - Minimize disruption to business operations and maintain access to required areas

  - Implement security measures for both the construction site and the building.

- Communication and Coordination:

  - A comprehensive project plan must be supplied with the technical bid spec for approval indicating a clear proposed timeframe, project milestone and dependencies.

  - Maintain clear communication with key contacts and follow the established meeting schedule.

  - Document site conditions and obtain necessary approvals and permits.

- Working Material and worker Experience:

  - All material used shall be of approved quality and profiles.

  - The service provider shall ensure that its team has relevant expertise and have necessary experience, equipment and tools to undertake the work.

# Service Elements

### Full-Service Agreement

#### Pre- Construction Phase

- Site Inspection:

  - Assess the installation site to record potential challenges.

  - Take measurements to ensure compatibility with provided dimensions.

- Customer Consultation:

  - Discuss customer requirements, preferences, and expectations.

  - Provide options regarding blind sizes, design, and additional features.

- Project Management and Coordination:

  - Develop and manage a detailed project plan

#### Construction Phase

- Project Management and Coordination:

  - Coordinate with all stakeholders and provide regular progress reports.

- Design Services:

  - Integrate with existing designs and ensure compliance with relevant codes and standards.

- Construction and Renovation Services:

  - Execute site preparation, construction, and renovation tasks while adhering to quality standards.

- Material and Equipment Management:

  - Procure and manage materials and equipment, with a focus on quality and sustainability.

- Health and Safety Management:

  - Implement safety protocols, comply with safety regulations, and manage risk effectively.

- Communication and Documentation:

  - Maintain clear communication and provide comprehensive documentation throughout the project.

#### Post-Construction Services:

- Cleanup:

  - Remove debris and surplus materials from the installation site.

  - Leave the area clean and safe for use.

- Customer Orientation:

- Demonstrate how to maintain and clean the blinds.

  - Provide information on warranties and after-installation support.

- Feedback and Follow-Up:

    - Obtain feedback from the SITA project team regarding the installation process.

    - Address any concerns or questions to ensure complete satisfaction.

# Bid Evaluation Stages

The bid evaluation process consists of four stages, according to the nature of the bid. A bidder must qualify for each stage to be eligible to proceed to the next stage of the evaluation. The stages are:

Table 5 Bid Evaluation Stages

| Stage | Description | Applicable for this bid YES |
| --- | --- | --- |
| Stage 1 | Administrative pre-qualification verification | YES |
| Stage 2 | Technical Mandatory requirement evaluation | YES |
| Stage 3 | Special Conditions of Contract verification | YES |
| Stage 4 | Price / B-BBEE evaluation | YES |

## Administrative requirements (Stage 1)

### Attendance of briefing session

A Compulsory briefing session and site visit will be held at both buildings. The bidder must sign the briefing session attendance register using the same information (bidder company name, bidder representative person name and contact details) as submitted in the bidder’s response document. Any bidder who fails to attend both the compulsory briefing session and, site visit held at both buildings will be disqualified.

### Registered Supplier

- Only responses from bidders who are registered as a Supplier on National Treasury’s Central Supplier Database (CSD) in terms of National Treasury’s Instruction Note 4A of 2016/17 will be considered for award on this RFx.

- In the case of joint ventures or consortiums the bidder must demonstrate that at least one of the parties to the bid response attended the briefing session.

## Technical returnable documents

### Instruction and evaluation criteria

- The bidder must comply with ALL the requirements as per the Technical Mandatory Requirements below by providing substantiating evidence in the form of documentation or information, failing which it will be regarded as “NOT COMPLY”.

- The bidder must provide a unique reference number (e.g. binder/folio, chapter, section, page) to locate substantiating evidence in the bid response.

- The bidder must comply with ALL the TECHNICAL MANDATORY REQUIREMENTS for the bid response to proceed to the next stage of the evaluation.

### Technical mandatory requirements (Stage 2)

Table 6: Technical Mandatory Requirements

| Mandatory Requirements | Substantiating evidence of compliance (used to evaluate bid) |  | Evidence reference (to be completed by bidder) |
| --- | --- | --- | --- |
| - BIDDER EXPERIENCE AND CAPABILITY REQUIREMENTS |  |  |  |
| The bidder must have completed construction work of similar scope and complexity for at least two (2) Corporate or Government clients within the past ten (10) years from the date of publication. | Provide, under Annex A, reference details from at least two (2) Corporate or Government clients for whom construction work of similar scope and complexity has been completed within the past ten (10) years from the date of publication, by completing and signing Annex A, section 5.1, table 10 - Company name; and - Reference Person Name, Tel and/or email; and - Project Scope of Work; and - Project Start and End-date. NOTE (1): SITA reserves the right to verify information provided. Note (2): Failure to complete Table 10 fully as indicated above will result in disqualification. |  | <provide unique reference to locate substantiating evidence in the bid response – see Annex A, par 5.1, table 10> |
| - CIDB REGISTRATION REQUIREMENT |  |  |  |
| The bidder, and/or its subcontractor (where applicable) must be registered with Construction Industry Development Board (CIDB) with a minimum rating of 4GB, or higher. |  | The bidder, and/or its subcontractor (where applicable) must complete and sign Annex B as proof of registration with the Construction Industry Development Board (CIDB), with a minimum grading of 4GB or higher. AND, The Bidder must attach proof of CIDB registration to Annex B. Note 1: SITA reserves the right to verify the information provided. Note 2: SITA reserves the right to validate the certificate of registration prior to the award of the bid. | <provide unique reference to locate substantiating evidence in the bid response – see Annex A, par 5.2 |

## Special Conditions of Contract Verification (Stage 3)

- The successful supplier will be bound by Government Procurement: General Conditions of Contract (GCC) as well as this Special Conditions of Contract (SCC), which will form part of the signed contract with the successful Supplier. However, SITA reserves the right to include or waive the condition in the signed contract.

- SITA reserves the right to:

  - Negotiate the conditions; or

  - Automatically disqualify a bidder for not accepting these conditions; or

- If the bidder qualifies the proposal with own conditions and does not specifically withdraw such own conditions when called upon to do so, SITA will invoke the rights reserved in accordance with subsection 4.3. (b) above.

### Special Conditions of Contract

### Contracting Conditions

- Formal Contract - The supplier must enter a formal written contract (agreement) with SITA.

- Right to Audit - SITA reserves the right, before entering a contract, to conduct or commission an external service provider to conduct a financial audit or probity to ascertain whether a qualifying bidder has the financial wherewithal or technical capability to provide the goods and services as required by this tender.

### Delivery Address

- The supplier must deliver the required products or services at as indicated in Section 2.2, Delivery Address:

| # Physical Address | GPS Coordinates (optional) |
| --- | --- |
| Erasmuskloof Head Office | 459 Tsitsa Street, Pretoria |
| Centurion Data Centre | John Vorster Drive, Centurion, Pretoria |

### Services and Performance Metrics

The Service Provider is responsible to provide the following services as specified in the Service Breakdown Structure (SBS) and the Work Breakdown Structure:

| SBS | Service Element | Service Grade | Service Level |
| --- | --- | --- | --- |
|  | - Take out old dilapidated suspended ceilings including grids to be replaced with new ceilings - Take out old carpets, light fittings and old blinds Remove Wi-Fi and CCTV accessories, Fire detectors and alarm, as required for setting out ceilings and layouts including new lighting system and fittings | Professional and quality workmanship | - Official SITA working hours are 08h00am – 16h30pm. All services to be done outside the official SITA working hours must be communicated and approved in advance |
|  | Patch and prepare the areas to receive new installations and fittings | Professional and quality workmanship | - Official SITA working hours are 08h00am – 16h30pm. All services to be done outside the official SITA working hours must be communicated and approved in advance |
|  | Install ceiling tiles, new LED lights and fittings, wall and door painting, new blinds and new carpet tiles. | Professional workmanship and quality material | - Official SITA working hours are 08h00am – 16h30pm. All services to be done outside the official SITA working hours must be communicated and approved in advance |
|  | Re-fit Wi-Fi and CCTV accessories, Fire detectors and alarm, as required for setting out ceilings and layouts. | Professional and quality workmanship | - Official SITA working hours are 08h00am – 16h30pm. All services to be done outside the official SITA working hours must be communicated and approved in advance |
|  | The successful bidder will have to demonstrate and present samples of paint colour and blinds as per the provided SITA specifications. The blinds and colour combination presented, will be agreed upon with SITA project management team, before the commencement of the project. |  |  |

| Work Breakdown Structure |  |  |  |
| --- | --- | --- | --- |
| W SBS | Area | Material and Activities | Delivery Timeframe |
| 1. | All Applicable Areas | - Take out old dilapidated suspended ceilings including grids to be replaced with new ceilings - Take out old carpets, light fittings and old blinds - Remove Wi-Fi and CCTV accessories, Fire detectors and alarm, as required for setting out ceilings and layouts including new lighting system and fittings | 1st phase |
| 2. | All Applicable Areas | Patch and prepare the areas to receive new paint, installations and fittings | 2nd phase |
| 3. | Training Areas | Replacing Ceiling tiles, new LED lights and fittings, wall and door painting, new blinds and new carpet tiles: All training rooms, offices and storerooms Carpet tiles and Skirting: - Stainproof SDX (Solution Dyed Nylon) tufted tile - 575g/m2 - 2.5mm, 4.5mm, 5.5mm (±0.5mm) - 8mm (±0.5mm) - 2 (SANS 10177-IV) - Permanent static control - Colour – dark blue with grey undertone - Skirting - Water resistant (W=8cm, D=2cm, H=240cm) T20mm x W90mm x L2400mm Walls: - Paint the walls with washable high quality acrylic PVA - Colour – light grey Ceiling: - Colour – New white ceiling tiles (600x600mm) and grid drops Blinds: - Custom Vertical Blind, Controls and Brackets - Colour: Light grey Lights and fittings: - Knightsbridge Surface Fitting with CAT2 Louvre - 5ft LED light lobe - Surface-Mounted Low Height Fitting - DN570C LED20S/840 – Dimmable LED Light Bulb Doors: - Paint the doors with washable high quality acrylic PVA - Colour – Dark grey | As per the project plan |

### Supplier Performance Reporting

- The successful bidder will provide progress report on a weekly basis to SITA project team/project manager, during the design, preparation work, installation and implementation phase until the project life cycle is completed.

- Scheduled monthly meetings to be scheduled as well as ADHOC meetings when required from both the successful bidder and SITA.

- The report type will be driven by the service level agreement; definitions of the content of each report type will be finalised at the time of concluding the contract service level agreement).

### Certification, Expertise and Qualification

- The bidder certifies that:

  - it has the necessary expertise, skill, qualifications and ability to undertake the work required in terms of the Statement of Work or Service Definition

  - it is committed to provide the Products or Services; and

  - perform all obligations detailed herein without any interruption to the Customer

  - it has been certified for the Products and Services required

### Logistical Conditions

- Hours of Work

  - Office hours are defined as business working hours of the customer and is Mondays to Fridays between 07:30 and 16:00

  - After hours of the customer during weekdays are from16:00 to 07:30

  - All mission critical sites will be managed on a 24 x 7 x 365 basis

- Tools of Trade

  - The bidder is expected to use its own resources (cell phone, laptops etc) to communicate with its own offices or outside of the SITA/Client buildings, including all tools and equipment to render the services effectively.

### Regulatory, Quality and Standards

- Products used to deliver the goods /services must comply with the South African Bureau of Standards (SABS)

- The National Building Regulations

- SANS standards (SANS 10400)

### Security screening and clearance requirements

  - Company security screening: The supplier may be required to undergo a company security screening conducted by the State Security Agency (SSA). Should the SSA find the supplier not suitable after the conduct of the security screening, the business relationship will be terminated. The following documentation will be required for the company security screening process to be conducted:

    - Copy of company registration documentation.

    - Copy(ies) of identity documentation of Director(s), Member(s) or Trustee(s);

    - Copy of valid tax clearance certificate.

  - Security suitability check for individuals: SITA may, at its own discretion and in line with its policies and procedures, require employees of the supplier to be subjected to a security suitability check before commencement of a project or delivering of a service. The security suitability check is conducted by SITA in order to ensure that individuals meet the minimum-security requirements and also to verify personal information. The supplier will be required to replace any employee(s) who is found to be not suitable after the conduct of the security screening. The following documentation will be required for the security suitability check:

    - Copy of identity document.

    - Copy(ies) of qualification(s) if SITA requires verification thereof.

    - Fingerprints – will be taken electronically.

    - Signed consent form for the conduct of background checks.

  - Security clearance: A security clearance, issued by either the SSA or Defence Intelligence (DI) is required if any employee of the supplier will have or may gain access to classified information throughout the duration of the project or in the process of delivering a service. The level of security clearance required – Confidential, Secret or Top Secret, will be determined at the sole discretion of SITA. The supplier will have to replace any employee who do not qualify for a security clearance or is found not suitable by the SSA or DI. The following documentation will be required for the security clearance process:

    - Completed Z204 or DD1057 security clearance application form.

    - Fingerprints.

    - Personal documentation of the applicant, including but not limited to, identity document, passport, marriage certificate (if applicable), divorce order (if applicable), qualifications, salary advice and bank statements.

### Confidentiality and non -disclosure conditions

- The Supplier, including its management and staff, must before commencement of the Contract, sign a non-disclosure agreement regarding Confidential Information

- Confidential Information means any information or data, irrespective of the form or medium in which it may be stored, which is not in the public domain, and which becomes available or accessible to a Party because of this Contract, including information or data which is prohibited from disclosure by virtue of:

  - the Promotion of Access to Information Act, 2000 (Act no. 2 of 2000).

  - being clearly marked "Confidential" and which is provided by one Party to another Party in terms of this Contract.

  - being information or data, which one Party provides to another Party or to which a Party has access because of Services provided in terms of this Contract and in which a Party would have a reasonable expectation of confidentiality.

  - being information provided by one Party to another Party during contractual or other negotiations, which could reasonably be expected to prejudice the right of the non-disclosing Party.

  - being information, the disclosure of which could reasonably be expected to endanger a life or physical security of a person.

  - being technical, scientific, commercial, financial and market-related information, know-how and trade secrets of a Party.

  - being financial, commercial, scientific or technical information, other than trade secrets, of a Party, the disclosure of which would be likely to cause harm to the commercial or financial interests of a non-disclosing Party; and

  - being information supplied by a Party in confidence, the disclosure of which could reasonably be expected either to put the Party at a disadvantage in contractual or other negotiations or to prejudice the Party in commercial competition; or

  - information the disclosure of which would be likely to prejudice or impair the safety and security of a building, structure or system, including, but not limited to, a computer or communication system; a means of transport; or any other property; or a person; methods, systems, plans or procedures for the protection of an individual in accordance with a witness protection scheme; the safety of the public or any part of the public; or the security of property; information the disclosure of which could reasonably be expected to cause prejudice to the defence of the Republic; security of the Republic; or international relations of the Republic; or plans, designs, drawings, functional and technical requirements and specifications of a Party, but must not include information which has been made automatically available, in terms of the Promotion of Access to Information Act, 2000; and information which a Party has a statutory or common law duty to disclose or in respect of which there is no reasonable expectation of privacy or confidentiality;

- Notwithstanding the provisions of this Contract, no Party is entitled to disclose Confidential Information, except where required to do so in terms of a law, without the prior written consent of any other Party having an interest in the disclosure.

- Where a Party discloses Confidential Information which materially damages or could materially damage another Party, the disclosing Party must submit all facts related to the disclosure in writing to the other Party, who must submit information related to such actual or potential material damage to be resolved as a dispute.

- Parties may not, except to the extent that a Party is legally required to make a public statement, make any public statement or issue a press release which could affect another Party, without first submitting a written copy of the proposed public statement or press release to the other Party and obtaining the other Party's prior written approval for such public statement or press release, which consent must not unreasonably be withheld.

### Guarantee and warranties

- The supplier confirms that:

  - The warranty of goods supplied under this contract remains valid for the duration of 5 years after the goods were delivered and installed with a sign off, including the clients signature

  - as at Commencement Date, it has the rights, title and interest in and to the Product or Services to deliver such Product or Services in terms of the Contract and that such rights are free from any encumbrances whatsoever;

  - the Product is in good working order, free from Defects in material and workmanship, and substantially conforms to the Specifications, for the duration of the Warranty period;

  - during the Warranty period any defective item or part component of the Product be repaired or replaced within 3 (three) days after receiving a written notice from SITA;

  - the Products is maintained during its Warranty Period at no expense to SITA;

### Intellectual Property Rights

- SITA retains all Intellectual Property Rights in and to SITA's Intellectual Property. As of the Effective Date, the Supplier is granted a non-exclusive license, for the continued duration of this Contract, to perform any lawful act including the right to use, copy, maintain, modify, enhance and create derivative works of SITA's Intellectual Property for the sole purpose of providing the Products or Services to SITA pursuant to this Contract; provided that the Supplier must not be permitted to use SITA's Intellectual Property for the benefit of any entities other than SITA without the written consent of SITA, which consent may be withheld in SITA's sole and absolute discretion. Except as otherwise requested or approved by SITA, which approval is in SITA's sole and absolute discretion, the Supplier must cease all use of SITA's Intellectual Property, at of the earliest of:

  - termination or expiration date of this Contract;

  - the date of completion of the Services; and

  - the date of rendering of the last of the Deliverables

- If so required by SITA, the Supplier must certify in writing to SITA that it has either returned all SITA Intellectual Property to SITA or destroyed or deleted all other SITA Intellectual Property in its possession or under its control

- SITA, always, owns all Intellectual Property Rights in and to all Bespoke Intellectual Property.

- Save for the license granted in terms of this Contract, the Supplier retains all Intellectual Property Rights in and to the Supplier’s pre-existing Intellectual Property that is used or supplied in connection with the Products or Services

- Provide SITA with the compliant Occupational Health and Safety File (required on site for period of installation and proof of compliance).

### Counter Conditions

- Bidders’ attention is drawn to the fact that amendments to any of the Bid Conditions or setting of counter conditions by bidders may result in the invalidation of such bids.

### Fronting

- The SITA supports the spirit of Broad Based Black Economic Empowerment and recognizes that real empowerment can only be achieved through individuals and businesses conducting themselves in accordance with the Constitution and in an honest, fair, equitable, transparent and legally compliant manner. Against this background the SITA will not condone any form of fronting.

- The SITA, in ensuring that bidders conduct themselves in an honest manner will, as part of the bid evaluation processes, conduct or initiate the necessary enquiries/investigations to determine the accuracy of the representation made in bid documents. Should any of the fronting indicators as contained in the Guidelines on Complex Structures and Transactions and Fronting, issued by the Department of Trade and Industry, be established during such enquiry/investigation, the onus will be on the bidder / contractor to prove that fronting does not exist. Failure to do so within a period of 14 days from date of notification may invalidate the bid / contract and may also result in the restriction of the bidder/contractor to conduct business with the public sector for a period not exceeding ten (10) years, in addition to any other remedies SITA may have against the bidder/contractor concerned.

### Business Continuity and Disaster Recovery Plans

The bidder confirms that they have written business continuity and disaster recovery plans that define the roles, responsibilities and procedures necessary to ensure that the required services under this bid specification is in place and will be maintained continuously in the event of a disruption to the bidder’s operations, regardless of the cause of the disruption.

### Supplier Due Diligence

SITA reserves the right to conduct supplier due diligence prior to final award or at any time during the Contract period and this may include pre-announced / non-announced site visits. During the due diligence process the information submitted by the bidder will be verified and any misrepresentation thereof may disqualify the bid or Contract in whole or parts thereof.

### Preference Goal Requirements conditions

- The Bidder’s commitment for the Preference Goal Requirements in this tender will be legally binding and the Bidder needs to perform against their commitment for the duration of the contract which will form part of the Contractual Agreement.

- The Bidder must sustain or improve the company’s BBBEE Level for the duration of the contact which will form part of the Contractual Agreement.

- Performance of Preference Goal Requirements will be determined annually. Bidders must submit their Preference status report indicating progress against the Bidder’s Preferential commitments within 30 days of the yearly anniversary of the contract.

- Bidders need to keep auditable substantive records / evidence and upon request by SITA/Department must be made available for audit and, or due diligence purposes.

- SITA reserves the right to require from a Bidder, either before a bid is adjudicated or at any time subsequently, to substantiate any claim with regards to preferences, in any manner required by SITA.

- SITA reserves the right to verify information / evidence provided by the Bidder.

- SITA/Department reserves the right to introduce a penalty of 1% of the overall annual year spent by SITA/Department for the prior year if the Bidder fails to comply to paragraphs (a), (b) and (c) above.

### Declaration of compliance and acceptance SCC

I (we), the bidder hereby declare that I (we) accept ALL the Special Conditions of Contract as specified in par 4.3.2 above and shall comply with all stated obligations:

Name of Bidder: _____________________________	Signature: _________________________

Date: ______________

## Price and Preference Points Evaluation (Stage 4)

### Costing and Preference Evaluation

- In terms of the SITA Preferential Procurement Policy (PPP), the following preference point system is applicable to all Bids:

  - the 80/20 system (80 Price, 20 B-BBEE) for requirements with a Rand value of up to R50 000 000 (all applicable taxes included); or

- The Applicable Preference Point system for this tender is the 80/20 preference point system.

- Points for this tender shall be awarded for:

  - Price; and

  - Preference points for specific goals.

- The maximum points for this tender will be allocated as follows, subject to par.2.

Table 7: Points allocation

| - Description | Points |
| --- | --- |
| - Price | 80 |
| - Preference points for specific goals | 20 |
| - Total points for Price and preference points for specific goals | 100 |

### Costing and Pricing Conditions

            - South African Pricing - The total price must be VAT inclusive and be quoted in South African Rand (ZAR).

- Total Price

  - All quoted prices are the total price for the entire scope of required services and deliverables to be provided by the bidder.

  - All additional costs as well as cost of delivery, labour, S&T, overtime, etc. must be included in this bid.

  - All services, accessories, upgrades and options required by the solution or specified by the client must be included in the quoted price. If not included, suppliers will be required to supply these accessories at no cost to the client.

  - SITA reserves the right to negotiate pricing with the successful bidder prior to the award as well as envisaged quantities

            - These conditions will form part of the Contract between SITA and the bidder. However, SITA reserves the right to include or waive the condition in the Contract.

            - The bidder must complete the declaration of acceptance as per par 4.4.4 below by marking with an “X” either “ACCEPT ALL”, or “DO NOT ACCEPT ALL”, failing which the declaration will be regarded as “DO NOT ACCEPT ALL” and the bid will be disqualified.

### Bid Pricing Schedule

Bidders must complete the bid pricing schedule in the Excel spreadsheet format provided and upload this as part of their submission.

### Declaration of Acceptance

|  | ACCEPT ALL | DO NOT ACCEPT ALL |
| --- | --- | --- |
| - The bidder declares to ACCEPT ALL the Costing and Pricing conditions as specified in par 4.4.2 above by indicating with an “X” in the “ACCEPT ALL” column, or - The bidder declares to NOT ACCEPT ALL the Costing and Pricing Conditions as specified in par 4.4.2 above by - - Indicating with an “X” in the “DO NOT ACCEPT ALL” column, and; - Provide reason and proposal for each of the condition not accepted. |  |  |
| Comments by bidder: Provide the condition reference, the reasons for not accepting the condition. |  |  |

## Preference Requirements

- The bidder must complete in full all the PREFERENCE requirements.

- Allocation of points per requirements: The points allocation of bidders’ responses to the requirements will be determined by the completeness, relevance and accuracy of substantiating evidence.

- Points will be allocated for each PREFERENCE requirement as per the criteria set in each section in the table 8 below.

- The bidder must provide a unique reference number (e.g. binder/folio, chapter, section, page) to locate substantiating evidence in the bid response. During evaluation, SITA reserves the right to treat substantiation evidence that cannot be in the bid response, as “NOT COMPLY”. The evidence needs to be attached to ANNEX A.

- Preference Goal Requirements

  - The applicable Preference Point system for this tender and points claimed is 80/20.

  - The specific Preferential Goal Requirements for this tender is indicated in table 8 below.

  - The Bidder must complete 80/20 preference point system and submit proof or documentation required in terms of this tender.

  - The Bidder must indicate their commitment to claim points for each of the preference points by signing at par 4.5 in the Invitation to Bid document.

  - Failure on the part of a bidder to submit proof or documentation required or to comply to paragraph (d) above in terms of this tender to claim preference points for the Preference Goal Requirements for this tender, will be interpreted to mean that preference points are not claimed.

  - The Bidder’s commitment for the Preference Goal Requirements in this tender will be legally binding and the Bidder needs to perform against their commitment for the duration of the contract which will form part of the Contractual Agreement.

  - The Bidder must sustain or improve the company’s BBBEE Level for the duration of the contact which will form part of the Contractual Agreement.

  - Performance of Preference Goal Requirements will be determined annually. Bidders must submit their Preference status report to SITA indicating progress against the Bidder’s Preferential commitments within 30 days after each quarter from the commencement date of the contract.

  - Bidders need to keep auditable substantive records / evidence and upon request by SITA must be made available for audit and, or due diligence purposes.

  - SITA reserves the right to require from a Bidder, either before a bid is adjudicated or at any time subsequently, to substantiate any claim with regards to preferences, in any manner required by SITA.

  - SITA reserves the right to verify information / evidence provided by the Bidder.

  - SITA reserves the right to introduce a penalty of 1% of the overall annual year spent by SITA for the prior year if the Bidder fails to comply to paragraphs (f), (g) and (h) above.

Table 8: Preference Goal Requirements

| Preferential Goal Requirements | Preferential Goal Requirements for (80/20) system |  |  |
| --- | --- | --- | --- |
| Preferential Goal Requirements allocated for this tender | Number of points allocated (80/20) system (To be completed by the organ of state) | Substantiating evidence and evidence reference to be completed by bidder. Evaluation per requirement: Each requirement indicated in the table below must be completed and points will be allocated based on the evidence required below for the (80/20) system | Evidence reference for the (80/20) system |
| B-BBEE Requirements |  |  |  |
| The allocation of points for bidders that meet a certain B-BBEE level as defined in the Broad-Based Black Economic Empowerment Act; | 20 | Evidence: The Bidder must provide a copy of relevant proof of B-BBEE status level of contributor level as defined in the Broad-Based Black Economic Empowerment Act. Points allocation: Points will be allocated in line with the BBBEE table 5 in section 4.6 | <provide unique reference to locate (80/20) system substantiating evidence in the bid response – Annex A, section 5> |
| Total Point Allocation: | 20 |  |  |

Table 9: B-BBEE Points as part of the Preference Goal requirements (Preferential Goal Requirements for (80/20) system)

Note: Bidder to select the section for points they wish to claim (Mark as Y=Yes) in the table below.

|  |  |  |  | Ownership |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Reference # | Contributor Level as defined in the Broad-Based Black Economic Empowerment Act | EME/QSEs | Black Owned (BO) (51% or more) | Black Woman Owned (BWO) (More than 30%) | Youth Owned | Owned by People living with disabilities | Score | Bidder to select the section for points they wish to claim (Mark as Y= Yes) |  |
|  |  |  | (A) | (B) | (C) | (D) | (E) | (F) |  |  |
|  | 1 | Level 1 | 6 | 4 | 4 | 4 | 2 | 20 |  |  |
|  | 2 | Level 1 | 6 | 4 | 2 | 2 | 0 | 14 |  |  |
|  | 3 | Level 1 | 6 | 4 | 2 | 0 | 0 | 12 |  |  |
|  | 4 | Level 1 | 6 | 4 | 0 | 0 | 0 | 10 |  |  |
|  | 5 | Level 2 and 3 | 4 | 2 | 1 | 1 | 1 | 9 |  |  |
|  | 6 | Level 2 and 3 | 4 | 2 | 1 | 1 | 0 | 8 |  |  |
|  | 7 | Level 2 and 3 | 4 | 2 | 1 | 0 | 0 | 7 |  |  |
|  | 8 | Level 2 and 3 | 4 | 2 | 0 | 0 | 0 | 6 |  |  |
|  | 9 | Level 4 and 5 | 2 | 1 | 0,5 | 0,5 | 0,5 | 4,5 |  |  |
|  | 10 | Level 4 and 5 | 2 | 0,5 | 0,5 | 0,5 | 0 | 3,5 |  |  |
|  | 11 | Level 4 and 5 | 2 | 0,5 | 0,5 | 0 | 0 | 3 |  |  |
|  | 12 | Level 4 and 5 | 2 | 0,5 | 0 | 0 | 0 | 2,5 |  |  |
|  | 13 | Level 6 | 0 |  | 0 | 0 | 0 | 0 |  |  |
|  | 14 | Level 7 | 0 |  | 0 | 0 | 0 | 0 |  |  |
|  | 15 | Level 8 | 0 |  | 0 | 0 | 0 | 0 |  |  |
|  | 16 | Non-Contributor | 0 |  | 0 | 0 | 0 | 0 |  |  |
|  | Total Maximum Score Allocation: |  | 20 |  |  |  |  |  |  |  |

F= A+B+C+D+E

Bidder substantiating evidence

# Technical Mandatory Requirement Evidence

## Bidder Experience and Capability Requirements

- Complete table below, noting that:

  - Provide reference details of customers to whom the project or service was delivered.

  - References may include multiple customers.

  - Project end-date must be current or not older than Ten (10) years from date this bid is advertised.

  - Scope of work must be related.

Table 10: References

| No | Company Name | Reference person name, contact details | Project Scope of Work | Project start and end date |
| --- | --- | --- | --- | --- |
| 1 | <Company name> | <Person Name> <Tel> <email> | < Provide scope details of a project from a customer to whom construction work was delivered for cooperate OR government buildings. > | Start Date: End Date: |
| 2 | <Company name> | <Person Name> <Tel> <email> | Provide scope details of a project from a customer to whom construction work was delivered for cooperate OR government buildings | Start Date: End Date: |

NOTE (1):

SITA reserves the right to verify information provided.

Note (2):

Failure to complete Table 10 fully as indicated above will result in disqualification.

## CIDB rating

## The Bidder must complete and sign ANNEX B as evidence that the Bidder, and/ or its Subcontractor is registered with the CIDB with a minimum rating of 4GB, or higher

# Preference Points Preferential Goals Evidence

- Bidder to complete the table(s) below in terms of points claimed for the specific bid and must provide the required evidence justifying the points claimed.

- The bidder must provide a copy of relevant proof of B-BBEE status level of contributor as defined in the Broad-based Black Economic Empowerment Act and attach it here.

- Annex B: CIDB REGISTRATION REQUIREMENT

The Bidder needs to complete and sign ANNEX B to confirm that the Bidder is registered with the Construction Industry Development Board (CIDB) with a minimum rating, of 4GB or higher.

      - The Bidder needs to indicate the Bidder’s CIDB rating by ticking next to the relevant CIDB rating in the table below:

| Service and Support (Milestones) | CIDB Rating (Minimum or Higher) | Bidder to Indicate the Bidder CIDB Rating here |
| --- | --- | --- |
| CIDB Rating | 4GB |  |
|  | Higher Rating |  |

I, the Supplier (Full names) …………………………………………………. representing (company name) …………………………………………………………….. hereby confirm that the Bidder is registered with Construction Industry Development Board (CIDB) and understand that it will form part of the contract and is legally binding.

Thus, done and signed at ……………………………………. On this………day of……………….20….

__________________

Signature

Designation: