Procurement approach: PA02

Evaluation Method: Price and Preference

Advertisement Duration: 5 working Days

Site Visit/Briefing Session: No

Pricing Option: Lump Sum

BSEC members: N/A

Advertisement: Nationally

Project Manager:

Programme Manager:

Cost Center (Linked to WBS):

Requestor:

Business Partner:

CC Manager:

Head:

Scope of Work:

Sentech will be doing a broadcast in mid-June. The exact date still needs to be firmed up. As such Sentech requires 9 MHz of ADHOC C-Band space segment capacity on a circular polarised geostationary satellite. Bidders are to supply information based on the mid- June timeline. The date will be confirmed at a later date.

- 9 MHz bandwidth.

- 38-hour duration. Exact times will be confirmed at a later date.

- Pricing must be quoted in ZAR.

- Assigned receive centre frequency should be above 3805 MHz

- Circular Polarised transmit and receive.

- Coverage area: Africa including the island nations, Western Europe, Western Russia, Western China, Western India, and the Middle East states.

- Due to topology and surrounding infrastructure, only satellite within the orbital location between 3° West  and 10° East will be considered.

- The earth station will be located at the following co-ordinates:

- Latitude: -25.7542°

- Longitude: 28.2219°

- The link budget, using the MODCODS listed in the table below must allow for the transmission to be successfully downlinked with a C-Band antenna with a mid-band gain of 37.5dBi at the lowest EIRP within the beam. The link availability must be 99.9% or better.

- Sentech’s Earth Station has a maximum uplink EIRP of 69 dBW, including losses.

- Bidders are expected to provide capacity that works within the specifications above to close the link.

- Bidders are required to provide detailed coverage maps and suggested link margins for the capacity proposed.

- Bidders are to complete the table 1.1 below, where the headings highlighted in yellow require information and submit it with the response. Sentech will be using DVB-S2 modulation standard.

Table 1.1

| SATELLITE: |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DVB-S2 MODCODS |  |  |  |  |  |  | Power Req | Minimum antenna size required for lowest EIRP |
| Bandwidth | Baudrate | MODCOD | FEC | Roll Off | Info Rate | RX C/N | Uplink EIRP | dBw Contour |
| 9 MHz | 7,2 | 8PSK | 2/3 | 25% |  |  |  |  |

End of Document.