Vegetable/ fruit cutter with accessories specification

- Variable speed

- Single-phase model

- Veg prep speed: 300 to 1000 rpm

- Cutter speed: 300 to 3500 rpm

- Bowl-based twin-blade assembly: small and large quantities

- Must include all 28 discs. Perfect glazed cut: 28 discs with high-precision blade sharpening

- Cutter: chopping, emulsifying, kneading, grinding and making sauces. Must include all 3 types of blades: Smooth blade, Coarse serrated blade. Designed for grinding & kneading and Fine serrated blade. Designed for herbs & spices

- Stainless steel cutter and vegetable slicer bowls.

- 4,5 l stainless steel bowl

- Metal motor base.

- Heavy-duty powerful induction motor

- Large hopper (Area 104 cm2): Fruits and vegetables such as cabbage, celery, lettuce, tomatoes, onions, apples, pears, kiwis, lemons, etc.

- Exactitube pusher: Ø 39 mm to cut up small-sized ingredients such as chillies, gherkins, dry sausage, asparagus, bananas, strawberries, etc.

- Cylindrical hopper Ø 58 mm: Long vegetables such as carrots, zucchini, cucumber, etc.

- Lateral ejection system: Space-saving. More convenient.

- Lid: Dry and liquid ingredients can be added while the machine is running.

- Ergonomic Soft Touch handle: Convenient and easy to grip.

- Easy to clean in dishwasher: All the parts can be easily removed and are dishwasher safe.