Scope of Work:

For the de-installation and re-installation of a 4.7 Meter Vertex antenna.

The Sentech King Williams Town transmitter site requires a new foundation for the 4.7-meter C-Band antenna that is currently on site. Sentech requires the services of a contactor to de-install and crate a Vertex 4.7-meter C-Band Satellite antenna along with its de-icing kit for reinstallation once the prepared plinth is ready. The GPS coordinates of the site is -32.67924450770152, 27.25989640731776. Bidders are to ensure that they have the correct vehicle to access the site.

The Project Objectives are:

- The de-installation of a Vertex 4.7-meter C-Band TVRO antenna including the de-icing components. A crane will be required to remove the reflector and pedestal.

- The contractor will inspect the antenna components, fastener hardware and de-icing kit to determine if replacements are necessary.

- The contractor will crate the antenna components, de-icing kit and fasteners for safe storage onsite/at the East London OC.

- Re-installation of the antenna, according to the manufactures guidelines once the plinth is ready

- The components of the antenna must be thoroughly cleaned before assembly

- Fasteners required for re-assembly that were identified during de-installation, are to be provided by the contractor for re-installation.

- The antenna will be installed and aligned, and the de-icing kit will be installed according to the installation manuals, on the prepared plinth. A crane will be required to re-install the antenna. Antenna structure earthing needs to be re-connected.

- The antenna will be optimized on the AMOS 17 satellite at 17° east. The qualification of the antenna. The antenna is to be qualified by providing a G/T measurement. The G/T methodology will be left to the discretion of the contractor.

- RF cables will be re-connected and the signal will be verified with the IRD lock.

- A signed As-built handover document must be provided to SENTECH as proof of completion of work.

- The contractor must include a sample of the OHS file in their RFQ submission.

- The contractor will be required to submit a complete OHS file to be approved by Sentech’s OHS department upon appointment. All personal working on the antennas will be required to have a valid mast climbing certificate as well as a valid medical fitness certificate. All work to be carried out in accordance with the OHSE act as applicable in South Africa.

- Site clean-up: All excess installation, packaging and waste material will be removed from site and disposed of by the installer.

- The contractor must include a R20 000,00 contingency fee in their price submission that will be used if there is a deviation from the scope of work.

Annexure1

OCCUPATIONAL HEALTH SAFETY AND ENVIRONMENTAL PROJECT FILE

PROJECT NAME:

The de-installation and re- installation of a 4.7 Meter Vertex antenna at King Williams Town Transmitter Site.

INDEX

- Occupational Health Safety Environmental Policy

- Legal appointments

  - 16 1 Appointee – CEO/MD/GM

  - 16 2 Appointee/ CR6.1  – Snr Rigger/Antenna Technician onsite

  - CR20(f)  - Crane Operator

  - GSR 14 – Scaffold Inspector or Cherry picker

  - CR 7.1 – Risk Assessor

  - GAR 9.2 – Incident Investigator

  - EMR 9.4 –Electrical portable tools inspector

  - SEC. 17.2 - H & S Rep

  - GSR 3.4 – First Aider

  - CR 27 – Fire Fighter & equipment’s Inspector

- Training Records supporting most Legal Appointments above

- Medical Fitness Certificate of Crane Operator

- Scope of Operation

- Section 37.2 agreement

- Public Liability Insurance

- Notification of construction work to Provincial Director (Annexure A) of Reg 3

- Letter of Good Standing

- Safety plan including the following:

  - Risk Assessment – indicate assessment criteria

  - Fall Protection Plan

  - Checklist

    - Portable Electrical Tools

    - Crane load testing certificate

    - Slings(DMR 18)

    - Safety Harness

    - Scaffold

    - Hand tools

    - Mobile Crane etc.

    - PPE

- Emergency plan including:

  - First Aid box with standard contents

  - IOD reporting procedure or routing diagram

  - IOD forms WCL1 or 2 or Annexure 1

  - Incident Investigation procedure and forms

  - Emergency numbers

- Induction & Toolbox Talks with registers and minutes

- Waste disposal plan e.g. wooden drum, cable wires etc.