- bottle top dispenser

- 5ml

Robust dosing instruments with excellent chemical resistance (chloroform, phenol, organic acids), high performance and simplified maintenance. Intended for the safe and reproducible liquid distribution in volumes ranging from 0.1 mL to 5 mL.  Autoclavable at 121o C fully assembled, quick disassembling.  It is very crucial for the dispenser to have all these features described below;

  - Feed tube FEP

  - Intake valve Ceramic

  - Valve body Ceramic - Aluminum oxide

  - Valve balls Ceramic - Aluminum oxide

  - Valve spring Platinum-iridium

  - Barrel Borosilicate glass

  - Barrel plate / base PTFE

  - Plunger Ground Borosilicate glass

  - Outlet valve Ceramic

  - Body   Ceramic - Aluminum oxide

  - Delivery jet assembly FEP

  - CapETFE