Part 3: Scope of Work

| Document reference | Title | No of pages |
| --- | --- | --- |
|  | This cover page | 1 |
| C3.1 | Purchaser’s Goods Information |  |
| C3.2 | Supplier’s Goods Information (insert at award stage or delete if not applicable) |  |
|  | Total number of pages |  |

C3.1: Purchaser’s Goods Information

Contents

Part 3: Scope of Work	1

C3.1: Purchaser’s Goods Information	2

1	Overview and purpose of the goods and services	4

2	Specification and description of the goods	4

2.1	Purchaser’s design	4

2.2	Procedure for submission and acceptance of Supplier’s design	4

2.3	Other requirements of the Supplier’s design	4

2.4	Use of Supplier’s design	5

2.5	Manufacture & fabrication	5

2.6	Factory acceptance testing (FAT)	5

2.7	Other tests and inspections and commissioning in place of use	5

2.8	Operating manuals and maintenance schedules	5

3	Supply Requirements	5

4	Specification of the services to be provided	6

5	Constraints on how the Supplier Provides the Goods	6

5.1	Programming constraints	6

5.2	Work to be done by the Delivery Date	6

5.3	Marking the goods	6

5.4	Constraints at the delivery place and place of use	6

5.5	Cooperating with Others	6

5.6	Services & other things to be provided by the Purchaser or Supplier	7

5.7	Management meetings	7

5.8	Documentation control	7

5.9	Health and safety risk management	8

5.10	Environmental constraints and management	8

5.11	Quality	8

5.12	Invoicing and payment	8

5.13	Insurance provided by the Purchaser	8

5.14	Contract change management	9

5.15	Provision of bonds and guarantees	9

5.16	Records of Defined Cost, payments & assessments of compensation events to be kept by the Supplier	9

6	Procurement	9

6.1	Subcontracting	9

6.1.1	Preferred subcontractors	9

6.1.2	Limitations on subcontracting	9

6.1.3	Spares and consumables	9

6.1.4	Other requirements related to procurement	10

6.1.5	Cataloguing requirements	10

7	List of drawings	10

7.1	Drawings issued by the Purchaser	10

C3.2	Supplier’s Goods Information	11

# Overview and purpose of the goods and services

Kriel Power Station is a coal-fired power station that is located between the towns of Kriel and Ogies at the Nkalanga District Municipality in Mpumalanga Province. The station has six (6) units installed with an installed capacity of 3 000 MW. Kriel Power Stations has various installed electrical and mechanical systems i.e., components, equipment, machines that require the use of bearings to support the rotating shafts to aid in the transmission of power and the rolling motion across the various plants on the station terrace.

Bearings are high wear items that either reach their design life or alternatively fail prematurely resulting in plant breakdowns. Considering the age of the plant, these failures, and the reduced life of the bearings often require replacement. When that happens it negatively, affects the stations spares management, and in some instances the bearing set spares have to be pirated from other local sister stations. Thus, the need for a five (5) year contract for the supply and delivery of the Bearing Set spares.

# Specification and description of the goods

| Item No | Material No | Full Description | Unit | QTY |
| --- | --- | --- | --- | --- |
| 1 | 70 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 95 MM; OUTSIDE DIAMETER: 200 MM; WIDTH: 45 MM; ROW: SINGLE; SUPPL P/N: 6319; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 2 | 72 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 65 MM; OUTSIDE DIAMETER: 140 MM; WIDTH: 33 MM; ROW: SINGLE; SUPPL P/N: NU313STD; NOTE - ITEM TO BE ORIGINAL PACKED. | EA | 75 |
| 3 | 74 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: SHIELD; INSIDE DIAMETER: 15 MM; OUTSIDE DIAMETER: 35 MM; WIDTH: 11 MM; ROW: SINGLE; SUPPL P/N: 6202-Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 4 | 83 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 15 MM; OUTSIDE DIAMETER: 35 MM; WIDTH: 11 MM; ROW: SINGLE; SUPPL P/N: 6202-2Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 5 | 84 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 70 MM; OUTSIDE DIAMETER: 150 MM; WIDTH: 35 MM; ROW: SINGLE; SUPPL P/N: 6314; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 6 | 89 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 7 MM; OUTSIDE DIAMETER: 22 MM; WIDTH: 7 MM; ROW: SINGLE; SUPPL P/N: 627-2Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE | EA | 75 |
| 7 | 20539 | BEARING, THRUST: ROLLING ELEMENT: ROLLER; MODEL NO: HTGD90222 | EA | 75 |
| 8 | 22767 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 240 MM; OUTSIDE DIAMETER: 265 MM; MATERIAL: STL; FOR CW PUMP; DUTY: EXTRA HEAVY; STYLE: OPEN TYPE; WIDTH: 255MM | EA | 75 |
| 9 | 22782 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 20 MM; OUTSIDE DIAMETER: 25.4 MM; LENGTH: 40 MM; MATERIAL: STL; DRAWING NO: HTGR 305950 POS 565 REV 1; HTGR 305950 REV 1; VAR 2, FOR SPEED GOVERNOR VAR 2; TO BBT | EA | 75 |
| 10 | 22783 | BEARING, SLEEVE: TYPE: BUSH; SUPPL P/N: 21563 | EA | 25 |
| 11 | 22786 | BEARING, SLEEVE: TYPE: BUSH; DRAWING NO: 12SK B1 010-001 REV 1; MODEL NO: HTGD90102; NS, FOR TURNING GEAR | EA | 25 |
| 12 | 26413 | BEARING, ASSEMBLY: TYPE: INSULATED; SUPPL P/N: E1-994X; FOR PRECIPITATORS MECHANICAL DISCHARGE SYSTEM; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 13 | 29804 | BEARING, ROLLER: TYPE: CUP/CONE, TAPER; INSIDE DIAMETER: 52 MM; OUTSIDE DIAMETER: 98.43 MM; WIDTH: 30.5 MM; ROW: SINGLE; MODEL NO: 3732-3767; NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 14 | 29810 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 72 MM; WIDTH: 17 MM; ROW: SINGLE; SUPPL P/N: 6207RS; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 15 | 29812 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 15 MM; OUTSIDE DIAMETER: 35 MM; WIDTH: 11 MM; ROW: SINGLE; SUPPL P/N: 6202-RS; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 16 | 29821 | BEARING, BALL: TYPE: SELF ALIGNING; STYLE: OPEN; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 90 MM; WIDTH: 20 MM; ROW: DOUBLE; SUPPL P/N: 1210KC3; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 17 | 29826 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 15 MM; OUTSIDE DIAMETER: 42 MM; WIDTH: 13 MM; ROW: SINGLE; SUPPL P/N: 6302; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 18 | 29837 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 62 MM; WIDTH: 16 MM; ROW: SINGLE; SUPPL P/N: 6206-2Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE | EA | 75 |
| 19 | 29838 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 80 MM; WIDTH: 21 MM; ROW: SINGLE; SUPPL P/N: 6307-2Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 20 | 29839 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 80 MM; WIDTH: 21 MM; ROW: SINGLE; SUPPL P/N: 6307; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 21 | 29840 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 40 MM; OUTSIDE DIAMETER: 90 MM; WIDTH: 23 MM; ROW: SINGLE; SUPPL P/N: 6308; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 22 | 29842 | BEARING, BALL: TYPE: CONRAD; STYLE: SEAL; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 62 MM; WIDTH: 16 MM; ROW: SINGLE; SUPPL P/N: 6206-Z-NR; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 23 | 29847 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 90 MM; OUTSIDE DIAMETER: 190 MM; WIDTH: 43 MM; ROW: SINGLE; SUPPL P/N: 6318; ITEM TO BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 24 | 29851 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: SHIELD; INSIDE DIAMETER: 12 MM; OUTSIDE DIAMETER: 32 MM; WIDTH: 10 MM; ROW: SINGLE; SUPPL P/N: 6201-Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 25 | 29853 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: SHIELD; INSIDE DIAMETER: 25 MM; OUTSIDE DIAMETER: 62 MM; WIDTH: 17 MM; ROW: SINGLE; SUPPL P/N: 6305-Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 26 | 29854 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: SHIELD; INSIDE DIAMETER: 40 MM; OUTSIDE DIAMETER: 80 MM; WIDTH: 18 MM; ROW: SINGLE; SUPPL P/N: 6208Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 27 | 29855 | BEARING, BALL: TYPE: CONRAD; STYLE: SHIELD; INSIDE DIAMETER: 45 MM; OUTSIDE DIAMETER: 85 MM; WIDTH: 19 MM; ROW: SINGLE; SUPPL P/N: 6209Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 28 | 29856 | BEARING, BALL: TYPE: CONRAD; STYLE: SHIELD; INSIDE DIAMETER: 60 MM; OUTSIDE DIAMETER: 130 MM; WIDTH: 31 MM; ROW: SINGLE; SUPPL P/N: 6312-Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 29 | 29857 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 70 MM; OUTSIDE DIAMETER: 150 MM; WIDTH: 35 MM; ROW: SINGLE; SUPPL P/N: 6314-2ZC3; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 30 | 29861 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 60 MM; OUTSIDE DIAMETER: 110 MM; WIDTH: 22 MM; ROW: SINGLE; SUPPL P/N: NU212; NU 212; NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 31 | 29873 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 75 MM; OUTSIDE DIAMETER: 160 MM; WIDTH: 37 MM; ROW: SINGLE; SUPPL P/N: N315STD; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 32 | 29875 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 80 MM; OUTSIDE DIAMETER: 170 MM; WIDTH: 39 MM; ROW: SINGLE; SUPPL P/N: N316 ECJ; NOTE - ITEM MUST BE ORIGINAL PACKED, SKF BRAND ONLY, NO OTHER BRANDS ACCEPTABLE | EA | 75 |
| 33 | 29883 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 1 IN; OUTSIDE DIAMETER: 2 IN; WIDTH: 3/8 IN; ROW: SINGLE; SUPPL P/N: EE9-2Z; NOTE: ITEM MUST BE ORIGINAL PACKED AND SEALED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 34 | 29900 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 110 MM; WIDTH: 27 MM; ROW: DOUBLE; SUPPL P/N: 21310 CC; 21310CCSTD; NOTE! ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 35 | 29909 | BEARING, BALL: TYPE: SELF ALIGNING; STYLE: OPEN; INSIDE DIAMETER: 25 MM; OUTSIDE DIAMETER: 52 MM; WIDTH: 15 MM; ROW: DOUBLE; REFERENCE NO: 1205; NOTE! ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 36 | 29923 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: OPEN; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 110 MM; WIDTH: 27 MM; ROW: SINGLE; CAGE MATERIAL: STL; SUPPL P/N: 7310B; REFERENCE NO: 017-733-617-410; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 37 | 29930 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 40 MM; OUTSIDE DIAMETER: 68 MM; WIDTH: 15 MM; ROW: SINGLE; SUPPL P/N: 6008-2Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 38 | 29931 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 15 MM; OUTSIDE DIAMETER: 32 MM; WIDTH: 9 MM; ROW: SINGLE; SUPPL P/N: 6002-2Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 39 | 29938 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 10 MM; OUTSIDE DIAMETER: 30 MM; WIDTH: 9 MM; ROW: SINGLE; SUPPL P/N: 6200-2RS; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 40 | 29940 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: OPEN; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 72 MM; WIDTH: 19 MM; ROW: SINGLE; SUPPL P/N: 6306; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE | EA | 75 |
| 41 | 29943 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 17 MM; OUTSIDE DIAMETER: 40 MM; WIDTH: 12 MM; ROW: SINGLE; SUPPL P/N: 6203; NOTE: THE ITEM MUST BE PROTECTIVE PACKED AND CLEARLY MARKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 42 | 29945 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 72 MM; WIDTH: 17 MM; ROW: SINGLE; SUPPL P/N: 6207; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 43 | 29947 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 75 MM; OUTSIDE DIAMETER: 130 MM; WIDTH: 25 MM; ROW: SINGLE; SUPPL P/N: 6215; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 44 | 29949 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 110 MM; WIDTH: 27 MM; ROW: SINGLE; SUPPL P/N: 6310; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 45 | 29951 | BEARING, BALL: TYPE: CONRAD; STYLE: SHIELD; INSIDE DIAMETER: 20 MM; OUTSIDE DIAMETER: 52 MM; WIDTH: 15 MM; ROW: SINGLE; SUPPL P/N: 6304-Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 46 | 29956 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: OPEN; INSIDE DIAMETER: 20 MM; OUTSIDE DIAMETER: 52 MM; WIDTH: 15 MM; ROW: SINGLE; CAGE MATERIAL: STL;SUPPL P/N: 6304; ITEM MUST BE ORIGINAL PACKED; IBI 14788; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 25 |
| 47 | 29957 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 25 MM; OUTSIDE DIAMETER: 52 MM; WIDTH: 15 MM; ROW: SINGLE; SUPPL P/N: 6205; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 48 | 29958 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 65 MM; OUTSIDE DIAMETER: 140 MM; WIDTH: 33 MM; ROW: SINGLE; SUPPL P/N: 313A; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 49 | 29959 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 15.88 MM; OUTSIDE DIAMETER: 39.69 MM; WIDTH: 11.11 MM; ROW: SINGLE; SUPPL P/N: RLS5; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 50 | 29960 | BEARING, BALL: TYPE: CONRAD; STYLE: SHIELD; INSIDE DIAMETER: 40 MM; OUTSIDE DIAMETER: 90 MM; WIDTH: 23 MM; ROW: SINGLE; SUPPL P/N: 6308Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 51 | 29972 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 15 MM; OUTSIDE DIAMETER: 35 MM; WIDTH: 11 MM; ROW: SINGLE; SUPPL P/N: 6202; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 52 | 30008 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 55 MM; OUTSIDE DIAMETER: 120 MM; WIDTH: 29 MM; ROW: SINGLE; SUPPL P/N: 6311-2RSC3; | EA | 75 |
| 53 | 30009 | BEARING, ROLLER: TYPE: TAPER; INSIDE DIAMETER: 54 MM; OUTSIDE DIAMETER: 120 MM; WIDTH: 44.5 MM; ROW: SINGLE; MODEL NO: 32311; NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 54 | 30013 | BEARING, BALL: TYPE: CONRAD; STYLE: SHIELD; INSIDE DIAMETER: 65 MM; OUTSIDE DIAMETER: 140 MM; WIDTH: 33 MM; ROW: SINGLE; SUPPL P/N: 6313ZC3E; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 55 | 30016 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 160 MM; OUTSIDE DIAMETER: 290 MM; WIDTH: 104 MM; ROW: SINGLE; MODEL NO: 23232 CCK-W33; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 56 | 30025 | BEARING, BALL: STYLE: OPEN; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 52 MM; WIDTH: 16 MM; ROW: SINGLE; SUPPL P/N: 51206; SINGLE DIRECTION; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 57 | 30026 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 62 MM; WIDTH: 9 MM; ROW: SINGLE; SUPPL P/N: 16007; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 58 | 30030 | BEARING, THRUST: ROLLING ELEMENT: BALL; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 70 MM; WIDTH: 14 MM; STYLE: DOUBLE SHIELD; MODEL NO: 51110; SINGLE DIRECTION, NOTE - ITEM MUST BE ORIGINAL PACKED; ROW SINGLE | EA | 75 |
| 59 | 30054 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 100 MM; OUTSIDE DIAMETER: 180 MM; WIDTH: 46 MM; ROW: DOUBLE; SUPPL P/N: 22220CCKC3; NOTE: THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING AND ITEM MUST BE ORIGINAL PACKED; SKF FAG RHP BRAND ONLY | EA | 75 |
| 60 | 30057 | BEARING, ROLLER: TYPE: SELF ALIGNING; SPHERICAL; INSIDE DIAMETER: 100 MM; OUTSIDE DIAMETER: 180 MM; WIDTH: 60.33 MM; ROW: DOUBLE; CAGE MATERIAL: BRS/STL; SUPPL P/N: 23220CC3; THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON AND ORIGINAL PACKED, NTERNAL FIT | EA | 75 |
| 61 | 30058 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 120 MM; OUTSIDE DIAMETER: 215 MM; WIDTH: 76 MM; ROW: DOUBLE; SUPPL P/N: 23224CC3; CYLINDRICAL BORE, NOTE: THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING AND ORIGINAL PACKED | EA | 75 |
| 62 | 30060 | BEARING, ROLLER: TYPE: SELF ALIGNING; SPHERICAL; INSIDE DIAMETER: 85 MM; OUTSIDE DIAMETER: 150 MM; WIDTH: 36 MM; ROW: DOUBLE; BORE SHAPE: TAPER; REFERENCE NO: 22217CCK; 22217CCKC3; NOTE - THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON AND ORIGINAL PACKED, COMPLETE WITH H317 ADAPTOR SLEEVE | EA | 75 |
| 63 | 30063 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 70 MM; OUTSIDE DIAMETER: 150 MM; WIDTH: 35 MM; ROW: SINGLE; SUPPL P/N: N314STD; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 64 | 30065 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 15.88 MM; OUTSIDE DIAMETER: 39.69 MM; WIDTH: 11.11 MM; ROW: SINGLE; SUPPL P/N: CRL5; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 65 | 30071 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 31.75 MM; OUTSIDE DIAMETER: 79.38 MM; WIDTH: 22.23 MM; ROW: SINGLE; SUPPL P/N: MRJ1.1/4J | EA | 75 |
| 66 | 30076 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 90 MM; OUTSIDE DIAMETER: 190 MM; WIDTH: 43 MM; ROW: SINGLE; SUPPL P/N: N318; NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 67 | 30090 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 160 MM; OUTSIDE DIAMETER: 290 MM; WIDTH: 80 MM; ROW: DOUBLE; SUPPL P/N: 22232CCKC3; REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON AND ORIGINAL PACKED | EA | 75 |
| 68 | 30091 | BEARING, ROLLER: TYPE: SELF ALIGNING; SPHERICAL; INSIDE DIAMETER: 55 MM; OUTSIDE DIAMETER: 120 MM; WIDTH: 43 MM; ROW: DOUBLE; BORE SHAPE: TAPER; SUPPL P/N: 22311CCK; NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 69 | 30092 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 170 MM; OUTSIDE DIAMETER: 288 MM; WIDTH: 88 MM; ROW: DOUBLE; MODEL NO: 23134 CCK; NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 70 | 30093 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 170 MM; OUTSIDE DIAMETER: 280 MM; WIDTH: 88 MM; ROW: DOUBLE; SUPPL P/N: 23134CCK-C3; NOTE - THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON AND ORIGINAL PACKED | EA | 75 |
| 71 | 30095 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 80 MM; OUTSIDE DIAMETER: 170 MM; WIDTH: 39 MM; ROW: SINGLE; SUPPL P/N: NU316C3 | EA | 75 |
| 72 | 30096 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 110 MM; OUTSIDE DIAMETER: 240 MM; WIDTH: 50 MM; ROW: SINGLE; SUPPL P/N: NU322CC3; NOTE - THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON AND ORIGINAL PACKED | EA | 75 |
| 73 | 30101 | BEARING, ROLLER: TYPE: NEEDLE; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 37 MM; WIDTH: 26 MM; ROW: SINGLE; SUPPL P/N: HK3026; NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 74 | 30105 | BEARING, ROLLER: TYPE: SELF ALIGNING; SPHERICAL; INSIDE DIAMETER: 190 MM; OUTSIDE DIAMETER: 320 MM; WIDTH: 104 MM; ROW: DOUBLE; MODEL NO: 23138CC- W33; REFERENCE NO: 23138CC- W33C3; NOTE - THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON AND ORIGINAL PACKED | EA | 75 |
| 75 | 30111 | BEARING, THRUST: ROLLING ELEMENT: BALL; INSIDE DIAMETER: 110 MM; OUTSIDE DIAMETER: 160 MM; WIDTH: 38 MM; STYLE: OPEN; SUPPL P/N: 51222; NOTE - ITEM MUST BE ORIGINAL PACKED; ROW SINGLE | EA | 75 |
| 76 | 30114 | BEARING, ROLLER: TYPE: SELF ALIGNING; SPHERICAL; INSIDE DIAMETER: 100 MM; OUTSIDE DIAMETER: 180 MM; WIDTH: 60.3 MM; ROW: DOUBLE; SUPPL P/N: 23220CCK/C3W33C3; NOTE - THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON AND ORIGINAL PACKED | EA | 75 |
| 77 | 30165 | BEARING, BALL: TYPE: SPHERICAL; STYLE: OPEN; INSIDE DIAMETER: 85 MM; OUTSIDE DIAMETER: 150 MM; WIDTH: 36 MM; ROW: DOUBLE; SUPPL P/N: 2217K; NOTE! ITEM MUST BE ORIGINAL PACKED,TAPERED BORE.; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 78 | 30166 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: SHIELD; INSIDE DIAMETER: 17 MM; OUTSIDE DIAMETER: 40 MM; WIDTH: 12 MM; ROW: SINGLE; SUPPL P/N: 6203Z; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 79 | 30181 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: SHIELD; INSIDE DIAMETER: 17 MM; OUTSIDE DIAMETER: 35 MM; WIDTH: 10 MM; ROW: SINGLE; SUPPL P/N: 6003-Z; ITEM MUST BE ORIGINAL PACKED; DUTY: EXTRA LIGHT; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 80 | 30182 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: OPEN; INSIDE DIAMETER: 60 MM; OUTSIDE DIAMETER: 130 MM; WIDTH: 31 MM; ROW: SINGLE; SUPPL P/N: 7312B; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 81 | 30185 | BEARING, BALL: TYPE: CONRAD; STYLE: SHIELD; INSIDE DIAMETER: 17 MM; OUTSIDE DIAMETER: 47 MM; WIDTH: 14 MM; ROW: SINGLE; SUPPL P/N: 6303Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 82 | 30186 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 12 MM; OUTSIDE DIAMETER: 32 MM; WIDTH: 10 MM; ROW: SINGLE; SUPPL P/N: 6201-2Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 83 | 30187 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 17 MM; OUTSIDE DIAMETER: 40 MM; WIDTH: 12 MM; ROW: SINGLE; SUPPL P/N: 6203-2Z; SOLE SOURCE: CLYDE BERGEMANN AFRICA PTY LTD; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 84 | 30201 | BEARING, ROLLER: TYPE: SELF ALIGNING; SPHERICAL; INSIDE DIAMETER: 80 MM; OUTSIDE DIAMETER: 140 MM; WIDTH: 33 MM; ROW: DOUBLE; SUPPL P/N: 22216CCKC3; NOTE - THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON AND ORIGINAL PACKED | EA | 75 |
| 85 | 30202 | BEARING, ROLLER: TYPE: SELF ALIGNING; SPHERICAL; INSIDE DIAMETER: 110 MM; OUTSIDE DIAMETER: 200 MM; WIDTH: 53 MM; ROW: DOUBLE; SUPPL P/N: 22222CCKC3; THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON AND ORIGINAL PACKED | EA | 75 |
| 86 | 30203 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 140 MM; OUTSIDE DIAMETER: 250 MM; WIDTH: 68 MM; ROW: DOUBLE; BORE SHAPE: TAPER; SUPPL P/N: 22228CCKC3; NOTE: THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING, ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 87 | 30204 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 130 MM; OUTSIDE DIAMETER: 230 MM; WIDTH: 64 MM; ROW: DOUBLE; BORE SHAPE: TAPER; SUPPL P/N: 22226CCK W33C3; NOTE - THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON AND MUST BE ORIGINAL PROTECTIVE PACKED | EA | 75 |
| 88 | 30216 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 20 MM; OUTSIDE DIAMETER: 42 MM; WIDTH: 12 MM; ROW: SINGLE; SUPPL P/N: 6004; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 89 | 30219 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 110 MM; OUTSIDE DIAMETER: 200 MM; WIDTH: 38 MM; ROW: SINGLE; REFERENCE NO: NU222C3; NU222; ITEM TO BE ORIGINAL PACKED, NOTE: THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVE ON THE BEARING | EA | 75 |
| 90 | 30229 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 65 MM; OUTSIDE DIAMETER: 120 MM; WIDTH: 23 MM; ROW: SINGLE; SUPPL P/N: N213; TWO LIPS ON INNER, PLAIN OUTER, NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 91 | 30237 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 22 MM; OUTSIDE DIAMETER: 57 MM; WIDTH: 17.5 MM; ROW: SINGLE; MODEL NO: CRM7; REFERENCE NO: CRM7C3; NOTE - THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON AND MUST BE ORIGINAL PROTECTIVE PACKED | EA | 75 |
| 92 | 30247 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 65 MM; OUTSIDE DIAMETER: 140 MM; WIDTH: 33 MM; ROW: SINGLE; SUPPL P/N: N313; NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 93 | 30257 | BEARING, ROLLER: TYPE: SELF ALIGNING; SPHERICAL; INSIDE DIAMETER: 75 MM; OUTSIDE DIAMETER: 130 MM; WIDTH: 31 MM; ROW: DOUBLE; REFERENCE NO: 22215CCKC3; THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING, ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 94 | 30258 | BEARING, ROLLER: TYPE: SELF ALIGNING; SPHERICAL; INSIDE DIAMETER: 85 MM; OUTSIDE DIAMETER: 150 MM; WIDTH: 36 MM; ROW: DOUBLE; SUPPL P/N: 22217CK | EA | 75 |
| 95 | 30262 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 100 MM; OUTSIDE DIAMETER: 215 MM; WIDTH: 73 MM; ROW: DOUBLE; SUPPL P/N: 22320CCKC3; MODEL NO: 22320CCK; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 96 | 30265 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: OPEN; INSIDE DIAMETER: 25 MM; OUTSIDE DIAMETER: 62 MM; WIDTH: 17 MM; ROW: SINGLE; SUPPL P/N: 6305; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 97 | 30266 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 80 MM; WIDTH: 21 MM; ROW: SINGLE; SUPPL P/N: 6307C3; NOTE:THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON THE BEARING; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 98 | 30268 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 55 MM; OUTSIDE DIAMETER: 120 MM; WIDTH: 29 MM; ROW: SINGLE; SUPPL P/N: 6311; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE | EA | 75 |
| 99 | 30269 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 60 MM; OUTSIDE DIAMETER: 130 MM; WIDTH: 31 MM; ROW: SINGLE; SUPPL P/N: 6312; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 100 | 30277 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 22.23 MM; OUTSIDE DIAMETER: 57.15 MM; WIDTH: 17.46 MM; ROW: SINGLE; SUPPL P/N: RMS7; SUPPLIER NOTE: THE ITEM MUST BE ORIGINAL PACKED, SEALED AND CLEARLY MARKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 101 | 30283 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 15 MM; OUTSIDE DIAMETER: 35 MM; WIDTH: 15.9 MM; ROW: DOUBLE; SUPPL P/N: 3202A; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 102 | 30288 | BEARING, BALL: TYPE: SPHERICAL; STYLE: OPEN; INSIDE DIAMETER: 12 MM; OUTSIDE DIAMETER: 32 MM; WIDTH: 14 MM; ROW: DOUBLE; SUPPL P/N: 2201STD; NOTE! ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE | EA | 75 |
| 103 | 30290 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: SHIELD; INSIDE DIAMETER: 20 MM; OUTSIDE DIAMETER: 47 MM; WIDTH: 14 MM; ROW: SINGLE; SUPPL P/N: 6204Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 104 | 30291 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 62 MM; WIDTH: 16 MM; ROW: SINGLE; SUPPL P/N: 6206; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 105 | 30310 | BEARING, BALL: TYPE: CONRAD; INSIDE DIAMETER: 25 MM; OUTSIDE DIAMETER: 47 MM; WIDTH: 12 MM; ROW: SINGLE; SUPPL P/N: 6005-NR; STYLE SNAP RING AND GROOVE; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 106 | 30321 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 17 MM; OUTSIDE DIAMETER: 40 MM; WIDTH: 16 MM; ROW: DOUBLE; SUPPL P/N: 4203; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 107 | 30326 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 72 MM; WIDTH: 19 MM; ROW: SINGLE; SUPPL P/N: NU306; NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 108 | 30332 | BEARING, THRUST: ROLLING ELEMENT: ROLLER SPHERICAL; INSIDE DIAMETER: 200 MM; OUTSIDE DIAMETER: 280 MM; WIDTH: 48 MM; MODEL NO: 29240; NOTE - ITEM MUST BE ORIGINAL PACKED; ROW SINGLE | EA | 75 |
| 109 | 30338 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 100 MM; OUTSIDE DIAMETER: 200 MM; WIDTH: 38 MM;ROW: SINGLE; SUPPL P/N: 6222-2ZC3; REFERENCE NO: 6222-2Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 110 | 30339 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 45 MM; OUTSIDE DIAMETER: 120 MM; WIDTH: 29 MM; ROW: SINGLE; SUPPL P/N: 6409C3; NOTE:THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING AND ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 111 | 30342 | BEARING, BALL: TYPE: CONRAD; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 55 MM; WIDTH: 13 MM; ROW: SINGLE; SUPPL P/N: 6006-NR; STYLE SNAP RING AND GROOVE; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 112 | 30354 | BEARING, ROLLER: TYPE: NEEDLE; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 41 MM; WIDTH: 25 MM; ROW: SINGLE; REFERENCE NO: B2216 | EA | 75 |
| 113 | 30359 | BEARING, ROLLER: TYPE: CUP/CONE, TAPER; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 90 MM; WIDTH: 25 MM; ROW: SINGLE; MODEL NO: 32210U; NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 114 | 30360 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 12 MM; OUTSIDE DIAMETER: 32 MM; WIDTH: 16 MM; ROW: DOUBLE; SUPPL P/N: 3201A; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 115 | 30369 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 40 MM; OUTSIDE DIAMETER: 80 MM; WIDTH: 18 MM; ROW: SINGLE; SUPPL P/N: 6208-2Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 116 | 30370 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 5 MM; OUTSIDE DIAMETER: 19 MM; WIDTH: 6 MM; ROW: SINGLE; SUPPL P/N: 635; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 25 |
| 117 | 30374 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 12 MM; OUTSIDE DIAMETER: 28 MM; WIDTH: 8 MM; ROW: SINGLE; SUPPL P/N: 6001-2Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 118 | 30375 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: OPEN; INSIDE DIAMETER: 15 MM; OUTSIDE DIAMETER: 42 MM; WIDTH: 13 MM; ROW: SINGLE; SUPPL P/N: 7302B; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 119 | 30376 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 17 MM; OUTSIDE DIAMETER: 35 MM; WIDTH: 10 MM; ROW: SINGLE; SUPPL P/N: 6003-2Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 120 | 30378 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 17 MM; OUTSIDE DIAMETER: 47 MM; WIDTH: 14 MM; ROW: SINGLE; SUPPL P/N: 6303-2RS; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 121 | 30379 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 17 MM; OUTSIDE DIAMETER: 62 MM; WIDTH: 17 MM; ROW: SINGLE; SUPPL P/N: 6403; NOTE: ITEM MUST BE ORIGINAL PACKED; DUTY: HEAVY; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT | EA | 75 |
| 122 | 30381 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 25 MM; OUTSIDE DIAMETER: 47 MM; WIDTH: 12 MM; ROW: SINGLE; SUPPL P/N: 6005; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 123 | 30382 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: SHIELD; INSIDE DIAMETER: 25 MM; OUTSIDE DIAMETER: 52 MM; WIDTH: 15 MM; ROW: SINGLE; SUPPL P/N: 6205Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 124 | 30385 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 25 MM; OUTSIDE DIAMETER: 62 MM; WIDTH: 17 MM; ROW: SINGLE; SUPPL P/N: 6305-2RS; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 125 | 30386 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 55 MM; WIDTH: 13 MM; ROW: SINGLE; SUPPL P/N: 6006; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 126 | 30387 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: SHIELD; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 62 MM; WIDTH: 16 MM; ROW: SINGLE; SUPPL P/N: 6206-Z; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 127 | 30392 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 72 MM; WIDTH: 17 MM; ROW: SINGLE; SUPPL P/N: 6207-2Z; DUTY: MEDIUM; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 128 | 30396 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 45 MM; OUTSIDE DIAMETER: 85 MM; WIDTH: 19 MM; ROW: SINGLE; SUPPL P/N: 6209-2Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 129 | 30397 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: SHIELD; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 110 MM; WIDTH: 27 MM; ROW: SINGLE; SUPPL P/N: 6310Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 130 | 30398 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 110 MM; WIDTH: 27 MM; ROW: SINGLE; SUPPL P/N: 6310-2Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 131 | 30399 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 60 MM; OUTSIDE DIAMETER: 95 MM; WIDTH: 18 MM; ROW: SINGLE; SUPPL P/N: 6012; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 132 | 30400 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 60 MM; OUTSIDE DIAMETER: 130 MM; WIDTH: 31 MM; ROW: SINGLE; SUPPL P/N: 6312-2ZR; DUTY: MEDIUM; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 133 | 30406 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 20 MM; OUTSIDE DIAMETER: 52 MM; WIDTH: 22 MM; ROW: DOUBLE; SUPPL P/N: 3304A; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 134 | 30407 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: OPEN; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 80 MM; WIDTH: 34.9 MM; ROW: DOUBLE; SUPPL P/N: 3307A; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 135 | 30414 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 12 MM; OUTSIDE DIAMETER: 32 MM; WIDTH: 14 MM; ROW: DOUBLE; REFERENCE NO: 4201; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 136 | 30418 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 12.7 MM; OUTSIDE DIAMETER: 33.34 MM; WIDTH: 9.53 MM; ROW: SINGLE; SUPPL P/N: RLS4; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 137 | 30439 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 110 MM; WIDTH: 27 MM; ROW: SINGLE; SUPPL P/N: NU310STD; NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 138 | 30443 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 100 MM; OUTSIDE DIAMETER: 180 MM; WIDTH: 46 MM; ROW: DOUBLE; SUPPL P/N: 22220CCC3; NOTE: THE BEARING REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING AND ORIGINAL PACKED | EA | 75 |
| 139 | 30446 | BEARING, ROLLER: TYPE: ANGULAR CONTACT; INSIDE DIAMETER: 75 MM; OUTSIDE DIAMETER: 161 MM; WIDTH: 55 MM; ROW: SINGLE; SUPPL P/N: 22315CC-STD; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 140 | 30454 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 140 MM; OUTSIDE DIAMETER: 300 MM; WIDTH: 102 MM; ROW: DOUBLE; SUPPL P/N: 22328CCKC3; NOTE: THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING AND ORIGINAL PACKED | EA | 75 |
| 141 | 30460 | CONE, BEARING: TYPE: SINGLE; INSIDE DIAMETER: 69.85 MM; WIDTH: 25.4 MM; BORE SHAPE: CYLINDRICAL; SUPPL P/N: 29675; NOTE! ITEM MUST BE ORIGINAL PACKED; IBI 21437; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 142 | 30469 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 45 MM; OUTSIDE DIAMETER: 100 MM; WIDTH: 40 MM; ROW: DOUBLE; SUPPL P/N: 3309BC3; REFERENCE NO: 3309B; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 143 | 30470 | BEARING, BALL: TYPE: SELF ALIGNING; STYLE: OPEN; INSIDE DIAMETER: 8 MM; OUTSIDE DIAMETER: 22 MM; WIDTH: 7 MM; ROW: SINGLE; SUPPL P/N: 608; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 144 | 30473 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 9 MM; OUTSIDE DIAMETER: 26 MM; WIDTH: 8 MM; ROW: SINGLE; SUPPL P/N: 629-2Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 145 | 30475 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 10 MM; OUTSIDE DIAMETER: 26 MM; WIDTH: 8 MM; ROW: SINGLE; SUPPL P/N: 6000-2Z; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 146 | 30481 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 15 MM; OUTSIDE DIAMETER: 35 MM; WIDTH: 11 MM; ROW: SINGLE; SUPPL P/N: 6202C3; NOTE:THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING AND ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 147 | 30485 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: OPEN; INSIDE DIAMETER: 85 MM; OUTSIDE DIAMETER: 150 MM; WIDTH: 28 MM; ROW: SINGLE; SUPPL P/N: 6217C3; THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING AND ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 148 | 30486 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 12 MM; OUTSIDE DIAMETER: 37 MM; WIDTH: 12 MM; ROW: SINGLE; SUPPL P/N: 6301; SOLE SOURCE: CLYDE BERGEMANN AFRICA PTY LTD; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 149 | 30489 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 72 MM; WIDTH: 19 MM; ROW: SINGLE; SUPPL P/N: 6306-2Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 150 | 30490 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 45 MM; OUTSIDE DIAMETER: 100 MM; WIDTH: 25 MM; ROW: SINGLE; SUPPL P/N: 6309-2PZC3; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 151 | 30491 | BEARING, BALL: TYPE: CONRAD; STYLE: SHIELD; INSIDE DIAMETER: 45 MM; OUTSIDE DIAMETER: 100 MM; WIDTH: 25 MM; ROW: SINGLE; SUPPL P/N: 6309Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 152 | 30494 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 65 MM; OUTSIDE DIAMETER: 140 MM; WIDTH: 33 MM; ROW: SINGLE; SUPPL P/N: 6313-2Z; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 153 | 30496 | BEARING, ROLLER: TYPE: SELF ALIGNING; SPHERICAL; INSIDE DIAMETER: 45 MM; OUTSIDE DIAMETER: 85 MM; WIDTH: 23 MM; ROW: DOUBLE; MODEL NO: 22209 CC; NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 154 | 30501 | BEARING, ROLLER: TYPE: SELF ALIGNING; SPHERICAL; INSIDE DIAMETER: 200 MM; OUTSIDE DIAMETER: 340 MM; WIDTH: 112 MM; ROW: DOUBLE; SUPPL P/N: 23140CCW33C3; NOTE - THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON AND ORIGINAL/PROTECTIVE PACKED | EA | 75 |
| 155 | 30506 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 110 MM; WIDTH: 27 MM; ROW: DOUBLE; SUPPL P/N: 21310CCK; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 156 | 30519 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 85 MM; OUTSIDE DIAMETER: 180 MM; WIDTH: 41 MM; ROW: SINGLE; SUPPL P/N: N317; TWO LIPS ON INNER, PLAIN OUTER, NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 157 | 30538 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 140 MM; OUTSIDE DIAMETER: 250 MM; WIDTH: 88 MM; ROW: DOUBLE; MODEL NO: 23228 CCK-W33; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 158 | 30552 | BEARING, ROLLER: TYPE: CUP/CONE, TAPER; INSIDE DIAMETER: 25.4 MM; OUTSIDE DIAMETER: 65.09 MM; WIDTH: 21.46 MM; ROW: SINGLE; MODEL NO: 23256B-23100; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 159 | 30570 | BEARING, BALL: TYPE: CONRAD; STYLE: SHIELD; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 90 MM; WIDTH: 20 MM; ROW: SINGLE; SUPPL P/N: 6210-2ZC3; NOTE: THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING AND ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 160 | 30575 | BEARING, ROLLER: TYPE: STRAIGHT; INSIDE DIAMETER: 22 MM; OUTSIDE DIAMETER: 58 MM; WIDTH: 18 MM; ROW: SINGLE; MODEL NO: MMRJ7/8; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 161 | 30576 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 25 MM; OUTSIDE DIAMETER: 62 MM; WIDTH: 17 MM; ROW: SINGLE; MODEL NO: NU 305; ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 162 | 30577 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 55 MM; OUTSIDE DIAMETER: 90 MM; WIDTH: 18 MM; ROW: SINGLE; MODEL NO: NU1011; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 163 | 30578 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 70 MM; OUTSIDE DIAMETER: 125 MM; WIDTH: 24 MM; ROW: SINGLE; SUPPL P/N: NU 214C3; MODEL NO: NU 214; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 164 | 30585 | BEARING, THRUST: ROLLING ELEMENT: BALL; INSIDE DIAMETER: 80 MM; OUTSIDE DIAMETER: 115 MM; WIDTH: 28 MM; STYLE: OPEN; SUPPL P/N: 51216; NOTE - ITEM MUST BE ORIGINAL PACKED, WITH WASHERS; ROW SINGLE | EA | 75 |
| 165 | 30586 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 90 MM; OUTSIDE DIAMETER: 190 MM; WIDTH: 43 MM; ROW: SINGLE; SUPPL P/N: N318 ECPC3; MODEL NO: N318 ECP; ITEM TO BE ORIGINAL PACKED, NOTE: THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVE ON THE BEARING | EA | 75 |
| 166 | 30587 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 20 MM; OUTSIDE DIAMETER: 52 MM; WIDTH: 15 MM; ROW: SINGLE; MODEL NO: NU 304; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 167 | 30588 | BEARING, ROLLER: TYPE: STRAIGHT; INSIDE DIAMETER: 150 MM; OUTSIDE DIAMETER: 270 MM; WIDTH: 45 MM; ROW: SINGLE; SUPPL P/N: NU230ECJC3; ONLY SKF, RHP OR FAG ACCEPTABLE AT KRIEL POWER STATION, NOTE! ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 168 | 30590 | BEARING, ROLLER: TYPE: TAPER; SUPPL P/N: 82680X-82620D; SPACER X1S82680X, ASSEMBLY NUMBER 49603, END PLAY, 008 INCH, NOTE! ITEM MUST BE PROTECTIVE PACKED, SPECIAL NOTE: PLEASE ENSURE THAT SPACER IS INCLUDED | EA | 75 |
| 169 | 30592 | BEARING, ROLLER: TYPE: CUP/CONE, TAPER; INSIDE DIAMETER: 114.3 MM; OUTSIDE DIAMETER: 234 MM; WIDTH: 66.68 MM; ROW: SINGLE; MODEL NO:HH224346-224310; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 170 | 30595 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 45 MM; OUTSIDE DIAMETER: 85 MM; WIDTH: 19 MM; ROW: SINGLE; SUPPL P/N: 6209-2RS; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 171 | 30596 | BEARING, BALL: TYPE: SPHERICAL; STYLE: OPEN; INSIDE DIAMETER: 65 MM; OUTSIDE DIAMETER: 120 MM; WIDTH: 23 MM; ROW: SINGLE; SUPPL P/N: 1213K; NOTE! ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 172 | 30605 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 500 MM; OUTSIDE DIAMETER: 720 MM; WIDTH: 167 MM; ROW: DOUBLE; SUPPL P/N: 230/500 CA/W33; NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 173 | 30608 | BEARING, ROLLER: TYPE: CUP/CONE, TAPER; INSIDE DIAMETER: 36.51 MM; OUTSIDE DIAMETER: 88.7 MM; WIDTH: 26 MM; ROW: SINGLE; MODEL NO: 44143-44348B; NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 174 | 30610 | BEARING, ROLLER: TYPE: STRAIGHT; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 82 MM; WIDTH: 21.5 MM; ROW: SINGLE; BORE SHAPE: TAPER; MODEL NO: JLM104948; COMPLETE, NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 175 | 30624 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 25 MM; OUTSIDE DIAMETER: 52 MM; WIDTH: 15 MM; ROW: SINGLE; SUPPL P/N: 6205-2RS; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 176 | 30634 | BEARING, ROLLER: TYPE: NEEDLE; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 42 MM; WIDTH: 12 MM; ROW: SINGLE; REFERENCE NO: HK | EA | 75 |
| 177 | 30638 | BEARING, THRUST: ROLLING ELEMENT: ROLLER; INSIDE DIAMETER: 500 MM; OUTSIDE DIAMETER: 870 MM; WIDTH: 224 MM; SUPPL P/N: 294-500EM; ROW SINGLE | EA | 25 |
| 178 | 30651 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 20 MM; OUTSIDE DIAMETER: 42 MM; WIDTH: 8 MM; ROW: SINGLE; SUPPL P/N: 16004; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 179 | 30835 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 65 MM; OUTSIDE DIAMETER: 120 MM; WIDTH: 23 MM; ROW: SINGLE; SUPPL P/N: 6213-2RS; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 180 | 31152 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 200 MM; OUTSIDE DIAMETER: 340 MM; WIDTH: 112 MM; ROW: DOUBLE; SUPPL P/N: 23140 CCK/W33C3; ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 181 | 31159 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 90 MM; WIDTH: 20 MM; ROW: SINGLE; SUPPL P/N: 6210-2Z; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 182 | 31175 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 85 MM; OUTSIDE DIAMETER: 180 MM; WIDTH: 41 MM; ROW: SINGLE; SUPPL P/N: 6317C3; DUTY: HEAVY; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 183 | 31181 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: SHIELD; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 90 MM; WIDTH: 20 MM; ROW: SINGLE; SUPPL P/N: 6210-Z; VENDORS ARE | EA | 75 |
| 184 | 31198 | BEARING, ROLLER: TYPE: CUP/CONE, TAPER; INSIDE DIAMETER: 80 MM; OUTSIDE DIAMETER: 170 MM; WIDTH: 62 MM; ROW: SINGLE; REFERENCE NO: 32316B; ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 185 | 31223 | BEARING, ROLLER: TYPE: CUP/CONE, TAPER; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 110 MM; WIDTH: 42.5 MM; ROW: SINGLE; SUPPL P/N: 32310; 32310STD | EA | 75 |
| 186 | 31244 | BEARING, THRUST: ROLLING ELEMENT: BALL; INSIDE DIAMETER: 40 MM; OUTSIDE DIAMETER: 60 MM; WIDTH: 16 MM; STYLE: DOUBLE SHIELD; SUPPL P/N: 51108; ITEM MUST BE ORIGINAL PACKED; ROW SINGLE | EA | 75 |
| 187 | 31274 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 60 MM; OUTSIDE DIAMETER: 110 MM; WIDTH: 22 MM; ROW: SINGLE; SUPPL P/N: 6212-2Z; ITEM TO BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 188 | 31279 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 19.05 MM; OUTSIDE DIAMETER: 47 MM; WIDTH: 31 MM; ROW: SINGLE; SUPPL P/N: 1020- 3/4DECG; FOR COAL FEEDER, ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 189 | 31315 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 52 MM; OUTSIDE DIAMETER: 195 MM; LENGTH: 125 MM; MATERIAL: STL; DRAWING NO: THGR 204400 POS 442 REV 1; THGR 204400 REV 1; FOR HP CONTROL VALVE | EA | 75 |
| 190 | 31327 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 40 MM; OUTSIDE DIAMETER: 80 MM; WIDTH: 23 MM; ROW: DOUBLE; MODEL NO: 22208CCK-W33; REFERENCE NO: 22208CCK-W33C3; WITH ADAPTER SLEEVE H308, NOTE - THE REFERENCE NUMBER MUST BE CAST AND NOT | EA | 75 |
| 191 | 31344 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 36 MM; OUTSIDE DIAMETER: 67 MM; LENGTH: 45 MM; MATERIAL: STL; DRAWING NO: R 406681POS 2076A/20676B REV 1; R 406681 REV 1; INTERMEDIATE SET, BFPT MAIN OIL PUMP | EA | 75 |
| 192 | 31433 | BEARING, ASSEMBLY: TYPE: ROLLER; INSIDE DIAMETER: 405 MM; OUTSIDE DIAMETER: 485 MM; WIDTH: 140 MM; REFERENCE NO: 101210704C3; 01B1600EX; 86-1217W; COOPER SPLIT TYPE; COMPRISING: 1 X INNER RACE AND CLAMPING RINGS SPL F/CR OSO NUMBER 4; 16 IN BORE 2 X HALF OUTER RACE NUMBER SPL F/CR OSO Y624 NUMBER 3; HALF CAGE AND ROLLERS; 16 IN BORE; ROW DOUBLE; AIR HEATER GUIDE BEARING: DRW 2Z4410/22; PART NO: TUTBLR1-6AH79; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 25 |
| 193 | 31549 | BEARING, SLEEVE: TYPE: LOCKING; INSIDE DIAMETER: 127 MM; OUTSIDE DIAMETER: 247.6 MM; LENGTH: 127 MM; MATERIAL: WHITE METAL; SIZE: 5 IN; SUPPL P/N: 17520; THRUST AND JOURNAL SLEEVE; FOR JAMES HOWDEN PA FANS DRIVE END BEARING, UNIT 4 TO 6, SUPPLIER NOTE: THE ITEM MUST BE PROTECTIVE PACKED; MATERIAL: WHITE METAL; TYPE: 5 IN PA FAN; IBI 30021; MACHINE THE BEARING TO 126.997 - 127.084MM ID, CYLINDRICAL BORE, MATERIAL WM90, SHAFT SPEED 1500RPM, ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE-METALLING OF WHITE METAL BEARINGS ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE THE WORK STARTS, THE SUPPLIER SHALL BE RESPONSIBLE OF THE FOLLOWING: MINIMISING DISTORTION OF THE BEARING SHELL DURING THE CASTING PROCESS; RESPONSIBLE FOR ENSURING THE MATING FACES OF BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND AND THE BEARING OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS; ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETORIORATION DURING TRANSPORTATION, HANDLING AND STORAGE; DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS; RECORD ALL THE RELEVANT DIMENSIONS AND FEATURES OF THE BEARING ON A DRAWING TO BE SUBMITTED TOGETHER WITH THE FINISHED BEARING, INSPECTION OF THE FINAL MACHINED SIZES MUST BE DONE ON THE MACHINE AND ONCE REMOVED IT MUST BE REPEATED ON A WORK BENCH AT ROOM TEMPERATURE, ALL INSPECTION REPORTS MUST ACCOMPANY THE BEARING DATAPACK AND THE SUPPLIER SHALL DETERMINE THE DEGREE OF BONDING BY MEANS OF AN ULTRASONIC METHOD COMPLYING WITH BS7585 PART1, OR EQUIVALENT METHOD A TOTAL BONDED AREA 95% SHALL BE ACHIEVED ALL JOINTS, EDGES AND | EA | 75 |
| 194 | 31554 | BEARING, ROLLER: TYPE: TAPER CONE/CUP; INSIDE DIAMETER: 37 MM; OUTSIDE DIAMETER: 19.05 MM; WIDTH: 69 MM; ROW: SINGLE; MODEL NO: 13621-13682; TIMKEN, FOR VOLUMETRIC COAL FEEDER 44115, NOTE! ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 195 | 31685 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 130 MM; WIDTH: 31 MM; ROW: SINGLE; SUPPL P/N: 6410; DUTY: MEDIUM; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 196 | 31808 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 65 MM; OUTSIDE DIAMETER: 140 MM; WIDTH: 33 MM; ROW: SINGLE; REFERENCE NO: NU313/C3; ITEM TO BE ORIGINAL PACKED, NOTE - THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVE ON THE BEARING | EA | 75 |
| 197 | 31813 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 110 MM; WIDTH: 27 MM; ROW: SINGLE; SUPPL P/N: NU310C3; NOTE - THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON AND ORIGINAL PROTECTIVE PACKED, SKF TYPE ONLY | EA | 75 |
| 198 | 31834 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 20 MM; OUTSIDE DIAMETER: 52 MM; WIDTH: 15 MM; ROW: SINGLE; SUPPL P/N: 6304C3; REFERENCE NO: 6304; DUTY: MEDIUM; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 199 | 31851 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 80 MM; WIDTH: 21 MM; ROW: SINGLE; SUPPL P/N: 6307-2ZC3; NOTE: THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING AND ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 200 | 31852 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 72 MM; WIDTH: 19 MM; ROW: SINGLE; MODEL NO: 6306-2RS1; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 201 | 31853 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 72 MM; WIDTH: 19 MM; ROW: SINGLE; SUPPL P/N: 6306-2ZC3; NOTE: THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING AND ORIGINAL PACKED;SKF BRAND ONLY; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 202 | 31854 | BEARING, ROLLER: TYPE: TAPER; INSIDE DIAMETER: 114.3 MM; OUTSIDE DIAMETER: 215 MM; WIDTH: 68 MM; ROW: SINGLE; MODEL NO: 932-938; COMPLETE, NOTE! ITEM MUST BE ORIGINAL PACKED, TIMKEN BRAND ONLY | EA | 75 |
| 203 | 31855 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 80 MM; WIDTH: 35 MM; ROW: DOUBLE; SUPPL P/N: 3307ATN9C3; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 204 | 31856 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 41 MM; OUTSIDE DIAMETER: 80 MM; WIDTH: 18 MM; ROW: SINGLE; SUPPL P/N: NU 208 ECKPC3; MODEL NO: NU 208 ECKP; COMPLETE WITH ADAPTER SLEEVE H208 WITH NUT, NOTE: THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING AND ORIGINAL PACKED | EA | 75 |
| 205 | 31900 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 90 MM; OUTSIDE DIAMETER: 159 MM; LENGTH: 150 MM; MATERIAL: STL; DRAWING NO: BBT TGR 305600 POS 20648 REV 1; INTERMEDIATE; FOR MAIN OIL DUPLEX GEAR PUMP | EA | 75 |
| 206 | 31905 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 110 MM; WIDTH: 27 MM; ROW: SINGLE; SUPPL P/N: 6310ZZC3; DUTY: MEDIUM; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 207 | 31967 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: SHIELD; INSIDE DIAMETER: 4.4 MM; OUTSIDE DIAMETER: 13 MM; WIDTH: 6 MM; ROW: SINGLE; DRAWING NO: BBT R304508 POS 1230 REV 1; R304508 REV 1; POS 15724, FOR MULTIPLICATION RELAY; BBT; DUTY: EXTRA LIGHT; VENDORS ARE RESPONSIBLE FOR ENSURING THAT | EA | 75 |
| 208 | 31968 | BEARING, BALL: TYPE: RADIAL GROOVED; STYLE: OPEN; INSIDE DIAMETER: 25 MM; OUTSIDE DIAMETER: 47 MM; WIDTH: 12 MM; ROW: SINGLE; DRAWING NO: HTGR 306948 POS 247 REV 1; HTGR 306948 REV 1; FOR MAIN AND BOILER FEED PUMP TURBINES OPERATING DEVICE, AS PER BBT; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 209 | 31988 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 65 MM; OUTSIDE DIAMETER: 140 MM; WIDTH: 58.74 MM; ROW: DOUBLE; SUPPL P/N: 3313AC3; COMPLETE WITH CIRCLIP ON OUTSIDE DIA, NOTE! THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 210 | 32009 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 100 MM; OUTSIDE DIAMETER: 180 MM; WIDTH: 34 MM; ROW: SINGLE; MODEL NO: N220; NOTE: THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING | EA | 75 |
| 211 | 32039 | BRG BAL:ANGULAR;2Z;ID 20 MM;OD 42 MM;STL | EA | 25 |
| 212 | 32053 | BEARING, BALL: TYPE: SPHERICAL; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 31.75 MM; OUTSIDE DIAMETER: 72 MM; WIDTH: 42.86 MM; ROW: SINGLE; SUPPL P/N: 1035-1.1/4; ITEM MUST BE ORIGINAL (PROTECTIVE) PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 213 | 32113 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 15.8 MM; OUTSIDE DIAMETER: 39.6 MM; WIDTH: 11 MM; ROW: SINGLE; SUPPL P/N: LLRJ5/8; ITEM MUST BE ORIGINAL PACKED. | EA | 75 |
| 214 | 32114 | BEARING, ROLLER: TYPE: STRAIGHT; INSIDE DIAMETER: 60 MM; OUTSIDE DIAMETER: 110 MM; WIDTH: 22 MM; ROW: SINGLE; SUPPL P/N: N 212C3; MODEL NO: N 212; NOTE: THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING | EA | 75 |
| 215 | 32139 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 85 MM; OUTSIDE DIAMETER: 180 MM; WIDTH: 41 MM; ROW: SINGLE; SUPPL P/N: NU317ECPC3 | EA | 75 |
| 216 | 32278 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 43 MM; OUTSIDE DIAMETER: 85 MM; WIDTH: 56 MM; ROW: SINGLE; SUPPL P/N: 1111KRR; COLLAR, FOR STOCK EQUIPMENT VOLUMETRIC COAL FEEDER NON DRIVE END, NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 217 | 32280 | BEARING, BALL: TYPE: MINIATURE CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 3 MM; OUTSIDE DIAMETER: 10 MM; WIDTH: 4 MM; ROW: SINGLE; SUPPL P/N: 623-2Z; DUTY: EXTRA LIGHT; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 218 | 32281 | BEARING, ROLLER: TYPE: SPLIT; INSIDE DIAMETER: 450 MM; OUTSIDE DIAMETER: 550 MM; WIDTH: 74 MM; ROW: DOUBLE; REFERENCE NO: 01B1600EX FCR OSOC3; 01B1600EX FCR OSO; GUIDE | EA | 25 |
| 219 | 32282 | BEARING, THRUST: INSIDE DIAMETER: 410 MM; OUTSIDE DIAMETER: 630 MM; WIDTH: 30 MM; DRAWING NO: Z3073D REV 1; REFERENCE NO: ET 11864-3; TYPE COOPER, FOR USE ON AIR HEATER, FOR AIR HEATER MAIN SHAFT | EA | 25 |
| 220 | 32283 | BEARING, BALL: TYPE: MINIATURE CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 3 MM; OUTSIDE DIAMETER: 9.5 MM; WIDTH: 4 MM; ROW: SINGLE; SUPPL P/N: 1J9689-0606; FOR USE ON FISHER CONTROL VALVE; DUTY: EXTRA LIGHT; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 221 | 32286 | BEARING, ROLLER: TYPE: CUP/CONE, TAPER; INSIDE DIAMETER: 165.1 MM; OUTSIDE DIAMETER: 336.55 MM; WIDTH: 69.85 MM; ROW: SINGLE; SUPPL P/N: HH437549-437510; TIMKEN OR SKF OR FAG OR RHP BRAND ONLY | EA | 75 |
| 222 | 32287 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 9.4 MM; OUTSIDE DIAMETER: 14.4 MM; LENGTH: 12.7 MM; MATERIAL: BRZ; SUPPL P/N: P22413B; FOR WALLACE AND TIERNAN ELECTRIC GRAVIMETRIC FEEDER ON WATER CLARIFICATION PLANT NORTH, SUPPLIER NOTE: THE ITEM MUST BE PROTECTIVE PACKED | EA | 75 |
| 223 | 32288 | BEARING, BALL: TYPE: SELF ALIGNING; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 45 MM; OUTSIDE DIAMETER: 85 MM; WIDTH: 27 MM; ROW: SINGLE; SUPPL P/N: 410031D; 27MM WIDE EXTENDED INNER RING, FOR COAL FEEDERS, NOTE! MATERIAL MUST BE ORIGINAL PROTECTIVE PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 224 | 32291 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 140 MM; OUTSIDE DIAMETER: 200 MM; LENGTH: 230 MM; MATERIAL: STL BABBITTED; SUPPL P/N: 310-2; MODEL NO: 200-415-1; FOR KSB PUMP LUVE | EA | 75 |
| 225 | 32292 | BEARING, THRUST: ROLLING ELEMENT: BALL; INSIDE DIAMETER: 45 MM; OUTSIDE DIAMETER: 64 MM; WIDTH: 9 MM; STYLE: DOUBLE SHIELD; MODEL NO: 51109-J9; NOTE - THAT SKF OR FAG OR RHP BRAND ACCEPTED; ROW SINGLE | EA | 75 |
| 226 | 32293 | BEARING, ASSEMBLY: TYPE: TAPER ROLLER; INSIDE DIAMETER: 114 MM; OUTSIDE DIAMETER: 228 MM; WIDTH: 117 MM; SUPPL P/N: 97450; 97901D; 90039; X1S-95475; MODEL NO: 97450- 97901D; DOUBLE ROW; COMPLETE; TIMKEN DOUBLE OUTER RACE ASSEMBLY; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 227 | 32294 | BEARING, THRUST: ROLLING ELEMENT: ROLLER NEEDLE; INSIDE DIAMETER: 40 MM; OUTSIDE DIAMETER: 60 MM; WIDTH: 3 MM; SUPPL P/N: AXK4060; COMPLETE WITH 2 WASHERS AS4060 EACH 1MM THICK, ROW SINGLE | EA | 75 |
| 228 | 32295 | BEARING, BALL: TYPE: CONRAD; STYLE: OPEN; INSIDE DIAMETER: 120 MM; OUTSIDE DIAMETER: 215 MM; WIDTH: 40 MM; ROW: SINGLE; SUPPL P/N: 6224C3; NOTE:THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING, ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 229 | 32296 | CUP, BEARING: TYPE: DOUBLE; OUTSIDE DIAMETER: 228.6 MM; WIDTH: 84.14 MM; REFERENCE NO: 97901D; 97450; WITH OIL GROOVE, TIMKEN, FOR USE WITH TAPER ROLLER BEARINGS | EA | 75 |
| 230 | 32299 | BEARING, THRUST: ROLLING ELEMENT: ROLLER NEEDLE; INSIDE DIAMETER: 55 MM; OUTSIDE DIAMETER: 78 MM; WIDTH: 3 MM; REFERENCE NO: AXK5578; NEEDLE, COMPLETE WITH WASHER AS5578, ITEM MUST BE ORIGINAL PROTECTIVE PACKED, ROW SINGLE | EA | 75 |
| 231 | 32300 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 80 MM; WIDTH: 35 MM; ROW: DOUBLE; SUPPL P/N: 307SFFC; M1985; (MRC ), FOR MODEL SP-214- P1(X-9883) DIESEL COMPRESSOR CLUTCH PILOT BEARING; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 232 | 32301 | BEARING, SLEEVE: TYPE: MOTOR; MATERIAL: WHITE METAL; REFERENCE NO: 1G050B0101; DRIVE END, FOR MITSUBISHI , TYPE:- MKB-R, FRAME 9000, 1492 KW, 18 POLE, 330 RPM, 3300 VOLT | EA | 25 |
| 233 | 32307 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 220 MM; OUTSIDE DIAMETER: 340 MM; LENGTH: 175 MM; MATERIAL: STL; DRAWING NO: THGD 90191 REV 1; THGD 90191 POS 3 AND 4 REV 1; OUTER SHELL, UPPER AND LOWER HALF, FOR BFPT THREE WEDGE , TO BBT | EA | 75 |
| 234 | 32308 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 180 MM; OUTSIDE DIAMETER: 250 MM; LENGTH: 133 MM; MATERIAL: WHITE METAL; DRAWING NO: BBT HTGD 90191 POS1/2 REV 1; HTGD 90191 REV 1; INNER SHELL, UPEER AND LOWER HALF, BBT, FOR BFPT THREE WEDGE | EA | 75 |
| 235 | 32310 | BEARING, SLEEVE: TYPE: MOTOR NDE; MATERIAL: WHITE METAL; REFERENCE NO: 1G050B0101; FOR MITSUBISHI , TYPE:- MKB-R, FRAME 9000, 1492 KW , 18 POLE, 330 RPM, 3300 VOLT | EA | 25 |
| 236 | 32311 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 420 MM; OUTSIDE DIAMETER: 620 MM; WIDTH: 150 MM; MODEL NO: 23084CA-W33; ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 237 | 32312 | BEARING, SLEEVE: TYPE: SPEED GOVERNOR POS 553; INSIDE DIAMETER: 36 MM; OUTSIDE DIAMETER: 58 MM; LENGTH: 26.5 MM; MATERIAL: WHITE METAL; DRAWING NO: BBT HTGR 305950 REV 1; REFERENCE NO: HTGR308489R0001; ON MAIN BOILER FEED PUMP, SUPPLIER NOTE: THE ITEM MUST BE PROTECTIVE PACKED, SEALED AND CLEARLY MARKED | EA | 75 |
| 238 | 32313 | BEARING, SLEEVE: TYPE: SPEED GOVERNOR POS 554; INSIDE DIAMETER: 36 MM; OUTSIDE DIAMETER: 98 MM; LENGTH: 25 MM; MATERIAL: WHITE METAL; DRAWING NO: BBT HTGR 305950 REV 1; REFERENCE NO: HTGR305950POS554; ON MAIN AND BFP TURBINES, SUPPLIER NOTE: THE ITEM MUST BE PROTECTIVE PACKED, SEALED AND CLEARLY MARKED | EA | 75 |
| 239 | 32314 | BEARING, BALL: TYPE: RADIAL; STYLE: OPEN; INSIDE DIAMETER: 6 MM; OUTSIDE DIAMETER: 13 MM; WIDTH: 3.5 MM; ROW: SINGLE; SUPPL P/N: 17228; DRAWING NO: HTGR306948 POS 228 REV 1; HTGR306948 REV 1; FOR MAIN AND BOILER FEED PUMP TURBINES; DUTY: EXTRA LIGHT; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 240 | 32315 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 95 MM; OUTSIDE DIAMETER: 122 MM; LENGTH: 80 MM; MATERIAL: STL; DRAWING NO: TGR408553 REV 1; TGR410127 REV 1;BBT TGR410127 POS 516D REV 1; MODEL NO: 1211; FOR DRIVING GEAR ASSEMBLY/GEARBOX | EA | 75 |
| 241 | 32316 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: SHIELD; INSIDE DIAMETER: 3 MM; OUTSIDE DIAMETER: 10 MM; WIDTH: 5 MM; ROW: SINGLE; SUPPL P/N: 14607; DRAWING NO: BBT R303790 REV 1; BBT R303790 POS 1223/3202/3206 REV 1; POSITION:- 607, FOR PRESSURE CONTROLLER, TYPE:- R55, HYDRAULIC PART, SHEMATIC DUTY: EXTRA LIGHT; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 242 | 32317 | BEARING, BALL: TYPE: RADIAL; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 85 MM; OUTSIDE DIAMETER: 180 MM; WIDTH: 41 MM; ROW: SINGLE; SUPPL P/N: 6317-2Z; NOTE! ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 243 | 32318 | BEARING: TYPE: RADIAL LSS; INSIDE DIAMETER: 100 MM; OUTSIDE DIAMETER: 210 MM; WIDTH: 100 MM; STYLE: SPLIT; MATERIAL: STL; SUPPL P/N: 790-1250-03-2; MODEL NO: GN25; DUTY HEAVY | EA | 75 |
| 244 | 32319 | BEARING: TYPE: RADIAL-AXIAL LSS; INSIDE DIAMETER: 100 MM; OUTSIDE DIAMETER: 210 MM; WIDTH: 90 MM; STYLE: SPLIT; MATERIAL: STL; SUPPL P/N: 790-1251-06-2; MODEL NO: GN25; DUTY: HEAVY | EA | 75 |
| 245 | 32320 | BEARING: TYPE: RADIAL; STYLE: SPLIT; MATERIAL: WHITE METAL; SUPPL P/N: 790-1219-05-2; MODEL NO: GN25; AXIAL HSS | EA | 75 |
| 246 | 32321 | BEARING, SLEEVE: TYPE: RADIAL HSS; INSIDE DIAMETER: 70 MM; OUTSIDE DIAMETER: 190 MM; MATERIAL: WHITE METAL; SUPPL P/N: 790-1213-02-2; MODEL NO: GN25; DUTY: MEDIUM; STYLE: SPLIT; WIDTH: 70 MM | EA | 75 |
| 247 | 32324 | BRG RLR:CYLINDRICAL;ID 22 MM;OD 62 MM | EA | 25 |
| 248 | 32325 | BEARING, THRUST: INSIDE DIAMETER: 140 MM; OUTSIDE DIAMETER: 179 MM; WIDTH: 18 MM; STYLE: DOUBLE SHIELD; SUPPL P/N: AX11140180; DRAWING NO: HTGD 90228 REV 1; HTGD 90228 POS 38 REV 1; FOR BFPT LUB OIL FILTER, 16 L/S | EA | 75 |
| 249 | 32326 | BEARING, THRUST: INSIDE DIAMETER: 200 MM; OUTSIDE DIAMETER: 242 MM; WIDTH: 28 MM; STYLE: DOUBLE SHIELD; MATERIAL: STL; SUPPL P/N: AX14200250; DRAWING NO: HTGD 90228 POS 38 REV 1; HTGD 90228 REV 1; FOR MAIN TURBINE LUB OIL FILTER, 25 L/S, SCHEMATIC POS 5765/5766 | EA | 75 |
| 250 | 32327 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: OPEN; INSIDE DIAMETER: 100 MM; OUTSIDE DIAMETER: 180 MM; WIDTH: 34 MM; ROW: SINGLE; SUPPL P/N: 6220C3; NOTE:THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 251 | 32329 | BRG BAL | EA | 25 |
| 252 | 32330 | BEARING, ASSEMBLY: TYPE: ROLLER; INSIDE DIAMETER: 31.75 MM; OUTSIDE DIAMETER: 69.01 MM; WIDTH: 19.85 MM; DRAWING NO: R304508 REV 1; R304508 POS 736 REV 1; TAPERED CONE/CUP, SINGLE ROW, PRESS BALL SET, TO CONSIST OF: 1 DEEP GROOVE 626-ZZ, 1 DEEP GROOVE 686, 1 MAGNETO E5; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 253 | 32331 | BEARING, ROLLER: TYPE: ADJUSTING; ROLLING ELEMENT: BALL; DRAWING NO: R 304508 REV 1; R 304508 POS 1230 REV 1; POS 739, FOR MULTIPLICATION RELAY, BBT | EA | 75 |
| 254 | 32332 | BEARING, ROLLER: TYPE: NEEDLE; INSIDE DIAMETER: 20 MM; OUTSIDE DIAMETER: 28 MM; WIDTH: 16 MM; ROW: SINGLE; MODEL NO: NK20-16; NOTE - SKF OR FAG OR RHP BRAND ONLY | EA | 75 |
| 255 | 32333 | BALL, BEARING: MATERIAL: STL; DIAMETER: 7 MM; SUPPL P/N: KU.7G28; SUPPLIER NOTE: THE ITEM MUST BE PROTECTIVE PACKED AND CLEARLY MARKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 256 | 32334 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 70 MM; OUTSIDE DIAMETER: 125 MM; WIDTH: 24 MM; ROW: SINGLE; SUPPL P/N: 6214-2Z; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 257 | 32335 | BEARING, ROLLER: TYPE: CUP/CONE, TAPER; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 64 MM; WIDTH: 22 MM; ROW: SINGLE; MODEL NO: M86610-M86649 | EA | 75 |
| 258 | 32386 | BEARING, BALL: TYPE: DEEP GROOVE; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 12 MM; OUTSIDE DIAMETER: 37 MM; WIDTH: 12 MM; ROW: SINGLE; SUPPL P/N: 6301-2RS; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 259 | 32466 | BEARING, ROLLER: TYPE: CUP/CONE, TAPER; INSIDE DIAMETER: 203.2 MM; WIDTH: 88.9 MM; ROW: SINGLE; SUPPL P/N: EE420801-EE421451D; ITEM 73, FOR DAVID BROWN GEARBOX. | EA | 75 |
| 260 | 32479 | BEARING, SLEEVE: TYPE: PLAIN TOP; INSIDE DIAMETER: 140 MM; OUTSIDE DIAMETER: 200 MM; LENGTH: 230 MM; MATERIAL: STL BABBITTED; SUPPL P/N: 310-2; LUVZ-250-430-1; DRAWING NO: 510 737 69 01 REV 1; REFERENCE NO: 2-151-146-348/1-2; FOR KSB BOILER CIRCULATION PUMP | EA | 75 |
| 261 | 32536 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 25.45 MM; OUTSIDE DIAMETER: 35 MM; LENGTH: 26.4 MM; MATERIAL: STL; SUPPL P/N: 5X158; MODEL NO: C30EMX4/C18X13; REFERENCE NO: 67514372; THRUST BEARING FOR CENTAC COMPRESSOR | EA | 75 |
| 262 | 32591 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 55 MM; OUTSIDE DIAMETER: 100 MM; WIDTH: 21 MM; ROW: SINGLE; SUPPL P/N: 6211-2Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 263 | 32593 | BEARING, BALL: TYPE: CONRAD; STYLE: SHIELD; INSIDE DIAMETER: 65 MM; OUTSIDE DIAMETER: 140 MM; WIDTH: 33 MM; ROW: SINGLE; SUPPL P/N: 6313-Z; NOTE: ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 264 | 32611 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 90 MM; WIDTH: 20 MM; ROW: SINGLE; REFERENCE NO: NU210/C3; ITEM TO BE ORIGINAL PACKED, NOTE - THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVE ON THE BEARING | EA | 75 |
| 265 | 32620 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SEAL; INSIDE DIAMETER: 55 MM; OUTSIDE DIAMETER: 90 MM; WIDTH: 18 MM; ROW: SINGLE; SUPPL P/N: 6011-2RSC3; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 266 | 32676 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: OPEN; INSIDE DIAMETER: 55 MM; OUTSIDE DIAMETER: 120 MM; WIDTH: 29 MM; ROW: SINGLE; SUPPL P/N: 7311B; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 267 | 32698 | BEARING, ROLLER: TYPE: CUP/CONE, TAPER; INSIDE DIAMETER: 25 MM; OUTSIDE DIAMETER: 62 MM; WIDTH: 18.25 MM; ROW: SINGLE; MODEL NO: 30305; ITEM MUST BE ORIGINAL PROTECTIVE PACKED | EA | 75 |
| 268 | 32700 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 20 MM; OUTSIDE DIAMETER: 47 MM; WIDTH: 14 MM; ROW: SINGLE; SUPPL P/N: 6204-2Z; ITEM MUST BE ORIGINAL PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 269 | 32711 | BEARING, ROLLER: TYPE: SELF ALIGNING; SPHERICAL; INSIDE DIAMETER: 150 MM; OUTSIDE DIAMETER: 270 MM; WIDTH: 73 MM; ROW: DOUBLE; SUPPL P/N: 22230CC/W33C3; NOTE - THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON; ALSO ORIGINALLY PACKED | EA | 75 |
| 270 | 32712 | BEARING, ROLLER: TYPE: SELF ALIGNING; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 110 MM; WIDTH: 40 MM; ROW: DOUBLE; SUPPL P/N: 22310CKC3; NOTE: THE REFERENCE NUMBER MUST BE CAST AND NOT ENGRAVED ON BEARING AND ORIGINAL PACKED | EA | 75 |
| 271 | 33099 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 235 MM; OUTSIDE DIAMETER: 370 MM; LENGTH: 210 MM; MATERIAL: WHITE METAL; MODEL NO: DLM3630 | EA | 75 |
| 272 | 33307 | BEARING, SLEEVE: TYPE: BUSH; DRAWING NO: HTGR 412450 REV 1; HTGR 412450 POS 7235/7236 REV 1; MODEL NO: RSK-G 630 40-400; POS 970FOR POWER ASSISTED CHECK VALVE IN HP EXHAUST | EA | 25 |
| 273 | 33310 | BEARING, SLEEVE: TYPE: BUSH; DRAWING NO: POS 16680 R 406681 REV 1; R 406681 REV 1; FOR BFPT MAIN OIL PUMP | EA | 25 |
| 274 | 33427 | BEARING, THRUST: INSIDE DIAMETER: 60 MM; OUTSIDE DIAMETER: 104 MM; WIDTH: 50 MM; MATERIAL: STL; MODEL NO: GN60; REFERENCE NO: 119-023- 061-864; FOR USE ON MECHANICAL/AUX LUB OIL PUMP ON MAAG GEARBOX , NOTE - THE ITEM MUST BE PROTECTIVE PACKED AND CLEARLY MARKED; TYPE JOURNAL | EA | 75 |
| 275 | 33499 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 8 MM; OUTSIDE DIAMETER: 11 MM; LENGTH: 8 MM; MATERIAL: BRASS; MODEL NO: B3113836C | EA | 75 |
| 276 | 33506 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 38 MM; OUTSIDE DIAMETER: 48 MM; LENGTH: 51 MM; MATERIAL: STAINLESS STEEL;DRAWING NO: K14367 REV 1; TRAVERSE REDUCTION UNIT | EA | 75 |
| 277 | 33554 | BEARING, SLEEVE: TYPE: PA FAN MOTOR; INSIDE DIAMETER: 114 MM; OUTSIDE DIAMETER: 155 MM; LENGTH: 127 MM; MATERIAL: WHITE METAL; SIZE: 4.5 IN; MOTOR JKD 400/6, 465KW, 3,3KV, LM2556/04, SUPPLIER NOTE; TO BE SUPPLY IN MATCH SETS AND PROTECTIVE PACKED; MACHINE THE BEARING TO 114,4-114,43 MM WITH A 0,10MM SHIM IN THE JOINTS. REMOVE THE SHIM AFTER MACHINING TO ACHIEVE A LEMON SHAPE BORE; MATERIAL WM90; SHAFT SPEED 1500 RPM. ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE-METALLING OF WHITE METAL BEARINGS. ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE WORK STARTS. THE SUPPLIER SHALL BE RESPONSIBLE FOR MINIMISING DISTORTION OF THE BEARING SHELL DURING THE CASTING PROCESS. THE SUPPLIER SHALL BE RESPONSIBLE FOR ENSURING THAT THE MATING FACES OF BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND. THE BEARING OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS. THE SUPPLIER SHALL ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETERIORATION DURING TRANSPORTATION, HANDLING AND STORAGE.THE SUPPLIER SHALL DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS. THE SUPPLIER SHALL RECORD ALL RELEVANT DIMENSIONS AND FEATURES OF THE BEARING ON A DRAWING TO BE SUBMITTED TOGETHER WITH THE FINISHED BEARING. INSPECTION OF THE FINAL MACHINED SIZES MUST BE DONE ON THE MACHINE AND ONCE REMOVED IT MUST BE REPEATED ON A WORK BENCH AT ROOM TEMPERATURE. ALL INSPECTION REPORTS MUST ACCOMPANY THE BEARING DATAPACK. THE SUPPLIER SHALL DETERMINE THE DEGREE OF BONDING BY MEANS OF AN | EA | 75 |
| 278 | 33595 | BEARING, SLEEVE: TYPE: ECCENTRIC BUSH; INSIDE DIAMETER: 24 MM; OUTSIDE DIAMETER: 32 MM; LENGTH: 46 MM; MATERIAL: ALUMINIUM; REFERENCE NO: D4075C; MASTER ORDER 55160 | EA | 25 |
| 279 | 33611 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 127 MM; OUTSIDE DIAMETER: 143 MM; LENGTH: 135 MM; MATERIAL: ALUMINIUM; SUPPL P/N: 1R16430; FOR INGERSOLL-RAND COMPRESSOR XLE, REPLACE BY 30181598 | EA | 25 |
| 280 | 34013 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 285 MM; OUTSIDE DIAMETER: 360 MM; LENGTH: 500 MM; MATERIAL: STL; SUPPL P/N: 378-315; SUCTION; STEEL, FOR SPARGO ASH PUMP 300X300; SIZE 500MM MAXIMUM; INSIDE DIAMETER X 490MM LONG AND RECESS 355MM OUT SIDE DIAMETER | EA | 75 |
| 281 | 34043 | BEARING, THRUST: ROLLING ELEMENT: ROLLER SPHERICAL; INSIDE DIAMETER: 20 MM; OUTSIDE DIAMETER: 27 MM; WIDTH: 10 MM; SUPPL P/N: B250; MODEL NO: 44115; FOR STOCK EQUIPMENT VOLUMETRIC COAL FEEDER, TENSION PULLEY; ROW SINGLE; TYPE SELF ALIGNING | EA | 75 |
| 282 | 34044 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 55 MM; LENGTH: 60 MM; MATERIAL: WHITE METAL; MODEL NO: GLY PG 505560 F; GLYCODUR DRY SLIDING | EA | 75 |
| 283 | 34047 | BEARING, SLEEVE: TYPE: DE PA FAN; INSIDE DIAMETER: 101.6 MM; OUTSIDE DIAMETER: 203.1 MM; LENGTH: 101.8 MM; MATERIAL: WHITE METAL; SIZE: 4 IN; FOR USE ON UNIT 1-3, SUPPLIER NOTE: THE ITEM MUST BE PROTECTIVE PACKED; MACHINE THE BEARING TO 101.602 - 101.659MM ID CYLINDICAL BORE MATERIAL WM90, SHAFT SPEED 1500RPM, ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE- METALLING OF WHITE METAL BEARINGS ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE THE WORK STARTS, THE SUPPLIER SHALL BE RESPONSIBLE OF THE FOLLOWING: MINIMISING DISTORTION OF THE BEARING SHELL DURING THE CASTING PROCESS; RESPONSIBLE FOR ENSURING THE MATING FACES OF BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND AND THE BEARING OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS; ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETERIORATION DURING TRANSPORTATION, HANDLING AND STORAGE; DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS; RECORD ALL THE RELEVANT DIMENSIONS AND FEATURES OF THE BEARING ON A DRAWING TO BE SUBMITTED TOGETHER WITH THE FINISHED BEARING, INSPECTION OF THE FINAL MACHINED SIZES MUST BE DONE ON THE MACHINE AND ONCE REMOVED IT MUST BE REPEATED ON A WORK | EA | 75 |
| 284 | 34052 | BEARING, SLEEVE: TYPE: THRUST JOURNAL; INSIDE DIAMETER: 127 MM; OUTSIDE DIAMETER: 247.6 MM; LENGTH: 128 MM; MATERIAL: WHITE METAL; SIZE: 5 IN; BUSH; PA FAN NON DRIVE END; FOR USE ON UNIT 4 TO 6, SUPPLIER NOTE: THE ITEM MUST BE PROTECTIVE PACKED; MACHINE THE BEARING TO 126,997 - 127.084MM ID, CYLINDRICAL BORE, MATERIAL WM90, SHAFT SPEED 1500RPM, ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE- METALLING OF WHITE METAL BEARINGS ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE THE WORK STARTS, THE SUPPLIER SHALL BE RESPONSIBLE OF THE FOLLOWING: MINIMISING DISTORTION OF THE BEARING SHELL DURING THE CASTING PROCESS; RESPONSIBLE FOR ENSURING THE MATING FACES OF BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND AND THE BEARING OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS; ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETORIORATION DURING TRANSPORTATION, HANDLING AND STORAGE; DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS; RECORD ALL THE RELEVANT DIMENSIONS AND FEATURES OF THE BEARING ON A DRAWING TO BE SUBMITTED TOGETHER WITH THE FINISHED BEARING, INSPECTION OF THE FINAL MACHINED SIZES MUST BE DONE ON THE MACHINE AND ONCE REMOVED IT MUST BE REPEATED ON A WORK BENCH AT ROOM TEMPERATURE, ALL INSPECTION REPORTS MUST ACCOMPANY THE BEARING DATAPACK AND THE SUPPLIER SHALL DETERMINE THE DEGREE OF BONDING BY MEANS OF AN ULTRASONIC METHOD COMPLYING WITH BS7585 PART1, OR EQUIVALENT METHOD A TOTAL BONDED AREA 95% SHALL BE ACHIEVED ALL JOINTS, EDGES AND LOADED AREAS SHALL BE 100% BONDED, NO ONE AREA OF BAD BONDING SHALL EXCEED MORE THAN | EA | 75 |
| 285 | 34054 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 267 MM; OUTSIDE DIAMETER: 317.55 MM; LENGTH: 267 MM; MATERIAL: WHITE METAL; SIZE: 10.5 IN; MODEL NO: 10-1/2; FEED BEARING, SIRROCCO FD; MACHINE THE BEARING TO 267.03 - 267.1 MM ID, CYLINDRICAL BORE, MATERIAL WM90, SHAFT SPEED 750RPM, ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE- METALLING OF WHITE METAL BEARINGS ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE THE WORK STARTS, THE SUPPLIER SHALL BE RESPONSIBLE OF THE FOLLOWING: MINIMISING DISTORTION OF THE BEARING SHELL DURING THE CASTING PROCESS; RESPONSIBLE FOR ENSURING THE MATING FACES OF BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND AND THE BEARING OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS; ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETORIORATION DURING TRANSPORTATION, HANDLING AND STORAGE; DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS; RECORD ALL THE RELEVANT DIMENSIONS AND FEATURES OF THE BEARING ON A DRAWING TO BE SUBMITTED TOGETHER WITH THE FINISHED BEARING, INSPECTION OF THE FINAL MACHINED SIZES MUST BE DONE ON THE MACHINE AND ONCE REMOVED IT MUST BE REPEATED ON A WORK BENCH AT ROOM TEMPERATURE, ALL INSPECTION REPORTS MUST ACCOMPANY THE BEARING DATAPACK AND THE SUPPLIER SHALL DETERMINE THE DEGREE OF BONDING BY MEANS OF AN ULTRASONIC METHOD COMPLYING WITH BS7585 PART1, OR EQUIVALENT METHOD A TOTAL BONDED AREA 95% SHALL BE ACHIEVED ALL JOINTS, EDGES AND LOADED AREAS SHALL BE 100% BONDED, NO ONE AREA OF BAD BONDING SHALL EXCEED MORE THAN 3% OF THE TOTAL AREA OF THE BEARING SURFACE, THE FOLLOWING | EA | 25 |
| 286 | 34055 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 330 MM; OUTSIDE DIAMETER: 380 MM; LENGTH: 285 MM; MATERIAL: WHITE METAL; FIG NO: 13; FEED SIRROCCO ID FAN | EA | 5 |
| 287 | 34056 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 200 MM; OUTSIDE DIAMETER: 408 MM; LENGTH: 210 MM; MATERIAL: STL WHITE METAL LINED; SPECIFICATION: 22/200; SUPPL P/N: EFNLB/22-200; FOR FD AND ID FAN; MOTORS WITH BEARINGS; MACHINE THE BEARING TO LEMON BORE WITH A 0.20MM SHIM IN THE JOINTS REMOVE THE SHIM AFTER MACHINING TO ACHIEVE A LEMON SHAPE BORE, MATERIAL WM90, SHAFT SPEED 600RPM, ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE-METALLING OF WHITE METAL BEARINGS ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE THE WORK STARTS, THE SUPPLIER SHALL BE RESPONSIBLE OF THE FOLLOWING: MINIMISING DISTORTION OF THE BEARING SHELL DURING THE CASTING PROCESS; RESPONSIBLE FOR ENSURING THE MATING FACES OF BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND AND THE BEARING OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS; ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETERIORATION DURING TRANSPORTATION, HANDLING AND STORAGE; DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS; RECORD ALL THE RELEVANT DIMENSIONS AND FEATURES OF THE BEARING ON A DRAWING TO BE SUBMITTED TOGETHER WITH THE FINISHED BEARING, INSPECTION OF THE FINAL MACHINED SIZES MUST BE DONE ON THE MACHINE AND ONCE REMOVED IT MUST BE REPEATED ON A WORK BENCH AT ROOM TEMPERATURE, ALL INSPECTION REPORTS MUST ACCOMPANY THE BEARING DATAPACK AND THE SUPPLIER SHALL DETERMINE THE DEGREE OF BONDING BY MEANS OF AN ULTRASONIC METHOD COMPLYING WITH BS7585 PART1, OR EQUIVALENT METHOD A TOTAL BONDED AREA 95% SHALL BE ACHIEVED ALL JOINTS, EDGES AND LOADED AREAS SHALL BE 100% BONDED, NO ONE AREA OF BAD | EA | 75 |
| 288 | 34059 | BEARING, SLEEVE: TYPE: ID FAN; INSIDE DIAMETER: 330 MM; OUTSIDE DIAMETER: 287.35 MM; LENGTH: 283 MM; MATERIAL: WHITE METAL; SIZE: 13 IN; SUPPLIER NOTE: THE ITEM MUST BE PROTECTIVE PACKED; MACHINE THE BEARING TO 330.61- 330.69MM ID, CYLINFRICAL BORE, MATERIAL WM90, SHAFT SPEED 600RPM, 13 IN SIROCCO ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE- METALLING OF WHITE METAL BEARINGS ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE THE WORK STARTS, THE SUPPLIER SHALL BE RESPONSIBLE OF THE FOLLOWING: MINIMISING DISTORTION OF THE BEARING SHELL DURING THE CASTING PROCESS; RESPONSIBLE FOR ENSURING THE MATING FACES OF BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND AND THE BEARING OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS; ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETERIORATION DURING TRANSPORTATION, HANDLING AND STORAGE; DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS; RECORD ALL THE RELEVANT DIMENSIONS AND FEATURES OF THE BEARING ON A DRAWING TO BE SUBMITTED TOGETHER WITH THE FINISHED BEARING, INSPECTION OF THE FINAL MACHINED SIZES MUST BE DONE ON THE MACHINE AND ONCE REMOVED IT MUST BE REPEATED ON A WORK BENCH AT ROOM TEMPERATURE, ALL INSPECTION REPORTS MUST ACCOMPANY THE BEARING DATAPACK AND THE SUPPLIER SHALL DETERMINE THE DEGREE OF BONDING BY MEANS OF AN ULTRASONIC METHOD COMPLYING WITH BS7585 PART1, OR EQUIVALENT METHOD A TOTAL BONDED AREA 95% SHALL BE ACHIEVED ALL JOINTS, EDGES AND LOADED AREAS SHALL BE 100% BONDED, NO ONE AREA OF BAD BONDING SHALL EXCEED MORE THAN 3% OF THE TOTAL AREA OF THE BEARING SURFACE, THE FOLLOWING | EA | 75 |
| 289 | 34061 | BEARING, SLEEVE: TYPE: MILL MOTOR; INSIDE DIAMETER: 127 MM; OUTSIDE DIAMETER: 157 MM; LENGTH: 127 MM; MATERIAL: WHITE METAL; SIZE: 5 IN; SUPPLIER NOTE: THE ITEM MUST BE PROTECTIVE PACKED AND MARKED; MACHINE THE BEARING TO 127,10-127,13MM WITH A 0,10MM SHIM IN THE JOINTS. REMOVE THE SHIM AFTER MACHINING TO ACHIEVE A LEMON SHAPE BORE; MATERIAL WM90; SHAFT SPEED 1000 RPM. ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE- METALLING OF WHITE METAL BEARINGS. ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE WORK STARTS. THE SUPPLIER SHALL BE RESPONSIBLE FOR MINIMISING DISTORTION OF THE BEARING SHELL DURING THE CASTING PROCESS. THE SUPPLIER SHALL BE RESPONSIBLE FOR ENSURING THAT THE MATING FACES OF BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND. THE BEARING OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS. THE SUPPLIER SHALL ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETERIORATION DURING TRANSPORTATION, HANDLING AND STORAGE.THE SUPPLIER SHALL DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS. THE SUPPLIER SHALL RECORD ALL RELEVANT DIMENSIONS AND FEATURES OF THE BEARING ON A DRAWING TO BE SUBMITTED TOGETHER WITH THE FINISHED BEARING. INSPECTION OF THE FINAL MACHINED SIZES MUST BE DONE ON THE MACHINE AND ONCE REMOVED IT MUST BE REPEATED ON A WORK BENCH AT ROOM TEMPERATURE. ALL INSPECTION REPORTS MUST ACCOMPANY THE BEARING DATAPACK. THE SUPPLIER SHALL DETERMINE THE DEGREE OF BONDING BY MEANS OF AN ULTRASONIC METHOD COMPLYING WITH BS 7585 PARK 1, OR EQUIVALENT METHOD. A TOTAL BONDED AREA OF 95% SHALL BE ACHIEVED. ALL JOINTS, | EA | 75 |
| 290 | 34064 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 135 MM; OUTSIDE DIAMETER: 155 MM; LENGTH: 120 MM; MATERIAL: BRASS; DRAWING NO: BBT TGD90102 POS 16 REV 1; TGD90102 REV 1; FOR TURNING GEAR 12SK | EA | 75 |
| 291 | 34065 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 135 MM; OUTSIDE DIAMETER: 155 MM; LENGTH: 120 MM; MATERIAL: BRASS; DRAWING NO: BBT TGD90102 POS 17 REV 1; TGD90102 REV 1; FOR TURNING GEAR 12SK | EA | 75 |
| 292 | 34068 | BEARING, SLEEVE: TYPE: MILL MOTOR; INSIDE DIAMETER: 91 MM; OUTSIDE DIAMETER: 121 MM; LENGTH: 80 MM; MATERIAL: WHITE METAL; REFERENCE NO: ERNLB 9-90; FOR HAWKER SIDDELY MOTOR 328 KW, SUPPLIER NOTE; THE ITEM MUST BE PROTECTIVE PACKED | EA | 75 |
| 293 | 34069 | BEARING: INSIDE DIAMETER: 110 MM; OUTSIDE DIAMETER: 221.5 MM; WIDTH: 109 MM; STYLE: OPEN; MATERIAL: WHITE METAL; MODEL NO: ERNLB 11- 100; FOR USE WITH HAWKER SIDDELY MOTOR | EA | 75 |
| 294 | 34078 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 19 MM; OUTSIDE DIAMETER: 25.5 MM; LENGTH: 32 MM; MATERIAL: BRASS; DRAWING NO: F105-223 REV 1; FIG NO: 9152; REFERENCE NO: 23114-434; A8; HANDWHEEL SHAFT , FOR HOPKINSON VALVE ACTUATOR | EA | 75 |
| 295 | 34079 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 19 MM; OUTSIDE DIAMETER: 25.5 MM; LENGTH: 16 MM; MATERIAL: BRASS; REFERENCE NO: 23114-414; ACT VALVE | EA | 75 |
| 296 | 34080 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 17 MM; OUTSIDE DIAMETER: 22 MM; LENGTH: 16 MM; MATERIAL: BRASS; REFERENCE NO: 23114-421; FOR HOPKINSON ACTUATOR | EA | 75 |
| 297 | 34081 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 25 MM; OUTSIDE DIAMETER: 70 MM; LENGTH: 107 MM; MATERIAL: STL BABBITTED; REFERENCE NO: 23113-166; FIXED CLUTCH BRISCH; REFERENCE F4; FOR HOPKINSON VALVE ACTUATOR FIGURE 9152, DRAWING NUMBER F105/223 | EA | 75 |
| 298 | 34082 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 20 MM; OUTSIDE DIAMETER: 27 MM; LENGTH: 25 MM; MATERIAL: BRASS; REFERENCE NO: C23113-365; ACT VALVE | EA | 75 |
| 299 | 34083 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 64 MM; OUTSIDE DIAMETER: 76 MM; LENGTH: 38 MM; MATERIAL: BRASS; REFERENCE NO: 23113-380; ACT VALVE | EA | 75 |
| 300 | 34084 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 9 MM; OUTSIDE DIAMETER: 13 MM; LENGTH: 14.4 MM; MATERIAL: BRASS; DRAWING NO: F105/223 REV 1; 9152 REV 1; REFERENCE NO: F16; LIMIT SWITCH TAKE OFF; FOR HOPKINSON VALVE ACTUATOR | EA | 75 |
| 301 | 34085 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 103 MM; OUTSIDE DIAMETER: 111 MM; LENGTH: 30 MM; MATERIAL: BAKELITE; REFERENCE NO: A2; C2217-401; OUTPUT SHAFT; BRISCH; FOR HOPKINSON VALVE ACTUATOR, FIGURE 9154 | EA | 75 |
| 302 | 34086 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 19 MM; OUTSIDE DIAMETER: 25.5 MM; LENGTH: 25 MM; MATERIAL: BRASS; DRAWING NO: F105/223 REV 1; FIG 9152 REV 1; REFERENCE NO: 23114-432; SPLINED | EA | 75 |
| 303 | 34087 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 17 MM; OUTSIDE DIAMETER: 22 MM; LENGTH: 22 MM; MATERIAL: BRASS; DRAWING NO: F105/223 REV 1; FIG 9152 REV 1; BEVEL SHAFT; BRISCH; REFERENCE A7, FOR HOKINSON VALVE ACTUATOR | EA | 75 |
| 304 | 34089 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 22 MM; OUTSIDE DIAMETER: 45 MM; LENGTH: 21 MM; MATERIAL: WHITE METAL; FOR 3 INCH BALL VALVE | EA | 75 |
| 305 | 34090 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 60 MM; LENGTH: 40 MM; MATERIAL: STL; DRAWING NO: BBT R 406681 POS 16681 REV 1; FOR BFPT MAIN OIL PUMP | EA | 75 |
| 306 | 34091 | BEARING, SLEEVE: TYPE: BUSH; DRAWING NO: R406681 REV 1; R406681 POS 16681A REV 1; POS 16681A, FOR BFPT MAIN OIL PUMP | EA | 25 |
| 307 | 34092 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 44.5 MM; OUTSIDE DIAMETER: 84 MM; LENGTH: 88 MM; MATERIAL: WHITE METAL; ST 44,4-146- 83 | EA | 75 |
| 308 | 34093 | BEARING, SLEEVE: TYPE: BUSH; DRAWING NO: HTGD 90237 POS 17 REV 1; HTGD 90237 REV 1; MODEL NO: 6SK; POS 17, FOR BFPT TURNING GEAR TYPE | EA | 75 |
| 309 | 34095 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 83 MM; OUTSIDE DIAMETER: 117 MM; LENGTH: 27 MM; MATERIAL: STL; DRAWING NO: BBT HTGD90237 POS 7 REV 1; HTGD90237 REV 1; FRONT, FOR BFPT TURNING GEAR TYPE:- 6SK | EA | 75 |
| 310 | 34096 | BEARING, SLEEVE: TYPE: BUSH; DRAWING NO: HTGD90237 REV 1; HTGD90237 POS 6 REV 1; MODEL NO: 6SK; REAR, FOR BFPT TURNING GEAR | EA | 75 |
| 311 | 34097 | BEARING, SLEEVE: TYPE: BUSH; DRAWING NO: HTGD 90237 POS 16 REV 1; HTGD 90237 REV 1; MODEL NO: 6SK; FOR BFPT TURNING GEAR | EA | 75 |
| 312 | 34099 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 92 MM; OUTSIDE DIAMETER: 122 MM; LENGTH: 110 MM; MATERIAL: STL; DRAWING NO: TGR408553 POS 561 REV 1; TGR410127 REV 1; TGR410127 POS 561 REV 1; TGR408553 REV 1; MODEL NO: 1211;FOR DRIVING GEAR ASSEMBLY AND GEARBOX , TO BBT | EA | 75 |
| 313 | 34102 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 49 MM; OUTSIDE DIAMETER: 90 MM; LENGTH: 48 MM; MATERIAL: STL; CLASIER VALVE | EA | 75 |
| 314 | 34182 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 80 MM; OUTSIDE DIAMETER: 105 MM; LENGTH: 75 MM; MATERIAL: BRZ; SUPPL P/N: 310-1; MODEL NO: LUVE 200-415-1; FOR CIRCULATING PUMP | EA | 75 |
| 315 | 34253 | BEARING, BALL: TYPE: CONRAD; STYLE: DOUBLE SHIELD; INSIDE DIAMETER: 42.86 MM; OUTSIDE DIAMETER: 85 MM; WIDTH: 49.21 MM; ROW: SINGLE; SUPPL P/N: B251; ER27T; SEALMASTER, SIZE 1-11/16 INCH, FOR VOLUMETRIC COAL FEEDER MODEL 44115, BABCOCK; ITEM MUST BE ORIGINAL(PROTECTIVE) PACKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 316 | 34261 | BEARING, ROLLER: TYPE: NEEDLE; INSIDE DIAMETER: 38.1 MM; OUTSIDE DIAMETER: 47.63 MM; WIDTH: 12.7 MM; ROW: SINGLE; SUPPL P/N: B248; DRAWING NO: L-A14418-A REV 1; COMPOUND GEAR, ITEM 25, FOR VOLUMETRIC COAL FEEDER 44115, DESIGN 1969, ITEM MUST BE PROTECTIVE PACKED | EA | 75 |
| 317 | 34587 | BEARING, PILLOW BLOCK: INSIDE DIAMETER: 112 MM; BOLT MOUNTING: 2; BOLT DIAMETER: 26 MM; BLOCK SIZE: WD 122 X LG 410 X HT 260 MM; SHAFT HEIGHT: 140 MM; HOUSING MATERIAL: CI; SUPPL P/N: SN524; DUTY: MEDIUM; BARREL | EA | 75 |
| 318 | 34589 | Bearing, Pillow block:inside diameter:130mm;bolt mounting:2; bolt diameter:M34;block size:wd 140 x LG 500 x HT 290 MM; Shaft height:150 MM;housing material:CI;Suppl P/N:SN528;Barrel | EA | 75 |
| 319 | 34590 | BEARING, PILLOW BLOCK: INSIDE DIAMETER: 202 MM; BOLT MOUNTING: 4; BOLT DIAMETER: M36; BLOCK SIZE: WD 130 X LG 540 X HT 345 MM; SHAFT HEIGHT: 170 MM; HOUSING MATERIAL: CI; SUPPL P/N: SN532; FOR SHAFT 140MM DIAMETER; BARREL | EA | 75 |
| 320 | 34593 | BEARING, FLANGE BLOCK: INSIDE DIAMETER: 40 MM; SHAPE: SQ; BLOCK SIZE: LG 130 X THK 35 MM; BOLT MOUNTING: 4; BOLT DIAMETER: M13; HOUSING MATERIAL: CI; SUPPL P/N: SF40; CYLINDRICAL, WITH GREASE NIPPLE AND SECURING GRUB SCREW, NOTE - ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 321 | 34606 | BEARING, PILLOW BLOCK: INSIDE DIAMETER: 38.1 MM; BOLT MOUNTING: 2; BOLT DIAMETER: 17 MM; BLOCK SIZE: WD 57.5 X LG 220 X HT 117 MM; SHAFT HEIGHT: 40 MM; ROLLING ELEMENT: ROLLER CYLINDRICAL; HOUSING MATERIAL: CI; REFERENCE NO: MP24; FOR 1-1/2 INCH SHAFT, NOTE: ITEM MUST BE ORIGINAL(PROTECTIVE) PACKED | EA | 75 |
| 322 | 34618 | BEARING, PILLOW BLOCK: INSIDE DIAMETER: 176 MM; BOLT MOUNTING: 2; BOLT DIAMETER: 35 MM; BLOCK SIZE: WD 150 X LG 505 X HT 294 MM; SHAFT HEIGHT: 150 MM; ROLLING ELEMENT: SPHERICAL; HOUSING MATERIAL: STL; SUPPL P/N: SNA528; PLUMMER BLOCK, WITHOUT SEALS; DUTY: HEAVY | EA | 75 |
| 323 | 34942 | BEARING, PILLOW BLOCK: INSIDE DIAMETER: 30 MM; BOLT MOUNTING: 2; BOLT DIAMETER: 13 MM; BLOCK SIZE: WD 39 X LG 154 X HT 80 MM; SHAFT HEIGHT: 32 MM; ROLLING ELEMENT: ROLLER CYLINDRICAL; HOUSING MATERIAL: CI; MODEL NO: SY30TF; COMPLETE; FOR 30MM SHAFT, NOTE; ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 324 | 34945 | BEARING, PILLOW BLOCK: INSIDE DIAMETER: 25 MM; BOLT MOUNTING: 2; BOLT DIAMETER: M12; BLOCK SIZE: WD 35 X LG 132 X HT 70 MM; SHAFT HEIGHT: 24 MM; ROLLING ELEMENT: ROLLER CYLINDRICAL; HOUSING MATERIAL: CI; MODEL NO: SY25TF; SHAFT SIZE 25MM, NOTE: ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 325 | 34953 | #N/A | EA | 25 |
| 326 | 34997 | BEARING, FLANGE BLOCK: INSIDE DIAMETER: 25 MM; SHAPE: SQ; BLOCK SIZE: LG 95 X THK 30 MM; BOLT MOUNTING: 4; BOLT DIAMETER: M12; HOUSING MATERIAL: CI; SUPPL P/N: UCF205-16; CYLINDRICAL, WITH GREASE NIPPLE AND SECURING GRUB SCREWS, ITEM MUST BE ORIGINAL PACKED | EA | 75 |
| 327 | 39252 | BEARING, PILLOW BLOCK: INSIDE DIAMETER: 114.4 MM; BOLT MOUNTING: 4; BOLT DIAMETER: M20; BLOCK SIZE: WD 120 X LG 405 X HT 245 MM; SHAFT HEIGHT: 125 MM; ROLLING ELEMENT: ROLLER; HOUSING MATERIAL: DI; SUPPL P/N: PB900X4.1/2; SPHERICAL, ONLY BROWING PLUMMER ACCEPTABLE, HEAVY DUTY, FOR USE ON ASH CRUSHER, NOTE - THE ITEM MUST BE SUPPLIED WITH A BUTTON TYPE GREASE NIPPLE INSTALLED | EA | 75 |
| 328 | 40550 | BEARING, SLEEVE: TYPE: PA FAN NDE; INSIDE DIAMETER: 101.6 MM; OUTSIDE DIAMETER: 203.1 MM; LENGTH: 111.7 MM; MATERIAL: WHITE METAL; SIZE: 4 IN; FOR UNIT 1 TO 3, SUPPLIER NOTE; THE ITEM MUST BE PROTECTIVE PACKED; MACHINE THE BEARING TO 101.602 - 101.659MM ID, CYLINDRICAL BORE, MATERIAL WM90, SHAFT SPEED 1500RPM, ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE-METALLING OF WHITE METAL BEARINGS ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE THE WORK STARTS, THE SUPPLIER SHALL BE RESPONSIBLE OF THE FOLLOWING: MINIMISING DISTORTION OF THE BEARING SHELL DURING THE CASTING PROCESS; RESPONSIBLE FOR ENSURING THE MATING FACES OF BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND AND THE BEARING OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS; ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETERIORATION DURING TRANSPORTATION, HANDLING AND STORAGE; DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS; RECORD ALL THE RELEVANT DIMENSIONS AND | EA | 75 |
| 329 | 40560 | BEARING, ROD END: INSIDE DIAMETER: 20 MM; OUTSIDE DIAMETER: 54 MM; WIDTH: 16 MM; SHANK DIAMETER: 28 MM; HOUSING MATERIAL: STL; REFERENCE NO: GIL17UK; THREADED, O/A LENGTH 90.0MM, BEARING ID 17.0MM X 14.0MM WIDE, CENTRE TO 67.0MM, HEAD WIDTH 12.0MM X 46.0MM OD, L/H THREADED TO M16 X 2.0MM PITCH X 24.0MM DEEP; THREADS 2MM; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 330 | 40563 | BEARING, ROD END: INSIDE DIAMETER: 17 MM; OUTSIDE DIAMETER: 46 MM; WIDTH: 14 MM; THREAD: 2 MM; SHANK DIAMETER: 24 MM; HOUSING MATERIAL: STL; REFERENCE NO: GIR17D0; THREADED, O/A LENGTH = 90MM, CENTRE TO ROD END = 67MM, HEAD WIDTH = 12MM X 46MM OD, R/H THREADED TO M16 X 2MM PITCH X 24MM DEEP, FOR F.D. FANS; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE | EA | 75 |
| 331 | 44959 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 48 MM; OUTSIDE DIAMETER: 62 MM; LENGTH: 75 MM; MATERIAL: ALUMINIUM; SUPPL P/N: 2- 123-010-454-05; FOR Q34 STD STEAM JACKETED SULPHUR PUMP, FOR SO3 PLANT | EA | 75 |
| 332 | 45331 | BEARING: TYPE: CARTRIDGE; INSIDE DIAMETER: 250 MM; OUTSIDE DIAMETER: 300 MM; SUPPL P/N: 19210- 740005; FOR COARSE ASH PUMP TYPE DENVER ORION | EA | 75 |
| 333 | 45508 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 32 MM; OUTSIDE DIAMETER: 50 MM; LENGTH: 38 MM; MATERIAL: STL; DRAWING NO: HTGR 305916/178 REV 1; FOR BFPT EMERGENCY LUBE OIL PUMP, M8 | EA | 75 |
| 334 | 47951 | BEARING: TYPE: HVB; DRAWING NO: 6252 REV 1; MODEL NO: 416-31; TYPE HVB, FOR SPARGO ASH CRUSHER HUBSIN, BIN NUMBER 416-31 | EA | 75 |
| 335 | 47977 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 135 MM; OUTSIDE DIAMETER: 158 MM; LENGTH: 260 MM; MATERIAL: STL; SUPPL P/N: 529.1; MODEL NO: 200-415-1; FOR KSB PUMP TYPE LUV | EA | 75 |
| 336 | 47978 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 135 MM; OUTSIDE DIAMETER: 160 MM; LENGTH: 260 MM; MATERIAL: STL; SUPPL P/N: 529-2; MODEL NO: 200-415-1; FOR PUMP TYPE LUV | EA | 75 |
| 337 | 48019 | PAD, BEARING: MATERIAL: STL; DRAWING NO: SULZER 0-104-608-249 REV 1; REFERENCE NO: HZB 302-580; THRUST; ASSEMBLY COMPLETE WITH RETAING RING; FOR 100PCT BOILER FEED PUMP BOOSTER PUMP; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 338 | 48124 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 275 MM; OUTSIDE DIAMETER: 385 MM; LENGTH: 160 MM; MATERIAL: WHITE METAL; OEM P/N: HTGD015008R0003, OEM: GE STEAM POWER SERVICE; DRAWING NO: HTGD90191/1 REV 1; HTGD90191/2 REV 1; INNER SHELL (UH/LH), FOR THREE WEDGE BODY FOR TURNING GEAR DIA 280 MM NO 1 | EA | 75 |
| 339 | 48157 | BEARING, ROLLER: TYPE: CYLINDRICAL; INSIDE DIAMETER: 27 MM; OUTSIDE DIAMETER: 40 MM; WIDTH: 21 MM; ROW: SINGLE; SUPPL P/N: 21074; DRAWING NO: TRG 305551 POS 74 REV 1; TRG 305551 REV 1; MODEL NO: AVES 315-125; REFERENCE NO: INA NATR 17-02-W9; TYPE:CONTACT ROLLER; FOR HP,IP AND LP BYPASS CONTROL VALVE SERVOMOTORS,BBT | EA | 75 |
| 340 | 48169 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 47.5 MM; OUTSIDE DIAMETER: 65 MM; LENGTH: 22.2 MM; MATERIAL: STL; DRAWING NO: HTGR 305950 POS 563 REV 1; HTGR 305950 REV 1; FOR SPEED GOVERNOR 1201, ON MAIN AND BFP TURBINES | EA | 75 |
| 341 | 48174 | PAD, BEARING: TYPE: LOWER; MATERIAL: STL; DRAWING NO: BBT HTGD90253 POS 23 REV 1; REFERENCE NO: 23HTGD90253; SLIDING; FOR THRUST BEARING PEDESTAL; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 342 | 48177 | BEARING, SLEEVE: TYPE: BUSH; DRAWING NO: TGR001854 REV 1; TGR 001855 REV 1; SHAFT NUT, POS 175, FOR THE FOLLOWING PUMPS:- 1) AUXILLIARY LUB OIL, TYPE: M40 12,5, 2) AUXILLIARY LUB OIL, TYPE: M20 12,5, 3) EMERGENCY LUB OIL, TYPE:- M20 11 | EA | 75 |
| 343 | 48180 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 85 MM; OUTSIDE DIAMETER: 110 MM; LENGTH: 72 MM; MATERIAL: STL; DRAWING NO: TGR305600/20650 REV 1; FOR MAIN OIL DUPLEX GEAR PUMP | EA | 75 |
| 344 | 49171 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 57 MM; OUTSIDE DIAMETER: 77 MM; LENGTH: 102 MM; MATERIAL: STAINLESS STEEL; SUPPL P/N: 335-037; DRAWING NO: 4781 REV 1; FOR SHAFT LINER AT LOWER STUFFING BOX ON SPARGO DRAIN PUMP STE/3V | EA | 75 |
| 345 | 50748 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 250 MM; OUTSIDE DIAMETER: 325 MM; MATERIAL: WHITE METAL; SUPPL P/N: BS2613-1970; SET 2 HALVES FOR DRIVE END AND NON DRIVE END ON EFP MOTOR TYPE, SIZE IK145125; 250MM SHAFT, NOTE - SUPPLY IN MATCH SETS AND SEALED FOR DAMAGES; STYLE OPEN; WIDTH 200MM | EA | 75 |
| 346 | 52055 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 9 MM; OUTSIDE DIAMETER: 14 MM; LENGTH: 19 MM; MATERIAL: BRASS; SUPPL P/N: BU13651B; DRAWING NO: K15396 REV 1; REFERENCE NO: BU13651B; GUNBLOWER GEARBOX SCREWED SPINDLE | EA | 25 |
| 347 | 53430 | BEARING, SLEEVE: TYPE: SPLIT BUSH; INSIDE DIAMETER: 60 MM; OUTSIDE DIAMETER: 100 MM; LENGTH: 111.5 MM; MODEL NO: E1-941B; FOR PRECIPITATOR | EA | 75 |
| 348 | 53438 | BEARING, SLEEVE: TYPE: SPLIT BUSH; INSIDE DIAMETER: 40 MM; OUTSIDE DIAMETER: 80 MM; LENGTH: 115 MM; MATERIAL: STL; DRAWING NO: 13.45/481 REV 1; FOR BRANDT PRECIPITATOR | EA | 75 |
| 349 | 73581 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 48 MM; OUTSIDE DIAMETER: 90 MM; LENGTH: 55 MM; MATERIAL: STL; DRAWING NO: BBT R 405720 POS 7204 REV 1; POS 108; FOR POWER ASSISTED CHECK VALVE IN EXTRACTION 4, TYPE:- 500L/KK 12,5/400, NW 600 | EA | 75 |
| 350 | 73588 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 30 MM; OUTSIDE DIAMETER: 55 MM; LENGTH: 35 MM; MATERIAL: STL; DRAWING NO: BBT R 405720 POS 7207/7208 REV 1; POS 108; FOR POWER ASSISTED CHECK VALVE IN EXTRACTION 5, TYPE:- 315L/RK 12,5/400 NW 400, SCHEMATIC POS | EA | 75 |
| 351 | 73597 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 38 MM; OUTSIDE DIAMETER: 60 MM; LENGTH: 40 MM; MATERIAL: STL; DRAWING NO: BBT R 405720 POS 7213/7214 REV 1; FOR POWER ASSISTED CHECK VALVE IN EXTRACTION 6, TYPE:- 250 RK 40/500 NW 300, BBT POS 108 | EA | 75 |
| 352 | 73606 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 75 MM; LENGTH: 50 MM; MATERIAL: STL; DRAWING NO: BBT R 405720 POS 7231/7232 REV 1; POS 108; FOR POWER ASSISTED CHECK VALVE IN EXTRACTION 6 TO BFPT, TYPE:- 315 RK40/400 NW400 | EA | 75 |
| 353 | 74064 | BALL, BEARING: MATERIAL: STL; DIAMETER: 40 MM; SUPPL P/N: SB40MM; ONLY SKF; RHP OR FAG BRANDS ACCEPTABLE AT KRIEL POWER STATION; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 354 | 74150 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 40 MM; OUTSIDE DIAMETER: 70 MM; MATERIAL: STL; SUPPL P/N: 112-2-46; MODEL NO: ARS112; GUIDE, FOR SULZER COMBINED BY-PASS AND RELIEF VALVE | EA | 75 |
| 355 | 74321 | PAD, BEARING: DIMENSIONS: WD 50 X LG 20 X THK 2 MM; MATERIAL: STL; REFERENCE NO: 15 ACT 14A GMY; 15ACT14AGMY; THRUST; VENDORS ARE RESPONSIBLE FOR ENSURING | EA | 75 |
| 356 | 74329 | PAD, BEARING: DIMENSIONS: WD 165 X LG 6 MM; MATERIAL: STL; WAIT INFO - MMD 1/9/00; THRUST 15 ACT 30A GMY; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 357 | 74392 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 45 MM; OUTSIDE DIAMETER: 73 MM; LENGTH: 69 MM; MATERIAL: STL; DRAWING NO: HTGR 001855 REV 1; HTGR 001855 POS 178 REV 1; HTGR 001854 POS 178 REV 1; HTGR 001854 REV 1; FOR THE FOLLOWING OIL PUMPS:- 1. AUXILLIARY LUB OIL TYPE:- M40 12.5, 2. AUXILLIARY LUB OIL TYPE:- M20 12,5, 3. EMERGENCY LUB OIL TYPE:- M20 11 | EA | 75 |
| 358 | 113803 | BEARING, SLEEVE: TYPE: BUSH; INSIDE DIAMETER: 40 MM; OUTSIDE DIAMETER: 65 MM; LENGTH: 25.5 MM; MATERIAL: STL; RING NB CLASS C | EA | 75 |
| 359 | 143547 | BRG PILW BLCK:2;M22;STL | EA | 25 |
| 360 | 177108 | BEARING, SLEEVE: TYPE: TURBINE NO 7; INSIDE DIAMETER: 220 MM; OUTSIDE DIAMETER: 340 MM; LENGTH: 200 MM; MATERIAL: WHITE METAL; COMPLETE TOP AND BOTTOM, POS 1 AND 2 TO BBT DRAWING, SUPPLIER NOTE: THE ITEM MUST BE PROTECTIVE PACKED | EA | 10 |
| 361 | 177135 | BEARING, SLEEVE: TYPE: CW PUMP MOTOR; INSIDE DIAMETER: 280 MM; OUTSIDE DIAMETER: 480 MM; LENGTH: 140 MM; MATERIAL: WHITE METAL; DRAWING NO: CG 79315 REV 1; LOWER GUIDE , PART NO:- 2, FOR MITSUBISHI 3 PHASE INDUCTION MOTOR TYPE MKB-R, 330 RPM, 1492 KW, 330V, FRAME 9000, SUPPLIER NOTE: THE ITEM MUST BE PROTECTIVE PACKED | EA | 75 |
| 362 | 177136 | BEARING, SLEEVE: TYPE: JOURNAL THRUST; INSIDE DIAMETER: 215 MM; OUTSIDE DIAMETER: 255 MM; LENGTH: 160 MM; MATERIAL: STL; COMBINATION, FOR BOILER FEED PUMP | EA | 75 |
| 363 | 212684 | BEARING, BALL: TYPE: ANGULAR CONTACT; STYLE: OPEN; INSIDE DIAMETER: 50 MM; OUTSIDE DIAMETER: 90 MM; WIDTH: 20 MM; ROW: SINGLE; SUPPL P/N: 7210BECB; NOTE: THE ITEM MUST BE PROTECTIVE PACKED AND CLEARLY MARKED; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 364 | 219366 | BEARING, PILLOW BLOCK: INSIDE DIAMETER: 4-1/2 IN; BOLT MOUNTING: 4; BOLT DIAMETER: M20; BLOCK SIZE: WD 220 X LG 419.1 X HT 238.1 MM; SHAFT HEIGHT: 120.65 MM; ROLLING ELEMENT: SPHERICAL; HOUSING MATERIAL: CI; SUPPL P/N: AMEP5408YF | EA | 75 |
| 365 | 221273 | BEARING, PILLOW BLOCK: INSIDE DIAMETER: 50 MM; BOLT MOUNTING: 2; BOLT DIAMETER: M16; BLOCK SIZE: WD 50 X LG 100 X HT 80 MM; SHAFT HEIGHT: 50 MM; ROLLING ELEMENT: SPHERICAL; HOUSING MATERIAL: CI; SUPPL P/N: 222 11 EK; DRAWING NO: 2163 REV 1; FOR USE IN MILLING PLANT; DUTY: HEAVY | EA | 75 |
| 366 | 226359 | PAD, BEARING: DIMENSIONS: WD 152 X LG 60 X HT 44 MM; MATERIAL: STL; SUPPL P/N: 583-6-0-03; DRAWING NO: 28.45-54321 REV 1; PRECIPITOR; NEW PRECIPS; FOR POTHEMUHLE; FIXED BEARING; FOR DISCHARGE FRAME RAPPING SYSTEM; FOR UNIT 1, 2, 3 AND 6; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 367 | 226360 | PAD, BEARING: DIMENSIONS: WD 60 X LG 80 X THK 32 MM; MATERIAL: MS; SUPPL P/N: 583-6-0-02; DRAWING NO: 28.45-54321 REV 1; NEW PRECIPITOR; PADS FOR ROTHEMUHLE; FREE BEARING; FOR DISCHARGE FRAME RAPPING SYSTEMS; FOR UNIT 1, 2, 3 AND 6; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 368 | 227594 | BEARING, SLEEVE: TYPE: JOURNAL; MATERIAL: WHITE METAL; SIZE: 10 IN; STYLE OPEN, HEAVY DUTY, FOR FD FAN, RUNNER DE AND NDE, ONE 2 HALVES, SUPPLY IN MATCH SETS AND SEALED FOR DAMAGES, BUBBLE WRAP | EA | 75 |
| 369 | 230238 | BEARING: TYPE: SPLIT; INSIDE DIAMETER: 150 MM; STYLE: DOUBLE SHIELD; MATERIAL: STAINLESS STEEL; SUPPL P/N: 150-01-B; A SERIES 01, TYPE COOPER, MUST BE SUITABLE FOR CONDITIONERS, WITH SHAFT | EA | 75 |
| 370 | 231600 | BEARING: TYPE: PLAIN; MATERIAL: WHITE METAL; SUPPL P/N: 949 659 69; 3 LOBE, SHAPE CYLINDRICAL, FOR THE CENTAC COMPRESSOR; SIZE 45MM DIA X 60MM | EA | 75 |
| 371 | 232762 | BEARING, SLEEVE: TYPE: JOURNAL; MATERIAL: WHITE METAL; DRAWING NO: GMD 0262301 R1 REV 1; TURBINE, SIZE 250MM, STYLE MEDIUM DUTY; STILTING PAD RADIAL, FOR SLIP RING SHAFT | EA | 75 |
| 372 | 236514 | BEARING: TYPE: CLASSIFIER BLADE BOTTOM; INSIDE DIAMETER: 75 MM; OUTSIDE DIAMETER: 130 MM; WIDTH: 31 MM; STYLE: OPEN; MATERIAL: STL; SUPPL P/N: DIB C 3659 007; MEDIUM DUTY; SCOOP, PLUMBER BLOCK, VOLTAGE RATINGS: 183 KN-DYN, 236 KN - STAT, USED ON THE EWAX BEARINGS, FOR INCLINED COAL CONVEYORS FROM 6A TO 6G | EA | 75 |
| 373 | 251383 | BEARING, SLEEVE: TYPE: FD FAN MOTOR; INSIDE DIAMETER: 225 MM; OUTSIDE DIAMETER: 350 MM; LENGTH: 200 MM; MATERIAL: WHITE METAL; DRAWING NO: HSR22/EARP225/SS250.SB250/I REV 0; FOR USE ON 3050 KW MOTOR (SERIAL NO:DO 5027/01), NDE/DE; SIZE: 22 MM; MACHINE THE BEARING TO LEMON BORE WITH 0.02MM SHIM IN THE JOINTS; REMOVE THE SHIM AFTER MACHINING TO ACHIEVE A LEMON SHAPE BORE, MATERIAL WM90, SHAFT SPEED 750RPM APPLICATION ACTOM FD FAN MOTOR, ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE-METALLING, ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE THE WORK STARTS, THE SUPPLIER SHALL BE RESPONSIBLE FOR THE FOLLOWING: MINIMISING DISTORTION OF THE BEARING SHELL DURING CASTING PROCESS; ENSURING THAT THE MATING FACES OF THE BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND ALSO THE OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS; ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETERIORATION DURING TRANSPORTATION, HANDLING AND STORAGE; DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS; RECORD ALL THE RELEVANT DIMENSIONS AND FEATURES OF THE BEARING ON A DRAWING TO BE SUBMITTED TOGETHER WITH THE FINISHED BEARING, INSPECTION OF THE FINAL MACHINED SIZES MUST BE DONE ON THE MACHINE AND ONCE REMOVED IT MUST BE REPEATED ON A WORK BENCH AT ROOM TEMPERATURE, ALL INSPECTION REPORTS MUST ACCOMPANY THE BEARING DATAPACK, THE SUPPLIER SHALL DETERMINE THE DEGREE OF BONDING BY MEANS OF AN ULTRASONIC METHOD COMPLYING WITH BS7585 PART1, OR EQUIVALENT METHOD A TOTAL BONDED AREA 95% SHALL BE ACHIEVED ALL JOINTS, EDGES AND LOADED AREAS SHALL BE | EA | 75 |
| 374 | 251384 | BEARING, SLEEVE: TYPE: PA FAN MOTOR; INSIDE DIAMETER: 100 MM; MATERIAL: WHITE METAL; REFERENCE NO: BOO 5031/01 AND 02; FOR USE ON 650 KW MOTOR; UNIT 4 TO 6; NDE/DE; SIZE: 14 MM | EA | 75 |
| 375 | 251385 | BEARING, SLEEVE: INSIDE DIAMETER: 250 MM; MATERIAL: WHITE METAL; REFERENCE NO: DO 5032; FOR USE ON ELECTRIC PUMP MOTOR; NDE/DE; SIZE: 22 MM | EA | 75 |
| 376 | 251386 | BEARING, SLEEVE: TYPE: PA FAN MOTOR; INSIDE DIAMETER: 110 MM; MATERIAL: WHITE METAL; REFERENCE NO: BOO 5030/01 AND 02; FOR USE ON 465 KW MOTOR; UNIT 1TO 3; NDE/DE; SIZE: 14 MM | EA | 75 |
| 377 | 251387 | BEARING, SLEEVE: TYPE: MILL MOTOR; INSIDE DIAMETER: 140 MM; MATERIAL: WHITE METAL; REFERENCE NO: BOO 5 029/01,02 AND 03; FOR USE ON 570 KW MOTOR; UNIT 4 TO 6, NDE/DE; SIZE: 14 MM | EA | 75 |
| 378 | 251388 | BEARING, SLEEVE: TYPE: MILL MOTOR; INSIDE DIAMETER: 110 MM; OUTSIDE DIAMETER: 240 MM; LENGTH: 100 MM; MATERIAL: WHITE METAL; DRAWING NO: 0.45/58901 REV 0; REFERENCE NO: BOO 5 028/01, 02 AND 03; FOR USE ON 328 KW MOTOR; UNIT 1 TO 3; NDE/DE; SIZE: 11 MM; MACHINE THE BEARING TO 110.14 WITH 0.10 MM SHIM IN THE JOINTS REMOVE THE SHIM AFTER MACHINING TO ACHIEVE A LEMON SHAPE BORE MATERIAL WM90, OLM NUMBER HRS11/ 1ARP110/SS125/SB125 SHAFT SPEED 1000 RPM, ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE-METALLING OF WHITE METAL BEARINGS ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE THE WORK STARTS, THE SUPPLIER SHALL BE RESPONSIBLE OF THE FOLLOWING: MINIMISING DISTORTION OF THE BEARING SHELL DURING THE CASTING PROCESS; RESPONSIBLE FOR ENSURING THE MATING FACES OF BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND AND THE BEARING OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS; ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETERIORATION DURING TRANSPORTATION, HANDLING AND STORAGE; DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS; RECORD ALL THE RELEVANT DIMENSIONS AND FEATURES OF THE BEARING ON A DRAWING TO BE SUBMITTED TOGETHER WITH THE FINISHED BEARING, INSPECTION OF THE FINAL MACHINED SIZES MUST BE DONE ON THE MACHINE AND ONCE REMOVED IT MUST BE REPEATED ON A WORK BENCH AT ROOM TEMPERATURE, ALL INSPECTION REPORTS MUST ACCOMPANY THE BEARING DATAPACK AND THE SUPPLIER SHALL DETERMINE THE DEGREE OF BONDING BY MEANS OF AN ULTRASONIC METHOD COMPLYING WITH BS7585 PART1, OR EQUIVALENT METHOD A TOTAL BONDED AREA 95% SHALL BE ACHIEVED ALL JOINTS, EDGES AND | EA | 75 |
| 379 | 252681 | BEARING, THRUST: ROLLING ELEMENT: BALL; OUTSIDE DIAMETER: 190 MM; DRAWING NO: 046051/54 REV 1; DEEP GROOVE, FOR USE ON WELLAND AND TEXHORN CONTROL VALVE, PN 4 00 DN 250 | EA | 75 |
| 380 | 565476 | BEARING, SLEEVE: TYPE: EFP MOTOR; INSIDE DIAMETER: 250 MM; OUTSIDE DIAMETER: 428.84 MM; LENGTH: 205 MM; MATERIAL: GR 420/12; MATERIAL: BS2786; TYPE: EXTERNAL FEED OIL; SPECIFICATION: HSR22 BEARING; SPARE BEARINGS FOR EFP MOTORS | EA | 75 |
| 381 | 576015 | BEARING, PILLOW BLOCK: INSIDE DIAMETER: 180 MM; BOLT MOUNTING: 4; BOLT DIAMETER: M30; BLOCK SIZE: WD 230 X LG 610 X HT 400 MM; ROLLING ELEMENT: BALL; HOUSING MATERIAL: CI; MANUF P/N: SD-3140; SUPPLIED WITH LABYRINTH SEALS | EA | 75 |
| 382 | 588609 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 100 MM; OUTSIDE DIAMETER: 209 MM; LENGTH: 80 MM; MATERIAL: WHITE METAL; SIZE: 4 IN; MANUF P/N: HSR11/EARP100/SS125/SB140/I; DRAWING NO: 0.45/58904 REV 0; BP11608 REV 0; APPLICATION: USED ON THE HORIZONTAL BEARING ASSEMBLY ON NEW U4 - 6 PA FAN MOTOR; MACHINE THE BEARING TO 100.08 - 100.14MM WITH A 0.10MM SHIM IN THE JOINTS REMOVE THE SHIM AFTER MACHINING TO ACHIEVE A LEMON SHAPE BORE, MATERIAL WM90, SHAFT SPEED 1500RPM, ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE- METALLING OF WHITE METAL BEARINGS ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE THE WORK STARTS, THE SUPPLIER SHALL BE RESPONSIBLE OF THE FOLLOWING: MINIMISING DISTORTION OF THE BEARING SHELL DURING THE CASTING PROCESS; RESPONSIBLE FOR ENSURING THE MATING FACES OF BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND AND THE BEARING OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS; ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETERIORATION DURING TRANSPORTATION, HANDLING AND STORAGE; DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS; RECORD ALL THE RELEVANT DIMENSIONS AND FEATURES OF THE BEARING ON A DRAWING TO BE SUBMITTED TOGETHER WITH THE FINISHED BEARING, INSPECTION OF THE FINAL MACHINED SIZES MUST BE DONE ON THE MACHINE AND ONCE REMOVED IT MUST BE REPEATED ON A WORK BENCH AT ROOM TEMPERATURE, ALL INSPECTION REPORTS MUST ACCOMPANY THE BEARING DATAPACK AND THE SUPPLIER SHALL DETERMINE THE DEGREE OF BONDING BY MEANS OF AN ULTRASONIC METHOD COMPLYING WITH BS7585 PART1, OR EQUIVALENT METHOD A TOTAL | EA | 75 |
| 383 | 588610 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 140 MM; OUTSIDE DIAMETER: 274 MM; LENGTH: 105 MM; MATERIAL: WHITE METAL; MANUF P/N: HRS14/EARP140/SS160/SB140/I; DRAWING NO: BP11611 REV 0; APPLICATION: USED ON THE HORIZONTAL BEARING ASSEMBLY ON NEW U4-6 MILL MOTOR; MACHINE THE BEARING TO 140,15-140,22MM WITH A 0,10MM SHIM IN THE JOINTS. REMOVE THE SHIM AFTER MACHINING TO ACHIEVE A LEMON SHAPE BORE; MATERIAL WM90; SHAFT SPEED 1000 RPM. ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE-METALLING OF WHITE METAL BEARINGS. ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE WORK STARTS. THE SUPPLIER SHALL BE RESPONSIBLE FOR MINIMISING DISTORTION OF THE BEARING SHELL DURING THE CASTING PROCESS. THE SUPPLIER SHALL BE RESPONSIBLE FOR ENSURING THAT THE MATING FACES OF BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND. THE BEARING OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS. THE SUPPLIER SHALL ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETERIORATION DURING TRANSPORTATION, HANDLING AND STORAGE.THE SUPPLIER SHALL DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS. THE SUPPLIER SHALL RECORD ALL RELEVANT DIMENSIONS AND FEATURES OF THE BEARING ON A DRAWING TO BE SUBMITTED TOGETHER WITH THE FINISHED BEARING. INSPECTION OF THE FINAL MACHINED SIZES MUST BE DONE ON THE MACHINE AND ONCE REMOVED IT MUST BE REPEATED ON A WORK BENCH AT ROOM TEMPERATURE. ALL INSPECTION REPORTS MUST ACCOMPANY THE BEARING DATAPACK. THE SUPPLIER SHALL DETERMINE THE DEGREE OF BONDING BY MEANS OF AN ULTRASONIC METHOD COMPLYING WITH BS 7585 PARK 1, OR EQUIVALENT | EA | 75 |
| 384 | 588611 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 110 MM; OUTSIDE DIAMETER: 209 MM; LENGTH: 80 MM; MATERIAL: WHITE METAL; MANUF P/N: BP11306; DRAWING NO: HSR11/EARP110/SS125/SB/25/I REV 0; APPLICATION: USED ON THE HORIZONTAL BEARING ASSEMBLY ON NEW U1-3 MILL MOTOR | EA | 75 |
| 385 | 588612 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 110 MM; OUTSIDE DIAMETER: 209 MM; LENGTH: 80 MM; MATERIAL: WHITE METAL; MANUF P/N: HSR11/EARP110/SS125/SB125/I; DRAWING NO: 0.45/58903 REV 0; BP8832 REV 0; USED ON THE HORIZONTAL BEARING ASSEMBLY ON NEW U1-3 PA FAN MOTOR; MACHINE THE BEARING TO 110.08- 110.14MM WITH A 0.10MM SHIM IN THE JOINTS REMOVE THE SHIM AFTER MACHINING TO ACHIEVE A LEMON SHAPE BORE, MATERIAL WM90, SHAFT SPEED 1500RPM, ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE-METALLING OF WHITE METAL BEARINGS ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE THE WORK STARTS, THE SUPPLIER SHALL BE RESPONSIBLE OF THE FOLLOWING: MINIMISING DISTORTION OF THE BEARING SHELL DURING THE CASTING PROCESS; RESPONSIBLE FOR ENSURING THE MATING FACES OF BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND AND THE BEARING OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS; ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETERIORATION DURING TRANSPORTATION, HANDLING AND STORAGE; DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS; RECORD ALL THE RELEVANT DIMENSIONS AND FEATURES OF THE BEARING ON A DRAWING TO BE SUBMITTED TOGETHER WITH THE FINISHED BEARING, INSPECTION OF THE FINAL MACHINED SIZES MUST BE DONE ON THE MACHINE AND ONCE REMOVED IT MUST BE REPEATED ON A WORK BENCH AT ROOM TEMPERATURE, ALL INSPECTION REPORTS MUST ACCOMPANY THE BEARING DATAPACK AND THE SUPPLIER SHALL DETERMINE THE DEGREE OF BONDING BY MEANS OF AN ULTRASONIC METHOD COMPLYING WITH BS7585 PART1, OR EQUIVALENT METHOD A TOTAL BONDED AREA 95% SHALL BE ACHIEVED ALL JOINTS, EDGES AND | EA | 75 |
| 386 | 603205 | BEARING, SLEEVE: TYPE: END FLANGE MOUNTED UNIT; INSIDE DIAMETER: 250 MM; OUTSIDE DIAMETER: 315 MM; LENGTH: 360 MM; MATERIAL: STL; SPECIFICATION: HSR28/EKRP250SS315/I; DRAWING NO: BP11612/BP11613 REV 0; FRAME 28 | EA | 75 |
| 387 | 622701 | BRG:R304482P0005;RADIAL;ID 90 MM;OD 110 | EA | 75 |
| 388 | 624429 | BEARING, ROLLER: TYPE: SPHERICAL; INSIDE DIAMETER: 35 MM; OUTSIDE DIAMETER: 72 MM; WIDTH: 23 MM; ROW: DOUBLE; BORE SHAPE: TAPER 1:12; STYLE: OPEN; CAGE MATERIAL: STL; ROLLING ELEMENT: ROLLERS; SUPPL P/N: 22207EK | EA | 75 |
| 389 | 649018 | BEARING, ROLLER: TYPE: SPHERICAL; INSIDE DIAMETER: 190 MM; OUTSIDE DIAMETER: 290 MM; WIDTH: 75 MM; ROW: DOUBLE; BORE SHAPE: TAPER; CAGE MATERIAL: STL; SUPPL P/N: 23038CAMKE4C3; IBI SS489 | EA | 75 |
| 390 | 659634 | BEARING, PLAIN: TYPE: SPHERICAL; INSIDE DIAMETER: 55 MM; OUTSIDE DIMENSION: 120 MM; WIDTH: 28 MM; | EA | 75 |
| 391 | 659635 | BEARING, BALL: TYPE: ANGULAR; STYLE: CLOSED; INSIDE DIAMETER: 56 MM; OUTSIDE DIAMETER: 124 MM; WIDTH: 28 MM; ROW: DOUBLE; CAGE MATERIAL: CS; SUPPL P/N: AAP1408226-07214; TA3000 CENTRIFUGAL COMPRESSOR BULLGEAR; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 392 | 659699 | BEARING, FLANGE BLOCK: ROLLING ELEMENT: CYLINDRICAL; INSIDE DIAMETER: 45 MM; SHAPE: SQ FLANGED; BLOCK SIZE: LG 137 MM; BOLT MOUNTING: 4; BOLT DIAMETER: M38; HOUSING MATERIAL: CAST IRON; PCD: 105MM; FIT CONVENTIONAL CYLINDRICAL SHAFT; SET SCREWS PREVENT SHAFT SLIPPAGE; ITEM SHOULD BE ON A PROTECTIVE PACKAGE AND CLEARLY MARKED | EA | 75 |
| 393 | 659701 | BEARING, FLANGE BLOCK: ROLLING ELEMENT: CYLINDRICAL; INSIDE DIAMETER: 75 MM; SHAPE: SQ FLANGED; BLOCK SIZE: LG 200 MM; BOLT MOUNTING: 4; BOLT DIAMETER: M40; HOUSING MATERIAL: CAST IRON; STATIC LOAD RATING: 48.3KN; DYNAMIC LOAD RATING: 67.3KN; FIT CONVENTIONAL CYLINDRICAL SHAFT; SET SCREWS PREVENT SHAFT SLIPPAGE; ITEM SHOULD BE ON A PROTECTIVE PACKAGE AND CLEARLY MARKED | EA | 75 |
| 394 | 662371 | BEARING, SLEEVE: TYPE: PLAIN JOURNAL; INSIDE DIAMETER: 95.3 MM; OUTSIDE DIAMETER: 160 MM; LENGTH: 95 MM; MATERIAL: STL 316L; SUPPL P/N: AAA3791334-00016 | EA | 75 |
| 395 | 662372 | BEARING: TYPE: PINNION SHAFT STAG 2; INSIDE DIAMETER: 38.1 MM; OUTSIDE DIAMETER: 95.25 MM; WIDTH: 50.8 MM; MATERIAL: BRONZE; SUPPL P/N: AAA3408544-03802 | EA | 75 |
| 396 | 662386 | BEARING: TYPE: PINNION SHAFT STAG 1; INSIDE DIAMETER: 47.625 MM; OUTSIDE DIAMETER: 95.25 MM; WIDTH: 50.8 MM; MATERIAL: BRONZE; SUPPL P/N: AAA3408544-03801 | EA | 75 |
| 397 | 662388 | BEARING, SLEEVE: TYPE: THRUST JOURNAL; INSIDE DIAMETER: 95.8 MM; OUTSIDE DIAMETER: 160 MM; LENGTH: | EA | 75 |
| 398 | 663958 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 265 MM; OUTSIDE DIAMETER: 295 MM; LENGTH: 1.2 M; MATERIAL: WHITE METAL; MODEL NO: 10-1/2; TYPE: FEED BEARING, SIRROCCO FD | EA | 75 |
| 399 | 664335 | BEARING, ROLLER: TYPE: SPHERICAL; INSIDE DIAMETER: 55 MM; OUTSIDE DIAMETER: 120 MM; WIDTH: 28 MM; ROW: SINGLE; CAGE MATERIAL: CS; SPECIFICATION: TA3000; SUPPL P/N: APP1408226-06311 | EA | 75 |
| 400 | 664338 | BEARING, ROLLER: TYPE: PATENTED; INSIDE DIAMETER: 56 MM; OUTSIDE DIAMETER: 124 MM; WIDTH: 28 MM; ROW: SINGLE; CAGE MATERIAL: CS; ROLLING ELEMENT: CYLINDRICAL; SPECIFICATION: TA3000; SUPPL P/N: AAP1408226-07214 | EA | 75 |
| 401 | 664380 | BEARING: TYPE: BULLGEAR; INSIDE DIAMETER: 60 MM; OUTSIDE DIAMETER: 700 MM; WIDTH: 50 MM; MATERIAL: STL HARDENED; SPECIFICATION: TA3000; SUPPL P/N: AAA3793822-03255; TEETH: 298 | EA | 75 |
| 402 | 693091 | BEARING, PILLOW BLOCK: INSIDE DIAMETER: 90 MM; BOLT MOUNTING: 2; BOLT DIAMETER: 37 MM; BLOCK SIZE: 340 MM; SHAFT HEIGHT: 112 MM; ROLLING ELEMENT: TAPER CYLINDRICAL; POLYMER RING CAGE RINGS; CARBURIZED INNER RING; INTER CHANGABLE SEALS | EA | 75 |
| 403 | 693110 | BEARING, ASSEMBLY: TYPE: GUIDE SEGMENT; OEM P/N: HTGD118479R0001, OEM: GE STEAM POWER SERVICE; DRAWING NO: HTGD90200 REV C; FIG NO: 9-3; VENDORS ARE RESPONSIBLE FOR ENSURING THAT THEY ARE PERFORMING AGAINST THE CORRECT DRAWING REVISION NUMBER (IF APPLICABLE). | EA | 75 |
| 404 | 694224 | BEARING, SLEEVE: TYPE: JOURNAL; INSIDE DIAMETER: 200 MM; OUTSIDE DIAMETER: 350 MM; LENGTH: 200 MM; MATERIAL: STL WHITE METAL LINES; USED FOR FD FANS; MACHINE THE BEARING TO LEMON BORE WITH A 0.20MM SHIM IN THE JOINTS REMOVE THE SHIM AFTER MACHINING TO ACHIEVE A LEMON SHAPE BORE SHAFT SPEED 750RPM, ALL WORK MUST BE DONE IN LINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE- METALLING OF WHITE METAL BEARINGS ALL QCP'S MUST BE APPROVED BY ESKOM BEFORE THE WORK STARTS, THE SUPPLIER SHALL BE RESPONSIBLE OF THE FOLLOWING: MINIMISING DISTORTION OF THE BEARING SHELL DURING THE CASTING PROCESS; RESPONSIBLE FOR ENSURING THE MATING FACES OF BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND AND THE BEARING OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATIONS; ENSURE THAT THE BEARING IS PROPERLY PACKAGED TO PREVENT DAMAGE OR DETERIORATION DURING TRANSPORTATION, HANDLING AND STORAGE; DETERMINE THE METHOD OF MACHINING TO ACHIEVE AN EVEN SURFACE FINISH FREE FROM WAVINESS, HIGH SPOTS AND SURFACE DEFECTS; RECORD ALL THE RELEVANT DIMENSIONS AND FEATURES OF THE BEARING ON A DRAWING TO BE SUBMITTED TOGETHER WITH THE FINISHED BEARING, INSPECTION OF THE FINAL MACHINED SIZES MUST BE DONE ON THE MACHINE AND ONCE REMOVED IT MUST BE REPEATED ON A WORK BENCH AT ROOM TEMPERATURE, ALL INSPECTION REPORTS MUST ACCOMPANY THE BEARING DATAPACK AND THE SUPPLIER SHALL DETERMINE THE DEGREE OF BONDING BY MEANS OF AN ULTRASONIC METHOD COMPLYING WITH BS7585 PART1, OR EQUIVALENT METHOD A TOTAL BONDED AREA 95% SHALL BE ACHIEVED ALL JOINTS, EDGES AND LOADED AREAS SHALL BE 100% BONDED, NO ONE AREA OF BAD BONDING SHALL EXCEED MORE THAN 3% OF THE TOTAL AREA OF THE | EA | 75 |
| 405 | 714637 | BEARING, SLEEVE: TYPE: EXTERNAL OIL FEED SLIDE NON-LOCATING; INSIDE DIAMETER: 250 MM; OUTSIDE DIAMETER: 380 MM; LENGTH: 200 MM; MATERIAL: WHITE METAL; SPECIFICATION: ISO11687-3 DIN31694; SIZE: 22-250 MM; OEM P/N: EMZLQ 22- 250; CAT NO: RH-1046; MACHINE THE BEARING TO LEMON BORE WITH 0.02MM SHIM IN THE JOINTS, REMOVE THE SHIM AFTER MACHINE TO ACHIEVE THE LEMON SHAPE BORE MATERIAL WM90; SHAFT SPEED: 1500RPM; APPLICATION: WEG EFP MOTOR, ALL WORK MUST BE DONE INLINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE-METALLING, ALL QCP'S TO BE APPROVED BY ESKOM BEFORE THE WORK STARTS, THE SUPPLIER SHALL BE RESPONSIBLE FOR THE FOLLOWING: MINIMISING DISTORTION OF THE BEARNG, SHELL DURING CASTING PROCESS, ENSURE THAT THE MATING FACES OF THE BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND, THE OUTSIDE | EA | 75 |
| 406 | 714646 | BEARING, SLEEVE: TYPE: EXTERNAL OIL FEED SLIDE; INSIDE DIAMETER: 250 MM; OUTSIDE DIAMETER: 380 MM; LENGTH: 200 MM; MATERIAL: WHITE METAL; SPECIFICATION: ISO11687-3 DIN31694; SIZE: 22-250 MM; OEM P/N: EMZLB 22-250; CAT NO: RH- 1046; MACHINE THE BEARING TO LEMON BORE WITH 0.02MM SHIM IN THE JOINTS, REMOVE THE SHIM AFTER MACHINE TO ACHIEVE THE LEMON SHAPE BORE MATERIAL WM90; SHAFT SPEED: 1500RPM; APPLICATION: WEG EFP MOTOR, ALL WORK MUST BE DONE INLINE WITH GGSS0526 ESKOM STANDARD SPECIFICATION FOR THE RE-METALLING, ALL QCP'S TO BE APPROVED BY ESKOM BEFORE THE WORK STARTS, THE SUPPLIER SHALL BE RESPONSIBLE FOR THE FOLLOWING: MINIMISING DISTORTION OF THE BEARNG, SHELL DURING CASTING PROCESS, ENSURE THAT THE MATING FACES OF THE BEARING HALVES ARE MATCHED AND THAT THE INTERNAL CIRCUMFERENCE OF THE BEARING IS ROUND, THE OUTSIDE DIAMETER MUST BE CHECKED FOR CONFORMANCE TO ORIGINAL SPECIFICATION | EA | 75 |

## Purchaser’s design

Not applicable to this contract

## Procedure for submission and acceptance of Supplier’s design

- For All material with a special code that require AIA intervention, The Supplier submits the Project quality (PQP/ Contract quality plan) plan for approval prior to commencement with any manufacturing or fabrication

- All goods or material are designed and manufactured in accordance with an accepted design standard and as per heading 2 the long item description table above

- All goods or material technical documents are accepted by the Employer prior to commencement with  manufacturing or fabrication

- All goods supplied shall include material certification  in line with the design code and standard

- The Supplier issues the data pack/ sheet for all goods supplied

- The data book of goods supplied includes but not limited NDT results, welding procedures etc.

- All goods are delivered with relevant material certificate in line with the PQP/ Contract quality plan and metallurgical relevant tests

## Other requirements of the Supplier’s design

Not applicable to this contract

## Use of Supplier’s design

The Purchaser is allowed to use the designs, drawings and documents for the purpose of verifying goods supplied and the fulfilment of PQP/ CQP/QCP

## Manufacture & fabrication

As per heading 2.2 above

## Factory acceptance testing (FAT)

As per heading 2.2 above

## Other tests and inspections and commissioning in place of use

Not applicable to this contract

## Operating manuals and maintenance schedules

Material data sheets should include storage procedure as well as maintenance philosophies and to be submitted upon delivery of all goods.

# Supply Requirements

- Supplier to deliver goods in line with lead times prescribed on Part C2 (Data by the Purchaser) upon receipt of order

- All purchased goods must be delivered at Kriel Power Station stores.

- The Supplier to ensures the goods are delivered during working hours and reasonable time must be allocated for gate access, off-loading, paperwork processing. Working Hours for Kriel Power Station are as follows: Monday to Thursday – 07h00 to 16h15 and Friday 07h00 to 12h00.

- Where deliveries are to be made outside the working ours, the Supplier need to confirm the availability of Kriel Power Station Stores Receiving personnel before dispatching the items

- The Supplier to be prepared and always ready to provide goods within an acceptable delivery period when urgently required.

- Supply and deliver order according to specification as per the SOW and handle defective material and premature failure of components.

- Perform quality verification.

- Provide onsite and offsite material storage procedure as per the equipment manufacturer requirements.

- Perform safe stock handling and transportation of critical components.

- Provide technical support to the Employer on delivered filters as per the SOW in liaising with respective equipment manufacturer.

- Transport goods and provide technical support

- All goods are packaged such that  inadvertent mechanical damage to components is not possible

- Damaged goods due to the fault of the Supplier are corrected at Supplier’s cost

- All intervention points and data books are supplied for all goods as signed off in line with PQP/ Contract quality plan and QCPs

Mandatory requirements

- The Supplier will                   always supply the Material with  a minimum of a 3.1 Material Certificate

- Material certs are delivered with the material at receiving

- The Supplier complies with the Technical evaluation strategy requirements through the duration of the contract ref no 555-EBP2072

- The Supplier will always obtain a signed declaration form from manufacturer, confirming that all the requirements of material preservation specifications according to any EN Code (e.g., EN 10204:2004) and/or local accreditation body will be met.

- Quality certification that the Supplier has been audited and authorized having a quality assurance system for material manufacture in accordance an ISO standard to produce/Sell the material grades and dimension ranges tendered for.

- Both the ISO 9001 Valid Certification and Supplier’s Signed QMS

# Specification of the services to be provided

As per Table on heading 2

# Constraints on how the Supplier Provides the Goods

## Programming constraints

- The Supplier submits a program to the Supply Manager as per the agreed lead times during negotiations which covers all milestones of the manufacturing process up to delivery to site.

- The Supplier shares  the program two (2) days after placement of each order  to the Supply Manager for acceptance

- The early warning as per clause 16.1 is used to communicate the changes on agreed delivery schedule/ programme

| goods and services | delivery date |
| --- | --- |
| Non-OEM bearings | 5 working days after purchase order issuance |

## Work to be done by the Delivery Date

The agreed delivery date of the goods is the date after all the Quality Control Processes and correction of defects.

## Marking the goods

No need to mark good except that the supplier must submit material data sheets for all goods supplied

## Constraints at the delivery place and place of use

- All deliveries should be arranged to be within Kriel Power Station normal working hours as stipulated in section 3 of this document.

- Kriel Power Station site access will be adhered to. The Supplier informs the Purchaser timeously to make all necessary arrangement.

## Cooperating with Others

- The Supplier cooperates with the stores personnel during delivery.

- The Supplier cooperates with the Purchasers team in ensuring that the goods are delivered in accordance with all requirements.

## Services & other things to be provided by the Purchaser or Supplier

Clause 23.2 requires that the Goods Information state what services and other things are to be provided by the Parties, each to the other as part of the supply process. This has nothing to do with the services identified in the Contract Data. It would include temporary services and things needed to facilitate the supply, such as cranes provided by the Purchaser for offloading of the goods.

## Management meetings

Regular meetings of a general nature may be convened and chaired by the Supply Manager as follows:

| Title and purpose | Approximate time & interval | Location | Attendance by: |
| --- | --- | --- | --- |
| Overall contract progress and feedback | Quarterly or as and when urgent need arises | either face to face or virtual at a convenient venue | Purchaser, Supplier, and Cross functional team members |
| Contract risk reduction or Early warning meting | When the need arises | TBC | Purchaser, Supplier, and CFT |
| Contractor partnership SHEQ meeting | Monthly | TBC | Statutory meeting, Supplier to attend |

Meetings of a specialist nature may be convened as specified elsewhere in this Goods Information or if not so specified by persons and at times and locations to suit the Parties, the nature and the progress of the manufacture of the goods.  Records of these meetings shall be submitted to the Supply Manager by the person convening the meeting within five days of the meeting.

All meetings shall be recorded using minutes or a register prepared and circulated by the person who convened the meeting.   Such minutes or register shall not be used for the purpose of confirming actions or instructions under the contract as these shall be done separately by the person identified in the conditions of contract to carry out such actions or instructions.

## Documentation control

All contractual communications will be in the form of properly compiled letters or forms attached to e-mails and not as a message in the email itself.

The manufacturing data books shall be submitted in hardcopy and soft copy format. The following documentation shall form part of the manufacturing data book:

- Material Certificates 3.1 certification

- Manufacturing QCP’s (All fully signed) All goods are accepted and signed off by Employer’s QC

- Storage procedures

- Material data sheets

## Health and safety risk management

- The Supplier complies with Section 10 of Occupational Health and Safety Act (Act No.85 of 1993) when manufacturing any goods for this contract.

- The Supplier shall comply with site health and safety requirements for Kriel Power Station when delivering goods.

- Site delivery safety requirements to be adhered to - And can be obtained through the Purchaser’s Manager In line with the SHE specification

- The Supplier submits safety file for approval before access is granted

- The Supplier maintains the safety file validity in line with site requirements

- The mode of transport for delivery should comply with site requirement in line with issued SHE specification.

- Supplier to update their safety file yearly and each time they make changes on their staff or personnel

- The Supplier ensures that when changing a team member, the new team gets inducted and all process of safety file approval is adhered to- before gaining access to site

The Supplier shall comply with the health and safety requirements contained in Annexure A of the Invitation to Tender

## Environmental constraints and management

The Supplier shall comply with the environmental criteria and constraints when doing deliveries at Kriel Power Station premises regarding:

- The vehicle used for delivery; by ensuring that there are no oil spillages, and the vehicle emission is not emitting beyond limits.

- The material used for supporting the goods being delivered are correctly disposed and are without harm to environment. The Supplier must comply with site Environmental management plan (EMP) and other requirement.

- The Supplier complies with Environmental aspect and impact register.

- The Supplier complies with all site’s environmental management procedures, especially the waste management and oil spillages

The Supplier shall comply with the environmental criteria and constraints contained in Annexure B of the Invitation to Tender

## Quality

- The Supplier compiles the baseline Quality Control plan

- Quality Control Plan (QCP) must be approved by both the Suppliers, Eskom engineer and QC inspector before commencing with any deliveries at the beginning of the contract

- Material certificates, mechanical testing certificate and NDT certificates should be part of the data package accompanying the QCPs.

- All delivered goods will be subjected to QC verification by an Employer’s Quality personnel at receiving at per detailed item description found in heading 2 table

- The Supplier must comply with Eskom’s Supply Quality Management Specification (Document No: 240-105658000) and ISO 9001:2015 requirements.

- The Employer will on frequent basis conduct the Supplier’s performance/ compliance in accordance to QM 58 and ISO 9001 and NCR will be raised for non-compliance

The Supplier shall comply with the quality criteria and constraints contained in Annexure C of the Invitation to Tender

## Invoicing and payment

List the information that is to be shown by the Supplier on his invoices.  State any other requirements relating to payment.   For example:

Within one week of receiving a payment certificate from the Supply Manager in terms of core clause 51.1, the Supplier provides the Purchaser with a tax invoice showing the amount due for payment equal to that stated in the Supply Manager’s certificate.

The Supplier shall address the tax invoice to Purchaser and include on each invoice the following information:

- Name and address of the Supplier and the Supply Manager;

- The contract number and title;

- Supplier’s VAT registration number;

- The Purchaser’s VAT registration number.

- Description of goods and services provided for each item invoiced based on the Price Schedule;

- Total amount invoiced excluding VAT, the VAT and the invoiced amount including VAT;

- (add other as required)

Add procedures for invoice submission and payment (e. g. electronic payment instructions)

## Insurance provided by the Purchaser

Not applicable to this contract

## Contract change management

The change management process to be followed as per the core clause 16.1. Any verbal instruction/communication must be backed with a written instruction; the use of minutes, letters or emails is accepted. Any communication must go through the purchaser’s manager

## Provision of bonds and guarantees

Not Applicable to this contract

## & assessments of compensation events to be kept by the Supplier

Early warning to be given by any of the parties as soon either becomes aware of matters that could increase the total of total prices, delay completion etc. All the compensation events will be implemented through the raising of an early warning. See NEC Core clause 16.1 and 63.1 and 63.2

# Procurement

## Subcontracting

### Preferred subcontractors

Tenderers shall subcontract a minimum of 10% of the contract value to the following designated groups:

- An EME or QSE which is at least 51% owned by black people.

- An EME or QSE which is at least 51% owned by black people who are youth.

- An EME or QSE which is at least 51% owned by black people who are women.

- An EME or QSE which is at least 51% owned by black people with disabilities.

- An EME or QSE which is 51% owned by black people living in rural or underdeveloped area or townships.

- A cooperative which is at least 51% owned by black people.

- A EME or QSE which is at least 51% owned by black people who are military veterans

The Supplier shall comply with the SD&L criteria contained in Annexure D of the Invitation to Tender

### Limitations on subcontracting

Awaiting Thomas’s input

### Spares and consumables

Not Applicable to this Contract

### Other requirements related to procurement

The CSI target for this contract is 3% per invoice paid

### Cataloguing requirements by the Supplier

### Not Applicable to this Contract (All items are stock items)

# List of drawings

## Drawings issued by the Purchaser

This is the list of drawings issued by the Purchaser at or before the Contract Date and which apply to this contract.

| Drawing number | Revision | Title |
| --- | --- | --- |

C3.2	Supplier’s Goods Information

This section of the Goods Information will always be contract specific depending on the nature of the goods and services. It is most likely to be required for supply contracts where the tendering supplier will have proposed specifications and schedules for the goods and services, which once accepted by the Purchaser prior to award of contract now become obligations of the Supplier per core clause 20.1.

This section could also be compiled as a separate file.