SPECIFICATIONS

Maintenance to Nursery, net house structure, at ARC, Addo.

Size of structure – (40m x 32m x ±3.5m)

Scope of work to be done;

Structure;

- Plant and secure inside and outside poles.

- Plant and secure anchors.

- Putting up cable to secure structure and level poles.

- Securing of cable with grips.

Netting;

- Covering of pole heads.

- Threading of net on steel wire.

- Pulling up roof and securing wire and net.

- Putting up side net and securing net.

Material needed;

| DESCRIPTION | AMOUNT |
| --- | --- |
| Outside poles, Creosote poles, 4.2 meter (125x150mm) | 22 |
| Inside poles, Creosote poles, 3.6 meter (100 125mm) | 22 |
| Stayrods, galvanised, 1.5m x 20mm | 18 |
| Concrete anchors 690mm (diameter) x 150mm (Middle thickness) | 18 |
| Oval steel wire 2.31mm x 1500 meter | 1 |
| Grip VDWG 240 (150 x 175mm) PLP GRIPS | 76 |
| Grip VBT 240 (100 x 125mm) PLP GRIPS | 54 |
| Shade net, 3m wide (40% black) | 100 meters |
| Hog rings HR 23 (10 000 pieces) | 10 000 pieces |