- Technical Specification: Flight Data Management System

Overall System Description.

The proposed system by the vendor should meet as a bare minimum the following requirements:

- Flight Data processing from RAW QAR source data in 32-bit ARINC 717/ 429 format with Standard Airbus data frame and SAA customized Data frame structure.

- Processed data Automatic validation.

- Access via standard internet web browsers for multiple users with Appropriate Permissions

- Ability to allow for data querying on processed data without reprocessing.

- Dashboards configurable to SAA requirements.

- Open architecture database for integration with the Skywise Airbus platform and IQSMS SMS data repository.

- Analysis module

Analysis tool with flight segmentation by Date, Time, Geographical location with relevant airport GPS coordinates

- Aircraft Type / Variant, Tail Reg, Flight Number, Sector, Flight Phase detection.

- Customizable Event definitions for development and refinement

- Automatic Event Detection

- Event validation by end user

- Event alerting

- Graphical and Tabular flight representation with selectable parameters for analysis

- Event set customization aligned with SAA SPIs

- Module interface with Analysis tools e.g., Google Maps with accurate geolocation

- Animation Module

- 3 D cockpit and aircraft configuration for event analysis

- Accurate Google Maps based Terrain and Airport rendering with regular AIP based updates.

- Customizable playback speed

- Integrated meteorological reports from available Met data sources on the 3D rendering.

- Reporting Module

- Event classification customizable to SAA SPIs

- Integration with IQSMS for automatic event matching with crew reports

- Ability to query data base for specific flight patterns and integration with the Animation module, Google Maps or Data processing software e.g., EXCEL.

- Automatic SPI tracking and reporting

- Automatic weekly, monthly, quarterly, and annual reports.

END.