1. GPS x 2

| Specifications Display type | Bright, transflective 65k color TFT, dual-orientation touchscreen; sunlight readable |
| --- | --- |
| Battery | Rechargeable lithium-ion (included) or 3 AA batteries (not included); NiMH or Lithium recommended |
| Battery life | Up to 16 hours (lithium-ion); up to 22 hours (AA batteries) or better |
| Memory/history | 2.7 GB or more |
| Interface | High-speed USB and NMEA 0183 compatible |

2. Soil moisture (TDR equipment)x3

Specifications

• Soil moisture rods :12cm

• Data logger

• Waterproof and rugged probe

• Stores reading with location id

3. Draminski twistGrainPro moisture meters x4

Specifications

 Measurement resolution: humidity – 0.1%, temperature – 0.1°C/°F

 Humidity measurement accuracy:± 0.5 % for normalized grain

± 1 % in the range up to 10 % of humidity

± 1.5 % in the range over 10 % of humidity and can increase if the sample’s humidity increases.

 Temperature measurement accuracy: ± 0.5°C in the range from 0°C to 85°C / ± 0.9°F in the range from 32°F to 185°F

 Temperature measurement range: from 0°C to 125°C

4. Chlorophyll meters x 2

Specifications

 Portable non-invasive measurements of chlorophyll content

 Repeatability: Within ±0.3 SPAD unit

 Built-in data logger   Hard carrying case and extra batteries

 Interface with any GPS receiver

 GPS cables   USB to 9 pin serial adapter

 Validated chlorophyll measurements by industry standards

 Device software interface for data download

 Device should record more than 4 000 measurements

 Water proof