MINIMUM REQUIREMENTS AND SPECIFICATION

SUPPLY, DELIVERY, INSTALLATION, COMMISSIONING AND MAINTENANCE OF ACCESS CONTROL SOFTWARE AND SYSTEM

- PURPOSE

  - The purpose of this document is to provide specifications regarding supply, delivery, installation, commissioning and maintenance of the latest access control software and system at the Department of Employment and Labour.

  - The system should monitor and record movement in and around the building through walkthrough metal detector, x-ray machine, turnstiles and readers.

  - The successful bidder shall provide once-off system and/or equipment and maintain for additional thirty-six (36) months commencing from expiry of initial manufacturer or standard warranty, guarantee or maintenance period of not less than twelve (12) months.

- SCOPE OF WORK

| No | Office | Botshabelo | Kroonstad | Bethlehem |
| --- | --- | --- | --- | --- |
| 2.1. | Walkthrough Metal Detector | 01 | 01 | 01 |
| 2.2. | X-Ray Inspection Unit | 01 | 01 | 01 |
| 2.3. | Turnstiles and Perimeter | 01 | 01 | 01 |
| 2.4. | Biometric and Card Readers | 02 | 04 | 04 |

- TECHNICAL SPECIFICATIONS

| No | Item Description | Compliance Yes/No | State Details of Offer |
| --- | --- | --- | --- |
| Technical |  |  |  |
| 3.1. | Wireless system |  |  |
| 3.2. | Walkthrough metal detector |  |  |
| 3.3. | X-Ray Inspection Unit |  |  |
| 3.4. | Turnstiles and perimeter |  |  |
| 3.5. | Biometric (fingerprint) and card readers |  |  |
| 3.6. | Live viewing monitor and CPU |  |  |
| 3.6. | 01 KVA uninterrupted power supply (UPS) |  |  |
| 3.7. | Retrievable data, stored for at least 06 months |  |  |
| 3.8. | User enrolment system |  |  |
| 3.9. | Emergency override system on computer |  |  |
| General |  |  |  |
| 3.10. | Design layout, install partition at the entry area |  |  |
| 3.11. | Establish enrolment stations |  |  |
| 3.12. | Experience in South Africa |  |  |
| 3.13. | Provide training from commencement of commissioning, training costs shall be included on the bid quotation. |  |  |
| 3.14. | Provide duly marked coloured brouchers (no internet copies) |  |  |
| 3.15. | Install original and new parts only |  |  |
| Warranty and Maintenance |  |  |  |
| 3.16. | Latest date technology – state date of initial manufacture of the model range offered. Technology suitable for Department of Employment and Labour |  |  |
| 3.17. | warranty must include repair or replacement on faulty equipment and part for two years |  |  |
| 3.18. | Repair and installation must be done on site, and information ICT section must be involved |  |  |
| 3.19. | Warranty will commence after formal acceptance and handover of the equipment |  |  |
| 3.20. | All total costs of warranty for two years, spares, repairs and replacement, call out, travel costs and other extra operational costs must be included in the quotation. There will be no extra costs that will be paid within the duration of warranty period |  |  |
| 3.21. | Support response on call out time should be within 24 hours on site |  |  |
| 3.22. | Spare parts must be available within three working days – state how that will be achieved, this must be indicated on your project execution plan |  |  |
| 3.23. | Normal service hours are from at least 08:00 to 17:00 during week days |  |  |
| 3.24. | Warranty must include services performed on all the equipment in bid documents |  |  |
| 3.25. | Bidders to state lifespan and end of support date of the equipment offered |  |  |
| 3.26. | All services and repairs should be done on the client’s premises. Repairs not being able to be carried out on site can be done at the service providers workshop as per the arrangement with clients at the cost of the service provider |  |  |
| 3.27. | Provide to technically supervision during testing, calibration and commissioning of the system. Final inspection and acceptance: During final inspection, a factory-trained representative of the installer shall demonstrate the system features, performance and operation. The bidder shall be expected to hand over the fully functional security system to Department of Employment and Labour Security Management or delegated personnel. The Bidder shall be expected to provide the warranty and guaranty in case of new equipment supplied and installed at DEL as well as a comprehensive Maintenance (Preventative and Emergency) Plan after the expiry of the warranty period. The system warranty of 12 months is required on all newly installed equipment’s. |  |  |
| 3.28. | Comply will all the building and electrical requirements. The certificate of compliance (COC) of the technician must be provided with the bid. |  |  |

- ADDITIONAL TECHNICAL SPECIFICATIONS: X-RAY INSPECTION UNIT

| No | Item Description | Compliance Yes/No | State Details of Offer |
| --- | --- | --- | --- |
| General |  |  |  |
| - 4.1. | A licence for the X-Ray machine in terms of the Hazardous Substances Act (Act 15 of 1973), including the ID numbers and SABS BIN numbers of the service technicians registered to carry out the servicing of the X-Ray machines in accordance with the requirements of the SABS must be submitted with the tender, failing which the tender will not be considered. In addition to complying with the specification the X-Ray Inspection unit shall meet the requirements of the SA Police Security Advisory Board, Pretoria |  |  |
| - 3.2. | The –ray inspection unit shall be complete with; - Dual Energy Detector System (Multi Energy Imaging); - Color monitor (Remotely operated) - Conveyor belt - Screening for full profile of inspection tunnel - Discharge roller trays |  |  |
| Construction |  |  |  |
| - 3.4. | The unit must incorporate a facility to be controlled either from the right of left hand side |  |  |
|  | In addition a facility must be incorporated so that the operation keyboard and TV monitor can be operated remotely (+- 5m) from the unit |  |  |
|  | Maximum height including the tunnel must be approximately 730mm |  |  |
|  | The unit must be quiet when operating |  |  |
| Name and telephone number of the tenderers contact person to make arrangements with; Name: _______________________________________________________ Tel No: _______________________________________________________ |  |  |  |

- ADDITIONAL TECHNICAL SPECIFICATIONS: WALKTHROUGH METAL DETECTOR

| No | Item Description | Compliance Yes/No | State Details of Offer |
| --- | --- | --- | --- |
| General |  |  |  |
|  | The metal detector shall consist of a freestanding walk through frame with an integral control unit and powered by safe low voltage DC. |  |  |
|  | The metal detector must have accurate detection of magnetic, non- magnetic and mixed-alloy metal weapons. |  |  |
|  | The metal detector shall be able to locate weapons accurately on people in transit, by means of “height on person” display divided into at least 60 zones. |  |  |
|  | The metal detector shall scan the entire area (Multizone) of the walk through area and detect metal objects on a person passing through. The “height on person “display shall be by means of illuminated LEDs, which are built into the antenna of the metal detector. |  |  |
|  | The LEDs must be programmed to provide a display either on both sides or on just one side of the gate. |  |  |
|  | The metal detector shall be completely tamper proof. |  |  |
|  | The program and sensitivity push buttons shall be so arranged that tampering by unauthorised persons is entirely eliminated. Programming must be protected by hardware key and by double password. |  |  |
|  | The metal detector shall not be adversely affected by stationary metal bars or structures in the vicinity of the unit or moving metal near the archway. Must have exceptional immunity to external interferences. |  |  |
|  | The metal detector must be capable of operating adjacent to an x-ray inspection unit, as well as automatic synchronisation between two or more metal detectors with distance of down to 5cm without the use of external cables. |  |  |
|  | Reliability must be confirmed with no periodic re-calibration and preventative maintenance required. |  |  |
|  | The metal detector shall be capable of operating in the following conditions: Min. temperature: -20 °C; Max. temperature: 70 °C; Max. relative humidity: 0 - 95% |  |  |
|  | The operation of the metal detector shall not be adversely effected by repositioning of the frame within certain limits of original adjusted position. |  |  |
| Construction: |  |  |  |
|  | The metal detector shall comprise of a freestanding walk through frame containing the detector coils and the micro-processor control unit, complete with a 3 metre length of flexible cable and a 16A 3-pin plug top. |  |  |
|  | The cord and plug shall comply with the relevant SABS specifications. |  |  |
|  | The frame and the control unit shall be of robust construction and the base of the frame shall be designed to ensure rigidity. |  |  |
|  | The unit shall be able to detect metal objects, magnetic and non-magnetic, in the upper and lower zones (Multizone) of the passage within the settings specified. |  |  |
|  | The finish shall be durable and maintenance free. |  |  |
|  | The type of material used for the construction of the frame and control unit must be stated by the tenderers. |  |  |
|  | The colour range in which the metal detectors are available must be stated by the tenderers. |  |  |
|  | All material consisting of metal shall be treated against corrosion. |  |  |
|  | The approximate dimensions of the frame shall be as follows: Walkthrough height: 2 metres, Walkthrough width: 720 mm (min.) and Overall external width of the frame shall not exceed 876mm |  |  |
| Control System |  |  |  |
|  | The system shall operate by means of automatic level control adjustable to environmental changes without the need to reset. |  |  |
|  | The unit shall be equipped with highly visible “height on person“ display, which shall be by means of illuminated LEDs, which are built into the antenna of the metal detector. |  |  |
|  | The control unit shall be equipped with the following: “ON/OFF” main switch and “Mains ON” indicator light. |  |  |
|  | Pre-selectable Functions: Sensitivity settings Transmission programs Speeds of serial transmission Transmission frequencies Detection speed bands The sensitivity setting shall be consistent at average walking speed. |  |  |
|  | Visual indication in the form of an LED bar graph indicator having at least four green lights and four red lights representing the “PROCEED” and “ALARM” zone respectively. The indicator shall give an indication of the metal on a person in accordance with the sensitivity settings of the selector switch. When the “ALARM” zone is activated it shall simultaneously activate an audible alarm having an adjustable continuous tone and adjustable volume. The alarm system will automatically reset after the metal has passed through the frame. |  |  |
|  | The system shall be modular to facilitate maintenance and repairs. |  |  |
| Discrimination: |  |  |  |
|  | The unit must be able to discriminate between possibly dangerous metal objects and personal effects such as coins, keys, watches, buckles etc. |  |  |
| Safety Features: |  |  |  |
|  | All electronic and electrical components shall be protected by lockable panels. |  |  |
|  | The metal detector shall not have any effect on heart pacemakers. |  |  |
|  | The metal detector shall not effect magnetic storage media or camera film. |  |  |
| Electrical Supply System: |  |  |  |
|  | The Metal detector shall be designed for connection to a 230+/- 5%, 50Hz, single phase, three wire (phase, neutral and earth) power supply. |  |  |
| Throughput: |  |  |  |
|  | The system shall accept a passage of at least 50 persons per minute without functional overload, with detection speed up to 16m/sec and reset time of 0.2 seconds. |  |  |
| Placing in Position and Testing: |  |  |  |
|  | The detector shall be placed in position, tested, commissioned and adjusted to the user. |  |  |
| Brochures: |  |  |  |
|  | Brochures furnishing descriptions and technical specifications etc., of the unit offered shall be submitted. |  |  |
|  | The following information is required: |  |  |
|  | Manufacturer Year of manufacture Country of origin Model number |  |  |
| Maintenance: |  |  |  |
|  | The unit must be maintenance free and with minimum future service. A statement confirming this is required. |  |  |
|  | Electronic modules must be easily exchangeable. |  |  |
|  | Spare parts must be locally stocked and availability guaranteed for a ten-year period starting from date of delivery. |  |  |
| Guarantee and Service: |  |  |  |
|  | The successful tenderer shall guarantee and service the complete unit for a period of 12 (twelve) months from date of delivery of every unit to site. This should be read in part with item 1.3. |  |  |
|  | During the period of guarantee the successful tenderer shall, at his own expense, carry out all necessary repair work including material and labour (excluding work required due to damage by others) in order to maintain the unit in working condition. |  |  |
|  | The successful tenderer shall, during the period of guarantee, repair the unit to the satisfaction of the department within 48 hours after he has been notified that the unit is not operating. |  |  |
|  | After the laps of the initial twelve months’ period of servicing, the successful tenderer might be required to enter into a service agreement with the department for a future period of twelve months. This agreement may subsequently be renewed for yearly periods. |  |  |
| Training: |  |  |  |
|  | The successful tenderer shall train and instruct operators designated by the user department in the operation of the unit. |  |  |
| Manuals: |  |  |  |
|  | Two manuals, each with the following information shall be handed over to the department when the unit is delivered to site: |  |  |
|  | Operating Instructions |  |  |
|  | Technical description with diagrams and instructions for maintenance and repairs. |  |  |
| Deviations: |  |  |  |
| Completion Time: |  |  |  |
| Time required for complete supply and delivery: |  |  |  |
| Delivery Arrangement: |  |  |  |
| Delivery arrangements shall be co-ordinated with: |  |  |  |
| Telephone: |  |  |  |
| Fax: |  |  |  |

- CRITERIA THAT WILL BE CONSIDERED IN EVALUATING

- The received bids shall be evaluated on functionality as a criterion

- The functionality shall be assessed, followed by evaluation in accordance with the 80/20 preference point systems.

- The bids must achieve minimum qualifying score of 70 percent for functionality, in order to be evaluated further in accordance with the 80/20 preference point systems.

NB:

- Bidders that score less than 70/100 points shall not qualify to move to the second phase of evaluation

- Recommended bidders will be required to present their bid proposition to the Department (logistical arrangements will be communicated according)

  - Functionality

| No | Evaluation Criteria | Point |
| --- | --- | --- |
|  | Project Execution Plan - contingency plan - response time - local support response - lead time for installation | 30 |
|  | Experience Relevant years of experience at least minimum of five (05) years, supported by stamped, signed proof of reference/testimonial from entities service was rendered. - five years and more (30) - four years (25) - three years (20) - two years (15) - one year and more (05) | 30 |
|  | Locality - within Free State (20) - outside Free State (15) | 20 |
|  | Financial Management (Bank Grading Certificate) - Grade A (20) - Grade B (15) - Grade C (10) - Grade D (05) | 20 |
| Total |  | 100 |

  - Preferential Point System: 80/20

| No | Evaluation Criteria | Points |
| --- | --- | --- |
| 6.2.1. | Price | 80 |
| 6.2.2. | Special Goals | 20 |
| Total |  | 100 |

  - Calculation of points for Special Goals

Points will be awarded to a bidder for attaining the B-BBEE status level of contribution in accordance with the table below:

| Special Goals | Number of Points |
| --- | --- |
| Women | 05 |
| SMME’s | 06 |
| HDI | 04 |
| Disabled | 03 |
| Locality | 02 |
| Non-compliant contributor | 0 |

NB: The goals below are now applicable to all procurement of goods and services above the value of R30K which are to be evaluated on an 80/20 principle up to a value not exceeding R500 000.00 including all applicable taxes.

As per the Preferential Procurement Regulations of 2022 in terms of the 80/20 Points system, the 20 points for Specific goals for the Department are allocated as follows:

- OTHER CONDITIONS

  - Any proposals received in response to this bid remain the (intellectual) property of the Department of Employment and Labour.

  - If there is tie between the bidders, SCM Practice Notes and Circulars will be applied to determine the successful bidder.

  - Bids should be held valid for a period of 90 days.

  - The successful bidder will be required to sign a Service Level Agreement (SLA) and a contract.

  - Prospective bidders must be registered on CSD and must have a corresponding commodity with the advertised Bid; the bidder is required to update CSD profile with relevant commodity.

  - Bidders must attach a valid letter of good standing from COID and Proof of Registration from UIF. Failure to attach will invalidate their bid.

  - Bidders who are not registered with COID and UI funds must attach a letter for tender purpose in their bid document. Failure to attach may invalidate the bid.

  - Bidders must attend a compulsory briefing session and site inspection on the specified date venue and time. Failure to attend and attach briefing session and site inspection certificate will invalidate the bid.

  - Failure to submit fully and accurately completed SBD forms will result in automatic disqualification.

- CLOSING DATE AND TIME

  - The closing date and time for this bid is the 28/09/2023 at 11:00. No bid document will be accepted after the specified date and time.

- COMPULSORY BRIEFING SESSION

  - The briefing session is compulsory, and will be scheduled as follows;

Date	19/09/2023

Time	11:00

Venue:	Allen Rautenbach (Civic Hall, next to Library)

302 Hill Street

Kroonstad

9500

  - Bidders will be issued with compulsory briefing session certificate, which must be attached to bidding documents. Failure to attached the certificate will amount to disqualification.

  - Bidders who arrives later than 12:00 will not be allowed to be part of the session

- COMPULSORY SITE INSPECTION

  - The site inspection is compulsory, and will be scheduled as follows;

Date	19/09/2023

Time	11:00

Venue:	Kroonstad Labour Centre

Date	20/09/2023

Time	11:00

Venue:	Bethlehem Labour Centre

Date	21/09/2023

Time	11:00

Venue:	Botshabelo Labour Centre

  - Bidders will be issued with compulsory site inspection certificate, which must be attached to bidding documents. Failure to attached the certificate will amount to disqualification.

  - Bidders who did not attend compulsory briefing session and/or arrives later than 12:00 will not be allowed to be part of the session