ANNEXURE A: RT18-2024 TECHNICAL SPECIFICATION REQUIREMENTS

RT18-2024

SUPPLY AND DELIVERY OF POST AND ANTE-MORTEM BLOOD ALCOHOL KIT TO THE STATE FOR THE PERIOD OF

36 MONTHS

# ITEM NUMBER: RT18-01-001 - ANTE-MORTEM BLOOD ALCOHOL KIT

| #### BACKGROUND |  |  |
| --- | --- | --- |
| The Forensic Chemistry Laboratories of the National Health Laboratory Services are responsible for the Alcohol Analysis of Ante-Mortem Blood Samples. The kits that these blood samples are packaged and procured by the South African Police services, The Department of Health, National Prosecution Authority, Department of justice, National Health Laboratory Services, National Road Agency and Metro Police from various municipalities. A transversal contract is established for organs of state that require to procure the kits from a centrally arranged contract. |  |  |
| The bidder must indicate in writing with Comply for compliance with the requirements and Do Not comply for non - compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not Comply. |  | COMPLY/ DO NOT COMPLY |
| A. | GENERAL REQUIREMENTS |  |
| 1. | Container |  |
| 1.1 | The container must consist out of a small stackable material (light weight e.g polystyrene). The material must be able to allow the control of the environmental temperature within the kit, regardless of the surroundings. |  |
| 1.2 | The material must be tamper evident. Should physical tampering be attempted, it must be apparent. |  |
| 1.3 | The material must not allow for easy access to the contents within the container at any time while it is sealed. |  |
| 1.4 | The container must be supplied in a sealed condition (see paragraph 5 below for details). |  |
| The bidder must indicate in writing with Comply for compliance with the requirements and Do Not comply for non - compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not Comply. |  | COMPLY/ DO NOT COMPLY |
| 1.5 | The kit must be small enough to allow for delivery and storage of numerous kits to e.g.10 be packaged and shipped together simultaneously. |  |
| 1.6 | The kit interior must contain sufficient space or compartments to house the contents listed below in a secure way without allowing for possible leakages, movement or breakage of the contents during transportation. |  |
| 1.7 | The exterior of the kit must be sealable/lockable with appropriate mechanism integrating seals, preferably containing holes on the sides in order to allow the insertion of the cable tie like seal or another seal mechanism. |  |
| 1.8 | The manufacturing date, expiry date, lot/batch number of the contents of the kit must be placed on the outside of the container in such a way that it cannot be concealed with the sample label and is clearly visible to the users. |  |
| 1.9 | The supplier must be ISO 13485:2003 certified and provide proof of validation data to establish the expiry date of the kit and its contents, specifically blood collection tube. The proof must be in documentary form with the testing results for this purpose. All documents provided must be signed and/or certified and be submitted with the bid reply. |  |
| 1.10 | The company must provide in documentary form, all standards adhered to in this testing process. This may include, but is not limited to ISO 13485:2003 standards, SANAS standards and must be traceable to the Standards International (SI). The documents in this regard must be attached to the bid submission. All documents provided must be signed and/or certified. |  |
| 1.11 | The polystyrene container must be equipped with a metal/foil indicator on the outside that would clearly define containers that were microwaved after sealing. The metal/ foil indicator must also show clear physical tampering if attempted to be pulled off. |  |
| The bidder must indicate in writing with Comply for compliance with the requirements and Do Not comply for non - compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not Comply. |  | COMPLY/ DO NOT COMPLY |
| 2. | Contents of Container |  |
| 2.1 | One 10 ml vacutainer test-tube for the collection of blood (see point 3. Below). |  |
| 2.2 | Two seals with the identical unique number in sequence of the original seal, meant for re-sealing. |  |
| 2.3 | One safety sterile needle in a peel pouch. |  |
| 3. | Vacutainer test-tube |  |
| 3.1 | Must be manufactured from glass and must be at least 10mL capacity |  |
| 3.2 | Must be sterile with documented length of sterility guarantee. |  |
| 3.3 | Must be capped to maintain vacuum of the tube (e.g. rubber-stoppered) |  |
| 3.4 | Must have 1 x plastic shell to fit over the top cap of test-tube. |  |
| 3.5 | Must contain an anti-clotting agent, pre-mixed with a preservative (sodium fluoride). |  |
| 3.6 | Must contain 1 x bar-coded carbonized label with data entry fields. |  |
| 4. | Preservative and anti-coagulant |  |
| 4.1 | Must be 99% potassium oxalate (POTOX), analytical reagent grade (AR) or higher as anti-clotting agent; |  |
| The bidder must indicate in writing with Comply for compliance with the requirements and Do Not comply for non - compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not Comply. |  | COMPLY DO NOT COMPLY |
| 4.2 | Must be Pre-mixed with the preservative sodium fluoride in the approximate ratio (1: 4, 33) and allow for a blood-filled tube to maintain a minimum concentration of 1% fluoride. |  |
| 4.3 | The chemical as referred to in points 4.1 and 4.2 must be pulverized to a fine powder when used in the vacutainer tube. |  |
| 5. | Seals must meet the following requirements: |  |
| 5.1 | Two seals for sealing the kit (pre- sampling) by the manufacturer at point of manufacture, with identical and unique numbering barcode. |  |
| 5.2 | Two seals for sealing the kit (post- sampling) by the medical officer after collecting the sample before dispatch to the Forensic Chemistry Laboratory (FCL). Must have identical and unique numbering barcode. Seals must be fixed and no expansion or slippage of seal possible. |  |
| 5.3 | Seals must be flexible and wide enough to allow a snug fit around the container without cracking, breaking or cutting into the container. Hollow tubes to protect holes if kit uses cable tie seals. |  |
| 5.4 | A unique serial number permanently infused. The serial number must have a prefix DD 000001 (barcode and numerical) that must be recorded with full serial tracking as per ISO standard. |  |
| 5.5 | The “post-sampling” seals must both have identical numbers, following sequentially onto the number on the two “pre-sampling” seals that also have identical numbers. |  |
| 6. | Label must meet the following requirements: |  |
| 6.1 | Must be carbonized or allow other duplication for data completion. |  |
| 6.2 | Must be attached to the test-tube and allow a tear-off/ removal of copy to affix elsewhere. |  |
| The bidder must indicate in writing with Comply for compliance with the requirements and Do Not comply for non - compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not Comply. |  | COMPLY/ DO NOT COMPLY |
| 6.3 | A second identical label that may be a carbon copy, and must contain adhesive to affix it on the outside of the container before re-sealing after sampling. |  |
| 6.4 | Both labels must be clearly marked with the outside and inside seal numbers. |  |
| 6.5 | Both labels must also contain the seal number in the form of a bar-code as mentioned in par 5.4 and must have the space for the following information to be filled out: |  |
| 6.5.1 | SAPS Station and CAS number; |  |
| 6.5.2 | SAPS 13 number; |  |
| 6.5.3 | Date and time of sampling; |  |
| 6.5.4 | Signature of Law Enforcement officer; |  |
| 6.5.5 | Rank, Initials, Surname and Persal Number of Law Enforcement officer; |  |
| 6.5.6 | Signature of Medical officer who drew the blood; |  |
| 6.5.7 | Name and Persal number of Medical officer who drew the blood. |  |
| 7. | Information leaflet |  |
| 7.1 | Information leaflet must have instructions on how to sample the blood. |  |
| 7.2 | Information leaflet must have instructions on how to attach the carbon copy label to the outside of the container. |  |
| 7.3 | Information leaflet must have guidelines on how to re-package the test-tube. |  |
| The bidder must indicate in writing with Yes for compliance with the requirements and No for non-compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not comply. |  | COMPLY/ DO NOT COMPLY |
| 7.4 | Information leaflet must have instructions on how to seal the container. |  |
| 7.5 | Information leaflet must have instructions on how to dispose the needle. |  |
| 7.6 | Information leaflet in the kit and printing on the individual items in the kit must be in a format that is bold and be readable in low light conditions. |  |
| B. | ADDITIONAL REQUIREMENTS |  |
| 8. | All containers in the Blood Alcohol Sampling Kit must be clean and sterilized to avoid the possibility of contamination. Proof of the clean and sterilized manufacturing process must form part of the bid submission. |  |
| 9. | A clinically clean environment and stringent quality control are critical to ensure clean containers and kits. Before the awarding of the contract, the manufacture’s site may be visited to assess the cleanness of the facility. |  |
| 10. | The production of the Potassium Oxalate must happen in a Bio Hazard safety zone and must be irradiated after vacutainer tube filling to ensure sterility. |  |
| 11. | The bidder must include a detailed procedure describing the preparation of the kit for the evaluation by the end user. Failure to submit the procedure will result in the exclusion of the bid proposal during the bid evaluation process. |  |
| 12. | At least 1 sample of the complete Blood Alcohol Sampling Kit must be delivered for evaluation with the bid proposal. |  |
| The bidder must indicate in writing with Comply for compliance with the requirements and Do Not comply for non - compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not Comply. |  | COMPLY/ DO NOT COMPLY |
| 13. | The uniquely bar-coded serial numbers of each kit must also appear on the outside of the outer container that will be used for delivery of the kits to the SAPS (for example should ten uniquely barcoded kits be packaged the stickers of all ten uniquely barcoded serial numbers should affix on the outside of the container). |  |
| 14. | Validation of the stability and other features of the vacutainer test-tubes must occur under local conditions. The bidder must supply upon request, at no additional cost, up to four hundred (400) of the specified tubes in batches and at times determined by the end-user for validation studies. |  |
| 15. | Where validation studies indicate a problem with the sodium fluoride concentration or stability, the bidder must source new test tubes that may be customised and allow additional concurrent validation of the other provided vacutainer tubes for up to four hundred (400) of the newly sourced tubes. This will occur upon request and at no additional cost. |  |
| 16. | The first ante mortem blood kits must be made available within the maximum of four (04) weeks after awarding of the contract. |  |
| 18. | This bid will be for the sampling of ante-mortem blood specimen for blood alcohol analysis by the Forensic Chemistry Laboratories in cases of suspected drunken driving. |  |
| 19. | The quantities cannot be guaranteed. Government will procure the kits as and when the need arises |  |
| 20. | If the bidder does not comply with ALL the special requirements, the bid will be disqualified. |  |
| 21. | This contract will be for the period of three years after the date of awarding of the contract. |  |
| The bidder must indicate in writing with Comply for compliance with the requirements and Do Not comply for non - compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not Comply. |  | COMPLY/ DO NOT COMPLY |
| 22. | If the bidder that is not the OEM manufacturer or OEM agent for the kits must provide documentary proof of long standing relationship with the manufacturer/supplier. Supplied consumables must be supported for a period of at least two (2) years. The bidder must be able to supply the necessary support and after sales service for the contract period. |  |
| 23. | The successful bidder must be able to provide, where necessary, evidence in Court, in person, on the product that was supplied. |  |
| 24. | All manufacturing and supply records of the product must be available throughout the contract period (as well as to the point of the last expiry date of the kits delivered) for inspection and verification purposes. |  |
| 25. | The bid price must include the price of each item (s) as: |  |
| 26. | Be inclusive of all other costs to provide the kits when orders are placed. |  |

# ITEM NUMBER: RT18-02-001 - POST-MORTEM BLOOD ALCOHOL KIT

| #### BACKGROUND |  |  |
| --- | --- | --- |
| The Forensic Chemistry Laboratories of the National Health Laboratory Services are responsible for the Alcohol Analysis of Ante-Mortem Blood Samples. The kits that these blood samples are packaged and procured by the South African Police services, The Department of Health, National Prosecution Authority, Department of justice, National Health Laboratory Services, National Road Agency and Metro Police from various municipalities. A transversal contract is established for organs of state that require to procure the kits from a centrally arranged contract. |  |  |
| The bidder must indicate in writing with Comply for compliance with the requirements and Do Not comply for non - compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not Comply. |  | COMPLY/ DO NOT COMPLY |
| A. | GENERAL REQUIREMENTS |  |
| 1. | Container |  |
| 1.1 | 20ml McCartney bottle made out of strong, crack resistant glass, stained brown, for the collection of blood, with a screw-on cap containing Teflon lining firmly adhered to the inner aspect of the screw-on lid, and an anti-leak mechanism that seals airtight; |  |
| 1.2 | The material must be tamper evident. Should physical tampering be attempted, it must be apparent. |  |
| 1.3 | The material must not allow for easy access to the contents within the container at any time while it is sealed. |  |
| 1.4 | The container must be supplied in a sealed condition (see paragraph 5 below for details). |  |
| The bidder must indicate in writing with Comply for compliance with the requirements and Do Not comply for non - compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not Comply. |  | COMPLY/ DO NOT COMPLY |
| 1.5 | The kit must be small enough to allow for delivery and storage of numerous kits to e.g.10 be packaged and shipped together simultaneously. |  |
| 1.6 | The kit interior must contain sufficient space or compartments to house the contents listed below in a secure way without allowing for possible leakages, movement or breakage of the contents during transportation. |  |
| 1.7 | The exterior of the kit must be sealable/lockable with appropriate mechanism integrating seals, preferably containing holes on the sides in order to allow the insertion of the cable tie like seal or another seal mechanism. |  |
| 1.8 | The manufacturing date, expiry date, lot/batch number of the contents of the kit must be placed on the outside of the container in such a way that it cannot be concealed with the sample label and is clearly visible to the users. |  |
| 1.9 | The supplier must be ISO 13485:2003 certified and provide proof of validation data to establish the expiry date of the kit and its contents, specifically blood collection tube. The proof must be in documentary form with the testing results for this purpose. All documents provided must be signed and/or certified and be submitted with the bid reply. |  |
| 1.10 | The company must provide in documentary form, all standards adhered to in this testing process. This may include, but is not limited to ISO 13485:2003 standards, SANAS standards and must be traceable to the Standards International (SI). The documents in this regard must be attached to the bid submission. All documents provided must be signed and/or certified. |  |
| 1.11 | The polystyrene container must be equipped with a metal/foil indicator on the outside that would clearly define containers that were microwaved after sealing. The metal/ foil indicator must also show clear physical tampering if attempted to be pulled off. |  |
| The bidder must indicate in writing with Comply for compliance with the requirements and Do Not comply for non - compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not Comply. |  | COMPLY/ DO NOT COMPLY |
| 2. | Contents of Container |  |
| 2.2 | Two seals with the identical unique number in sequence of the original seal, meant for re-sealing. |  |
| 2.3 | One safety sterile needle in a peel pouch. |  |
| 3. | Vacutainer test-tube |  |
| 3.1 | Must be manufactured from glass and must be at least 10mL capacity. |  |
| 3.2 | Must be sterile with documented length of sterility guarantee. |  |
| 3.3 | Must be capped to maintain vacuum of the tube (e.g. rubber-stoppered) |  |
| 3.4 | Must have 1 x plastic shell to fit over the top cap of test-tube. |  |
| 3.5 | Must contain an anti-clotting agent, pre-mixed with a preservative (sodium fluoride). |  |
| 3.6 | Must contain 1 x bar-coded carbonized label with data entry fields. |  |
| 4. | Preservative and anti-coagulant |  |
| 4.1 | Must be 99% potassium oxalate (POTOX), analytical reagent grade (AR) or higher as anti-clotting agent; |  |
| The bidder must indicate in writing with Comply for compliance with the requirements and Do Not comply for non - compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not Comply. |  | COMPLY DO NOT COMPLY |
| 4.2 | Must be Pre-mixed with the preservative sodium fluoride in the approximate ratio (1: 4, 33) and allow for a blood-filled tube to maintain a minimum concentration of 1% fluoride. |  |
| 4.3 | The chemical as referred to in points 4.1 and 4.2 must be pulverized to a fine powder when used in the vacutainer tube. |  |
| 5. | Seals must meet the following requirements: |  |
| 5.1 | Two seals for sealing the kit (pre- sampling) by the manufacturer at point of manufacture, with identical and unique numbering barcode. |  |
| 5.2 | Two seals for sealing the kit (post- sampling) by the medical officer after collecting the sample before dispatch to the Forensic Chemistry Laboratory (FCL). Must have identical and unique numbering barcode. Seals must be fixed and no expansion or slippage of seal possible. |  |
| 5.3 | Seals must be flexible and wide enough to allow a snug fit around the container without cracking, breaking or cutting into the container. Hollow tubes to protect holes if kit uses cable tie seals. |  |
| 5.4 | A unique serial number permanently infused. The serial number must have a prefix DD 000001 (barcode and numerical) that must be recorded with full serial tracking as per ISO standard. |  |
| 5.5 | The “post-sampling” seals must both have identical numbers, following sequentially onto the number on the two “pre-sampling” seals that also have identical numbers. |  |
| 6. | Label must meet the following requirements: |  |
| 6.1 | Must be carbonized or allow other duplication for data completion. |  |
| 6.2 | Must be attached to the McCartney bottle and allow a tear-off/ removal of copy to affix elsewhere. (Second identical label that will serve as a carbon copy, and should contain adhesive to affix it on the outside of the container before re-sealing after sampling) |  |
| The bidder must indicate in writing with Comply for compliance with the requirements and Do Not comply for non - compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not Comply. |  | COMPLY/ DO NOT COMPLY |
| 6.3 | A second identical label that may be a carbon copy, and must contain adhesive to affix it on the outside of the container before re-sealing after sampling. |  |
| 6.4 | Both labels must be clearly marked with the outside and inside seal numbers. |  |
| 6.5 | Both labels must also contain the seal number in the form of a bar-code as mentioned in par 5.4 and must have the space for the following information to be filled out: |  |
| 6.5.1 | SAPS Station and CAS number; |  |
| 6.5.2 | Death register number |  |
| 6.5.3 | Date and time of sampling; |  |
| 6.5.4 | Signature of Law Enforcement officer; |  |
| 6.5.5 | Rank, Initials, Surname and Persal Number of Law Enforcement officer; |  |
| 6.5.6 | Signature of Medical officer who drew the blood; |  |
| 6.5.7 | Name and Persal number of Medical officer who drew the blood. |  |
| 7. | Information leaflet |  |
| 7.1 | Information leaflet must have instructions on how to sample the blood. |  |
| 7.2 | Information leaflet must have instructions on how to attach the carbon copy label to the outside of the container. |  |
| 7.3 | Information leaflet must have guidelines on how to re-package the test-tube. |  |
| The bidder must indicate in writing with Yes for compliance with the requirements and No for non-compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not comply. |  | COMPLY/ DO NOT COMPLY |
| 7.4 | Information leaflet must have instructions on how to seal the container. |  |
| 7.5 | Information leaflet must have instructions on how to dispose the needle. |  |
| 7.6 | Information leaflet in the kit and printing on the individual items in the kit must be in a format that is bold and be readable in low light conditions. |  |
| B. | ADDITIONAL REQUIREMENTS |  |
| 8. | All containers in the Blood Alcohol Sampling Kit must be clean and sterilized to avoid the possibility of contamination. Proof of the clean and sterilized manufacturing process must form part of the bid submission. |  |
| 9. | A clinically clean environment and stringent quality control are critical to ensure clean containers and kits. Before the awarding of the contract, the manufacture’s site may be visited to assess the cleanness of the facility. |  |
| 10. | The production of the Potassium Oxalate must happen in a Bio Hazard safety zone and must be irradiated after vacutainer tube filling to ensure sterility. |  |
| 11. | The bidder must include a detailed procedure describing the preparation of the kit for the evaluation by the end user. Failure to submit the procedure will result in the exclusion of the bid proposal during the bid evaluation process. |  |
| 12. | At least 1 sample of the complete Blood Alcohol Sampling Kit must be delivered for evaluation with the bid proposal. |  |
| The bidder must indicate in writing with Comply for compliance with the requirements and Do Not comply for non - compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not Comply. |  | COMPLY/ DO NOT COMPLY |
| 13. | The uniquely bar-coded serial numbers of each kit must also appear on the outside of the outer container that will be used for delivery of the kits to the SAPS (for example should ten uniquely barcoded kits be packaged the stickers of all ten uniquely barcoded serial numbers should affix on the outside of the container). |  |
| 14. | Validation of the stability and other features of the vacutainer test-tubes must occur under local conditions. The bidder must supply upon request, at no additional cost, up to four hundred (400) of the specified tubes in batches and at times determined by the end-user for validation studies. |  |
| 15. | Where validation studies indicate a problem with the sodium fluoride concentration or stability, the bidder must source new test tubes that may be customised and allow additional concurrent validation of the other provided vacutainer tubes for up to four hundred (400) of the newly sourced tubes. This will occur upon request and at no additional cost. |  |
| 16. | The first post / ante mortem blood kits must be made available within the maximum of four (04) weeks after awarding of the contract. |  |
| 18. | This bid will be for the sampling of post mortem blood specimen for blood alcohol analysis |  |
| 19. | The quantities cannot be guaranteed. Government will procure the kits as and when the need arises |  |
| 20. | If the bidder does not comply with ALL the special requirements, the bid will be disqualified. |  |
| 21. | This contract will be for the period of three years after the date of awarding of the contract. |  |
| The bidder must indicate in writing with Comply for compliance with the requirements and Do Not comply for non - compliance with the requirements in the relevant column of every specification field. Failure to do this will be interpreted as Do Not Comply. |  | COMPLY/ DO NOT COMPLY |
| 22. | If the bidder that is not the OEM manufacturer or OEM agent for the kits must provide documentary proof of long standing relationship with the manufacturer/supplier. Supplied consumables must be supported for a period of at least two (2) years. The bidder must be able to supply the necessary support and after sales service for the contract period. |  |
| 23. | The successful bidder must be able to provide, where necessary, evidence in Court, in person, on the product that was supplied. |  |
| 24. | All manufacturing and supply records of the product must be available throughout the contract period (as well as to the point of the last expiry date of the kits delivered) for inspection and verification purposes. |  |
| 25. | The bid price must include the price of each item (s) as: |  |
| 26. | Be inclusive of all other costs to provide the kits when orders are placed. |  |