BLOEM WATER

TENDER NO: BW253/IRM/MAP/22

UPGRADING OF THE INTABAZWE RISING MAIN

C3.6	Occupational Health and Safety specifications

CONTENTS

PART	HEADING	PAGE NO.

1.	PREAMBLE	 C3.6-1

2.	SCOPE OF HEALTH AND SAFETY SPECIFICATION DOCUMENT	 C3.6-1

3.	DEFINITIONS	 C3.6-1

4.	OCCCUPATIONAL HEALTH AND SAFETY PLAN	 C3.6-3

4.1.	Contents of the occupational health and safety plan	 C3.6-3

4.1.1.	Structure and organisation of OHSACT responsibilities and appointments	 C3.6-3

4.1.2.	Communication of the health and safety management programme	 C3.6-6

4.1.3.	Health and safety internal audit and inspection	 C3.6-7

4.1.4.	Health and safety incident/accident investigations and reporting	 C3.6-7

4.2.	Health and safety training	 C3.6-8

4.2.1.	Health and Safety Induction Training	 C3.6-8

4.2.2.	Other Training	 C3.6-8

4.2.3.	Health and Safety Budget	 C3.6-9

4.2.4.	Administration	 C3.6-9

4.2.5.	Health and Safety Management Programme	 C3.6-9

4.2.6.	Personal Protective Equipment (PPE)	 C3.6-9

4.2.7.	Employee Site Facilities	 C3.6-10

4.2.8.	Health and Safety Signage	 C3.6-10

4.2.9.	Health and Safety Notice Board	 C3.6-11

4.2.10.	Training	 C3.6-11

4.2.11.	First Aid Station	 C3.6-12

4.3.	Logbooks and Registers	 C3.6-12

4.3.1.	Health and Safety Appointments	 C3.6-12

4.3.2.	Logbooks and Registers	 C3.6-12

4.3.3.	Record Keeping Responsibilities	 C3.6-13

5.	OCCUPATIONAL HEALTH AND SAFETY FILE	 C3.6-14

5.1.	Contents of the Occupational health and Safety file	 C3.6-14

6.	PROJECT STANDARDS FOR OCCUPATIONAL HEALTH AND SAFETY	 C3.6-14

6.1.	Administrative and legal	 C3.6-14

6.2.	Training	 C3.6-19

6.3.	Public safety, security and emergency	 C3.6-20

6.4.	Personal Protective Equipment	 C3.6-21

6.5.	Personal Protective Equipment (Continued).	 C3.6-22

6.6.	Personal Protective Equipment (Continued).	 C3.6-23

6.7.	Working at Heights (including roof work)	 C3.6-23

6.8.	Scaffolding/Formwork/Support Work	 C3.6-23

6.9.	Scaffolding/Formwork/Support Work (Continued)	 C3.6-24

6.10.	Ladders	 C3.6-25

6.11.	Electricity	 C3.6-25

6.12.	Electricity (Continued)	 C3.6-26

6.13.	Excavations	 C3.6-26

6.14.	Tools	 C3.6-26

6.15.	Tools (Continued)	 C3.6-27

6.16.	Transport & Materials Handling Equipment	 C3.6-27

6.17.	Site Plant and Machinery	 C3.6-27

6.18.	Site Plant and Machinery (Continued)	 C3.6-28

6.19.	Plant & Storage Yards/Site Workshops Specifics	 C3.6-29

6.20.	Plant & Storage Yards/Site Workshops Specifics (Continued)	 C3.6-29

6.21.	Workplace Environment, Health and Hygiene	 C3.6-30

7.	RISK ASSESSMENT PROGRAMME	 C3.6-31

7.1.	Methodology	 C3.6-31

7.2.	Contents of the risk assessment programme	 C3.6-32

7.3.	Base-line risk assessment	 C3.6-33

7.4.	Issue-based risk assessment	 C3.6-34

7.4.1.	Task orientated risks:	 C3.6-34

7.4.2.	Task orientated risks: (Continued)	 C3.6-35

7.4.3.	Task orientated risks (Continued).	 C3.6-37

7.4.4.	Plant and machinery risks:	 C3.6-37

7.4.5.	Equipment risks:	 C3.6-38

7.4.6.	Physical risks	 C3.6-39

7.4.7.	Ergonomically risks	 C3.6-39

7.4.8.	Behavioural risks	 C3.6-39

7.4.9.	Occupational Health risks	 C3.6-40

7.5.	Continuous risk assessment	 C3.6-40

7.6.	Safe work procedures	 C3.6-40

# PREAMBLE

This “Health and Safety Specifications” document is governed by the Occupational Health and Safety Act, 1993 (Act No 85 of 1993), hereinafter referred to as the Act, with specific reference to Construction Regulation 4(1) (a) Notwithstanding this, cognisance should be taken of the fact that no single ACT or it’s set of Regulations can be read in isolation.

Included in these specifications is set rules to assist the principal contractor, contractors (sub-contractors) and client of the project in controlling and managing health and safety issues on the construction site, as stipulated in the OHSACT.

The specifications and rules do not relief the principal contractor, contractors (sub-contractors) or their employees from any legal obligation under the requirements of the “Basic Conditions of Employment Act” or the “Occupational Health and Safety ACT”.

The specifications and rules will apply for the duration of the project.  Should the principal contractor or contractors (sub-contractors) not comply, it will be deemed as a breach of contract.

The principal contractor will carry full responsibility and accountability regarding the adherence to any health and safety issues when contractors (sub-contractors) are used to carry out any construction work on the project.  It must be noted that the client may stop any contractor from executing construction work, which is not in accordance with the client’s health and safety specifications or rules for the project or which poses a threat to the health and safety of any person.

# SCOPE OF HEALTH AND SAFETY SPECIFICATION DOCUMENT

The health and safety specifications pertaining to this project cover the subjects contained in the index and are intended to outline the normal as well as any special requirements of the Client pertaining to the health and safety matters applicable to the project.

The stipulations in this specification, as well as those contained in all other documentation pertaining to the project, including contract documentation and technical specifications shall not be interpreted in any way whatsoever, to countermand or nullify any stipulation of the ACT, Regulations and Safety Standards which are promulgated under, or incorporated into the Act.

# DEFINITIONS

The most important definitions in the Act and Regulations pertaining to this specification document are hereby extracted.

“Purpose of the Act”-

To provide for the health and safety of persons at work and the health and safety of persons in connection with the use of plant and machinery; the protection of persons other than persons at work against hazards to health and safety arising out of or in connection with the activities of persons at work; to establish an advisory council for occupational health and safety; and to provide for matters connected therewith.

“Agent”-

means any person who acts as a representative for a client;

“Client”-

means any person for whom construction work is performed;

“Construction Work” is defined as any work in connection with-

(a)	the erection, maintenance, alteration, renovation, repair, demolition or dismantling of or addition to a building or any similar structure;

(b)	the installation, erection, dismantling or maintenance of a fixed plant where such work includes the risk of a person falling;

(c)	the construction, maintenance, demolition or dismantling of any bridge, dam , canal, road, 	railway, runway, sewer or water reticulation system or any similar civil engineering structure; or

(d)	the moving of earth, clearing of land, the making of an excavation, piling, or any similar type of work;

“Contractor”-

means an employer, as defined in Section 1 of the Act, who performs construction work and includes Principal Contractors;

“Health and Safety File”-

means a file, or other record in permanent form, containing the information required a contemplated in the regulations;

“Health and Safety Plan”-

means a documented plan which addresses hazards identified and includes safe work procedures to mitigate, reduce or control the hazards identified;

“Health and Safety Specification”-

means a documented specification of all health and safety requirements pertaining to the associated works on a construction site, so as to ensure the health and safety of persons;

‘Method Statement”-

means a document detailing the key activities to be performed in order to reduce as reasonably as practicable the hazards identified in any risk assessment;

“Principal Contractor”-

means an employer, as defined in section 1 of the Act who performs construction work and is appointed by the client to be in overall control and management of a part of or the whole of a construction site;

“Risk Assessment”-

means a program to determine any risk associated with any hazard at a construction site, in order to identify the steps needed to be taken to remove, reduce or control such hazard.

# OCCCUPATIONAL HEALTH AND SAFETY PLAN

The principal contractor has to demonstrate to the Client that it has developed a suitable and sufficiently documented occupational health and safety plan for the specific project appointed, as well as the necessary competencies, experience and resources to perform the construction work safely.

## Contents of the occupational health and safety plan

The health and safety management programme should at least provide a detailed overview of the following matters:

(a)	Structure and organisation of OHSACT responsibilities and appointments

(b)	Management of the project’s occupational health and safety hazards and risks.

(c)	Communication of the health and safety management programme.

(d)	Programme for occupational health and safety internal audits and inspections

(e)	Programme for occupational health and safety investigations regarding incidents/accidents.

### Structure and organisation of OHSACT responsibilities and appointments

#### Overall Supervision and Responsibility for Occupational Health and Safety

    - The Client and/or its Agent on its behalf to ensure that the Principal Contractor, appointed in terms of Construction Regulation 4(1) (c), implements and maintains the agreed and approved H&S Plan.  Failure on the part of the Client or Agent to comply with this requirement will not relieve the Principal Contractor from any one or more of his/her duties under the Act and Regulations.

    - The Chief Executive Officer of the Principal Contractor in terms of Section 16(1) of the Act to ensure that the Employer (as defined in the Act) complies with the Act.  The pro forma Legal Compliance Audit may be used for this purpose by the Principal Contractor or his/her appointed contractor.

    - All OH&S Act (85/1993), Section 16 (2) appointee/s as detailed in his/her/their respective appointment forms to regularly, in writing, report to their principals on matters of health and safety per routine and ad hoc inspections and on any deviations as soon as observed, regardless of whether the observation was made during any routine or ad hoc inspection and to ensure that the reports are made available to the principal Contractor to become part of site records (Health & Safety File).

    - The Construction Supervisor and Assistant Construction Supervisor/s appointed in terms of Construction Regulation 6 to regularly, in writing, report to their principals on matters of health and safety per routine and ad hoc inspections and on any deviations as soon as observed, regardless of whether the observation was made during any routine or ad hoc inspection and to ensure that the reports are made available to the principal contractor to become part of site records (Health & Safety File).

    - All Health and Safety Representatives (SHE-Reps) shall be appointed, act and report as per Section 17(1) and 18 of the Act, respectively.

#### Specific Supervision Responsibilities for Occupational Health and Safety

Several appointments or designations of responsible and/or competent people in specific areas of construction work are required by the Act and Regulations. The following competent appointments, where applicable, in terms of the Construction Regulations, are required to ensure compliance to the Act, Regulations and Safety Standards.

Required appointments as per the Construction Regulations:-

| Item | Regulation | Appointment | Action by |
| --- | --- | --- | --- |
| 1. | 4(1) (c) | Principal contractor for each phase or project | Employer |
| 2. | 5(3)(b) | Contractor | Principal Contractor |
| 3. | 5(11) | Contractor | Principal Contractor / Contractor |
| 4. | 6(1) | Construction supervisor | Principal Contractor / Contractor |
| 5. | 6(2) | Construction supervisor sub-ordinates | Principal Contractor / Contractor |
| 6. | 6(6) | Health and Safety Officer (full-time or part-time) | Principal Contractor / Contractor |
| 7. | 7(1) | Risk assessor | Principal Contractor / Contractor |
| 8. | 7(4) | Health and Safety trainer/instructor | Principal Contractor / Contractor |
| 9. | 8(1)(a) | Fall protection planner | Principal Contractor / Contractor |
| 10. | 10(a) | Formwork & support work supervisor | Principal Contractor / Contractor |
| 11. | 10(e) + (f) | Formwork & support work inspector | Principal Contractor / Contractor |
| 12. | 11(1) | Excavation supervisor | Principal Contractor / Contractor |
| 13. | 11(3)(b)(ii) (b) | Professional engineer or technologist | Principal Contractor / Contractor |
| 14. | 11(3)(k) | Excavation explosives expert | Principal Contractor / Contractor |
| 15. | 12(1) | Demolition work supervisor | Principal Contractor / Contractor |
| 16. | 12(2) + (3) | Demolition expert | Principal Contractor / Contractor |
| 17. | 12(11) | Demolition explosives expert | Principal Contractor / Contractor |
| 18. | 14(2) | Scaffold supervisor | Principal Contractor / Contractor |
| 19. | 15(1) | Suspended platform supervisor | Principal Contractor / Contractor |
| 20. | 15(2)(c) | Suspended platform compliance plan developer | Principal Contractor / Contractor |
| 21. | 15(8)(c) | Erection and maintenance suspended platform expert | Principal Contractor / Contractor |
| 22. | 15(13) | Outrigger inspector | Principal Contractor / Contractor |
| 23. | 16 (2) | Boatswain’s chair inspector | Principal Contractor / Contractor |
| 24. | 17(8)(a) | Material hoist inspector | Principal Contractor / Contractor |
| 25. | 18(1) | Batch plant supervisor | Principal Contractor / Contractor |
| 26. | 18(7) | Batch plant operator | Principal Contractor / Contractor |
| 27. | 19(2)(b) | Power tool inspector | Principal Contractor / Contractor |
| 28. | 19(2)(g)(i) | Power tool controller | Principal Contractor / Contractor |
| 29 | 20(f) | Tower crane operator | Principal Contractor / Contractor |
| 30. | 21(1)(d)(i) | Construction vehicle and mobile plant operator | Principal Contractor / Contractor |
| 31. | 21(1)(j) | Construction vehicle and mobile plant inspector | Principal Contractor / Contractor |
| 32. | 22(d) | Temporary electrical installations inspector | Principal Contractor / Contractor |
| 33. | 22(e) | Temporary electrical installations controller | Principal Contractor / Contractor |
| 34. | 26(a) | Stacking and storage supervisor | Principal Contractor / Contractor |
| 35. | 27(h) | Fire equipment inspector | Principal Contractor / Contractor |

The list may be used as a reference or tool to determine which components of the ACT and Regulations would be applicable to a particular site.  This list must not be assumed to be exclusive or comprehensive.

### Communication of the health and safety management programme

The communication principles to be applied should cover the following:

(a)	Occupational health and safety goals for the project and arrangements for monitoring and reviewing occupational health and safety performance.

- Arrangements for:

  - regular liaison between stake holders on site

  - consultation with the workforce

(c)	Selection and control of contractors (sub-contractors)

(d)	The exchange of occupational health and safety information between all stake holders (client, contractors (sub-contractors), designers, etc.)

- site security

- site induction, onsite training

- sanitation facilities

- fist-aid facilities

- reporting and investigation of accidents and incidents

- the production and approval of risk assessments and method statements

- site rules

- fire and emergency procedures

(e)	Reporting to the client, i.e. results and action of occupational health and safety inspection, incident investigations and minutes of safety committee meetings.

(f)	Reporting of incidents to the Department of Labour and compensation insurer where appropriate.

### Health and safety internal audit and inspection

The Client and/or its Agent on the Client’s behalf will be conducting periodic audits at times agreed with the principal contractor.  This audit will monitor and ensure that the principal contractor has implemented, adhering to and is maintain the agreed and approved health and safety plan.  Non – conformances will be highlighted for ratification to endure that the Client is not unduly exposed regarding the requirements as stipulated by the OHSACT.

A representative and/or the relevant health and Safety Representative(s) of the principal contractor must accompany the Client and/or it’s Agent on it’s behalf on all audits and inspections.

The principal contractor shall conduct his/her own inspections/internal audits at regular intervals.  Copies of these inspections/internal audits must be handed to the Client and/or its Agent.

Copies of Health and Safety Committee meeting minutes must be available to the Client and/or its Agent, reflecting recommendations made by the committee to the principal contractor for reference purposes.

### Health and safety incident/accident investigations and reporting

The principal contractor shall report all incidents/accidents where an employee is injured on duty to the extent that it is:

(a)	fatal

(b)	unconscious

(c)	loses a limb or part of

(d)	becomes ill

(e)	permanent physical defect

The principal contractor shall report all investigations regarding incidents, where:

(a)	major incident (safety, health or environmental) occurred

(b)	health or safety of any person was endangered.

(c)	hazardous/danger substance was spilled

(d)	uncontrolled release of any substance under pressure occurred

(e)	machinery or any part thereof fractured or failed resulting in flying, falling or uncontrolled moving objects.

(f)	uncontrolled running of machinery

## Health and safety training

The contents and syllabi of all training required by the Act and Regulations including any other related or relevant training as required must be made available to the Client if so required.

### Health and Safety Induction Training

All employees of the principal and any other contractor (sub–contractor) must be in possession of proof of health and safety induction training.

This training will include:

- risk identification

- safe work procedures (SWP)

- personal protective equipment (PPE) the use and the necessity thereof

- health and safety outside the workplace

- legal impact of health and safety matters

- introduction to the” Workmen’s Compensation Act”

- site security

- sanitation facilities

- first aid facilities

- fire and emergency procedures

- roll of health and safety representatives and the selection of them

- working hours and general working conditions

- incident reporting and procedures

- incident investigation and procedures

### Other Training

All employees in jobs requiring competency in terms of the Act and Regulations must be in possession of valid proof of training/experience to be credited for competency.

Specific occupational health and safety training requirements for this project includes:

- occupational health and safety representatives

- first aider – Level 1

- risk assessor

All operators, drivers and users of construction vehicles, mobile plant and other equipment must be in possession of valid proof of training and medical fitness certificates as required by the Act.

### Health and Safety Budget

To enable the Client to comply with Construction Regulation 4(1) (h), the principal contractor has to demonstrate to the Client that sufficient provision has been made to implement and managed the health and safety plan of the principle contractor.

A detailed schedule of costs therefore has to be included in the health and safety plan submitted. This includes the following subjects:

### Administration

- Compile a health and safety plan

- Notification of construction work

- Proof of good standing with the compensation fund or with a licensed compensation insurer

### Health and Safety Management Programme

- Appointment of a Safety Officer(s) (full-time or part-time)

- Appointment of a Health and Safety Consultant if required

- Other cost relating to the implementation and managing of the Health and Safety Management Programme

### Personal Protective Equipment (PPE)

The Principal contractor is required to identify the hazards in workplace and deal with them on an ongoing basis.  He/she must either remove them or, where impracticable take steps to protect workers and make it possible for them to work safely and without risk to health under the hazardous conditions.

Personal protective equipment should, however, be the last resort and there should always first 	be an attempt to apply engineering and other solutions to mitigating hazardous situations before the issuing of personal protective equipment is considered.

Where it is not possible to create an absolutely safe and healthy workplace the principal contractor is required to inform employees regarding this and issue, free of charge, suitable equipment to protect them from any hazards being present and that allows them to work safely and without risk to health in the hazardous environment.

It is a further requirement that the principal contractor maintains the said equipment, that he/she instructs and trains the employees in the use of the equipment and ensured that the prescribed equipment is used by the employee/s.

Employees do not have the right to refuse to use and/or wear the equipment prescribed by the employer and, if it is impossible for an employee to use or wear prescribed protective equipment through health or any other reason, the employee cannot be allowed to continue working under the hazardous condition(s), for which the equipment was prescribed but an alternative solution has to be found that may include relocating the employee.

The principal contractor may not charge any fee for protective equipment prescribed by him or her but may charge for equipment under the following conditions:

- Where the employee requests additional issue in excess of what is prescribed

- Where the employee has patently abused or neglected the equipment leading to early failure

      - Where the employee has lost the equipment

All employees shall, as a minimum, be required to wear the following personal protective equipment on the project:

- Head protection. e.g. hardhats

- Eye protection, e.g. goggles

- Hearing protection, e.g. earplugs

- Footwear, e.g. safety shoes

- Hand protection e.g. gloves

- Clothing, e.g. overalls

- Fall protection, e.g. harnesses (no monkey chain type harness will be allowed on site)

- Inhalation protection, e.g. dust masks

### Employee Site Facilities

- Adequate facilities shall be provided for employees on site, i.e.

- toilets for each sex

- hand wash facility

- drinking water

### Health and Safety Signage

- Access to the construction site must be controlled

- Health and safety signage to inform visiting public, employees, client, etc must be prominently displayed

- The following signage shall be displayed :

- No Unauthorised Entry

- Danger:  Construction Work in Progress

- Visitors to Report to Site Office

- Site Office

- Excavation work in progress

This signage must not be assumed to be exclusive or comprehensive

### Health and Safety Notice Board

A health and safety notice board (1000mm X 600mm) shall be erected on site with the following information displayed:

- Safety notices

- Safety awareness poster

- Site rules

- Information regarding contact numbers of

- Doctor

- Ambulance

- Hospital

- Fire Brigade

- Safety Officer

- Project Manager

- First aider

- Site evacuation map and zones

### Training

Provision to train employees regarding competency as required by the Act and Regulations, this will include the following activities:

- Health and Safety Representative(s)

Every contractor on site, with 20 or more employees must have a health and safety representative (Section 17 (1) ), and where the total number of all employees on site is less than 20, the principal contractor shall ensure that at least one health and safety representative is available on site.

- First Aider

Every contractor with ten or more employees shall have a person with a valid certificate of training in First Aid – Level 1. The first aider shall permanently be available on site

- Risk Assessor

Every contractor performing construction work shall before commencement of any construction work and during construction work, cause a risk assessment to be performed by a competent person.

The training provision should be read in conjunction with the ACT, Construction Regulations or any other Regulation and Safety Standards which were or will be promulgated under the Act or incorporated into the Act and be in force or come into force during the effective duration of the project.

### First Aid Station

Every contractor with five or more employees shall have a first aid box on site.  The first aid box shall contain suitable first aid equipment which includes at least the equipment stipulated in the Annexure of the General Safety Regulations.

A stretcher and cervical collar shall be available at the first aid station.

## Logbooks and Registers

The following logbooks and registers shall be implemented and managed in terms of the Act and Regulations:

### Health and Safety Appointments

The health and safety appointments and proof of competency shall be as stipulated in the Act and Regulations

### Logbooks and Registers

The following logging shall be carried out and recorded for the initial start of the project:

(a)	Fire extinguishing

(b)	First aid

        - incident/accident reporting

        - incident/accident investigation

(c)	Portable electrical equipment

(d)	Scaffolding

(e)	Ladders

(f)	Gas welding and cutting

(g)	Excavation

(h)	Construction vehicles

(i)	Earth leakage testing

(j)	Information signage

(k)	Health and Safety inspections by Health and Safety Representatives

(i)	Monthly Health and Safety Committee meeting minutes

(m)	Attendance register

        - safety induction training

        - toolbox talks

Other logbooks/registers shall be implemented during duration of the project as stipulated by the Act and Regulations.

### Record Keeping Responsibilities

The following record keeping responsibilities has been identified for this project:

| ITEM | CONSTRUCTION REGULATION No. | RECORD TO BE KEPT | RESPONSIBLE PERSON |
| --- | --- | --- | --- |
| 1 | 3(3) | Notification to Provincial Director – Annexure A Available on site | Principal Contractor |
| 2 | 4(3) | Copy of Principal Contractor’s Health & Safety Plan Available on site | Client |
| 3 | 5(6) | Copy of Principal Contractor’s Health & Safety Plan As well as each Contractor’s Health & Safety Plan Available on request | Principal Contractor |
| 4 | 5(7) | Health and Safety File opened and kept on site (including all documentation required i.t.o OHSA & Regulations Available on request | Principal Contractor |
| 5 | 5(8) | Consolidated Health and Safety File handed to Client on completion of Construction work. To include all documentation required i.t.o. OHSA & Regulations and records of all drawings, designs, materials used and similar information on the structure | Principal Contractor |
| 6 | 5(9) | Comprehensive and Updated List of all Contractors on site, the agreements between the parties and the work being done Included in Health and Safety file and available on request | Principal Contractor |
| 7 | 6(7) | Keep record on the Health and Safety File of the Input by Construction Safety Officer [CR 6(7)] at design stage or on the Health and Safety Plan | Contractor |
| 8 | 7(2) | Risk Assessment – Available on site for inspection | Contractor |
| 9 | 7(9) | Proof of Health and Safety Induction Training | Every Employee on site |
| 10 | 8(3) | Construction Supervisor [CR 6(1)] has latest updated version of Fall Protection Plan [CR 8(1)] | Contractor |
| 11 | 9(2)(b) | Inform contractor in writing of dangers and hazards relating to construction work | Designer of Structure |
| 12 | 9(3) | All drawings pertaining to the design of structure On site available for inspection | Contractor |
| 13 | 11(3)(h) | Record of excavation inspection – On site available on request | Contractor |
| 14 | 21(1)(j) | Findings of daily inspections (prior to use) of Construction Vehicles and Mobile Plant | Contractor |
| 15 | 22(d) | Record of temporary electrical installation inspections[once a week] and electrical machinery [daily before use] in a register and kept on site | Contractor |

# OCCUPATIONAL HEALTH AND SAFETY FILE

## Contents of the Occupational health and Safety file

The minimum contents of the health and safety file shall include:

(a)	Health and safety plan

(b)	Brief description of work

(c)	Health and safety specifications provided by the Client

(d)	Risk assessment

(e)	Safe work procedures

(f)	Notification of construction work

(g)	Letter of good standing with the compensation fund or approved compensation insurer

(h)	Program of construction work

(i)	Minutes/audits of health and safety inspections and meetings

(j)	Documentation supporting the managing of the health and safety programme

(k)	As-built drawings

(l)	Arrangements with contractors (sub-contractors) and/or mandatories.

# PROJECT STANDARDS FOR OCCUPATIONAL HEALTH AND SAFETY

## Administrative and legal

| OHS Act Section/ Regulation | Subject | Standard |
| --- | --- | --- |
| Construction. Regulation 3 | Notice of carrying out construction work | Department of Labour notified. Copy of notice available on site |
| General Admin. Regulation 4 | *Copy of OH&S Act (Act 85 of 1993) | Updated copy of Act & Regulations on site. Readily available for perusal by employees. |
| COID Act Section 80 | *Registration with compensation insurer | Written proof of registration / letter of good standing available on site |
| Construction. Regulation 4 & 5(1) | H&S specification and programme | H&S Specifications received from client and / or its agent on its behalf OH&S programme developed & updated regularly. |
| Section 8(2) (d) Construction. Regulation 7 | *Hazard identification and risk assessment | Hazard identification carried out / recorded. Risk Assessment and - plan drawn up / updated. RA plan available on site employees / sub-contractors informed / trained. |
| Section 16(2) | *Assigned duties (Managers) | Responsibility of complying with the OH&S Act assigned to other person/s by CEO. |
| Construction. Regulation 6(1) | Designation of person responsible on site | Competent person appointed in writing as construction supervisor with job description. |
| Construction. Regulation 6(2) | Designation of assistant for above | Competent person appointed in writing as construction supervisor with job description. |
| Section 17 & 18 General administrative. Regulations 6 & 7 | *Designation of Health and Safety representatives | More than 20 employees – one H&S representative, one additional H&S representative for each 50 employees or part thereof. Designation in writing, period and area of responsibility specified in terms of GAR 6 & 7. Meaningful H&S representatives reports. Reports actioned by management |
| Section 19 & 20 General administrative. Regulations 5 | *Health & Safety committee/s | H&S Committee/s established. All H&S representatives shall be members of H&S committees. Additional members are appointed in writing. Meetings held monthly, minutes kept. Actioned by Management. |
| Section 37(1) & (2) | *Agreement with mandataries / (sub-) contractors | Written agreement with (sub-) contractors. List of (sub-) contractors displayed. Proof of registration with compensation insurer / letter of good standing. Construction supervisor designated. Written arrangements re. H&S representatives and H&S committee. Written arrangements re. First Aid. |
| Section 24 & General administrative. Regulation 8 COID Sections 38, 39 & 41 | *Reporting of incidents (Department of Labour) | Incident Reporting Procedure displayed. Incident Reporting Procedure displayed. All incidents in terms of Section 24 reported to the Provincial Director, Department of Labour, within 3 days. (Annexure 1)(WCL 1 or 2) and to the client and/or its agent on its behalf. Cases of occupational disease reported. Copies of reports available on site. Records of First Aid injuries to be kept. |
| General administrative. Regulation 9 | *Investigation and recording of incidents | All injuries which resulted in the person receiving medical treatment other than first aid, recorded and investigated by investigator designated in writing. Copies of reports (Annexure 1) available on site and tabled at H&S committee meeting. Action taken by site management |
| Construction. Regulation 8 | Fall prevention and protection | Competent person appointed to draw up and supervise the fall protection plan. Proof of appointees competence available on site. Risk assessment carried out for work at heights fall protection plan drawn up / updated. Available on site. |
| Construction. Regulation 8(5) | Roof work | Competent person appointed to plan and supervise roof work. Proof of appointees competence available on site. Risk assessment carried out. Roof work plan drawn up / updated. Roof work inspect before each shift. Inspection register kept. Employees medically examined for physical and psychological fitness. Written proof on site. |
| Construction. Regulation 9 | Structures | Information re. the structure being erected received from the designer including: - geo-science technical report where relevant - the design loading of the structure - the methods and sequence of construction - anticipated dangers / hazards / special measures to construct safely Risk assessment carried out. Method statement drawn up. All above available on site. Structures inspected before each shift. Inspections register kept. |
| Construction. Regulation 10 | Formwork and support work | Competent person appointed in writing to supervise erection, maintenance, use and dismantling of support and formwork. Design drawings available on site. Risk Assessment carried out. Support and formwork inspected: - before use/inspection - before pouring of concrete - weekly whilst in place - before stripping / dismantling - Inspection register kept |
| Construction Regulation 14 | Scaffolding | Competent persons appointed in writing to: * erect scaffolding (Scaffold Erector/s) * act as Scaffold Team Leaders * inspect Scaffolding weekly and after inclement weather (Scaffold Inspector/s) Written Proof of Competence of above appointees available on Site Copy of SABS 085 available on Site Risk Assessment carried out Inspected weekly/after bad weather. Inspection register/s kept |
| Construction. Regulation 11 | Excavations | Competent person/s appointed in writing to supervise and inspect excavation work. Written proof of competence of above appointee/s available on site. Risk Assessment carried out. Inspected: - before every shift - after any blasting - after an unexpected fall of ground - after any substantial damage to the shoring - after rain. Inspections register kept |
| Construction. Regulation 12 | Demolition work | Competent person/s appointed in writing to supervise and control demolition work. Written proof of competence of above appointees available on site. Risk Assessment carried out. Engineering survey and method statement available on site. Inspections to prevent premature collapse carried out by competent person before each shift. Inspections register kept. |
| Construction. Regulation 19 | Explosive powered tools | Competent person appointed to control the issue of the explosive powered tools & cartridges and the service, maintenance and cleaning. Register kept of above. Empty cartridge cases/nails/fixing bolts returns recorded Cleaned daily after use Work areas are demarcated! |
| Construction. Regulation 22 / Electrical machinery. Regulation 9 & 10 / Electrical regulations. | *Inspection and maintenance of electrical installation and equipment (including portable electrical tools) | Competent person appointed in writing to inspect / test the installation and equipment. Written Proof of competence of above appointee available on site. Inspections: * Electrical Installation & equipment inspected after installation, after alterations and quarterly. Inspection Registers kept Portable electric tools, electric lights and extension leads must be uniquely identified/numbered. Weekly visual inspection by User/Issuer/Storeman. Registers kept |
| Construction. Regulation 26 / General Safety Reg. 8(1)(a) | *Designation of stacking and storage supervisor | Competent person/s with specific knowledge and experience designated to supervise all stacking and storage. Written proof of competence of above appointee available on site. |
| Construction. Regulation 27 Environmental. Regulation 9 | *Designation of a person to co-ordinate emergency planning and fire protection | Person/s with specific knowledge and experience designated to co-ordinate emergency contingency planning and execution and fire prevention measures Emergency evacuation plan developed: - Drilled / practiced - Plan and records of drills / practices available on site All Fire extinguishing equipment identified and on registered on a register. Inspected weekly. Inspection Register kept. Serviced annually. |
| General Safety. Regulation 2 | Personal safety equipment (PSE) | PSE Risk Assessment carried out. Items of PSE prescribed/use enforced. Records of issue kept. Undertaking by employee to use/wear PSE. PSE remain property of employer, not to be removed from premises GSR 2(4) |
| General Safety. Regulation 3 | *First aid | Every workplace provided with sufficient number of first aid boxes. (Required where 5 persons or more are employed). First Aid freely available. Equipment as per the list in the OH&S Act. One qualified first aider appointed for every 50 employees. (Required where more than 10 persons are employed). List of first aid officials, certificates, name of person/s in charge of first aid box/es displayed. Location of first aid box/es clearly indicated. Signs instructing employees to report all injuries/illness including first aid injuries |
| General Safety. Regulation 9 | *Inspection and use of welding / flame cutting equipment | Competent person/s with specific knowledge and experience designated to inspect electric arc, gas welding and flame cutting equipment. Written proof of competence of above appointee available on site. All new vessels checked for leaks, leaking vessels NOT taken into stock but returned to supplier immediately. Purpose made storage available for full and empty vessels. Equipment identified / numbered and entered into a register. Equipment inspected weekly. Inspection register kept separate. |
| Hazardous chemical substances. Construction. Regulation 23 | * Control of storage and usage of HCS and flammables | Competent person/s with specific knowledge and experience designated to control the storage and usage of HCS (including flammables). Written proof of competence of above appointee available on site. Risk assessment carried out. Register of HCS kept/used on site. Separate/purpose made storage available for full and empty containers. |
| Vessels under pressure. Regulation, 1996 | Vessels under pressure (VUP) | Competent Person/s with specific knowledge and experience designated to supervise the use, storage, maintenance, statutory inspections & testing of VUP’s Written Proof of Competence of above appointee available on Site Risk Assessment carried out Certificates of Manufacture available on Site Register of VUP's on Site Inspections & Testing by Approved Inspection Authority (AIA): - after installation/re-erection or repairs. - every 36 months. - register / log kept of inspections, tests. - modifications and repair. |
| Construction. Regulation 21 | Construction. Vehicles and earth moving equipment | Operators/Drivers appointed to: - carry out a daily inspection prior to use. - drive the vehicle / plant that he/she is competent to operate / drive. Written proof of competence of above appointee available on site. Record of daily inspections kept. |
| General Safety. Regulation 13A | *Inspection of ladders | Competent person appointed in writing to inspect ladders. Ladders inspected at arrival on site and weekly there after. Inspections register kept. Application of the types of ladders (wooden, aluminum etc.) regulated by training and inspections and noted in register. |

## Training

| SUBJECT | STANDARD |
| --- | --- |
| Company OH&S Policy Section 7(1) Company/site OH&S Rules Section 13(a) Induction and task safety training Section 13(a) General OH&S training Section 13(a) Occupational Health and Safety Promotion | Policy signed by CEO and published / circulated to employees. Policy displayed on employee notice boards. Management and employees committed. Rules published. Rules displayed on employee notice boards. Rules issued and employees effectively informed or trained: written proof. Follow-up to ensure employees understand/adhere to the policy and rules. All new employees receive OH&S induction training. Training includes Task Safety Instructions. Employees acknowledge receipt of training. Follow-up to ensure employees understand/adhere to instructions. All current employees receive specified OH&S training: written proof. Operators of plant & equipment receive specified training. Follow-up to ensure employees understand/adhere to instructions. Incident experience board indicating e.g. - No of hours worked without an injury. - No of days worked without an injury. Mission, vision and goal. Star grading- board kept up to date. Safety Posters displayed & changed regularly. Employee notice board for OH&S notices. Site OH&S competition. Participation in regional OH&S competition Suggestion scheme |

## Public safety, security and emergency

| SUBJECT | STANDARD |
| --- | --- |
| *Notices and signs | Notices and signs at entrances / along perimeters indicating “No Unauthorized Entry”. Notices and signs at entrance instructing visitors and non - employees what to do, where to go and where to report on entering the site/yard with directional signs. e.g. “Visitors to report to Office”. Notices and signs posted to warn of overhead work and other hazardous activities e.g. General warning signs. |
| Site safeguarding | Nets, canopies, platforms, fans etc. to protect members of the public passing / entering the site. |
| *Security measures | Access control measures/register in operation. Security patrols after hours during weekends and holidays. Sufficient lighting after dark. Guard has access to telephone/ mobile/other means of emergency communication. |

## Personal Protective Equipment

| SUBJECT | STANDARD |
| --- | --- |
| *Emergency drill and evacuation | Adequate No. of employees trained to use fire fighting Equipment. Emergency evacuation plan available, displayed and practiced. |
| * PPE needs analysis | Need for PPE identified and prescribed in writing. |
|  | PPE remain property of Employer, not to be removed from premises GSR. 2(4) |
| * Head Protection | All persons on site wearing Safety Helmets including Sub-contractors and |
|  | Visitors (where prescribed) |
| * Foot Protection | All employees on site wearing Safety Footwear including Gumboots for concrete |
|  | / wet work and non-slip shoes for roof work. |
|  | Visitors to wear same upon request or where prescribed |
| * Eye and Face Protection | Eye and Face (also Hand and Body) Protection (Goggles, Face Shields, Welding |
|  | Helmets etc.) used when operating the following: |
|  | Jack/ Kango Hammers |
|  | * Angle/ Bench Grinders |
|  | * Electric Drills (Overhead work into concrete / cement / bricks |
|  | * Explosive Powered tools |
|  | * Concrete Vibrators / Pokers |
|  | * Hammers & Chisels |
|  | * Cutting / Welding Torches |
|  | * Cutting Tools and Equipment |
|  | * Guillotines and Benders |
| * Eye and Face Protection | * Shears |
|  | * Sanders and Sanding Machines |
|  | * C02 and Arc Welding Equipment |
|  | * Skill/Bench Saws |
|  | * Spray Painting Equipment etc |
| * Hearing Protection | Hearing Protectors (Muffs, Plugs etc.) used when operating the following: |
|  | * Jack / Kango Hammers |
|  | * Explosive Powered Tools |
|  | * Wood/Aluminium Working Machines e.g. saws, planers, routers |
| * Hand Protection | Protective Gloves worn by employees handling / using: |
|  | * Cement / Bricks / Steel / Chemicals |
|  | * Welding Equipment |
|  | * Hammers & Chisels |
|  | * Jack / Kango Hammers etc. |

## Personal Protective Equipment (Continued).

| SUBJECT | STANDARD |
| --- | --- |
| * Respiratory Protection | Suitable/efficient prescribed Respirators worn correctly by employees handling / using: * Dry cement * Dusty areas * Hazardous chemicals * Angle Grinders * Spray Painting etc. |
| * Fall Protection | Suitable Safety Belts / Fall Arrest Equipment correctly used by persons working |
|  | on / in unguarded, elevated positions e.g.: |
|  | * Scaffolding |
|  | * Riggers |
|  | * Lift shafts |
|  | * Edge work |
|  | * Ring beam edges etc. |
|  | Other methods of fall prevention applied e.g. catch nets |
| * Protective Clothing | All jobs requiring protective clothing (Overalls, Rain Wear, Welding Aprons etc.) Identified and clothing worn. |
| * PPE Issue & Control | Identified Equipment issued free of charge. |
|  | All PPE maintained in good condition. (Regular checks). |
|  | Workers instructed in the proper use & maintenance of PPE. |
|  | Commitment obtained from wearer accepting conditions and to wear the PPE. |
|  | Record of PPE issued kept on H&S File. |
|  | PPE remain property of Employer, not to be removed from premises GSR 2(4) |
| * Scrap Removal System | All items of Scrap/Unusable Off-cuts/Rubble and redundant material removed from working areas on a regular basis. (Daily) Scrap/Waste removal from heights by chute/hoist/crane. |
| Scrap Removal System | Nothing thrown/swept over sides. |
|  | Scrap disposed of in designated containers/areas |
|  | Removal from site/yard on a regular basis. |
| Stacking | Stacking: |
|  | * Stable, on firm level surface/base. |
|  | * Prevent leaning/collapsing |
|  | * Irregular shapes bonded |
|  | * Not exceeding 3x the base |
|  | * Stacks accessible |
|  | * Removal from top only. |

## Personal Protective Equipment (Continued).

| Storage | Storage: |
| --- | --- |
|  | * Adequate storage areas provided. |
|  | * Functional — e.g. demarcated storage areas/racks/bins etc. |
|  | * Special areas identified and demarcated e.g. |
|  | flammable gas, cement etc. |
|  | * Neat, safe, stable and square. |
|  | * Store/storage areas clear of superfluous material. |
|  | * Storage behind sheds etc. neat/under control. |
|  | * Storage areas free from weeds, litter etc. |
|  | and kept to a minimum in the work areas. |
| *Waste Control / Reclamation | (Nails removed/bent over in re-usable timber). Issue of hardware/nails/screws/cartridges etc. controlled and return of unused items monitored. controlled and return of unused items monitored. |
| Sub-contractors (Housekeeping) | Sub-contractors required to comply with Housekeeping requirements |

## Working at Heights (including roof work)

| SUBJECT | STANDARD |
| --- | --- |
| Openings | Unprotected openings adequately guarded/fenced/barricaded/catch nets installed |
|  | Roof work discontinued when bad/hazardous weather |
|  | Fall protection measures (including warning notices) when working close to edges |
|  | or on fragile roofing material |
|  | Covers over openings in roof of robust construction/secured against displacement |

## Scaffolding/Formwork/Support Work

| SUBJECT | STANDARD |  |
| --- | --- | --- |
| Access/System Scaffolding | Foundation firm / stable |  |
|  | Sufficient bracing. |  |
|  | Tied to Structure/prevented from side or cross movement |  |
|  | Platform boards in good condition/sufficient/secured. |  |
|  | Handrails and toe boards provided. |  |
|  | Access ladders / stairs provided. |  |
|  | Area/s under scaffolding tidy. |  |
|  | Safe/unsafe for use signs |  |
|  | Complying with OH&S Act/SABS 085 |  |

## Scaffolding/Formwork/Support Work (Continued)

| SUBJECT | STANDARD |
| --- | --- |
| Free Standing Scaffolding | Foundation firm / stable. Sufficient bracing. Platform boards in good condition/sufficient/secured. Handrails and toe boards provided. Access ladders / stairs provided. Area/s under scaffolding tidy. Safe/unsafe for use signs. Height to base ratio correct. Outriggers used /tied to structure where necessary. Complying with OH&S Act/SABS 085. |
| *Mobile Scaffolding | Foundation firm / stable. |
|  | Sufficient bracing. |
|  | Platform boards in good condition/sufficient/secured. |
|  | Handrails and toe boards provided. |
|  | Access ladders / stairs provided. |
|  | Area/s under scaffolding tidy. |
|  | Safe/unsafe for use signs |
|  | Wheels / swivels in good condition |
|  | Brakes working and applied. |
|  | Height to base ratio correct. |
|  | Outriggers used where necessary |
|  | Complying with OH&S Act/SABS 085 |
| Formwork/ Support Work | All components in good condition. |
|  | Foundation firm / stable. |
|  | Adequate bracing/stability ensured. |
|  | Good workmanship / uprights straight and plumb. |
|  | Good cantilever construction. |
|  | Safe access provided. |
|  | Areas under support work tidy. |
| Special Scaffolding | Same standards as for system scaffolding. |
|  | Special Scaffolding e.g. Cantilever, Jib and Truss scaffolds erected to an acceptable standard and inspected by specialists. |
| Edges & Openings | Edges barricaded to acceptable standards. |
|  | Manhole openings covered / barricaded. |
|  | Stairs provided with handrails. |
|  | Lift shafts barricaded / fenced off. |

## Ladders

| SUBJECT | STANDARD |
| --- | --- |
| *Physical Condition/Use & Storage | Stepladders - hinges/stays/braces/stiles in order. |
|  | Extension ladders - ropes/rungs/stiles/safety latch/hook in order. |
|  | Extension/ Straight ladders secured or tied at the bottom / top. |
|  | No joined ladders used |
|  | Wooden ladders are never painted except with varnish |
|  | Aluminium ladders NOT to be used with electrical work |
|  | All ladders stored on hooks / racks and not on ground. |
|  | Ladders protrude 900 mm above landings / platforms / roof. |
|  | Fixed ladders higher than 5 m have cages/Fall arrest system |

## Electricity

| SUBJECT | STANDARD |
| --- | --- |
| *Electrical Distribution | Colour coded / numbered / symbolic sign displayed. |
|  | Area in front kept clear and unobstructed, |
| Boards & Earth Leakage | Fitted with inside cover plate / openings blanked off/no exposed “live” conductors / terminals/Door kept close |
|  | Switches / circuit breakers identified. |
|  | Earth leakage protection unit fitted and operating. |
|  | Tested with instrument: Test results within 15 —30 milliamps Aperture/Opening/s |
|  | provided for the plugging in and removal of extension leads without the need to open the door |
|  | Apertures and openings used for extension leads to be protected against the elements and especially rain |
| *Electrical Installations & Wiring | Temporary wiring / extension leads in good condition / no bare or exposed wires. |
| *Electrical Installations & Wiring | Earthing continuity / polarity correct: |
|  | Looking at the open connectors to connect the wiring, the word “Brown” has the letter ‘R’ in it, so the b’R’own wire connects to the "R‘ight hand connector. “Blue” has the letter ‘L’ in it, so the b’L’ue wire connects to the L'eft‘ hand connector. |
|  | Cables protected from mechanical damage and moisture. |
|  | Correct loading observed e.g. no beating appliance used from lighting circuit etc. |
|  | Light fittings/lamps protected from mechanical damage/moisture. |
|  | Cable arrestors in place and used inside plugs |

## Electricity (Continued)

| SUBJECT | STANDARD |
| --- | --- |
| *Physical condition of Electrical Appliances & Tools | Electrical Equipment and Tools: (includes all items plugging in to a 16 Amp supply socket) |
|  | Insulation / casing in good condition. |
|  | Earth wire connected/intact where not of double insulated design |
|  | Double insulation mark indicates that no earth wire is to be connected. |
|  | Cord in good condition/no bare wires/secured to machine & plug. |
|  | Plug in good condition, connected correctly and correct polarity. |

## Excavations

| SUBJECT | STANDARD |
| --- | --- |
| Excavations deeper than 1.5m | Shored / Braced to prevent caving / falling in. |
|  | Provided with an access ladder. |
|  | Excavations guarded/barricaded/lighted after dark in public areas |
|  | Soil dumped at least 1 m away from edge of excavation |
|  | On sloping ground soil dumped on lower side of excavation |
|  | All excavations are subject to daily inspections |

## Tools

| SUBJECT | STANDARD |
| --- | --- |
| *Hand Tools | Shovels / Spades /Picks: |
|  | * Handles free from cracks and splinters |
|  | * Handles fit securely |
|  | * Working end sharp and true |
|  | Hammers: |
|  | * Good quality handles, no pipe or reinforcing steel handles. |
|  | * Handles free from cracks and splinters |
|  | Handles fit securely |
|  | Chisels: |
|  | * No mushroomed heads / heads chamfered |
|  | * Not hardened |
|  | * Cutting edge sharp and square |
|  | Saws: |
|  | * Teeth sharp and set correctly |
|  | * Correct saw used for the job |

## Tools (Continued)

| SUBJECT | STANDARD |
| --- | --- |
| *Explosive Powered Tools. | Only used by trained / authorised personnel. |
|  | Prescribed warning signs placed / displayed where tool is in use. |
|  | Work area must be properly isolated/demarcated during use of tool. |
|  | Inspected at least monthly by competent person and results recorded. |
|  | Issue and return recorded including cartridges / nails and unused cartridges / nails / empty shells recorded. |
|  | Cleaned daily after use. |

## Transport & Materials Handling Equipment

| SUBJECT | STANDARD |
| --- | --- |
| *Site Vehicles | All Site Vehicles, Dumpers, Bobcats, Loaders etc: checked daily before use by driver / operator. |
|  | Inventory of vehicles used/operated on site |
|  | Inspection by means of a checklist / results recorded. |
|  | No persons riding on equipment not designed or designated for passengers. |
|  | Site speed limit posted, enforced and not exceeded. |
|  | Drivers / Operators trained / licensed and carrying proof |
|  | No unauthorised persons allowed to drive / operate equipment. |

## Site Plant and Machinery

| SUBJECT | STANDARD |  |
| --- | --- | --- |
| Brick Cutting Machine | Operator Trained. |  |
|  | Only authorised persons use the machine. |  |
|  | Emergency stop switch clearly marked and accessible. |  |
|  | Area around the machine dry and slip/trip free/clear of off-cuts |  |
|  | All moving drive parts guarded/electrical supply cable protected |  |
|  | Operator using correct PPE - eye/face/hearing/foot/hands/body. |  |
| *Electric Arc Welder | Welder Trained. |  |
|  | Only authorised / trained persons use welder. |  |
|  | Earth cable adequately earthed to work. |  |
|  | Electrode holder in good condition/safe |  |
|  | Cables, clamps & lugs/connectors in good condition. |  |
|  | Area in which welding machine is used is dry/protected from wet. |  |
|  | Welder using correct PPE - eye/face/foot/body/respirator. |  |
|  | Correct transparent screens & warning signs placed |  |

## Site Plant and Machinery (Continued)

| SUBJECT | STANDARD |
| --- | --- |
| *Woodworking Machines | Operators Trained. Only authorised persons use machines, Provided with guards. Guards used. Operators using correct PPE - eye/face/feet/hearing Circular saws strictly operated according to prescribed methods and settings Only prescribed saw blades (cross-cut, ripping blade, smooth cut, aluminium) shall be used for various applications |
| *Compressors | Relief valves correctly set and locked / sealed. |
|  | Maximum Safe Working Pressure (MSWP) indicated on face of pressure gauge, not on glass cover. |
|  | All drives adequately guarded. |
|  | Receiver/lines drained daily |
|  | Hoses good condition/clamped, not wired |
|  | Compressed air NEITHER used to dust off clothing/ PPE/ and work areas NOR on bare skin |
| Concrete Mixer / Batch Plant | Top platform provided with guardrails. |
|  | Dust abatement methods in use. |
|  | Operators using correct PPE – eye / hands / respirators. |
| Concrete Mixer / Batch Plant | All moving drive parts guarded. |
|  | Emergency stops identified / indicated and accessible. |
|  | Area kept clean/dry/and free from tripping and slipping hazards. |
|  | Operators overseer identified and crane signals displayed and used. |
| *Gas Welding Flame & Cutting Equipment - | Only authorized/trained persons use the equipment. Torches and gauges in good condition. Flashback arrestors fitted at cylinders and gauges. Hoses in good condition/correct type/all connections with clamps Cylinders stored, used and transported in upright position, secured in trolley/ cradle/to structure. All cylinders regularly checked for leaks, leaking cylinders returned immediately. Fire prevention/control methods applied/hot work permits |

## Plant & Storage Yards/Site Workshops Specifics

| SUBJECT | STANDARD |  |
| --- | --- | --- |
| Section 8(2) (1) General Machinery Regulation 2(1): Supervision of the Use & Maintenance of Machinery | Person/s with specific knowledge and experience designated in writing to Supervise the Use & Maintenance of Machinery |  |
|  | Critical items of Machinery identified/numbered/placed on |  |
|  | register/inventory C4-28 |  |
|  | Inspection/maintenance schedules for abovementioned |  |
|  | Inspections/maintenance carried out to above schedules |  |
|  | Results recorded |  |

## Plant & Storage Yards/Site Workshops Specifics (Continued)

| SUBJECT | STANDARD |
| --- | --- |
| General Machinery Regulation 9(2): Notices re. Operation of Machinery | Schedule D Notice posted in Work areas |
| Vessels under Pressure | Person/s with specific knowledge and experience designated in writing |
|  | to Supervise the Use & Maintenance of VuP’s |
| Regulation 13(1)(b): Supervision of the Use & Maintenance of Vessels under Pressure | VuP’s identified/numbered/placed on register/Manufacturers plate intact |
|  | Inspection/maintenance schedules for abovementioned |
|  | Inspections/maintenance carried out to above schedules |
|  | Results recorded/Test certificates available |
| Lock-out Procedure Ergonomics | Lock-out procedure in operation |
|  | Ergonomics survey conducted — results on record |
|  | Survey results applied |
| Demarcation & Colour Coding | Demarcation principles applied |
|  | All services, pipes, electrical installation, stop-start controls, emergency controls etc. colour coded to own published or SABS standard |
|  | Employees trained to identify colour coding |
| Portable & Bench Grinders | Area around grinder clear/trip/slip free. Bench grinders mounted securely/grinder generally in good condition/No excessive vibration. On/Off switch/button clearly demarcated/accessible. Adequate guards in place. Tool rest— secure/square/max. 2 mm gap, perpendicular to drive shaft. Stone/disk - correct type and size/mounted correctly/dressed. Use of Eye protection enforced. |
| Battery Storage & Charging | Adequately ventilated, ignition free room/area/no smoking sign/s. Batteries placed on rubber/wooden surface. Emergency shower/eye wash provided. No acid storage in area. Prescribed methods in place and adhered to when charging batteries. |
| Ancillary Lifting Equipment | Chain Blocks/Tirfors/jacks/mobile gantries etc.identified/Numbered on register. Chains in good condition/links no excessive wear/checked daily. Lifting hooks — throat pop marked/safety latch fitted. SWL/MML marked/displayed. |
| Presses / Guillotines / Shears | Only operated by trained/authorized persons. Interlocks/lock-outs fitted/PPE worn or used at all times |

## Workplace Environment, Health and Hygiene

| SUBJECT | STANDARD |
| --- | --- |
| *Lighting | Adequate lighting in places where work is being executed e.g. stairwells and basements. Light fittings placed / installed causing no irritating/blinding glare. Stroboscopic effect eliminated (not only reduced) where moving objects or machinery is used. |
| *Ventilation | Adequate ventilation / extraction / exhausting in hazardous areas e.g. chemicals / adhesives / welding / petrol or diesel/ motors running and in confined spaces / basements. |
| *Noise | Tasks identified where noise levels exceeds 85 dB at any one time. All reasonable steps taken to reduce noise levels at the source. Hearing protection used where noise levels could not be reduced to below 85 dB. |
| *Heat Stress | Measures in place to prevent heat exhaustion in heat stress problem areas e.g. steel decks, when the WBGT index reaches 30. (See Environmental Regulation 4). Cold drinking water readily available at all times. |
| *Ablutions | Sufficient hygiene facilities provided - 1 toilet per 30 employees (National Building Regulations prescribe chemical toilets for Construction sites) |
|  | Toilet paper available. |
|  | Sufficient showers provided. |
|  | Facilities for washing hands provided |
|  | Soap/cleaning agent available for washing hands |
|  | Means of drying hands available |
|  | Lock-up changing facilities/area provided. |
|  | Ablution facilities kept hygienic and clean. |
| *Eating/Cooking Facilities | Adequate storage facilities provided. |
|  | Weather protected eating area provided, separate from changing area |
|  | Refuse bins with lids provided. |
|  | Facilities kept clean and hygienic. |
| *Pollution of Environment | Measures in place to minimize dust generation. |
|  | Accumulation or littering of empty cement pockets, plastic wrapping / bags, packing materials etc prevented. |
| *Pollution of Environment | ‘Spillage / discarding of oil, chemicals and dieseline into storm water and other |
|  | drains or into existing or newly dug holes/cavities on site expressly prohibited. |
| *Hazardous Chemical Substances | All substances identified and list available e.g. acids, flammables, poisons etc. |
|  | Material Safety Data Sheets (MSDS) indicating hazardous properties and |
|  | emergency procedures in case of incident on file and readily available. |
|  | Substances stored safely. |

# RISK ASSESSMENT PROGRAMME

## Methodology

Prior to drafting the Health and Safety Plan, and in consideration of the information contained here-in, the contractor shall set up a Risk Assessment Programme to identify and determine the scope and details of any risk associated with any hazard at the construction site, in order to identify the steps needed to be taken to remove, reduce or control such hazard. This Risk Assessment and the steps identified will be the basis or point of departure for the Health and Safety Plan.

Due to the wide scope and definition of construction work, every construction activity and site will be different, and circumstances and conditions may change even on a daily/hourly basis.  Therefore, due caution is to be taken by the principal contractor when dealing with the identification of hazards and risks.

The risk assessment to be conducted for this project shall be defined as workplace risk assessment. Workplace risk assessment is a set of ongoing management and engineering activities of the project, aimed at ensuring that the health, safety and environmental hazards/risks of the project are identified, understood and minimised to a reasonable, achievable and tolerable level.

The following guide can be helpful to conduct and manage risk assessments:

Step 1:	Identifying the current as well as emerging hazard, risks or exposures.

Step 2:	Aim to identify major hazards, don’t waste time on the minor and detail except if such hazard has the potential be repeat itself on a frequent basis.

Step 3:	Involve as many people as possible in the ongoing risk assessment process especially those at risk.

Step 4:	Gather all the information and analyse it.

Step 5:	Look at what actually could or has occurred including non-routine operations.

Step 6:	Use a systematic approach to ensure all hazards are adequately addressed.

Step 7:	Assess the risks identified or the risk that occurred by taking into account the effectiveness of current as well as controls under consideration.

Step 8:	Ensure the process is practical, realistic, cost and business effective.

Step 9:	Always record the assessment in writing including i.e. assumptions, date and why a particular decision has been made.

## Contents of the risk assessment programme

All risk assessments shall be conducted in terms of an acceptable methodology, prior to commencement of work, according to the provisions of CR7 and should cover at least the following were applicable

- Movement of construction vehicles

- Bulk earth works

- All work near overhead power lines and underground cables

- Locating underground cables/existing services

- Hand excavation of trenches

- Mechanical excavation of trenches

- All work carried out inside trenches, including compacting, pipe laying, backfilling etc.

- Temporary stockpiling and removal of excavated material

- Transporting material

- Cutting into existing sewage line

- All work carried out from scaffolding

- Exposure to raw sewerage

- All work carried out on live electrical installations

- Welding in confined spaces

- Working at heights

- Tower crane operation

- Piling

- Formwork/Support work

- Demolition work

- Working with hand tools

- Working with portable electrical tools

- Traffic accommodation

- All health hazards that can be present during any of the above activities and should include individual dusts, gases, fumes, vapours, noise, extreme temperatures, illumination, vibration and ergonomic hazards due to any of the above activities

The above list is by no means exhaustive and should not be limited to these activities but must cover all activities that forms part of the said construction work.  Each activity must be split down to individual tasks and all associated hazards identified and listed in the risk assessment.  This ensures that critical tasks and subsequent critical hazards are not missed.

The risk assessment to be included in the Health and Safety Plan must clearly indicate :

- The methodology used to dot the risk assessments

- Breakdown of processes and activities covered

- High risks anticipated

All risk assessments are to be conducted by a competent person as appointed. The plan must include a declaration in this regard or the risk assessment must contain the signature(s) of this appointed persons.

Risk assessments are to be handed to the agent prior to commencement of work.

The agent reserves the right to stop any work if such work is not conducted in terms of the recommendations of risk assessment.

Risk assessments are to cover safety as well as health hazards.

## Base-line risk assessment

The following risks were anticipated for this project, and shall form part of the principal contractors base-line risk assessment.

| ITEM | SIGNIFICANT HAZARDS | REFERENCE CONSTRUCTION REG.NO |
| --- | --- | --- |
| 1. | SAFETY |  |
| 1.1 | Site security | Sect 9 OHS ACT |
| 1.2 | Sub-contractors | 5(3), 5(5), 5(9), 5(10), 5(12) |
| 1.3 | Fall protection ± 4m | 8 |
| 1.4 | Construction vehicles and mobile plant | 21 |
| 1.5 | Site housekeeping | 25 |
| 1.6 | Traffic to and from the site | 21 |
| 1.7 | Personal protective equipment (PPE) | 8, 15, 23 |
| 1.8 | SHE incident investigation | 8, 9 |
| 1.9 | Emergency planning | 27 |
| 1.10 | Excavation Work Mechanical ±5m | 11 |
| 1.11 | Excavation work-Shoring | 11 |
| 1.12 | Material-Storage and stacking | 8, 25, 26 |
| 1.13 | Fire precaution/protection | 27 |
| 1.14 | Demolition work | 12 |
| 2. | HEALTH |  |
| 2.1 | Construction welfare facilities | 28 |
| 2.2 | Sanitation, plant hygiene | 28 |
| 2.3 | HIV/Aids Awareness | Client |
| 3. | ENVIRONMENTAL |  |
| 3.1 | Waste management | 23, 27 |
| 3.2 | Water environment | 24 |
| 3.3 | Housekeeping | 25 |
| 3.4 | Fire precautions/protection | 27 |

## Issue-based risk assessment

The purpose of an issue-based risk assessment is to conduct a detailed assessment study of the activities, occupations and tasks identified during the baseline risk assessment as posing a significant risk.

The aim output of the issue-based risk assessment is:

- input into continuous risk assessment

- action plans for the treatment of significant risk

- input to training program, standards, procedures, codes of practice and management systems

The following risks were anticipated as issue-base risks for this project.

### Task orientated risks:

| Activity/ Task | Hazard/Risk To Safety | Risk to Health | Risk to Environment |
| --- | --- | --- | --- |
| Stripping topsoil | Vehicles reversing over equipment and employees. Dirt in eyes and face | Noise induced hearing loss from heavy machinery. Multiple injuries. | Uncontrolled dust. Diesel and oil spillage from heavy machinery. Stripping of protected fauna and flora |
| Tipping | Trucks reversing over equipment and employees. Trucks toppling over while tipping | Multiple injuries and fatalities. | Diesel and oil spillage |
| Excavate and load | Tip trucks reversing over personnel. Truck colliding with excavator. Overloading of trucks. Collision with other vehicles. Unguarded trenches, slipping and falling down | Multiple injuries and fatalities. | Uncontrolled dust. Diesel and oil spillage from heavy machinery |
| Processing and compacting | Machine reversing. Congestion of machines and Noise | Noise induced hearing loss. Multiple injuries | Diesel and oil spillage. |
| Excavation Mechanical | No non workers allowed in excavation area, Excavated material away from edge of excavation, Mechanical malfunction. Machine topples over. Machine collision. Underground services. Unguarded trenches, slipping and falling down. | Multiple Injuries and fatalities. Electrical shock. Gases from burst pipes, etc. | Cutting down of protected trees, etc. Check with Dept. Of Environment Affairs. Diesel and oil Spillage |

### Task orientated risks: (Continued)

| Activity/ Task | Hazard/Risk To Safety | Risk to Health | Risk to Environment |
| --- | --- | --- | --- |
| Excavation by hand | Caving in of un shored sides, collapse of excavation, personnel and equipment falling in. Unguarded trenches, slipping and falling down. Heat related illness, unsafe picks or shovels, Cuts and lacerations. | Complications resulting from repetitive work. Multiple injuries, fatalities, |  |
| Backfilling | Sides of excavation collapse, live burials of personnel | Multiple injuries and fatalities |  |
| Formwork | Shuttering lying around, exposed nails, poorly erected scaffolding falls, injuries, fractures, death | Dermatitis from coming into contact with degreasers | Spillage of oils, degreasers onto ground, etc C4-34 |
| Stripping of formwork | Falling shutter boards, timber on ground nails in timber, falls from scaffold while stripping | Dermatitis form coming into contact with degreasers | Spillage of oils, degreasers, onto ground, etc. |
| Cement mortar and concrete mixing | Inhaling cement dust, | Ingestion, inhaling of cement, contact with skin may cause Dermatitis | Pollution, spillage of cement |
| Brickwork | Slipping/ tripping, falling of bricks from heights, unsafe stacking of bricks, cutting of bricks, twisting and straining of back muscles, unsafe scaffolding, | Straining of back, falls, fractures, struck by, finger cuts, eyes | Pollution, spillage of cement |
| Plastering | Slipping/ Tripping, poorly erected scaffolding. | Chemical reaction in wet cement causes Dermatitis | Pollution, spillage of cement |
| Carpentry and joinery | Unnecessary material, shuttering laying around, injuries falls, struck by, unsafe scaffolding | Multiple injuries, fractures, bruises |  |
| Structural steelwork | Falling objects, sharp/cutting edges, struck by e.g. lifting material with cranes, unsafe scaffolding, hand, foot injuries, | Multiple injuries, bruises, cuts, fractures, fatalities |  |
| Metal works | Falling objects, sharp/cutting edges, strike by, unsafe scaffold, hand foot injuries | Multiple injuries, bruises, cuts, fractures, fatalities |  |

### Task orientated risks (Continued).

| Activity/Task | Hazard/Risk To Safety | Risk to Health | Risk to Environment |
| --- | --- | --- | --- |
| Scaffolding erection and dismantling | Falls, fractures, fatalities, falling objects, collapsing of scaffold, no toe boards, no handrails, overloading of scaffold | Multiply injuries, fractures, fatalities, |  |
| Ladders | Falling from, damaged steps/legs, no cross bracing, homemade ladders | Multiple injuries, fractures, fatalities, |  |
| Hazardous chemical substances | Diesel | Absorption by the skin | Hazardous chemical pollution |
| Fuel supplies | Sparks can cause fire or explosion hazard, eyes, skin ingestion | Burns, inhaling fumes, skin ingestion, eye, | Spillage pollution, |
| Public and Site visitors | Uncontrolled access | Multiple injuries, fatalities | Uncontrolled access |
| Fire extinguishers | Uncontrolled used can cause multiple injuries | Inhaling of CO |  |
| Wheel barrows | Back injuries, thorn mussels, | Back injuries, thorn mussels |  |

### Plant and machinery risks:

| Activity/Task | Hazard/Risk To Safety | Risk to Health | Risk to Environment |
| --- | --- | --- | --- |
| Grader | Fall under wheels, or blade | Multiple injuries, fatalities | Diesel and oil pollution |
| TLB and Tractor | Fall under, struck by boom or bucket, falling material, | Multiple injuries, fatalities | Diesel and oil pollution |
| Front end loader | Fall under, stuck by, falling material, | Multiple injuries, fatalities | Diesel and oil pollution, dust |
| Excavator | Fall under, struck by, run over, | Multiple injuries, fatalities | Diesel and oil pollution, dust |
| Dumpers | Fall under, struck by, run over, | Multiple injuries, fatalities | Diesel and oil pollution, dust |
| Water tanker | Fall under wheels, | Multiple injuries, fatalities | Diesel and oil pollution, dust |
| Water pumps | Open axle | Multiple injuries, fatalities | Polluted water |
| Concrete mixer | Loose clothing caught in machine, limbs trapped or severed, inhaling cement dust, eye injuries | Eye injuries, Multiple injuries, fatalities | Cement dust, diesel and oil pollution |
| Plate compactor | Bruises, back injuries, hands and fingers caught in v-belts | Fingers and hands caught in v-belts back injuries, |  |
| Driving vehicles | Accidents, passengers, un road worthy vehicles, driver, roads and weather, alcohol, ignore speed limit, be aware of other vehicles | Multiple injuries, fatalities | Oil and diesel/ petrol |

### Equipment risks:

| Activity/Task | Hazard/RiskTo Safety | Risk to Health | Risk to Environment |
| --- | --- | --- | --- |
| Portable electrical tools | Electric shock, damaged cords, Unsafe plugs, cords run throw water, dust, flying sparks/ chips, burns, noise, fire, jamming of drill bits/of cutting blades, wood splints, | Electric shock cuts and bruises, eye and hand injuries |  |
| Hand tools | Damage handles, mushroom heads chisels, pipe handles, | Cuts and bruises, eye and hand injuries, fractures, |  |
| Flame cutting acetylene and oxygen set and LP gas | Damage gauges, cracked hoses, fall over, explosive gasses, burns, damage torches, fire hazard | Multiple injuries, burns, | Air pollution |
| Welding | Arc eyes, burns, fumes, fire hazard, | Arc eyes, burns | Fire hazard damage to property |
| Battery charger | Electric shock, explosive – fumes, fire risk, battery acid | Electric shock Inhaling fumes, |  |

### Physical risks

| Activity/Task | Hazard/Risk To Safety | Risk to Health | Risk to Environment |
| --- | --- | --- | --- |
| Noise | Ear damage – hearing lost | Noise induced hearing lost | Noise pollution |
| Vibration | Kidney problems | May result in kidney complications, | Noise pollution |
| Hot and humid work areas | Exhaustion | Heat exhaustion and heat stroke |  |
| Fire prevention | Combustible refuse: paper, plastics, flammable liquids: petrol, diesel, thinners, turpentine, incorrect fire extinguishers, obstructions in front of fire equipment, | Inhaling smoke and fumes, burns, fatalities | Smoke and fumes pollution |

### Ergonomically risks

| Activity/Task | Hazard/Risk To Safety | Risk to Health | Risk to Environment |
| --- | --- | --- | --- |
| Visual work place | Falls from heights, material falling from heights, tripping over material, | Multiple injuries |  |
| Awkward Postures | Back injuries, etc. | Health complications from unnatural postures |  |
| Heavy manual lifting | Back injuries, etc., | Back injuries, strains |  |

### Behavioural risks

| Activity/Task | Hazard/Risk To Safety | Risk to Health | Risk to Environment |
| --- | --- | --- | --- |
| Working in the direct path of energy release | Multiple injuries, | Multiple health complications, |  |
| Man/machine interaction | Loose clothing caught in machines, limbs trapped or severed | Multiple injuries, fatalities |  |
| Focusing on activity | Trips, falls, fractures, etc | Multiple injuries, fractures, |  |
| Using right tool for the job | Injuries, cuts bruises, fractures, etc | Injuries, cuts, bruises, fractures |  |

### Occupational Health risks

| Activity/Task | Hazard/Risk To Safety | Risk to Health | Risk to Environment |
| --- | --- | --- | --- |
| Hygiene | Health risk | Risk for health | C4-38 |
| Alcohol and other drugs | Health risk | Health risk self and for others |  |

## Continuous risk assessment

The purpose of a continuous risk assessment is to operationally identify occupational SHE hazards for the purpose of treating significant risks.

The output of continuous risk assessment is :

- Immediate treatment of risks, in order of significance and in a manner determined during the issue-based risk assessment.

- Information that feeds back into issue-based risk assessment.

The following activities will form part of the basis of continuous risk assessment, which was identified and developed from the issue-based risk assessments:

- Safe work procedures (SWP)

- Planned inspections

- Method statements

- Internal SHE audits

## Safe work procedures

Safe Work Procedures are to form part of the Risk Assessment and must be compiled for all the above-identified activities.

The safe work procedures must address the following elements:

- The work method to be followed to conduct work safely

- Mitigation/reducing and controlling of identified risks and hazards

- Responsibilities of competent persons

- Required personal protective equipment

- Correct equipment/tools/machinery to be used

- Reference to relevant registers to be completed

- Reference to applicable risk assessment

The following two tables proved information on all factors to be taken into account when the Risk Assessments and Safe Work Procedures are compiled:

| Physical | Chemical | Biological | Mechanical | Psycho-social |
| --- | --- | --- | --- | --- |
| Noise | Liquids | Insects | Guards | Stress |
| Vibration | Dusts | Fungi | Hand tools | Work pressure |
| Ionising radiation | Fumes | Bacteria | Machinery | Monotony |
| Non-ionising radiation | Fibres | Viruses |  | Unsociable hours |
| Health and cold | Mists |  |  | Ergonomically: |
| Electricity | Gases |  |  | * Posture |
| Pressure | Vapours |  |  | * Movement |
|  |  |  |  | * Repetitive tasks |

| System | Stress/Agency | Illness/Disease |
| --- | --- | --- |
| Musculoskeletal | Lifting/loads | Muscular pain syndromes |
|  | Repetitive strain | Tenosynovitis |
|  | Abnormal postures | Bursitis |
|  | Whole body vibration | Osteoarthritis |
| Sensory | Noise | Hearing loss |
| Skin | Cement (chromates), RUBGER | Allergic contract dermatitis |
|  | Thinners, epoxies | Irritant contact dermatitis |
|  | Tar, pitch | Acne, Skin cancer |
|  | Solar radiation | Keratoses, Cancer |
| Respiratory | Silica | Silicosis, TB |
|  | Asbestosis | Asbestosis, Cancer |
|  | Spray paints, wood, epoxies | Asthma |
|  | Irritant dusts, welding fumes | Bronchitis |
|  | Organic Solvents | Headaches, Dizziness, Caner |
| Psychosomatic | Physical stress | Head aches |
|  | Psychosocial stress | Depression |
|  |  | Fatigue |
|  |  | Substance abuse |
| Nervous System | Lead | Peripheral and central neuropathy |
|  | Organic solvents | Headaches, Dizziness, Mood disorder, Dementia, Cancer |

The following example illustrates a typical SWP for the use of an Angel Grinder under general operational conditions:

Activity: General use of an Angle grinder

Task steps: General operation to cut a steel pipe (150mmØ)

Risk identified (Ref: HIRA No: (X):

- Cuts

- Lacerations

- Eye injuries

- Sprains/strains

- Electric shock

Equipment Required:

- Angle grinder

- Electrical extension

- Power source

Registers: Portable electrical equipment register

Responsible person:

Portable electrical equipment inspection:

Construction Regulation 22(d)

Duties:

- Inspect the tool daily before use

- Record finding of the inspections, take corrective action, and report deviations.

Grinder operator:

Duties:

- Inspect tool daily according to Safe working procedure (before starting checks)

- Report deviation to supervisor immediately.

Supervisor:

Duties:

- Conduct regular planed job observation inspections.

- Take on the spot corrective action and note all in SWP Register

PPE:

- Goggles

- Face Shield

- Hard hats

- Hearing protection

- Safety boots

- Overall

- Gloves

Safe Work Procedure:

- Operator checks before starting work:

- No exposed wired

- Flexible cord is serviceable

- Plugs and extension socket free from damage

- Safety switch is serviceable

- No breaks or damage to Machines outer body

- Correct flange and bolting nut are in place for the type of disc being used.

- Guard and handles are secure

- No defect or damage to the disc

- No flammable materials close by

- Other employees are a safe distance away.

- Safe operating procedure

Use two hands when operating the grinder

- Always allow the grinder to “run up” to operating speed before applying it to the job.

- Hold grinder against the work piece with minimum pressure (to ensure it doesn’t dig in and kick back)

- Keep the grinding disc at a 15-to-30-degree angle to the work

- Adopt a comfortable distance (feet apart, well balanced, clear view of the job.

- Never use grinder between legs while sitting on the floor

- Stop the grinder at regular intervals for a short break to vest your rest your arms and hands.

- Disconnect the power and place the grinder on bench with the disc facing upwards when not in use.

- Never put a grinder down until the disc stop running.

- Remove the plug from the power point before changing discs

END OF SECTION