|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

BASELINE RISK ASSESSMENT

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: TRANSPORTATION OF MATERIALS TO SITE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Transportation of material to site | - Unsafe road conditions - Un-road worthy vehicles - Equipment and material not safely secured - Incompetent drivers - Driving under the influence of alcohol - Inclement weather - Speeding - Slippery road | - Overturning vehicles - Vehicle collisions - Bumping pedestrians / employees | - Injuries - Property damages - Third party liability | M | - Adherence to the speed limit - Only competent/ authorised drivers should operate the vehicle - Inspection of vehicles - Equipment and material to be properly secured - Alcohol testing to be done - The road to be paved to prevent accidents - Traffic control to be implemented to avoid collisions |
| Offloading of material | - Faulty lifting machinery & equipment - Suspended load - Poor housekeeping | - Malfunctioning - Falling on employees - Obstructed walkways by materials | - Injuries | M | - Inspect lifting equipment prior to use. - Ensure the safe working load prior to use - Train the employees in manual lifting - Ensure proper housekeeping - The correct PPE must be worn - Designate the stacking areas and put signs - Stacking and storage inspector must be appointed and in charge |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: SITE ESTABLISHMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Site establishment | - Sharp objects/ wires - Uneven surface - Faulty connection - Poor ergonomics - Falling objects - Inadequate security services - Not enough welfare facilities e.g. toilets, change rooms and lockers | - Cuts - Slips and trips - Damage to services - Using the environment as ablution facilities | - Injuries - Back strains and injuries - Crime, theft, fights - Contracting of communicable diseases - Soil, water pollution | M | - Supervisors to plan during site set up and induct employees - A competent electrician must be appointed to connect electrical wires to the site offices and Distribution Board. - Ensure there are welfare facilities on site for health and hygiene purposes - Awareness on hygiene and use of ablution facilities - Detailed Risk Assessment must be drawn before any work commences on site. |
| Installing containers | - Using lifting equipment - Faulty equipment - Faulty slings / chains | - Wind - Incompetent personnel - Heavy load - Failing of lifting equipment | - Serious injuries - Property damage | H | - Check wind speed prior to using the crane. - Inspect the crane, slings and chains before use. - Load test the crane before use - Only carry loads certified to be carried by the crane |
| Entry and exit | - No access control | - Unauthorised entry into the construction site | - Injuries - Theft | M | - Appoint a full time, registered security guard on site |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: SITE ESTABLISHMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Electrical installation | - Electricity - Incompetent personnel - Wrong tools - Damaged cables | - Contact with live electricity - Incompetent person connecting electricity - Electric shocks | - Electrocution - Serious injuries | H | - Follow lock out procedure - Ensure that equipment are earthed to an approved earthing point - Ensure a zero potential test is performed for electricity is isolated - Inspect all tools - Use correct tools for the job - Appoint a competent electrician/ technician - Wear task specific PPE - Ensure that there are no exposed wires on the cables |
| Stacking and storage | - Unsafe stacks of materials | - Falling of pallets and material on employees | - Injuries - Property damage | M | - Supervision of all stacking of materials on site - Materials of same base and heights stacked together - Barricade the stacking area - Unsafe stacks to be removed immediately - Never stack materials during knocking off time or late at night - Use task specific PPE |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: EXCAVATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Excavation | - Deep excavations | - Collapse - Falling - Unsafe entry and exit | Fatalities | H | - Geotechnical surveys to be conducted prior to construction - Shoring of the excavation - No unauthorised entry into the project site. - Fencing off of the excavation and prompt supervision of the area. - Full time security on site to monitor entry and exit. - Safe method of entry and exit to be provided for employees - Spoil material to be stored at least 2m away from the excavation - Signage to be displayed indicating deep excavations |
|  | - Incompetent operator - Faulty excavator | - Overturning - Bumping against people | - Serious injuries | H | - Only competent personnel may operate the excavator - All excavators must be inspected prior to use |
| Mechanical excavation | - Ignition on | - Accidental movement | - Fatalities - Property damages | H | - Operators should ensure that the ignition is off prior to leaving the plant. |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: EXCAVATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
|  |  |  |  |  | - Keys should be removed from the ignition and returned to the office for safekeeping, |
| Mechanical excavation | - Deep excavation - Lack of safety signage | - Employees falling into excavations | - Fatalities | H | - Ensure that deep excavation signage is placed and visible. - Deep excavations must be fenced off and access controlled. |
|  | - Spoil material | - Collapse of excavation walls. - Entrapment | - Fatalities | H | - Excavated soil to be placed at least 2m away from the excavation wall. |
|  | - Underground service | - Contact with plant | - Injuries - Property damages - Electrocution | H | - All underground services to be identified and marked prior to excavation. |
|  | - Unsafe access and exit | - Falling | - Injuries | H | - TLB / Excavator may not be used by employees to access and exit the excavation |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: CONTACT WITH UNDERGROUND SERVICES |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Working close to Eskom line | - High voltage | - Electrocution | - Fatalities - Power cuts - Property damages | H | - Proper scanning of services prior to starting with excavations - Obtain wayleaves from Eskom prior to starting with work - Do not use lifting equipment in close proximity to power lines unless authorized to do so. - Appoint an OHRVS to supervise all work done in close proximity to power lines - Contact Eskom if services are identified for isolation - Develop a method statement and risk assessment - Train employees in the method statement as well as risk assessment - Develop emergency response procedures for explosions |
| Working close to gas lines | - Gas leaks | - Explosion - Suffocation - Inhalation | - Fatalities - Property damages | H | - Proper scanning of services prior to starting with excavations - Obtain wayleaves from service provider prior to starting with work - Develop a method statement and risk assessment - Develop emergency response procedures for leaks and explosions |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: MECHANICAL LIFTING |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Lifting using a crane | - Overhead lines | - Contact with crane | - Electrocution - Property damages | H | - All lifting activities to be planned at least 2m away from overhead lines - Obtain wayleaves from Eskom - Appoint ORHVS - Proper earthing |
|  | - Wind/ Unfavourable weather | - Deflection - Loss of control | - Injuries - Property damages | H | - Do not use during unfavourable weather conditions - Lift Plan to be completed prior to any lift taking place by crane supervisor and engineer; - Method statements and risk assessments to be submitted and adhered to for technical and/or non-routine lifts; |
|  | - Incompetent operator - Medically unfit operator | - Loss of control - Hitting structures and people | - Injuries - Property damages | H | - Only trained and competent operators many operate the crane - Tower crane operator subject to regular health checks; |
|  | - Overloading | - Collapse of material | - Injuries - Property damages | H | - Display safe working load on the crane - Operator to adhere to crane safe working load limits, weight of load and COG established before lifting; - Crane load indicator to be functioning correctly, and not to be overridden under any circumstances; |
|  | - Faulty crane | - Malfunctioning | - Injuries - Property damages | H | - Cranes to be inspected by competent persons prior to being used |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: MECHANICAL LIFTING |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
|  |  |  |  |  | - All faulty cranes to be tagged and locked out - Ensure all lifting tackle and equipment have valid test certificate before use (6 monthly 3rd party inspections); |
|  | - Mast section / counterweight | - Collapse of Mast Section / Counterweight | - Property damages - Fatalities | H | - Ensure proper placement and securing of mast sections and counterweights |
|  | - Limited space / access to worksite | - Falling of material - Crane boom colliding with structures | - Property damages | H | - Proper planning and spacing of activities on site. - Determine the best route for the crane to access the work site - Maintain safe distances and use communication systems to coordinate with other cranes. - Temporary relocation of tower complex residents where required - Demarcate the site and not allow any movements where lifting operations are taking place - Develop a comprehensive lifting plan - Tower crane to be operated by a competent operator |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: MECHANICAL LIFTING |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
|  |  |  |  |  | - Proper communication with other teams on site |
| Accessing the cabin | - Heights | - Falling from heights | - Fatality | H | - Use safety harnesses and ensure proper anchoring points and access |
| Tower Crane supervision | - Lifting operations around workers; - Workers in crane operating radius; | - Persons underneath load struck by materials; - Load swinging / spinning uncontrolled; - Falling materials; - Damage to lifting slings from sharp edges; - Over-sailing of material over live traffic routes; - High winds – adverse weather; | - Injuries - Fatality - Property damages |  | - Lift controlled by designated trained rigger/banks man using approved hand signals only; - No personnel situated or walking under elevated loads, avoid over-sailing of personnel if practicable; - No over-sailing of loads over live traffic routes without road closure; - No lifts undertaken when wind speed exceeds 40kph; - No blind lifts without visual and communication aids, radios etc.; - Tag lines shall be used to guide materials while lifting, 2 lines minimum 10m in length; - Slings protected from sharp edges with rubber etc.; - Riggers to undertake visual inspection of all lifting equipment daily; |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: MECHANICAL LIFTING |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
|  |  |  |  |  | - Tower crane allowed to ‘weathervane’ in high winds; |
| - Jumping and dismantling of Tower Crane | - Works at Height; | - Fall from height; - Falling materials from height; - Use of untrained personnel; - Dismantling in adverse weather; | - Injuries - Fatality |  | - Ensure crane is jumped or dismantled to standards by trained competent erection personnel; - No dismantling or jumping in adverse weather or high winds; - Concise Dismantling Method Statement compiled and available in work area for review; - Area below works cleared and barriers implemented during jumping or dismantling; |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: FALSE WORK |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Transporting shutters | - Defective strapping - Incompetent driver - No inspection - Overloading of truck | - Falling load - Loss of control of vehicle | - Injuries - Property damages | M | - Ensure that the load is properly secured in the truck. - Driver must have a valid PDP - Driver must inspect truck and ensure that it is roadworthy - Driver must keep the speed limit while driving |
| Offloading of shutters | - No stop blocks - Ignition on - Untrained crane operator - No load testing | - Accidental movement of truck - Falling loads - Uncontrollable movement of crane | - Fatalities - Property damages | H | - Place stop blocks behind the wheels. - Operators should ensure that the ignition is off prior to leaving the plant. - Keys should be removed from the ignition and returned to the office for safekeeping, - Ensure that crane operator and rigger are trained and medically fit to operate the crane. - Ensure that the crane and all lifting equipment are load tested and SWL is displayed. |
| Installing shutters | - Untrained personnel - No PPE - Poor housekeeping - Defective hand tools - Defective shutters | - Falling shutters - Cuts - Hitting employees | - Injuries | M | - Ensure that shuttering is only done by trained personnel. - Provide employees with relevant PPE. - Ensure proper housekeeping on site - Inspect tools and shutters prior to installing |
| Dismantling shutters | - No PPE - No supervision | - Falling shutters - Cuts | - Injuries | M | - Ensure that dismantling is done under supervision of a competent person. |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: FALSE WORK |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
|  | - Untrained personnel - Defective tools | - Hitting employees |  |  | - Ensure that dismantling is only done by trained personnel. - Provide employees with relevant PPE. - Ensure proper housekeeping on site. - Inspect tools and shutters prior to installing |
| - Formwork installation | - Works at Height; - Manual Handling; - Worker competency; - Lifting materials to height; (crane) | - Fall from Heights; - Falling objects; - Manual Handling - Incorrect Design and Erection of formwork resulting in collapse; - Use of unsupervised or untrained personnel to erect formwork; - Erection of Formwork; - ​ | - Injuries - Property damages | M | - Safety harness and lanyard are to be used and hooked off to a secure anchor point when working around exposed edges; - No shock lanyard to be used below 6m in height; - All hand tools to be tied off when working at height over workers below; - Correct manual lifting techniques of formwork communicated to workers; - The erection of temporary structures shall be undertaken under supervision of a competent person under control of the designated Temporary Works Supervisor in accordance with the TW designer’s requirements; - All formwork tie rods to be same design/make of wing nuts and not allow slippage under load, double nut in areas where a risk of collapse onto workers under load is possible; |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: FALSE WORK |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
|  |  |  |  |  | - Formwork to be supported following a suitable proprietary bracing system as per design, inadequate or unsuitable fixings shall not be used; - Permit to load to be completed by designated persons before loading of concrete onto formwork; - Temporary Works register to be completed by designated person and available for review; - All workers to be briefed by supervision daily before shift commences; - Area below to be clear of workers during erection, barriers and signage to be implemented; - Formwork to be inspected daily before work by designated Temporary Works Co- ordinator; - Formwork inspected after damage or incident likely to affect structural stability such as high winds, heavy rain or electrical storm; - No overloading of formwork, TWC shall follow design SWL when distributing materials; |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: STEEL FIXING |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Offloading of steel with mobile crane | - Untrained operator - Untrained rigger - SWL of crane not indicated - No guide ropes to control load | - Collapse of load onto people - Uncontrollable movement of the crane | - Serious injuries - Fatalities - Property damages | H | - Ensure that the crane operator and rigger are trained and medically fit. - Ensure that inspection on crane is conducted prior to use. - Ensure that SWL is indicated - Ensure that there is good communication and visibility between the operator and rigger. - Guide ropes must be in place to control overhead loads - Ensure that truck is parked in a zero-energy position and have stop blocks in place. - Unauthorised persons not allowed when crane is in operation |
| Dismantling & installation of steel | - Untrained personnel - No PPE - Poor housekeeping - Defective hand tools - Incorrect pushing and pulling of steel | - Steel falling on feet and legs - Trips and falls | - Injuries | M | - Ensure that only trained employees are dismantling the steel - Ensure that employees have the relevant PPE. - Ensure proper housekeeping - Ensure that hand tools are used prior to use. - Ensure that the pulling and pushing system is correctly done and monitored. |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: STEEL FIXING |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Cutting steel with a grinder | - Grinder - Faulty connection points - No inspections - Untrained operator - No PPE - Damaged cutting disc | - Electrocution - Cuts | - Injuries - Amputation | M | - Ensure all grinders and connections are inspected - Ensure that safety guards are in place on the grinder - Handle and switch of the grinder should be inspected and tested prior to use. - Only trained employees may use grinders - Proper PPE to be used - Inspect cutting disc prior to use |
| Binding steel with wire (using nip pliers) | - Defective tools - No PPE | - Cuts | - Injuries | L | - Inspect tools prior to using - Ensure that the wire used to bind steel is bent at the end to prevent eye injuries - Ensure that proper PPE is provided |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: CONCRETE WORKS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Concrete works | - Moving bucket | - Hitting employees | - Injuries | M | - Workers to stand clear of the path of the bucket - Banksman to direct the loading and offloading activities |
|  | - Working at heights | - Falls from heights | - Injuries | H | - Only work on platforms that are fully boarded and handrails installed |
|  | - Concrete splashing | - Skin contact | - Dermatitis - Skin burns | M | - Use of skin protection - Provide gloves and safety boots - Have MSDS |
|  | - Concrete splashing | - Eye contact | - Eye injuries | M | - Provide safety goggles |
| Operating concrete truck | - Incompetent operator | - Hitting employees; - Bumping vehicles and properties | - Injuries - Property damages | M | - Only trained, competent and appointed operators may operate a concrete truck - Flagman to be posted to direct the operator |
|  | - Speeding | - Hitting employees; - Bumping vehicles and properties | - Injuries - Property damages | M | - Site speed limits to be adhered to at all times |
|  | - Manual handling of chutes | - Incorrect handling | - Injuries | M | - Training in correct handling methods - Provide employees with gloves. - Provide leather gloves for concrete chute |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: CONCRETE WORKS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Pouring concrete with a bucket by means of a mobile crane | - Incompetent operator | - Hitting employees; - Bumping vehicles and properties | - Injuries - Property damages | M | - Only trained, competent and appointed operators may operate a concrete truck. - Flagman to be posted to direct the operator |
|  | - Unauthorised / unsuitable crane | - Hitting employees; - Collapse of material | - Injuries - Property damages | M | - Crane to be inspected by a competent person prior to use. - Unsuitable crane to be tagged and locked out |
| Siting and setting up of concrete pump | - Working in proximity to existing structures and services; | - Overturning due to incorrect siting or incorrect pump outrigger extension; - Collision of jib with overhead services; - Blocking access routes with pump; - Jib striking stationary structures; | - Injuries - Property damages | M | - Ensure pump is set up/rigged out correctly, is on firm level ground as per siting plan with outriggers fully extended; - Solid barriers should be situated, where practicable, to protect pump from traffic routes around working radius of cab; - Consideration given to overhead services and existing structures when siting pump; |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: CONCRETE WORKS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Pumping concrete (Pump Operator) | - Operator competency; - Pump planning; - High pressures; | - Incident due to use of defective hoses / equipment; - Concrete spillage; - ​ | - Injuries - pollution | M | - Pumping Plan to be completed prior to any pour taking place by Temporary Works Coordinator; - Method statements and risk assessments to be submitted and adhered to; - Pump load indicator to be functioning correctly, and not to be overridden under any circumstances; - Operator to adhere to safe working load limits; - Daily inspections of hoses, pipes, connections etc. prior to use. Damaged equipment immediately removed from service; - Pump hopper guards to be in place at all times; - Plastic sheeting placed under hopper to contain spills; |
| Pumping operation supervision | - Pumping operations around workers; | - Worker struck by delivery vehicle; | - Injuries |  | - No over-sailing of hose over live traffic routes without road closure; |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: CONCRETE WORKS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
|  | - Workers in pump operating radius; - Reversing concrete delivery vehicles; | - Hose swinging / spinning uncontrolled; - Pump operator unable to see task – high level pumping; - Over-sailing of material over live traffic routes; - Use of concrete – protective gear; - Working on rebar – risk of injury; | - Property damages |  | - Supervision to undertake visual inspection of all pumping gear and equipment daily; - Workers to wear correct PPE at all times, nitrile gloves, wellingtons, glasses; - Flagmen controlling delivery truck movements; - Spotters to direct pump operators at high level in clear view at all times; - All concrete soaked workwear to be removed immediately; - Planks placed on rebar for worker access, clear access maintained |
| Crane parking | Unstable ground | - Crane overturning | - Injuries - Property damages | M | - Crane to be established on solid ground with outriggers fully extended and pads in place |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: WORKING AT HEIGHTS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Working at heights | - Heights - Unfit employees - Using hand tools - Unsecured tools and equipment | - Falls - Falling onto employees | - Injuries | M | - Employees to use proper PPE including safety harnesses when working at heights. - Inspect all tools prior to use. - Provide training for using safety harnesses correctly. - Employees working at heights must be certified fit to work. - Hand tools must be attached to lanyards when working at heights. - Use tool bags - Use netting system below each level. - No work should be done on the ground when work at heights is in progress. - Proper signage to be displayed. |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | TBC |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: SCAFFOLD WORK |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Scaffold work | Improper erection | - Collapse - Falling of employees and equipment | - Property damages - Fatalities | H | - Only a competent scaffold erector may erect a scaffold. - Scaffold to be erected on firm, stable base - Check the stability of the scaffold prior to allowing employees to climb on them. - Mobile scaffolds must have their wheels locked before any person is allowed to climb on them. - All parts of the work platform and scaffold must have safe means of access - Unattended or incomplete scaffold should be prevented against unauthorized access. - Suitable access ladder must be provided - A fall arrest harness must be worn. - Never use worn out fall arrest equipment. - Suitable equipment for rescue must be available - Proper bracing of the scaffold - Scaffold to be securely tied to a structure |
| Working on scaffold | Overloading | - Collapse | - Property damage | H | - Display the safe working load of the scaffold |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: SCAFFOLD WORK |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
|  |  | - Falling of employees and equipment | - Fatalities |  | - Never exceed the SWL of the scaffold |
| Working on scaffold | Overhead lines | - Contact with overhead lines - Contact with employees | - Property damages - Electrocution | H | - Erect the scaffold away from power lines - Keep tools away from overhead lines - Fall arrest equipment to be attached |
|  | Unstable ground conditions | - Collapsing of scaffold | - Injuries - Property damage | H | - Ensure that the foundation / footing of the scaffold is stable and levelled - Only trained and competent persons may erect a scaffold |
|  | Non-compliant scaffolding | - Collapsing of scaffold | - Injuries - Property damage | H | - All platforms to be fully boarded and boards used to comply with SANS 085 - All components of the scaffold must comply with SANS 085 |
| Work platform | Inadequate space | - Falling persons and objects | - Fatalities - Property damages | H | - Work platform should be at least 1m wide and capable of carrying the load that the type of work carried out may require |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: SCAFFOLD WORK |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
|  |  |  |  |  | - To prevent persons from falling from the outside edges of work platforms, guardrails and mid-rails must be fitted. - If loose objects or other materials may be present on the platform, edge protection/ toe boards must be provided on work platforms. |
| Edge protection | Working close to edges | - Falling | - Fatalities | H | - To prevent persons from falling, edge protection must be erected around the perimeter of the work. This must comprise the guardrail and mid-rail designed to withstand any reasonable force, which is expected to fall against it. - Only a competent person may erect an edge protection system - Edge protection can be in the form of hook on boards used as toe boards |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: PIPE JACKING |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| - Pipe jacking | - Working in confined spaces | - Lack of oxygen - Harmful gases - Entrapment | - Asphyxiation - Serious injuries - Fatalities | H | - Develop a confined space procedure. - Develop a permit to work system. - Only authorized and trained personnel to enter confined spaces. - Ensure that the gases are purged before entry. - Provide employee with oxygen mask - Have an employee trained in first aid ready. - Confined space may not be smaller than 800mm in diameter. |
|  | - Working at heights | - Falling | - Fatalities - Serious injuries | H | - Proper PPE to be issued - Provide rigid edge protection. |
|  | - Lifting operations | - People hit by load - Collapsing of pipe - Incorrect slinging | - Injuries | M | - No people are allowed to be at the jacking site. - Slinger to be identifiable and wear reflective PPE. - Lifting machinery to be inspected prior to use. - Lifting machinery to be load tested. |
|  | - Deep excavation | - Falling | - Fatalities | H | - Barricade the jacking pit - No unauthorized entry - Site should be fenced off and locked out. - Signage should be put up |
|  | - Jetting rig | - Crushing by horizontal move of the rig; - People hit by drop load | - Injuries | M | - Enter the shaft only once the load has reached the bottom. - Signaler to make the people in the shaft aware of the moving load. |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: PIPE LAYING & WELDING |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Pipe laying | - Steel - Employees working around the excavation | - Static electricity - Rough edges - Heavy steel - Hit by pipe | - Electrocution - Injuries | M | - Ensure that pipes are earthed - Provide gloves to employees - Heavy pipes to be lifted mechanically |
| Pipe laying | - Crane | - Failure of the crane - Employees | - Fatalities | H | - Inspect the crane prior to use. - Only trained and competent person to operate the crane - Employees to steer clear of lifting operations - Faulty cranes to be tagged and not used - Wind speed and direction to be considered before using the crane - Crane to be load tested - Only certified loads to be carried on the crane |
| Tie ins & pressure testing | - Pressurized water | - Hitting employees - Burst | - Fatalities - High water losses | E | - Ensure that the water is shut down - Ensure that the correct class of pipe is used as well as valve specifications are correct - Develop and follow a method statement - Use only competent person to perform the task. - Don’t exceed pipe operating pressure, ensure pipe is correct diameter and is not damaged. |
| Pipe welding | - Welding equipment | - Poor maintenance - ​ | - Injuries | M | - Welding equipment is visually checked before each use; |
|  | - Welding | - Sparks | - Fire | H | - Welding screens to be used |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

|  |  | - ​ | - Sparks | - Burns | L | - Fire resistant overalls and apron to be worn. - Develop method statement for welding inside the pipe |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | - ​ | - Glare | - Arc eyes | M | - Welding glasses to be used |
|  |  | - Confined space | - Entrapment - Inability to move | - Injuries - Suffocation | H | - Develop method statement for welding inside the pipe. - Ensure that the welder is able to fit in properly inside the pipe and move with ease - Use a different method of welding where the pipe’s internal diameter is less than 800mm. |
| Compacting |  | - Compactor - Noise - Dust - Vibration | - Overexposure to noise and vibration - Collapsing excavation wall - Inhalation of dust | - Noise induced hearing loss - Raynaud’s Syndrome - Injuries Respiratory problems | M | - Use of earmuffs by compactor operator - Dust mask to be worn by compactor operator - Compactor operator to be declared competent before using the machine - Provide operator with anti-vibration gloves and provide rest periods in between. |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: MECHANICAL WORKS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Installation of pumps and valves | Heavy pumps and valves | Falling | - Injuries - Property damages | M | - Use proper lifting equipment - Ensure that only the relevant people are in the work area. - Ensure that lifting equipment is load tested. - Only competent personnel to undertake this task |
|  | Steel fixing | - Falling - Slip | - Injuries | L | - PPE, including safety boots and goggles must be used |
| Cutting and drilling | - Drilling - Drill bit - Drill sharp metal fibres - High Noise Levels - Cutting Grinder/Disc | - Vibration - Cutting edges - Eye penetration - Finger cuts - Expose to high noise level area - Uncontrolled disc - Electrical equipment failure - Sharp window edges | - Damaged hearing - Carpal tunnel syndrome - Cuts/ injuries - Eye irritation / blindness - Injuries - Eye injuries | M | - Use hearing protection when exposed to excessive noise levels (greater than 85 dB over an 8-hour work period) - Assess noise level with sound level meter if possibility exists that level may exceed 85dB. - Rotate drilling tasks to minimize worker exposure to equipment vibration. - Use right size of a drill to drill different layers of the ground - Assess manual guide carefully to ensure correct usage of portable electrical devices. |
| Welding | - Fumes | - Inhalation | - Respiratory problems | M | - Wear respiratory protection |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: MECHANICAL WORKS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
|  | - Sparks | - Contact with skin | - Skin burns | L | - Personal Protective Equipment to include face, eye and skin protection |
|  | - Sparks | - Fire | - Damage to property - Fatalities | H | - Provide fire extinguisher - Provide screens - Remove all sources of combustion and hazardous chemicals from welding area |
|  | - Welding arc | - Starring welding arc | - Eye irritation | L | - Safety goggles to be worn |
| Install generator | - Lifting equipment | - No SWL displayed - Faulty equipment - Employees standing around | - Serious injuries - Property damages | M | - Load testing of lifting equipment - Inspection of lifting tackle and equipment prior to use - Display SWL - Only competent people to operate the lifting equipment - Access control to the lifting area |
| Fueling the generator | - Spillages | - Slips, trips, falls | - Injuries | L | - Prevent spillages - Use drip trays - Use funnels for fueling |
|  |  | - Fire | - Injuries | M | - No smoking allowed near the generator |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: MECHANICAL WORKS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
|  | - Diesel fumes | - Inhalation | - Respiratory problems | L | - Provide employees with respirators |
|  |  | - Skin contact | - Skin irritation | L | - Provide employees with gloves, safety boots and overalls |
|  | - Running engine | - Explosion | - Injuries | M | - Switch engine off before refueling and make sure fuel cap is replaced. - No smoking allowed near the generator |
| Running the engine | - Noise | - Over-exposure | - Noise-induced hearing loss | M | - Provide employees with hearing protection - Provide rest periods for employees |
|  | - Vibration | - Over exposure | - Raynaud’s Syndrome | L | - Provide employees with vibration gloves. - Rotate employees or provide rest periods. |
|  | - Fumes | - Inhalation | - Respiratory problems | L | - Provide employees with respirators - Never use indoors |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: ASBESTOS REMOVAL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| Removal of asbestos pipe | - Asbestos cement dust | - Inhalation of asbestos fibers | - Asbestosis | H | - No asbestos pipe should be cut. The entire length of pipe should be removed (remove clamps) - Appoint a registered Asbestos Contractor |
| Disposal of asbestos pipe | - Asbestos fibers | - Inhalation of fibers - Incorrect disposal | - Asbestosis | H | - Appoint a registered Asbestos Contractor for the removal and disposal of asbestos and asbestos containing waste. - Appoint an AIA to conduct an asbestos survey to determine the level of exposure of the employees. - Obtain a clearance certificate from the AIA - Obtain a safe disposal certificate from the Asbestos contractor and keep records. |
| Disposal of asbestos pipe | - Asbestos waste | - Inhalation of asbestos dust - Contamination of materials | - Lung infections | M | - Ensure that asbestos waste is removed within 90 days of accumulation by a registered asbestos contractor. - Provide employees with suitable dust masks - Demarcate and cover asbestos waste (soils) |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: TRAFFIC MANAGEMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | - Hazard | - Risk | - Consequence | Rating | - Controls |
| Pipe installation next to the road | - Moving vehicles and pedestrians - Improperly placed signage - Incorrect signage displayed - Poor communication between flagmen | - Road accidents - People getting knocked down by cars - ​ | - Fatalities - Serious injuries - Property damages | H | - Adhere to traffic management procedures - Proper training of flagpersons - Ensure good communication between flagmen - Display correct road signage, and put reflective barricades for additional visibility, especially at night and when it is dark / foggy. - Employees should wear reflective PPE - Keep area clean & clear of obstacles. - Obtain wayleaves from the roads agency (JRA) |
| Pipe installation via trenchless methods under the road | - Vibration | - Road collapse | - Fatalities - Property damages | H | - Only experienced service providers for pipe jacking and horizontal directional drilling should be appointed. - Pipe jacking and HDD to be done within the permitted timeframes issued by JRA and SANRAL. - Ensure that wayleaves from JRA and SANRAL are always valid, and are complied with. |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: GENERAL ACTIVITIES ON SITE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| - Working on site | - Unhappy community | - Community coming to survey site - Violence against employees | - Property damages - Serious injuries | H | - Develop an emergency response procedure - Meet with the community and all stakeholders prior to commencement of the project - Have the contact details of the nearest police station / JMPD offices |
|  | - Unfavourable weather conditions | - Exposure to temperature extremes | - Heat exhaustion - Frost bite | M | - Provide employees with water for cooling down. - Provide employees with warm jackets and gloves during winter - Provide 5-minute rest periods for every 30 minutes of exposure to temperature extremes. - No work to be undertaken in rainy conditions |
|  | - Housekeeping | - Trips and falls | - Injuries | M | - Ensure that proper housekeeping is maintained on site at all times. |
| - Working in open spaces | - Snakes | - Bites - Poisoning | - Fatalities - Serious injuries | H | - Inspect the area for snakes prior to entering - Conduct snake awareness training - Know the do’s and don’ts of what to do when coming across snakes |
| - Working in open spaces | - Bees | - Bites | - Allergic reaction | M | - Inspect the area for bees / wasps prior to entering - Conduct bees awareness training - Know the do’s and don’ts of what to do when coming across bees |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

| ACTIVITY: GENERAL ACTIVITIES ON SITE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task | Hazard | Risk | Consequence | Rating | Controls |
| - Working in open spaces | - Sharp objects | - Getting pricked by sharp objects | - Tetanus - Injuries | M | - All employees to get Tetanus vaccination. - Provide employees with proper safety boots |
| - Working in open spaces | - Criminals | - Getting mugged - ​ | - Loss of personal possession | M | - Personal belongings such as phones and car keys to be safely put in pockets while working. - Employees to report any suspicious activities to the local police. - Equipment to be safety stored while not in use |
| - Working in open spaces | - Criminals | - Employees being attacked | - Injuries - Fatalities | H | - Ensure that employees do not work in isolation. - Employees to report any suspicious activities to the local police. - Develop an emergency response procedure |
| - Working in open spaces | - Open excavations | - Falling inside | - Injuries | M | - Employees to be vigilant while working on site |
| - Working in open spaces | - Water bodies | - Falling inside | - Serious injuries - Fatalities | H | - Employees to be vigilant while working on site - Emergency procedures to be developed |

|  | OCCUPATIONAL HEALTH & SAFETY (OHS) SPECIFICATION: BASELINE RISK ASSESSMENT |  |
| --- | --- | --- |
|  | PROJECT NUMBER: | JW14402 |
|  | PROJECT LOCATION: | FOREST HILL |
|  | PROJECT DESCR: | CONSTRUCTION OF 2,5Ml WATER TOWER AT FOREST HILL, JOHANNESBURG SOUTH |

RISK ASSESSMENT MATRIX

| Likelihood | Consequences |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Insignificant (minor problem easily handled by normal day to day processes | Minor (Some disruption possible e.g.,damage equal to R150k | Moderate (significant time / resources required. E.g., damage equal to R500k | Major (Operations severely damaged. E.g., damages equal to R1m | Catastrophic (business survival is at risk. Damage equal to R5m – 10m |
| Almost certain (90% chance) | High | High | Extreme | Extreme | Extreme |
| Likely (between 50- 90%) | Moderate | High | High | Extreme | Extreme |
| Moderate (between 10- 50%) | Low | Moderate | High | Extreme | Extreme |
| Unlikely (between 3- 10%) | Low | Low | Moderate | High | Extreme |
| Rare (<3%) | Low | Low | Moderate | High | High |