# ETHEKWINI MUNICIPALITY

Occupational Health & Safety Unit

Baseline Risk Assessment in terms of Construction Regulation 5.1(a)

| Document Title | Baseline Risk Assessment in terms of Construction Regulation 5.1(a) |
| --- | --- |
| Client | eThekwini Municipality – road provision |
| Project Name | Upgrade of 121612STR from gravel to concrete surface Ntuzuma ward 45 |
| Contract Number | 1R- |
| Revision | 01 |
| Date | 22/09/2022 |
| Internal Reference no. | BRA 34/09/2022 |
| Compiled by (Safety officer) | Siziwe Chiliza Signature: |
| Reviewed by (Manager: Safety& Risk) | Arty Zondi Signature: |

| MAIN ACTIVITY |  |  | Site Establishment |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | REF NO |  | SUB ACTIVITY |  | SAFETY RISKS | HEALTH RISK |  | ENVIROMENTAL RISK |  | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| 1.1 |  |  | Travel to site | - Potholes - Undulating roads - Poor visibility |  | - Fatigue | - Oil/diesel spillages |  | - Community livestock - Other vehicles driving recklessly - Pedestrians - Construction works on public road |  | - All drivers to be deemed competent - No staff to travel in rear of vehicles with equipment - All equipment to be secured during transport - All vehicles to be road worthy - No drivers/Operators to be under the influence of alcohol or drugs of any kind. - Inspection Records must be kept for all vehicles and plant. |

| REF NO |  | SUB ACTIVITY |  | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.2 |  | Site clearance and de-vegetation | - Poor traffic control - Incompetent drivers - inadequate signage’s - Moving of construction vehicles and vehicle colliding - Snakes insect bites & stings |  | - Back strain - cuts, - Abrasion - heat exhaustion - noise exposure - dust inhalation | - Spilling of oil diesel, petrol - Contamination of ground | - Tripping hazard - dust inhalation - intermittent noise level | - All staff to have medicals done by an occupational medical practitioner prior to commencement of work. All workers to have valid medicals with annexure 3 documents signed and stamped by Occupational Medical Practitioner with records in file. - All staff to work under supervision - Dust suppression is to be maintained - Responsible 8.1 to ensure Instructions are given on Manual handling using Safe Work Procedure (SWP) for Manual Handling. - Only Licensed and Appointed Mechanical Aids Operators (i.e. Crane Operators, TLB operators, Bobcat Operators, etc.) to operate plant on site. - Inspection & Maintenance records must be maintained for all mechanical aids on site. |

| REF NO |  | SUB ACTIVITY |  | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.3 |  | Site fencing | - Pinch points from manual handling - Trip hazards - Underground services |  | - Heat exposure - Dust - Noise - Chemical burns/ contact dermatitis from cement and eye injuries from cement splashes | Hydrocarbon spills | - Unauthorised persons entering site - School learners entering site - Damage to underground services | - All contractor staff and local labour to be inducted of all “no go” areas - All staff have company logo PPE with name tags - Contractor staff to work within designated areas only - Contractor to conduct DSTI’s (Daily Safety Task Instruction) and communicated at the beginning of the shift daily. DSTI close out to be conducted and attendance registered signed by all staff. - Flagman to be provided to ensure vehicles are guided onto site. - Wear hand protection (PVC Gloves), safety glasses and face shield always by the operator of the concrete chute. - Concrete spillages to be cleaned up and concrete truck chute to be cleaned in concrete wash area. - Stacking & Storage Supervisor to be appointed to control lay down areas. |

| REF NO |  | SUB ACTIVITY |  | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4 |  | Placing of mobile offices | - Use of mechanical aids for offloading - Failure of lifting equipment - Fall from heights - Manual handling - Mechanical aids overturning - Overhead power lines |  | - Noise - Hand and body injuries - Heat exposure - Dust exposure | Hydrocarbon spills | - Exposure to dust - Interaction of machinery and learners | - All operators to be deemed competent - All riggers to be deemed competent - All lifting equipment to be. load tested - Flagman/banksman to be present on-site during movement of plant - Access control to be managed on site - All staff to have the required PPE - Staff working at heights to be trained for heights safety - Contractor to conduct DSTI’s (Daily Safety Task Instruction) and communicated at the beginning of the shift on a daily basis. DSTI close out to be conducted and attendance registered signed by all staff. - Ensure Offloading area is clear, level, secure and free from slip trip and falling hazards. - Mechanical Aids must be operated in accordance with the operators training and manufacturers guidelines |

| REF NO |  | SUB ACTIVITY |  | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.5 |  | Connection of power supply | Electrocution Hand injuries |  | Heat exhaustion | None | Interruption of power supply | - Only authorised person to do electrical connection - A COC is to be provided for all electrical works done - Monthly inspections to be done for all temporary electrical connections - Contractor is to be responsible for power consumption - Required agreements to be in file for electrical usage |
| 1.6 |  | Water supply | Injuries during the laying of water pipes |  | Drinking water which is not safe for human consumption | None | Theft of water by residents | - All drinking water is to be labelled/sign posted - Water use is to be managed on site - The location of standpipes to be correctly positioned to mitigate theft of water from site |
| 1.8 |  | Construction of staff facilities: eat area/toilets/ change area | - Manual handling - Fall from heights - Hand injuries - Use of portable electrical power tools |  | - Electrocution - Dust exposure - Heat exposure | None | None | - Separate facilities for both male and female - Drinking water to be provided for staff on site - If portable toilets are to be used required service slips are to be maintained on site - Staff facilities area to be suitable for staff use |

| MAIN ACTIVITY |  | Construction Works |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REF NO |  | SUB ACTIVITY |  | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| 2.1 |  | Site survey | - Trips and falls - Injuries from use of hand tools |  | - Heat exhaustion - Dust exposure | Use of cement for markings contamination | - Trips and falls - Unauthorised entrance | - Works to be supervised - DSTI to be done - Lime to be used for ground markings - Required PPE to be used - All survey pegs to be made visible - If steel is to be used rebar caps are to be fitted onto them |
| 2.2 |  | Mechanical excavations | - Machinery collides with people - Collision with machinery - Fall into open excavation - Collapse of side walls |  | - Dust exposure - Back injuries - Noise exposure | Hydrocarbon spills | - Dust exposure - Collision with machinery | - DSTI to be done - All staff to have the required PPE - All plant to be inspected before use - All open excavations to be netted off and sign posted - Excavated spoil to be 500mm away from edge of excavation - Nil staff to be in any excavation when machinery in use |
| 2.3 |  | Manual excavations | - Slips and falls - Fall into open excavations - Collapse of side walls |  | - Dust exposure - Back injuries - Heat exhaustion | None | Fall into open excavations | - All staff to be trained in safe works procedure - All spoil to be away from edge of excavation - Ladders to be provided for access and egress from excavations |

| REF NO |  | SUB ACTIVITY |  |  | SAFETY RISKS | HEALTH RISK |  | ENVIROMENTAL RISK |  | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.4 |  |  | Unsafe stacking and storage practices | - Manual handling injuries - Pinch points - Slips and trips |  |  | Back injuries | None | None |  | - Required PPE to be used - DSTI to be done - All works to be supervised - All steel to be stacked within a designated area |
| 2.5 |  |  | Installation of Temporary Electrical Installations | - Use of hand tools injuries - Trips and falls - Collapse of excavation side walls - Contact with machinery |  |  | - Chemical burns from concrete - Heat exhaustion - Dust exposure | Ground contamination | Collision with pedestrians |  | - DSTI to be done - Flagman to direct all trucks to site - All staff to have the required PPE when handling concrete - Nil staff to be in foundation during discharge of concrete into excavation - Delivery trucks to use designated access - Washing of truck chutes to be done within a concrete wash area |
| 2.6 |  |  | Housekeeping | - Manual handling injuries - Trips and falls |  |  | - Dust exposure - Back strains - Ergonomically injuries | Ground contamination due to mixing of mortar on natural ground | Unauthorised access to site |  | - Proper stacking of bricks to be done - Movement of bricks to be done with proper wheelbarrows - All staff to have required PPE - A platform to be created for mixing of mortar - Housekeeping to be maintained on site |

| REF NO |  | SUB ACTIVITY |  | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.7 |  | Site Induction | - Struck by machinery - Manual handling injuries - Fires |  | - Dust exposure - Heat exhaustion - HAVS - Noise | Hydrocarbon spills | Unauthorised access | - All staff to have required PPE - DSTI to be done - All works to be supervised - Safe works procedure to be done and communicated to staff - Risk assessments to be communicated by means of training |
| 2.8 |  | Setting up of temporary road works signage | - Struck by machinery - Placing of incorrect signage at locations |  | incident Damage to property | Environmental degradation | None | The Contract - Manager must ensure that a suitable site-specific Traffic Management Plan is developed and implemented and a competent Traffic Safety Officer |
| 2.9 |  | Public vehicular and pedestrian traffic travelling on the public road during construction work | - injuries - car collision |  |   incident Damage to property | None | None | - All works to be supervised All All temporary road traffic signage must comply with the - SARTSM - Flagman to direct all vehicles to site |
| 2.10 |  | Layer works (asphalt layer) | - Use of hand tools injuries - Trips and falls - Contact with machinery Respiratory failure - Burns |  | - Chemical burns from asphalt - Heat exhaustion - High temperature of product | Ground contamination | none | - Flagman to direct all vehicles - All staff to have the required PPE when handling asphalt - Delivery trucks to use designated access - All spills to be cleaned and disposed Enforced reverse alarms to be fitted Application of dust masks |

| REF NO |  | SUB ACTIVITY |  | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.11 |  | Backfilling | - Manual handling injuries - Trips and falls |  | - Dust exposure - Back strains - Ergonomically injuries - Heat exhaustion | Ground contamination due to mixing of mortar on natural ground | Unauthorised access to site | - All staff to have required PPE - A platform to be created for mixing of mortar - Waste skips to be provided on site - All asphalt waste to be properly disposed on site - Housekeeping to be maintained on site |
| 2.12 |  | Refuelling plant and equipment | Extremely flammable, high hazard. The liquid can release vapours at temperature below ambient which forms flammable mixtures |  | Heat exhaustion | Ground contamination due to mixing of mortar on natural ground | Unauthorised access to site | - Required appointments to be in file - All works to be supervised Remove all sources of ignition. Contain and - collect spillage with non-combustible absorbent materials. Place in a suitable container for disposal in accordance with waste regulations |

| REF NO | SUB ACTIVITY | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| --- | --- | --- | --- | --- | --- | --- |
| 2.13 | Excavator Plate compactor, Bomag roller, Front end loader, Mobile crane TLB, milling machine, paver, excavator, skid steer(bobcat), rollers (steel drum/ rubber tyre) grader, insist recycler, chip spreader, bitumen distributor(tanker) | - Collapse of scaffolding - Slips and falls - Fall from heights/ elevated positions | Heat exhaustion | None | Unauthorised access to site | - All staff will be deemed fit for work with medicals records in file - Required PPE will be provided for staff - All works to be supervised |

| REF NO | SUB ACTIVITY | SAFETY RISKS |  | HEALTH RISK |  | ENVIROMENTAL RISK |  | PUBLIC RISK |  | RECOMMENDED RISK CONTROL MEASURE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.15 | Employees working in close proximity to moving machinery Exposure to noise | noise |  | noise |  | none |  | No unauthorised access |  | - All power tools to be inspected before use. Records to be in file - Housekeeping to be maintained on site Employee to make use of SABS approved hearing protection, supervisor to ensure that the correct P.P.E is used. Task to be supervised by a competent supervisor |
| 2.16 | - Hazardous chemical substances |  | injuries Contact with skin/ eyes Inhalation or indigestion |  | Skin irritation, burns or infections Could cause loss in eyesight |  |  |  | - Required PPE to be used - DSTI to be done - All works to be supervised - All steel to be stacked within a designated area - Exposed rebar to be fitted with rebar caps - All power tools to be inspected before use. |  |
| 2.17 | Neighbouring business and public exposed to the nature of the construction activities |  | - Dust exposure - Heat exhaustion |  | Dust pollution |  | Unauthorised access to site |  | Communication with neighbouring business is critical. Health risk must be communicated to all - employees’ dust, noise generated out of the construction work must be managed - Fence constructed to approved standard |  |

| REF NO | SUB ACTIVITY | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| --- | --- | --- | --- | --- | --- | --- |
| 2.18 | Emergency Preparedness, Emergency Drill & Evacuation | No training No implementation Planning done Failure to have a basic site-specific Emergency Management Plan | Person/s unprepared to respond to the emergency at hand | none | none | The Construction Manager must ensure that a site specific Emergency Management Plan is developed for implementation. Adequate number of employees trained in the use of fire equipment |
| 2.17 | Utilizing incompetent Sub- Contractors | - Use of hand tools injuries - Trips and falls - Contact with machinery | - Chemical burns from concrete - Heat exhaustion - Dust exposure | Ground contamination | Collision with pedestrians | The Construction Manager must be reasonably satisfied that the Sub Contractors intended to be appointed have the necessary competencies and resources to carry out the work safely |
| 2.18 | Brickwork at manholes | - Manual handling injuries - Use hand tools injuries | - Ergonomical injuries - Heat exhaustion | None | None | - DSTI to be done - Risk assessment and SWP to be communicated to all staff - All power tools to be inspected before use - Fall arrest equipment to be used at all heights |

| REF NO | SUB ACTIVITY | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| --- | --- | --- | --- | --- | --- | --- |
| 2.19 | Mixing mortar | - Unsafe - electrocution - loading strain | Loading / unloading material | none | none | The Construction Manager must ensure that a SWP is developed implemented about safe lifting and loading procedures All work needs to be supervised |

| REF NO | SUB ACTIVITY | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| --- | --- | --- | --- | --- | --- | --- |
| 2.20 | Install Geosynthetic layer | Poor Ergonomics may result in muscular skeletal injuries | - Ergonomical injuries - Heat exhaustion | None | None | The Construction Manager must ensure that workers are trained in the risk of ergonomically injuries and methods to mitigate the risks |

| MAIN ACTIVITY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| REF NO | SUB ACTIVITY | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| 3.3 | Construction of drains | - Trips and falls - Manual handling injuries - Underground services - Open excavations | - Dust exposure - Heat exposure - Ergonomical injuries - Noise exposure | None | None | - All works to be supervised - DSTI to be done - Risk assessments & SWP to be done - All open excavations to be netted off - Access and egress to be provided for all open excavations - All spoil to be away from excavation side wall |

| MAIN ACTIVITY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| REF NO | SUB ACTIVITY | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| 3.4 | Pouring Ready Mix concrete | - Noise - Dust - Bumping against, - Concrete truck tipping over | - noise induced hearing loss | None | None | - flag person required and Visual check required - Wearing required PPE |
| 3.5 | Waste removal | - Noise - Dust - Bumping against, - Struck by flying items | - cuts and abrasions - dust inhalation - noise induced hearing loss | None | None | - Safe & proper use of hand tools - Wearing required PPE - practise of proper manual lifting of material |

| MAIN ACTIVITY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| REF NO | SUB ACTIVITY | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| 3.9 | Inlet and outlet structures | - Use of hand tools injuries - Trips and falls - Contact with machinery - Working in a restricted area - Manual handling | - Dust exposure - Ergonomical injuries - Chemical burns | None | None | - Safe & proper use of hand tools - Wearing required PPE - practise of proper manual lifting of material - Supervise the works by a competent supervisor |
| 3.10 | Working with hot asphalt | - lack of proper access / working platform |  Heat exhaustion | None | None | - DSTI to be done - Risk assessment and SWP to be communicated to all staff - Scaffold to be erected by competent person - Scaffold to be erected as per SANS 10085 - Supervise the works by a competent supervisor - Staff to be trained for work at heights - Fall arrest equipment to be used at all heights - Adequate lighting to be provided |

| REF NO | SUB ACTIVITY | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| --- | --- | --- | --- | --- | --- | --- |
| 6.3 | Cleaning tools with solvent | - Use of power tool – electrocution - Manual handling - Work in restricted area |  Dust exposure | None | None | - All power tools to be inspected before use - DSTI to be done - Risk assessment/SWP to be done and in file - All staff to have required PPE |
| 6.5 | Manual lifting of tool | - Fall from heights - Fall of - equipment from heights | - Dust exposure - Heat exhaustion - Manual exertion | None | None | - Work platform to be inspected - Full supervision at all times - Staff to have fall arrest equipment in use - Manual lifting techniques to be practiced at all times |
|  | Welding, flame cutting, soldering and similar operations | - Fumes inhalation - Eye injury | - Chest pain - heat | none | none | - Proper PPE required - Close supervision |

| MAIN ACTIVITY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| REF NO | SUB ACTIVITY | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| 7.1 | Inlet and outlet structures | Struck by vehicle | heat | none | none | Traffic management plan required |
| 7.2 | Applying tack | Eye injury | contamination | none | none | Close supervision all the time |

| MAIN ACTIVITY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| REF NO | SUB ACTIVITY | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| 8.1 | Site levelling using excavator/ bob cat | - Trips and falls - Exposed survey pegs - Use of hand tools | - Heat exposure - Noise exposure | Hydrocarbon spills | - Pedestrians exposed to machinery - Accidents | - Works to be supervised - Staff to use required PPE - Dust suppression to be maintained - Refuelling to be done in designated areas - Rebar caps to be fitted on exposed pegs - Risk assessments and SWP to be done for staff - All plant operators to be deemed competent - Flagman to be present on site |
| 8.2 | Backfill and compaction of other layers | - Manual handling - Trips and falls - Use of mobile plant accidents | - Heat exhaustion - Dust exposure - Noise exposure | Hydrocarbon spills | - Pedestrians exposed to machinery - Accidents | - Works to be supervised - Staff to use required PPE - Dust suppression to be maintained - Refuelling to be done in designated areas - Rebar caps to be fitted on exposed pegs - Risk assessments and SWP to be done for staff - All plant operators to be deemed competent - Flagman to be present on site |
| REF NO | SUB ACTIVITY | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| 8.4 | Concreting of walkways/ driveway | - Use of hand tools injuries - Trips and falls - Collapse of excavation side walls - Contact with machinery | - Chemical burns from concrete - Heat exhaustion - Dust exposure | Ground contamination | Collision with pedestrians | - DSTI to be done - Flagman to direct all trucks to site - All staff to have the required PPE when handling concrete - Nil staff to be in foundation during discharge of concrete into excavation - Delivery trucks to use designated access - All spills to be cleaned and disposed |

| MAIN ACTIVITY |  | COVID 19 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REF NO |  | SUB ACTIVITY |  | SAFETY RISKS |  |  | HEALTH RISK |  |  | ENVIROMENTAL RISK |  |  | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| 9.1 |  | Travel to site Deliveries -use of public transport |  | - Nil decontamination of public transport - Passengers in public transport not using face masks - Nil social distancing in public transport - Nil decontamination of supplies and deliveries |  |  | - Staff contracting the virus during use of public transport - Staff contracting virus during handling of contaminated supplies |  |  | None |  |  | None | - All staff to ensure that they use a face mask and sanitise when using the public transport - Contractor could arrange for a dedicated transport for all staff to and from work. - Staff are not to use their work PPE when travelling on public transport. - All staff to follow decontamination procedures when entering work site |
| 9.2 |  | Management to prepare site before construction works commence |  | - Nil decontamination of offices and facilities - Failure to align policies and procedures to COVID 19 virus |  |  | - Returning staff not provided with COVID related PPE and disinfectants - Possible contamination to staff |  |  | none |  |  | Unauthorised persons entering site | - Management to ensure all offices, facilities are disinfected with proper disinfectants - Overgrown vegetation to be cut before any staff can return to site - Required COVID 19 PPE is to be purchased and made available for staff when project commences |
|  | SUB ACTIVITY |  | SAFETY RISKS |  |  | HEALTH RISK |  |  | ENVIROMENTAL RISK |  |  | PUBLIC RISK |  | RECOMMENDED RISK CONTROL MEASURE |
|  | Site preparation |  | - Failure to display related signage’s for site access due to COVID virus. - Nil PPE and sanitisers (COVID related) for staff to return to work - Management not inducted/educate d on COVID 19 |  |  | Staff returning to work are not educated on policy changes Management unable to provide correct information to staff on COVID 19 issues |  |  | none |  |  | none |  | - Required access control signage’s (COVID related) to be displayed at main entrance - Management to rearrange facilities area to ensure social distancing - Policies and procedures (COVID related) to be amended and approved before site can reopen. This is to be communicated to all staff - An induction program (COVID 19) is to be prepared which is to be communicated to all staff when project commences - Required sanitisers and disinfectants to be purchased and made available before project commences - Contractor to ensure required staff information form pertaining to COVID 19 is available which is to be filled out by each staff - Adequate hazardous waste bins to be made available on site |
| 9.3 | Entering of construction work site |  | Staff fail to use required PPE |  |  | Staff may be infected with the virus |  |  | none |  |  |  |  |  All staff to follow the required social distancing when at main entrance |
| REF NO | SUB ACTIVITY |  | SAFETY RISKS |  | HEALTH RISK |  |  | ENVIROMENTAL RISK |  |  | PUBLIC RISK |  |  | RECOMMENDED RISK CONTROL MEASURE |
|  | Entering construction work |  | - Congestion of staff at main entrance to site - Failure of contractor to screen staff entering site - Staff not maintaining social distancing |  |  Staff spreading of the virus to other staff |  |  | none |  |  | none |  |  | - Due to lockdown, staff are to declare their movement, medical history during lockdown. These records are to be kept on site for record/tracing purposes - All staff are to be tested as per contractor’s protocol to enter the site. Taking of staff temperature and desensitising of entire staff. An infrared thermometer is to be used. - Contractor to ensure all staff conducting the required testing is provided with the required PPE and is supervised by management. - Staff are not permitted to enter the site with their work PPE. Work PPE is to be left on site when leaving site to prevent possible contamination of work PPE - Any staff tested with a high temperature or signs of COVID is to follow the quarantine procedure until staff can be tested by the DOH for possible COVID 19 infection. |

| 9.4 | Work tasks | - Nil social distancing on site - Staff socialising on site - Staff not using their required PPE to prevent COVID infections | - Spreading of the virus to other staff - Possible infections | none | Possible closure of site due to infection | - Identified tasks are to be evaluated to encourage social distancing - All staff to have the required PPE when on site - Staff to decontaminate all tools and workspace prior to any activity - Monitoring to be done by management - Inductions to be done for all staff on social distancing whilst at work. - Failure of staff to comply with COVID control measures are to be removed off site. |
| --- | --- | --- | --- | --- | --- | --- |

| REF NO | SUB ACTIVITY | SAFETY RISKS | HEALTH RISK | ENVIROMENTAL RISK | PUBLIC RISK | RECOMMENDED RISK CONTROL MEASURE |
| --- | --- | --- | --- | --- | --- | --- |
| 9.5 | Use of facilities during break intervals | - Nil social distancing - Nil hygiene been maintained at break intervals - Nil paper towel provided for staff at wash facilities - Lack of chairs for staff at facilities area | - Possible contamination of staff - Staff not practising adequate hygiene control measures - Possible spread of virus - Inadequate provision of hand wash /sanitisers for staff | none | none | - Breaks are to be staggered to allow social distancing of staff - All staff are to take their breaks at the identified facilities area - Layout of facilities area to be planned to encourage social distancing - Table and chairs to be disinfected before and after all break intervals - Toilets to be disinfected at frequent interval. - Adequate hand wash and sanitisers are to be provided for staff to frequently wash their hands - Paper towels to be provided for staff use - Labelled bins to be provided for all hazardous waste - Staff are not to share any meals or utensils - Staff are not to leave the site to purchase any meals |
| 9.6 | Use of jackhammer | - Injuries - Noise - Loss limb | - vibration | none | none | - Tools need to be inspected - Safety team supervisor |
| 9.10 | Ground water | - Injuries - Wet soil - Soil erosion | - Ground seepage | Rock formations Water Wet soil | none | - Proper PPE required - Close supervision |
| 9.11 | Slope stability | - Multiple injuries - Cracks caused by compacting - fatality | - Excavation collapsed | Rock formation |  | - Racking back required - Close supervision - PPE required |
| 9.12 | excavation | - Collapsed and cause multiple injury - fatality | - vibration |  |  | - Competent person to check on excavation - Close supervision on blasting - Notification for blasting |