TENDER NO.: 
WMM LM 00062 W13 ECDC









                      WARD 13 ECDC


WINNIE MADIKIZELA - MANDELA LOCAL MUNICIPALITY

PROVINCE OF EASTERN CAPE


TENDER NO.: WMM LM 00062 W13 ECDC  


	C4: SITE INFORMATION


C4.1

Nature of Ground

No Test Pit is available
The tenderer will be permitted to excavate trial holes in the area of the works at his own expense provided that they are properly safeguarded and reinstated. Should the tenderer wish to excavate his own trial holes; he shall first ascertain in conjunction with the engineer, the position of any underground services, which may exist in the area. The tenderer shall indemnify the municipality against the cost of repairing any underground services damaged by the Tenderer or his agent, while carrying out such excavations.

C4.2

Spoil Material
No indiscriminate spoiling of material will be allowed. All unsuitable or surplus material shall be spoiled off site to a spoil site/municipal dump, chosen by the Contractor.
C4.3

Finishing off the Site
The site shall be finished off in accordance with the specification as well as to the requirements of all applicable environmental standards.
C4.4

Existing Services
Although every effort has been made to depict existing services (water mains, electric cables, telephone cables etc), as accurately as possible on the contract drawings, insofar as they are known, variations do arise and the Contractor shall exercise extreme care when working in the area. Items have been allowed in the schedule of quantities for dealing with and protecting services.

The Contractor shall take whatever precautions are required to protect these services from damage during the period of the Contract.
C4.5

Proving of Underground Services
It is stressed that all services in a particular area must be proven before commencing work in that area.

Insofar as bulk earthworks are concerned, where services are indicated on the drawings or where from site observations can reasonably be expected that such services are likely to exist where excavations are to take place, the Contractor shall without instructions from the Engineer carefully excavate by hand to expose and prove their positions.
When a service is not located in its expected position, the Contractor shall immediately report such circumstances to the Engineer who will decide that further searching or other necessary action is to be carried out and shall instruct the Contractor accordingly.

Should any service be damaged by the Contractor in carrying out the works, and should it be found that the procedure laid down in this clause has not been followed, then all costs in connection with the repair of the service will be to the Contractor’s account.

Providing of services shall be completed at least two weeks in advance of the actual programmed date for commencing work in the area. The position of these services located must be coordinated and leveled by the Contractor, and the information given in writing to the Engineer’s representative.

The requirements of this clause do not relieve the Contractor of any obligations as detailed under the General Conditions Contract or the Special Conditions of Contract.

C4.6

Drawings
See attached drawings.

CONSTRUCTION OF WARD 13 ECDC








� LINK Excel.Sheet.8 "C:\\Users\\user\\Desktop\\PROJECTS\\MBIZANA LOCAL MUNICIPALITY\\TENDER DOC\\TENDER DOC\\MAKHOSONKE TENDER DOC\\Consultants Input Sheet - Regravelling Document.xls" Sheet1!R3C3 \a \f 4 \r \* MERGEFORMAT �Error! Not a valid link.�











The Contract – Part 4 – Site Information



C4.1 


