SPECIFICATIONS

Maintenance to LARGE 1HA, net house structure, at ARC, Addo.

Size of structure – (L x B x H) 130m X 93m X 5m

Scope of work to be done;

Structure;

- Plant and secure inside and outside poles.

- Plant and secure anchors.

- Putting up cable to secure structure and level poles.

- Securing of cable with grips.

Netting;

- Covering of pole heads.

- Threading of net on steel wire.

- Pulling up roof and securing wire and net.

- Putting up side net and securing net.

Material needed;

| DESCRIPTION | AMOUNT |
| --- | --- |
| Poles, Creosote poles, 4.6 meter (100x125mm) | 60 |
| Stayrods, galvanised, 1.5m x 20mm | 64 |
| Beton anchors 690mm (diameter) x 150mm (Middle thickness) | 64 |
| Oval steel wire 2.31mm x 1500 meter | 6 |
| Grip VDWG 240 (150 x 175mm) PLP GRIPS | 250 |
| Grip VBT 240 (100 x 125mm) PLP GRIPS | 360 |
| Shade net, 3m wide (40% white, with 2 draw cords) | 2100 meters |
| Hog rings HR 23 (10 000 pieces) | 5x10 000 = 50 000 pieces |

Equipment needed for job; to be supplied by contrator;

- Excavator, 5 ton mini excavator, for poles and anchors.

- Cherry picker or similar equipment to work at a height of 5 meters.