| Mobile Community Data Collection Unit |  |
| --- | --- |

|  | 1. Background |
| --- | --- |

The Human Sciences Research Council (HSRC) through the Centre for Community Based Research are implementing the ART resupply and delivery optimisation in Umgungundlovu District.

Our community-based study will explore approaches to improve the provision of ART and HIV related medication in the community using mobile trailers to provide convenient faclities in the community setting to receive care and conduct surveys.

|  | 2. Objective: To design and produce 2 mobile community data collection units. |
| --- | --- |

A registered truck body builder is required to provide 2 trailers that will provide mobile spaces for community surveys and patient care visits. Each trailer must comply with the following specifications:

- Carry a stationary load of minimum 850 kilograms.

- Design a chassis with the necessary capacity axle, rims and tyres to carry an upper structure that includes 2 consulting rooms, a toilet and storeroom as below:

  - Consulting rooms (minimum 1750mm x 2500mm)

  - Ablutions (minimum 1200mm x 1800mm)

  - Storage area (minimum 1200mm x 700mm)

- Trailer chassis:

  - Main chassis using minimum 76 x 38 x  6mm channel irons

  - Trailer bed supporting the floor from minimum 150 x 50 x 20 x 2mm lip channel

  - Trailer floor must be rubberized

  - Minimum 1800 kilogram 50mm braked axle

  - 3000 kilogram brake coupler

  - 15 inch steel rims with SABS approved tyres (Goodyear/Dunlop or Bridgestone 245 x 75 R15)

  - Jockey wheel

  - Hand operated adjustable stabilisers x 4 (each corner)

- Upper body:

  - Minimum 0.8mm chromadek clad 40mm insulated polyurethane panels

  - Reinforced corners and edges for durability

  - Appropriate light clusters in compliance with road traffic rules and regulations

- Consulting Rooms (x2)

  - 1 x chromadek clad standard door (minimum 813mm x 2032 mm)

  - 1 x aluminium frame sliding window (minimum 75mm wide x 50 mm height)

  - 30W White finish extractor fan (minimum 30cm diameter)

  - 2500mm x 500mm bench with upholstered cushions (anti tear vinyl) with storage area under

  - 1500mm x 500 mm bench with upholstered cushions (anti-tear vinyl) with storage under,

  - Foldable table (500mm wide x 750 mm long) with stand

  - 10w LED bulkhead light with switch (switch to control light and extractor)

  - 2 x 3pin plug point with USB C charging

  - Minimum 350mm (d) x 600mm (l) Chromadek counter fitted with drop in round sink (minimum 30cm diameter) with cold water tap, plumbed to central septic tank.

  - Storage cabinets fitted under counter.

- Bathroom

  - Slimline 7w LED bulkhead light with switch

  - 1 x chromadek clad standard door (minimum 813mm x 2032 mm)

  - 30cm diameter extractor fan connected to light switch

  - Flushable standard toilet connected to gravity fed water tank for flushing

  - Gravity fed water tank (minimum 50 litres) to basin with tap

  - Rubberised non-slip floor

  - Waste septic tank for toilet minimum capacity of 150l

  - Waste tank to be fitted with standard outlet for vacuum cleaning of waste

  - Window insert in door (minimum 250m x 150mm) – privacy glass

- Storage area

  - Minimum 3000W 12V inverter, minimum 2kva LiON battery

  - Slimline 7w LED bulk with switch

  - DB board wired to all lights and electricals

  - 2 x 3pin plug point with USB C charging

  - 1 x chromadek clad standard door (minimum 813mm x 2032 mm)

  - Rubberised floor

Indicative Floor Plan For Guidance (ruler in metres):

Standard Requirements For All Service Providers:

- Proof of registration as a registered truck body builder with Department of Transport with minimum 10 years experience.

- Design of the community survey trailer as per the specifications and sketch. Technical drawing to be provided with quotation.

- Comply with minimum local content threshold of 80%.

- Production and manufacture of the community survey trailer (x 2) and delivered on completion.

- Ensure all specifications detailed are complied with.

- Ensure compliance with all Department of Transport regulations for trailers.

- Certify, register and warrant roadworthiness of the trailer – supplier to provide COR for trailers (x2).

- Supply trailers fully licensed and registered in the name of the HSRC.

- Provide fair use warranty on all parts for a minimum of 2 years.

Other Information:

- Briefing: Tuesday, 09h30 11 December 2023

- Closing date for quotes: Wednesday, 24 January 2024