Soil Penetrometer (digital) specification

- MECHANICAL:

- Frame: 316L Stainless Steel Pillars; H4 Etched Aluminium spacers and frames; 304 Stainless Steel Fittings

- Penetration: 80cm Depth, Readings stored every 1cm; Silver steel rod 10 mm diameter

- Transport Case: Aluminum weather-proof case

- PHYSICAL:

- Height 1250 mm; Footprint 290 x 580mm with feet; Weight 21 Kg; Shipping Weight 25 Kg; Case Size 1300 x 270 x 200 mm high

- LOADCELL:

- Type SUB-G-200, Repeatability 0.01%; Temperature drift 0.015% per 1ºC, Accuracy error 0.03%; Calibration As per ASAE Standard S313.1 (1986)

- CONE:

- Base area 0.2 in 2 / 130 mm 2; Diameter	0.505 in / 12.8 mm; Max load 5000 Kpa; Apex angle 30 deg; Length 50 mm; Material 416L S/Steel

- DRIVE:

- Winding ratio 4.8 : 1; Chain 12.6mm pitch 1950mm long; Penetration 1000 mm/min at 1 second/rev.

- ELECTRONICS:

- Housing Detachable powder-coated weather-proof case (IP65 rated)

- Physical Length 220mm  Width 120mm  Height 90mm

- Microprocessor 8086 with 64 Kb RAM and 256 Kb ROM

- Keyboard	12 key membrane pad

- Display Liquid crystal, alpha-numeric 2 line, 40 characters each

- Data Output RS 232 at 9600 baud, 8 data bits, 1 stop bit, no parity, hardware handshake

- Power 4 Volt 3Ah rechargeable battery

- Charger Universal 5W, input 5-15V