Agricultural Research Council, Roodeplaat-east, Roodeplaat, ARC-Plant Protection Research

P/Bag X134, Pretoria Queenswood, 0121 South Africa

Tel: 012 8088177 (Int: +27-12-8088177)

REQUEST FOR QUOTATION

| RFQ NUMBER: | ARC-PHP/STELLENBOSCH/04/2023 |
| --- | --- |
| CLOSING DATE: | 09-08-2023 |
| DESCRIPTION | Laboratory Equipment |
| ADDRESS | ARC-PHP off Adam Tas road (Behind Distell), Vredenburg farm/campus Stellenbosch, BANHOEK 7600 |
| ENQUIRY | Name: Rirhandzu Hlatshwayo SCM ARC: Plant Protection Research Email: ppriquote@arc.agric.za |

1.         YOU ARE HEREBEY INVITED TO SUBMIT QUOTATIONS FOR THE REQUIREMENTS OF AGRICULTURAL RESEARCH COINCIL

|  | Quantity | UOM |
| --- | --- | --- |
| REQ-070864 - Fridge |  |  |
| -Free standing stainless steel fridge - Energy efficiency class: A++ - Total net capacity: 324 litres - Noise level dB(A) re 1 pW: 39 - Doors inox EasyClean, side panels Pearl grey -6 safety glass shelves of which 5 are height adjustable, - Internal LED lights - LED electronic control - constant temperature by intelligent sensor technology - Supercool function - Auto defrost - Interior Fan - Dimensions: H 176 cm x L 60 cm x D 65 cm - Door left hinged, reversible - Height adjustable front feet, roles in the back - Connected load 90 W - Rated Voltage 220 - 240 V -2 year warranty | 1 | EA |
| REQ-070866 - Repeater pipette |  |  |
| - Volume range: 20-200 µl - Adjustable volume - Electronic air displacement pipette - A single-channel pipette - Must have four pre-programmed pipetting modes (pipet, repetitive, mix, and reverse) and one custom mode - 2 year warranty | 1 | EA |
| REQ-070867 - Mini centrifuge |  |  |
| -Easy-to-use with dual start/stop function -Maximum speed 7000rpm/2680xg -Supplied with an 8 place 1.5/2.0ml microtube rotor and 16 x PCR tube rotor & 0.2/0.5ml tube adapters -Compact design -Whisper quiet operation, noise level ≤ 45dB -Ideal for quick spin downs of microtubes and PCR tubes -Simply close the lid for acceleration and open the lid for deceleration, or use the on/off switch -Accelerates and brakes in seconds -Rotor change without tools -2 YEAR WARRANTY |  |  |
| REQ-070869 - Waterbath |  |  |
| - Operating temperature range from 5 °C above room temperature to 99 °C (temperature adjustment range from 10 to 99 °C). - Stainless steel inner tank - Stainless steel gable cover - Digital display for displaying setpoint/actual temperature values - Overheating protection - Must have a water drainage tube - Capacity: 22liter to 35 liters - 2 year warranty |  |  |
| REQ-070870 - Vortex machine |  |  |
| -Must have three-position power switch: an on (continuous operation), off and touch (triggered by pressing the attachment) switch -variable speed control that must range between 600 to 2700 rpm -Easy-to-change mixing heads that can snap on and off and must come with a standard rubber head -Tough metal housing combined with a cast-metal base and rubber feet to provide a stable platform -two year warranty |  |  |
| REQ-070872 - PCR machine |  |  |
| • Sample capacity: 96 x 0.2 ml tubes, 0.2 ml tube strips, or 1 x 96-well plate • Maximum ramp rate, °C/sec: 4 • Average ramp rate, °C/sec: 2.5 • Temperature range: 4–100°C • Temperature accuracy: ±0.5°C of programmed target • Temperature uniformity: ±0.5°C well-to-well within 30 sec of arrival at target temperature • Display: 5.7'' VGA color touch screen • Port: 1 USB A • Memory: 500 typical programs; unlimited with USB flash drive expansion • Must have thermal gradient capability • Gradient range: 30–100°C • Temperature differential range: 1–25°C • Installation and calibration • 2-year warrantee |  |  |
| REQ-070880 - Benchtop centrifuge |  |  |
| • Max. RCF: 21,300 × g • Max. RCF with fixed-angle rotor: 21300 × g • Display large, brightly lit LCD • Speed: 100 – 15,060 rpm • Max. capacity: 24 × 1.5/2.0 mL • Acceleration time1: 15 s • Deceleration time1: 15 s • Timer: 10 s to 9:59 h, with continuous run function • SOFT brake function: 10 ramps • Noise level: < 56 dB(A) • Cooling: non-refrigerated • 2-year warrantee |  |  |
| REQ-074933 - Culture cabinet |  |  |
| To supply 1 off Culture cabinet: • Trolley 1240 long 1730mm high • 80 trays per trolley • 112 off Ø29 bottle holes per tray • Super-elastic non marking casters • Galvanized swivel & braked wheel brackets • 304 Stainless steel covered sides • Tray locking bars • Trays, frame, and Tray inserts electro galvanized mild steel • Delivery included - 2-year warrantee |  |  |
| REQ-074936 - Multi-channel pipette |  |  |
| Multi-channel pipette system • Eppendorf Research® Plus or an equivalent product • Manual pipette • 8-channel, volume (0.5-10 μL) • Mechanical air-cushion pipette for accurate, easy & ergonomic pipetting of aqueous solutions • The entire pipette must be autoclaveable • Material must have a high UV and chemical resistance • 2-year warrantee |  |  |
| REQ-074937 - Vortex machine |  |  |
| -Must have three-position power switch: an on (continuous operation), off and touch (triggered by pressing the attachment) switch -variable speed control that must range between 600 to 2700 rpm -Easy-to-change mixing heads that can snap on and off and must come with a standard rubber head -Tough metal housing combined with a cast-metal base and rubber feet to provide a stable platform -two year warranty |  |  |
| REQ-074940 - Pipettes |  |  |
| Pipette set one • Manual pipettes • 2-year warrantee • Eppendorf Research Plus or an equivalent product • Mechanical air-cushion pipette for accurate, easy & ergonomic pipetting of aqueous solutions • The entire pipette must be autoclaveable • Material must have a high UV and chemical resistance • Pipette sizes  P10 (1-10 µl)  P20 (2-20 µl)  P100 (10-100 µl)  P200 (20-200 µl)  P1000 (100-1000 µl) Pipette set two • Manual pipette • 2-year warrantee • Eppendorf Research Plus, Gilson or equivalent product • Mechanical air-cushion pipette for accurate, easy & ergonomic pipetting of aqueous solutions • The entire pipette must be autoclaveable • Material must have a high UV and chemical resistance • Pipette sizes  P10 (1-10 µl)  P20 (2-20 µl)  P100 (10-100 µl)  P200 (20-200 µl)  P1000 (100-1000 µl) |  |  |
| REQ-074941 - Vortex machine |  |  |
| -Must have three-position power switch: an on (continuous operation), off and touch (triggered by pressing the attachment) switch -variable speed control that must range between 600 to 2700 rpm -Easy-to-change mixing heads that can snap on and off and must come with a standard rubber head -Tough metal housing combined with a cast-metal base and rubber feet to provide a stable platform -two year warranty |  |  |
| REQ-074942 - Specialised freezer |  |  |
| Auto defrost Uniformed temperature and high-performance refrigeration Temperature, door ajar, and power failure (with battery backup) alarms. 2 solid door Upright Freezer No plumbing needed Capacity 1250 litre minimum, would prefer 1500 litre Reliable temperature control and performance Clear temperature display Temperature range: ± -10 to -22 °C Exterior and interior steel body Low heat output Supplied with at least 4-8 adjustable shelves Shelf capacity 30kg Whisper quiet operation < 50 dBA 2-year warranty |  |  |
| REQ-074944 - Pipettes |  |  |
| Pipette set one • Manual pipettes • 2-year warrantee • Eppendorf Research Plus or an equivalent product • Mechanical air-cushion pipette for accurate, easy & ergonomic pipetting of aqueous solutions • The entire pipette must be autoclaveable • Material must have a high UV and chemical resistance • Pipette sizes  P10 (1-10 µl)  P20 (2-20 µl)  P100 (10-100 µl)  P200 (20-200 µl)  P1000 (100-1000 µl) Pipette set two • Manual pipette • 2-year warrantee • Eppendorf Research Plus, Gilson or equivalent product • Mechanical air-cushion pipette for accurate, easy & ergonomic pipetting of aqueous solutions • The entire pipette must be autoclaveable • Material must have a high UV and chemical resistance • Pipette sizes  P10 (1-10 µl)  P20 (2-20 µl)  P100 (10-100 µl)  P200 (20-200 µl)  P1000 (100-1000 µl) |  |  |
| REQ-074946 - Roll-label printer |  |  |
| • Print in black and red* to highlight important information • Wide range of connectivity options • PC, Mac, iOS and Android compatibility • High speed printing of up to 110 labels per minute • Use continuous rolls and cutter to print labels up to one metre in length • Stand-alone printing function where no PC required • Compact and contemporary styling (small footprint) • Easy to change label/tape rolls, 12mm to 62mm width • Thermal printing with no ink or toner needed • Ability to print labels up to 62mm wide at 300dpi • Image. logo and barcode printing • Optional li-ion battery for printing away from power sources • Software Development Kits available for Microsoft Windows**, iOS and Android devices to add label printing to your own solutions • P-touch Editor with Microsoft Oce add-in • Advanced label design software for PC/Mac (P-touch Editor) • USB • Wide range of labels available in paper and plastic film material, pre-sized labels or continuous rolls • 2-year warranty |  |  |
| REQ-074947 - Freezer |  |  |
| • Free-standing upright freezer • Stainless steel • No frost fridge – internal central chiller unit with automatic defrost function • A++ energy efficiency • Must have Temperature setting • Must have warning signal for open door • Must have warning signal for malfunction • Total unit net capacity: 225 Litre • Total unit gross capacity: 246 Litre • Noise level: 42 dB • Approximate dimensions: 1760 x 600 x 650 mm • 2-year warranty |  |  |
| REQ-074950 - Real-time PCR machine |  |  |
| - Sample capacity: 96 x 0.2 ml tubes, 0.2 ml tube strips, or 1 x 96-well plate - Maximum ramp rate, °C/sec: 4 - Average ramp rate, °C/sec: 2.5 - Temperature range: 4–100°C - Temperature accuracy: ±0.5°C of programmed target - Temperature uniformity: ±0.5°C well-to-well within 30 sec of arrival at target temperature - Display: 5.7'' VGA color touch screen - Port: 1 USB A - Memory: 500 typical programs; unlimited with USB flash drive expansion - Must have thermal gradient capability - Gradient range: 30–100°C - Temperature differential range: 1–25°C - Installation and calibration - 2-year warrantee |  |  |

- Essential administrative  requirements:

- Only bidders registered on the Central Supplier Database (CSD) will be considered.  Bidders shall include the CSD registration number and full CSD report with the bid proposal.

- Only bidders that claim specific goals will be considered for scoring on the 20 points as per the 80/20 principle.

- Completed and signed Standard Bidding Documents (SBD) forms included in the bid document.

- The above specified goods/services should be delivered / rendered to the ARC-Plant Protection Research at above-mentioned delivery address.

- The particulars of the guarantee that will apply to the goods quoted for, with the particular regards to the period and extent of the warranty must be clearly stated.  Where services are required, service providers must submit documentation pertaining the relevant experience.

- Your written quotation must be emailed depending on the instructions given in the email or advert.

- All price quotations that have a rand value of R 2,000.00 to below R 50,000,000.00, including VAT, will be evaluated by applying the 80/20 principle as prescribed by the Preferential Procurement Policy Framework Act 5 of 2022 and its Regulations.

- The lowest acceptable price will score 80 points, Specific goals for the tender and points claimed are indicate per the table below:

| The specific goals allocated points in terms of this tender | Number of points allocated (80/20 system) (To be completed by the organ of state) | Number of points claimed (80/20 system) (To be completed by the tenderer) |
| --- | --- | --- |
| Percentage (%) Ownership by HDIs | Points 8 |  |
| 91-100% | 8 |  |
| 81-90 | 7 |  |
| 71-80 | 6 |  |
| 61-70 | 5 |  |
| 51-60 | 4 |  |
| 41-50 | 3 |  |
| 21-40 | 2 |  |
| 1-20 | 1 |  |
| 0% | 0 |  |
| Percentage (%) ownership by Women | Points (4) |  |
| 81-100 | 4 |  |
| 51-80 | 3 |  |
| 31-50 | 2 |  |
| 1-30 | 1 |  |
| 0% | 0 |  |
| Percentage (%) Ownership ( by Youth | Points (4) |  |
| 81-100 | 4 |  |
| 51-80 | 3 |  |
| 31-50 | 2 |  |
| 1-30 | 1 |  |
| 0% | 0 |  |
| Percentage Ownership by PwD | Points (2) |  |
| 51-100% | 2 |  |
| 1-50 | 1 |  |
| 0% | 0 |  |
| RDP Goals | Points (2) |  |
| EME’s/QSE’s | 2 |  |

- Standard conditions:

- The validity of the quotations must be indicated.

- Prices quoted should be in South African Rand and inclusive of VAT costs such as delivery, insurance, taxes, etc.

- No price adjustments or amendment of the delivery particulars contained in paragraph 8.2 will be considered by the ARC.

- The supplier accepts full responsibility for the proper execution and fulfilment of the goods/services quoted for.

- ARC reserves the right to accept or reject any special terms and conditions that may qualify the goods/services to be provided.

- Quotes should be submitted on an official letterhead and duly signed.

- Goods and services should be supplied / rendered upon receipt of a purchase order from the ARC.

- The General Conditions of Contract issued by National Treasury are applicable.

- The ARC supply chain management code of conduct is applicable.

- Standard Bidding Documents (SBD) forms must be signed and returned together with the quotation.  Failure to comply may result to disqualification of your quotation.

- Your quotation must indicate the delivery date.

- The ARC reserve the right to do due diligence on the quotations.

- The ARC reserves the right to benchmark prices quoted.

# SBD 6.1

PREFERENCE POINTS CLAIM FORM IN TERMS OF THE PREFERENTIAL PROCUREMENT REGULATIONS 2022

This preference form must form part of all tenders invited.  It contains general information and serves as a claim form for preference points for specific goals.

NB:	BEFORE COMPLETING THIS FORM, TENDERERS MUST STUDY THE GENERAL CONDITIONS, DEFINITIONS AND DIRECTIVES APPLICABLE IN RESPECT OF THE TENDER AND PREFERENTIAL PROCUREMENT REGULATIONS, 2022

- GENERAL CONDITIONS

  - The following preference point systems are applicable to invitations to tender:

- the 80/20 system for requirements with a Rand value of up to R50 000 000 (all applicable taxes included); and

- the 90/10 system for requirements with a Rand value above R50 000 000 (all applicable taxes included).

  - To be completed by the organ of state

(delete whichever is not applicable for this tender).

- The applicable preference point system for this tender is the 90/10 preference point system.

- The applicable preference point system for this tender is the 80/20 preference point system.

- Either the 90/10 or 80/20 preference point system will be applicable in this tender. The lowest/ highest acceptable tender will be used to determine the accurate system once tenders are received.

  - Points for this tender (even in the case of a tender for income-generating contracts) shall be awarded for:

- Price; and

- Specific Goals.

  - To be completed by the organ of state:

The maximum points for this tender are allocated as follows:

|  | POINTS |
| --- | --- |
| PRICE | 80 |
| SPECIFIC GOALS | 20 |
| Total points for Price and SPECIFIC GOALS | 100 |

  - Failure on the part of a tenderer to submit proof or documentation required in terms of this tender to claim points for specific goals with the tender, will be interpreted to mean that preference points for specific goals are not claimed.

  - The organ of state reserves the right to require of a tenderer, either before a tender is adjudicated or at any time subsequently, to substantiate any claim in regard to preferences, in any manner required by the organ of state.

- DEFINITIONS

- “tender” means a written offer in the form determined by an organ of state in response to an invitation to provide goods or services through price quotations, competitive tendering process or any other method envisaged in legislation;

- “price” means an amount of money tendered for goods or services, and includes all applicable taxes less all unconditional discounts;

- “rand value” means the total estimated value of a contract in Rand, calculated at the time of bid invitation, and includes all applicable taxes;

- “tender for income-generating contracts” means a written offer in the form determined by an organ of state in response to an invitation for the origination of income-generating contracts through any method envisaged in legislation that will result in a legal agreement between the organ of state and a third party that produces revenue for the organ of state, and includes, but is not limited to, leasing and disposal of assets and concession contracts, excluding direct sales and disposal of assets through public auctions; and

- “the Act” means the Preferential Procurement Policy Framework Act, 2000 (Act No. 5 of 2000).

- FORMULAE FOR PROCUREMENT OF GOODS AND SERVICES

  - POINTS AWARDED FOR PRICE

3.1.1   THE 80/20 OR 90/10 PREFERENCE POINT SYSTEMS

A maximum of 80 or 90 points is allocated for price on the following basis:

# 80/20	or	90/10

Ps=801-Pt-PminPmin	or	Ps=901-Pt-PminPmin

Where

Ps	=	Points scored for price of tender under consideration

Pt	=	Price of tender under consideration

Pmin	=	Price of lowest acceptable tender

  - FORMULAE FOR DISPOSAL OR LEASING OF STATE ASSETS AND INCOME GENERATING PROCUREMENT

    - POINTS AWARDED FOR PRICE

A maximum of 80 or 90 points is allocated for price on the following basis:

# 80/20	               or	            90/10

Ps=801+Pt-PmaxPmax	or	Ps=901+Pt-PmaxPmax

Where

Ps	=	Points scored for price of tender under consideration

Pt	=	Price of tender under consideration

Pmax	=	Price of highest acceptable tender

- POINTS AWARDED FOR SPECIFIC GOALS

  - In terms of Regulation 4(2); 5(2); 6(2) and 7(2) of the Preferential Procurement Regulations, preference points must be awarded for specific goals stated in the tender. For the purposes of this tender the tenderer will be allocated points based on the goals stated in table 1 below as may be supported by proof/ documentation stated in the conditions of this tender:

  - In cases where organs of state intend to use Regulation 3(2) of the Regulations, which states that, if it is unclear whether the 80/20 or 90/10 preference point system applies, an organ of state must, in the tender documents, stipulate in the case of—

- an invitation for tender for income-generating contracts, that either the 80/20 or 90/10 preference point system will apply and that the highest acceptable tender will be used to determine the applicable preference point system; or

- any other invitation for tender, that either the 80/20 or 90/10 preference point system will apply and that the lowest acceptable tender will be used to determine the applicable preference point system,

then the organ of state must indicate the points allocated for specific goals for both the 90/10 and 80/20 preference point system.

Table 1: Specific goals for the tender and points claimed are indicated per the table below.

(Note to organs of state: Where either the 90/10 or 80/20 preference point system is applicable, corresponding points must also be indicated as such.

Note to tenderers: The tenderer must indicate how they claim points for each preference point system.)

| The specific goals allocated points in terms of this tender | Number of points allocated (90/10 system) (To be completed by the organ of state) | Number of points allocated (80/20 system) (To be completed by the organ of state) | Number of points claimed (90/10 system) (To be completed by the tenderer) | Number of points claimed (80/20 system) (To be completed by the tenderer) |
| --- | --- | --- | --- | --- |
| Percentage (%) Ownership by HDIs |  | Points 8 |  |  |
| 91-100% 8 |  |  |  |  |
| 81-90 7 |  |  |  |  |
| 71-80 6 |  |  |  |  |
| 61-70 5 |  |  |  |  |
| 51-60 4 |  |  |  |  |
| 41-50 3 |  |  |  |  |
| 21-40 2 |  |  |  |  |
| 1-20 1 |  |  |  |  |
| 0% 0 |  |  |  |  |
| Percentage (%) ownership by Women |  | Points (4) |  |  |
| 81-100 4 |  |  |  |  |
| 51-80 3 |  |  |  |  |
| 31-50 2 |  |  |  |  |
| 1-30 1 |  |  |  |  |
| 0% 0 |  |  |  |  |
| Percentage (%) Ownership ( by Youth |  | Points (4) |  |  |
| 81-100 4 |  |  |  |  |
| 51-80 3 |  |  |  |  |
| 31-50 2 |  |  |  |  |
| 1-30 1 |  |  |  |  |
| 0% 0 |  |  |  |  |
| Percentage Ownership by PwD |  | Points (2) |  |  |
| 51-100% 2 |  |  |  |  |
| 1-50 1 |  |  |  |  |
| 0% 0 |  |  |  |  |
| RDP Goals |  | Points (2) |  |  |
| EME’s/QSE’s 2 |  |  |  |  |
| Total |  |  |  |  |

DECLARATION WITH REGARD TO COMPANY/FIRM

  - Name of company/firm…………………………………………………………………….

  - Company registration number: …………………………………………………………...

  - TYPE OF COMPANY/ FIRM

Partnership/Joint Venture / Consortium

One-person business/sole propriety

Close corporation

Public Company

Personal Liability Company

(Pty) Limited

Non-Profit Company

State Owned Company

[Tick applicable box]

  - I, the undersigned, who is duly authorised to do so on behalf of the company/firm, certify that the points claimed, based on the specific goals as advised in the tender, qualifies the company/ firm for the preference(s) shown and I acknowledge that:

- The information furnished is true and correct;

- The preference points claimed are in accordance with the General Conditions as indicated in paragraph 1 of this form;

- In the event of a contract being awarded as a result of points claimed as shown in paragraphs 1.4 and 4.2, the contractor may be required to furnish documentary proof to the satisfaction of the organ of state that the claims are correct;

- If the specific goals have been claimed or obtained on a fraudulent basis or any of the conditions of contract have not been fulfilled, the organ of state may, in addition to any other remedy it may have –

  - disqualify the person from the tendering process;

  - recover costs, losses or damages it has incurred or suffered as a result of that person’s conduct;

  - cancel the contract and claim any damages which it has suffered as a result of having to make less favourable arrangements due to such cancellation;

  - recommend that the tenderer or contractor, its shareholders and directors, or only the shareholders and directors who acted on a fraudulent basis, be restricted from obtaining business from any organ of state for a period not exceeding 10 years, after the audi alteram partem (hear the other side) rule has been applied; and

  - forward the matter for criminal prosecution, if deemed necessary.

……………………………………….

SIGNATURE(S) OF TENDERER(S)

SURNAME AND NAME:	 ……………………………………………………….

DATE:			………………………………………………………

ADDRESS:		………………………………………………………

………………………………………………………

………………………………………………………

………………………………………………………

SBD4

BIDDER’S DISCLOSURE

- PURPOSE OF THE FORM

Any person (natural or juristic) may make an offer or offers in terms of this invitation to bid. In line with the principles of transparency, accountability, impartiality, and ethics as enshrined in the Constitution of the Republic of South Africa and further expressed in various pieces of legislation, it is required for the bidder to make this declaration in respect of the details required hereunder.

Where a person/s are listed in the Register for Tender Defaulters and / or the List of Restricted Suppliers, that person will automatically be disqualified from the bid process.

- Bidder’s declaration

2.1 	Is the bidder, or any of its directors / trustees / shareholders / members / partners or any person having a controlling interest in the enterprise,

employed by the state?						YES/NO

2.1.1	If so, furnish particulars of the names, individual identity numbers, and, if applicable, state employee numbers of sole proprietor/ directors / trustees / shareholders / members/ partners or any person having a controlling interest in the enterprise, in table below.

| Full Name | Identity Number | Name of State institution |
| --- | --- | --- |

2.2	Do you, or any person connected with the bidder, have a relationship with any person who is employed by the procuring institution? YES/NO

2.2.1     If so, furnish particulars:

……………………………………………………………………………………

……………………………………………………………………………………

2.3 	Does the bidder or any of its directors / trustees / shareholders / members / partners or any person having a controlling interest in the enterprise have any interest in any other related enterprise whether or not they are bidding for this contract?				YES/NO

    - If so, furnish particulars:

…………………………………………………………………………….

…………………………………………………………………………….

- DECLARATION

I, the undersigned, (name)……………………………………………………………………. in submitting the accompanying bid, do hereby make the following statements that I certify to be true and complete in every respect:

3.1 	I have read and I understand the contents of this disclosure;

3.2	I understand that the accompanying bid will be disqualified if this disclosure is found not to be true and complete in every respect;

3.3 	The bidder has arrived at the accompanying bid independently from, and without consultation, communication, agreement or arrangement with any competitor. However, communication between partners in a joint venture or consortium will not be construed as collusive bidding.

3.4 	In addition, there have been no consultations, communications, agreements or arrangements with any competitor regarding the quality, quantity, specifications, prices, including methods, factors or formulas used to calculate prices, market allocation, the intention or decision to submit or not to submit the bid, bidding with the intention not to win the bid and conditions or delivery particulars of the products or services to which this bid invitation relates.

3.4	The terms of the accompanying bid have not been, and will not be, disclosed by the bidder, directly or indirectly, to any competitor, prior to the date and time of the official bid opening or of the awarding of the contract.

3.5 	There have been no consultations, communications, agreements or arrangements made by the bidder with any official of the procuring institution in relation to this procurement process prior to and during the bidding process except to provide clarification on the bid submitted where so required by the institution; and the bidder was not involved in the drafting of the specifications or terms of reference for this bid.

  - I am aware that, in addition and without prejudice to any other remedy provided to combat any restrictive practices related to bids and contracts, bids that are suspicious will be reported to the Competition Commission for investigation and possible imposition of administrative penalties in terms of section 59 of the Competition Act No 89 of 1998 and or may be reported to the National Prosecuting Authority (NPA) for criminal investigation and or may be restricted from conducting business with the public sector for a period not exceeding ten (10) years in terms of the Prevention and Combating of Corrupt Activities Act No 12 of 2004 or any other applicable legislation.

I CERTIFY THAT THE INFORMATION FURNISHED IN PARAGRAPHS 1, 2 and 3 ABOVE IS CORRECT.

I ACCEPT THAT THE STATE MAY REJECT THE BID OR ACT AGAINST ME IN TERMS OF PARAGRAPH 6 OF PFMA SCM INSTRUCTION 03 OF 2021/22 ON PREVENTING AND COMBATING ABUSE IN THE SUPPLY CHAIN MANAGEMENT SYSTEM SHOULD THIS DECLARATION PROVE TO BE FALSE.

………………………………	 ..……………………………………………

Signature	                          Date

………………………………	………………………………………………

Position 	Name of bidder

the power, by one person or a group of persons holding the majority of the equity of an enterprise, alternatively, the person/s having the deciding vote or power to influence or to direct the course and decisions of the enterprise.

Joint venture or Consortium means an association of persons for the purpose of combining their expertise, property, capital, efforts, skill and knowledge in an activity for the execution of a contract.