| # CATEGORY 2 - Fibre Installation Training |  |  |
| --- | --- | --- |
| The following requirements are set as the minimum content requirements for fibre optic courses. The courses shall be structured by the supplier into modules/levels so as to advance the learners knowledge from basic to advanced. |  |  |
| Fibre Optic Advanced Courses | Mandatory Compliance | Scoring Criteria |
| The Learner shall be able to demonstrate and perform the installation of OPGW, ADSS, and Wrapped fibre. The learner must demonstrate the use of the installation machinery required (winch and Tensioner, use of pulleys etc.), tensioning, associated safety and earthing. The learners need to be able to identify incorrect/poor installation practises, incorrect/poor installation of attachment hardware and highlight any safety issues. | 100% | Proof to be demonstrated in the course content 100% |
| - Theory of Installing of ADSS, planning, attachment hardware required, methods of installing, tensioning, safety, ADSS practical installation. | 100% | Proof to be demonstrated in the course content 100% |
| - Theory of installing OPGW, planning, attachment hardware required, method of installing, tensioning, safety, OPGW practical installation. | 100% | Proof to be demonstrated in the course content 100% |
| - Theory of installing wrapped fibre, planning, attachment hardware required, method of installing, tensioning, safety, Wrapped Fibre practical installation. | 100% | Proof to be demonstrated in the course content 100% |
| - Theory of installing duct cable, planning of manholes and ducted routes, hauling, safety during installation, safety accessing manholes post installation (gas detection, vermin etc.), Duct cable practical installation. | 100% | Proof to be demonstrated in the course content 100% |