SPECIFICATIONF FOR WIRELESS SCREEN SHARING EQUIPMENT

| Base Unit dimensions |  |
| --- | --- |
| Dimensions (HxWxD) | 39mm x 200mm x 202mm |
| Power supply | Standard 110/220 V AC plug |
| Power consumption | Operational: 50W (max) Standby |
| Weight | 900 gr |
| General |  |
| Operating system | Windows 10 or higher macOS 11 (BigSur) and higher Android v11 and higher (Share App) iOS 14 and higher (Share App) |
| System requirements | For a smooth experience with Microsoft Teams or Zoom Minimum: Intel i3 dual-core processor / 8GB RAM / OS : Windows 10 latest build or Mojave latest build Recommended: Intel i5 4-core processor / 8GB RAM / OS: Windows 10 latest build or Mac OS latest build |
| Video outputs | 4K UHD (3840*2160) @ 30Hz. HDMI 1.4b |
| Video inputs | 1920x1080 @30Hz. HDMI 1.4b |
| Audio output | USB, SPDIF, jack, HDMI |
| Connections | 1x Ethernet LAN 1Gbit 1x USB-C 2.0 (back); 2x USB-A 2.0 (back); 1x USB -A 2.0 (front) Audio analog line out on mini jack socket (3.5mm), digital S/PDIF |
| Share Buttons | 2 |
| Share App | Desktop & Mobile |
| Native protocols | Airplay, Google Cast, Miracast |
| Maximum number of simultaneous connections (with Buttons and/or App) | 32 |
| Noise Level | Max. 25dBA @ 0-30°C Max. 30dBA @ 30-40°C |
| Authentication protocol | WPA2-PSK in stand alone mode WPA2-PSK or IEEE 802.1X using the Share Button in network integration mode |
| Wireless transmission protocol | IEEE 802.11 a/g/n/ac and IEEE 802.15.1 |
| Reach | Max. 30m (100 ft) between Share Button and Share Base Unit Frequency band 2.4 GHZ and 5 GHz (DFS) |
| Frequency band | 2.4 GHZ and 5 GHz (DFS channels supported in select number of countries) |
| Temperature range | Operating: 0°C to +40°C (+32°F to +104°F) Max: 35°C (95°F) at 3000m Storage: -20°C to +60°C (-4°F to +140°F) |
| Humidity | Storage: 0 to 90% relative humidity, non-condensing Operation: 0 to 85% relative humidity, non-condensing |
| Anti-theft system | Kensington lock |
| Certifications | FCC/CE |
| Touch screen support & Interactivity | Yes |
| Wireless conferencing | via App or Button |
| Local view | High quality |
| Network connection | LAN & WiFi (dual) |
| Management and reporting | Yes |
| Warranty | 5 years coverage via SmartCare |