Removal of the Existing Doors, Supply, Deliver and Install Safety Doors (Steel) and Roller Shutter Doors at Various Substations in the Western Cape

.

PROJECT DESCRIPTION: Removal of the Existing Doors, Supply, Deliver and Install Safety Doors (Steel) and Roller Shutter Doors at Various Substations in the Western Cape

TRANSACTION:

Request for Quotations

END USER: Infrastructure

(REGION / DEPARTMENT):  Western Cape

- Introduction

The 3kV DC Traction Substations supply power for running of trains and the 11kV AC substation supply power for signalling. Both 11kV AC and 3kV DC Substations have been experiencing theft and vandalism for quite some time in the past years. The existing doors in the substations have proven to be the weakest link as they are not strong enough to prevent perpetrators from breaking into. Hence several theft and vandalism occurred.

It is crucial that the 11kV AC Signal Supply Substations and 3kV DC Traction Substations be kept as safe and secured as possible hence a need for these stell doors to be installed as they play an integral part in the foundation of our security. Failure to install the steel doors may results in more cases on theft in the substations.

- Scope of Works

The scope of work for this project shall entail the following activities:

Removal of the existing doors from the various substations and Installing of the new Steel Doors. It also involves the removal of the existing roller shutter door and installing new ones. The installation of new bricks works to accommodate the new frames where necessary.

  - Description of the works

The scope of works shall include the following activities:

Single Steel Doors

- The contractor shall remove rust on all doors shall be inclusive of all the moving parts and shall be removed by:

- Using a Steel brush to remove most of rust.

- Grinding with 150/180 grit metal sandpaper until rust is removed.

- Wipe using cloth dipped in lacquer thinner.

- Paint the door using NS4 or similar/equivalent

- Apply final coat which is an oil-based paint and must be grey in colour.

- Lubricate all moving parts and locking bolts using Q20 or similar/equivalent.

- Removal of the existing Single Doors and frames and deliver them at the Salt River Depot.

- Supply, deliver and make necessary installation of new bricks work to accommodate the new single steel doors and frames complete.

- Fully install the new single Steel Doors and frames.

Double Boors

- Removal of the existing double doors with frames and deliver them to Salt River Depot.

- Supply, deliver and make necessary installation of new bricks work to accommodate new double doors with frames complete.

- Supply, deliver and make necessary installation of new bricks work to accommodate the new single steel doors and frames complete

- Supply, deliver and fully install the double doors and frame complete.

Roller Shutter Doors

- Removal of the existing roller shutter doors with frames and wicket gated roller shutter doors with frames and deliver them to Salt River Depot.

- Supply, deliver and install the full functioning galvanised solid slat gear operated Roller Shutter Doors and Frames.

- Supply, deliver and install the full functioning galvanised solid slat Roller Shutter Doors with wicket gate and Frames.

- Provide weatherproof padlocks as per specification.

Roller Shutter Doors and Steel doors

- The contractor shall rectify any damage to all areas on completion of the works.

- The contractor is responsible for Verification and confirmation of Doors measurement during the site inspection.

- 10% contingency that will be managed and controlled by the PRASA Project Manager and approval will be required for the use of this contingency from the Prasa Regional engineering Manager.

Figure 1: Steel Door

- Specification

Below are the Dimensions for the existing doors and Roller Shutter doors to be supplied and installed.

AREA A: SOUTH

Below are the names of the Substations where Installation will be made:

Diep River 11kV and Diep River 3kV Subs

Fish Hoek 3kV and 11kV Sub

Wynberg 11kV Sub

Newlands 1kV Sub

Crawford 3kV Sub

Falsebay 33kV and Falsebay 3kV Sub

| DIEP RIVER 11kV AC Sub |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| Main Door | Single Door |  | 2.12 | 0.93 |
| Back Door | Roller Shutter Door |  | 2.5 | 1.95 |
| Transformer Room Door | Roller shutter door |  | 2.5 | 1.95 |
| Transformer Room Door | Roller shutter door |  | 2.5 | 1.95 |
| Total number of Doors |  | 1 Single Doors and 3 Roller Shutter Doors |  |  |

| DIEP RIVER 3kV DC Sub |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| Main Door | Single Door |  | 2.17 | 0.91 |
| Back Door | Single Door |  | 2.17 | 0.91 |
| Battery Room | Single Door |  | 1.73 | 0.93 |
| Basement Door | Single Door |  | 1.73 | 0.93 |
| Main Door | Roller Shutter Door |  | 2.15 | 1.55 |
| Main Door | Roller Shutter Door |  | 2.15 | 1.55 |
| Total number of Doors |  | 4 Single doors and 2 Roller Shutter Door |  |  |

| FISH HOEK Sub 3kV and 11kV Subs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| Transformer Room Door | Single Door |  | 2.07 | 0.93 |
| Main Door | Single Door |  | 2.07 | 0.93 |
| Main Door | Roller Shutter Door |  | 3.25 | 2.5 |
| Main Door | Roller Shutter Door |  | 3.25 | 2.5 |
| Total |  | 2 single Door and 2 Roller Shutter Doors |  |  |

| WYNBERG 11kV AC Sub |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| Main Door | Single Door |  | 2.075 | 0.905 |
| Total |  | 1 Single Door |  |  |

| NEWLANDS 11kV AC Sub |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| Main Door | Single Door |  | 2.075 | 0.905 |
| Total |  | 1 Single Door |  |  |

| CRAWFORD 3kV DC Sub |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length(in Metres) | Width(in Metres) |
| Back Door | Single Door |  | 2.06 | 0.895 |
| Wave filter Door | Single Door |  | 2.025 | 0.84 |
| Wave filter Door | Single Door |  | 2.02 | 0.83 |
| Battery Room Door | Single Door |  | 2.075 | 0.885 |
| Main Door | Single Door |  | 2.075 | 0.83 |
| Main Door | Roller Shutter Door |  | 3.25 | 2.5 |
| Total number of Doors |  | 5 Single Doors and 1 Roller Shutter Door |  |  |

| FALSE BAY 33kV Sub |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type(Single/Double) |  | Length(in Metres) | Width(in Metres) |
| Side Door | Single Door |  | 2.07 | 0.93 |
| Main Door | Roller Shutter Door with single wicket gate |  | 3.65 | 3.01 |
| Total |  | 1 Single Doors and 1 Roller Shutter Door with wicket gate. |  |  |

| FALSE BAY 3kV DC Sub |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type(Single/Double) |  | Length(in Metres) | Width(in Metres) |
| Main Door | Single Door |  | 2.12 | 0.93 |
| Battery Room Door | Single Door |  | 2.12 | 0.93 |
| Basement Door | Single Door |  | 2.12 | 0.93 |
| Main Door | Single Door |  | 2.12 | 0.93 |
| Transformer Door | Roller Shutter door |  | 4.8 | 4.3 |
| Transformer Door | Roller Shutter door |  | 4.8 | 4.3 |
| Total |  | 4 Single Doors and 2 Roller shutter door |  |  |

Area B: NORTH

Below are the names of the Substations where doors will be installed:

Maitland 11kV

Maitland 3kV Tie

Goodwood 3kV Sub

Cape Town 3kV Tie

Cape Town 11kV

Salt River 3kV Sub

Salt River 11kV Sub

Kensington 3kV tie

Kensington 11kV AC tie

Bellville 11kV

Modderdam 3kV

Elsies Rivier 11kV

Kasselsvlei 3kV

Sarepta 11kV Sub

Strand 3kV Sub

| MAITLAND 11kVAC |  |  |  |
| --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) | Length (in Metres) | Width (in Metres) |
| Main Door | Single Door | 2.5 | 0.99 |
| Total Number of Doors | 1 Single Doors |  |  |

| MAITLAND 3kV DC |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (S ingle/Double) |  | Length(in Metres) | Width(in Metres) |
| Main Door | Single Door |  | 2.14 | 0.99 |
| Bus Bar Chamber 1 Door | Single Door |  | 2.14 | 0.99 |
| Bus Bar Chamber 2 Door | Single Door |  | 2.14 | 0.99 |
| Main Door | Single Door |  | 2.14 | 0.99 |
| Total number of Doors |  | 4 Single Doors |  |  |

| GOODWOOD 3kV DC SUB |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| Battery Room Door | Single Door |  | 2.08 | 0.9 |
| Wave Filter Door | Single Door |  | 2.08 | 0.92 |
| Wave Filter Door | Single Door |  | 2.08 | 0.925 |
| Back Door | Single Door |  | 2.09 | 0.95 |
| Main Door | Roller Shutter Door with wicket gate |  | 3.65 | 3.01 |
| Total number of Doors |  | 4 Single Doors and 1 Roller Shutter Door with wicket gate |  |  |

| CAPE TOWN TIE 3kV DC Sub |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) |  | Width (in Metres) |
| Main Door | Single Door |  | 2.2 |  | 1.02 |
| Battery Room Door | Single Door |  | 2.195 |  | 1.02 |
| Main Door | Single Door |  | 2.19 |  | 1.02 |
| Main | Roller Shutter Door |  | 3.65 |  | 3.01 |
| Total number of Doors |  | 3 Single Doors and 1 Roller Shutter Door |  |  |  |
| CAPE TOWN 11KVAC SUB |  |  |  |  |  |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |  |
| Main Door | Roller Shutter Door |  | 2.095 | 0.92 |  |
| Main door | Roller Shutter Door with wicket gate |  | 3.4 | 1.6 |  |
| Total number of Doors |  | 1 Roller Shutter Door and 1 Roller Shutter with wicket gate Door |  |  |  |

| SALT RIVER 3kV DC Sub |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type(Single/Double) |  | Length(in Metres) | Width(in Metres) |
| Door BTN Bus Bar | Single Door |  | 2.12 | 0.935 |
| Toilet Door | Single Door |  | 2.09 | 0.935 |
| Battery Room Door | Single Door |  | 2.1 | 0.945 |
| Wave filter Room Door 1 | Single Door |  | 2.1 | 0.945 |
| Wave filter Room Door 2 | Single Door |  | 2.1 | 0.945 |
| main | Roller Shutter Door with wicket gate |  | 2.56 | 1.42 |
| Total number of Doors |  | 5 Single Doors and 1 Roller Shutter Door with wicket gate |  |  |

| SALT RIVER 11kV AC SUB |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| Main Door | Single Door |  | 2.1 | 0.92 |
| Main Door | Roller Shutter Door with wicket gate |  | 3.4 | 1.6 |
| Total number of Doors |  | 1 Single Door and 1 Roller Shutter Door with wicket gate |  |  |

| KENSINGTON 3kV DC Tie Sub |  |  |  |
| --- | --- | --- | --- |
| Door No./Name | Door Type(Single/Double) | Length(in Metres) | Width(in Metres) |
| Main Door | Single Door | 2.14 | 0.905 |
| Battery Room Door | Single Door | 2.1 | 0.91 |
| Bus Bar Chamber Do | Roller Shutter door | 2.9 | 1.6 |
| Total No. of doors | 2 Single Doors and 1 Roller shutter door |  |  |

| KENSINGTON 11kV AC Sub |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type(Single/Double) |  | Length(in Metres) | Width(in Metres) |
| Main Door | Single Door |  | 2.5 | 0.98 |
| Main Door | Roller Shutter door |  | 3.8 | 1.6 |
| Main Door | Roller Shutter door |  | 3.8 | 1.6 |
| Main Door | Roller Shutter door |  | 3.8 | 1.6 |
| Total number of Doors |  | 1 Single Door and 3 Roller Shutter door |  |  |

| BELLVILLE 11kV AC |  |  |  |
| --- | --- | --- | --- |
| Door No./Name | Door Type(Single/Double) | Length(in Metres) | Width(in Metres) |
| Main Door | Single Door | 2.08 | 0.92 |
| Main Door | Double door | 3.08 | 1.42 |
| Main Door | Double door | 3.08 | 1.42 |
| Main Door | Double door | 3.08 | 1.42 |
| Total number of Doors |  |  | 1 Single door and 3 Double Doors |

| BELLVILLE 3kV DC |  |  |  |
| --- | --- | --- | --- |
| Door No./Name | Door Type(Single/Double) | Length(in Metres) | Width(in Metres) |
| Main Door | Single Door | 2.72 | 0.91 |
| Main Door | Single Door | 2.72 | 0.91 |
| Main Door | Single Door | 2.72 | 0.91 |
| Main Door | Double Door | 3.08 | 1.42 |
| Total number of Doors |  |  | 3 Single Doors and 1 Double Door |

| MODDERDAM 3kV |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| Main Door | Single Door |  | 1.97 | 0.92 |
| Battery Door | Single Door |  | 2.04 | 0.92 |
| Wave filter room | Single Door |  | 2.04 | 0.875 |
| Wave filter room | Single Door |  | 2.04 | 0.875 |
| Main Door | Single Door |  | 1.97 | 0.92 |
| Main Door | Double Door |  | 3.08 | 1.42 |
| Main Door | Roller Shutter door with single wicket gate. |  | 3.65 | 2.45 |
| Total number of Doors |  | 5 Single Doors, 1 Double door, and 1 Roller Shutter doors with Wicked gate. |  |  |

| ELSIS RIVER 11kV |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type(Single/Double) |  | Length(in Metres) | Width(in Metres) |
| Main Door | Single Door |  | 2.4 | 0.97 |
| Main door | Double Door |  | 3.4 | 1.6 |
| Main door | Double Door |  | 3.4 | 1.6 |
| Main door | Double Door |  | 3.4 | 1.6 |
| Total number of Doors |  | 1 Single Door and 3 Double doors |  |  |

| Kasselsvlei 3kV DC Tie |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| Main Door | Single Door |  | 2.4 | 0.97 |
| Main door | Single Door |  | 2.4 | 0.97 |
| Main door | Single Door |  | 2.4 | 0.97 |
| Main door | Double Door |  | 3.4 | 1.6 |
| Total number of Doors |  | 3 Single Doors and 1 Double Door |  |  |

| Eersterivier 3kV DC |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| Main door | Single Door |  | 2.4 | 0.97 |
| Main door | Single Door |  | 2.4 | 0.97 |
| Battery room door | Single Door |  | 2.4 | 0.97 |
| Main door | Single Door |  | 2.4 | 0.97 |
| Main door | Roller shutter door with wicket gate |  | 3.65 | 3.01 |
| Total number of Doors |  | 4 Single Door and 1 Roller Shutter door with Wicket gate. |  |  |

| STRAND 3kV DC |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| Main Door | Single Door |  | 2.095 | 0.92 |
| Main Door | Single Door |  | 2.095 | 0.92 |
| Battery room | Single Door |  | 2.095 | 0.92 |
| Main Door | Roller Shutter Door with wicket gate. |  | 3.65 | 3.01 |
| Total number of Doors |  | 3 Single Doors and 1 Roller Shutter Door. |  |  |

Area C: CENTRAL

Below are the names of the Substations where doors will be installed:

Chris hani 3kV DC Tie

Lentegeur 11kV and 3kV Tie

Lavistown 11kV Sub

Kapteinsklip 11kV Sub

Langa 11kV and Langa 3kV Sub

| Chris Hani 3kV Tie |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| Main Door | Single Door |  | 2.45 | 0.96 |
| Main Door | Single Door |  | 2.45 | 0.96 |
| Main Door | Single Door |  | 2.45 | 0.96 |
| Main Door | Double Door |  | 3.4 | 1.6 |
| Total number of Doors |  | 3 Single Doors and 1 Double Door |  |  |

| LENTEGEUR 11kV and 3kV Sub |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| MAIN DOOR | Single Door |  | 2.1 | 0.81 |
| Main Door | Single Door |  | 2.00 | 0.83 |
| Main Door | Single Door |  | 2.1 | 0.81 |
| Main Door | Single Door |  | 2.00 | 0.81 |
| Main Door | Double Door |  | 3.4 | 1.6 |
| Main Door | Roller Shutter door |  | 3.6 | 1.8 |
| Main Door | Roller Shutter door |  | 3.6 | 1.8 |
| Main Door | Roller Shutter door |  | 3.6 | 1.8 |
| Total number of Doors |  | 4 Single Doors, 1 Double Door and 3 Roller Shutter Doors. |  |  |

| LANGA 11kV AC |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Door No./Name |  | Door Type(Single/Double) |  |  | Length(in Metres) |  | Width(in Metres) |
| MAIN DOOR |  | Single Door |  |  | 2.41 |  | 0.8 |
| Main Door |  | Single Door |  |  | 2.47 |  | 0.91 |
| Main doors |  | Double doors |  |  | 3.4 |  | 1.6 |
| Main doors |  | Double doors |  |  | 3.4 |  | 1.6 |
| Main doors |  | Double doors |  |  | 3.4 |  | 1.6 |
| Total number of Doors |  |  | 2 Single doors and 3 Double doors |  |  |  |  |
| LANGA 3kV |  |  |  |  |  |  |  |
| Door No./Name | Door Type(Single/Double) |  |  | Length(in Metres) |  | Width(in Metres) |  |
| Main Door | Single Door |  |  | 2.04 |  | 0.9 |  |
| Back door | Single Door |  |  | 2.04 |  | 0.9 |  |
| Wave Filter Door | Single Door |  |  | 2.04 |  | 0.9 |  |
| Wave filter Door | Single Door |  |  | 2.04 |  | 0.9 |  |
| Main Door | Double Door |  |  | 2.64 |  | 2.5 |  |
| Main Door | Roller Shutter Door with Single Wicket Gate |  |  | 3.05 |  | 2.4 |  |
| Total No. of Doors | 4 Single Doors,1 Double door and 1 Roller shutter door with wicked gate. |  |  |  |  |  |  |

Area D: Cape Flats

Below are the names of the Substations where doors will be installed:

Hazendal

Rappenburg

| HAZENDAL 11kV Sub |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| Main Door | Single Door |  | 2.45 | 0.96 |
| Main Door | Double Door |  | 3.4 | 1.6 |
| Main Door | Double Door |  | 3.4 | 1.6 |
| Main Door | Double Door |  | 3.4 | 1.6 |
| Total number of Doors |  | 1 single Door and 3 Roller shutter doors |  |  |

| Rappenburg 3kV DC Tie |  |  |  |  |
| --- | --- | --- | --- | --- |
| Door No./Name | Door Type (Single/Double) |  | Length (in Metres) | Width (in Metres) |
| Main Door | Single Door |  | 2.15 | 0.985 |
| Battery Room Door | Single Door |  | 2.19 | 0.985 |
| Bus bar Chamber Door | Single Door |  | 2.2 | 0.985 |
| Main Door | Roller Shutter Door |  | 3.4 | 1.6 |
| Total number of Doors |  | 3 Single Doors and 1 Roller Shutter Door |  |  |

Below is a Specification for Weatherproof Padlock for Roller shutter Doors and Roller Shutter doors with Wicket Gate.

- 51mm weatherproof padlock

- Solid and robust build

- Hardened steel shackle with protective rubber cover

- Suitable for high security applications and exposed conditions

- Double ball bearing locking for added security

- Keway cover cap to prevent rust

- 5 pin cylinders

- Security rating: 6

Below is a table showing overall number of doors per Area.

| Areas | Single Doors | Double Doors | Roller Shutter Doors | Roller shutter Doors with wicket gate |
| --- | --- | --- | --- | --- |
| Area South | 10 | 0 | 10 | 1 |
| Area North | 41 | 9 | 8 | 6 |
| Area Central | 13 | 6 | 3 | 1 |
| Area Cape flats | 4 | 0 | 4 | 0 |
|  | 68 | 15 | 25 | 8 |

# Table 1: Number of Doors per Corridor

Below is a list of Specifications

| No | Specification |  |
| --- | --- | --- |
|  | Description | Specification, Drawing No |
| 1 | Safety arrangements and procedural compliance With the occupational health and safety act (Act 85 of 1993) and applicable regulations. | E.4E Transnet (Jan 2004) |
| 2 | Specification for works on, over, under or adjacent to railway lines and near high voltage equipment | E71 |
| 3 | Single Steel Doors Specification | Single Steel Doors Specification |
| 4 | Double Steel door Spec | Double Steel door Spec |
| 5 | Double Steel Drawing | Double Steel Drawing |
| 6 | Roller Shutter Door Spec | Roller Shutter Door Spec |

# Table 2: Specifications

# 4. CIDB Grading

The Projects requires a CIDB Grading 3GB / 3EB or higher

5. General Specifications

The Contractor shall implement measures required in terms of Covid-19 regulations. No separate payment will be made for such measures, as the contractor shall allow for this in his/her rates tendered for the Works.

6. Mandatory Requirements

6.1.    Mandatory Requirements

If you do not submit/meet the following mandatory documents/requirements, your bid will be automatically disqualified:

| No. | Description of requirement |  |
| --- | --- | --- |
| a) | Completion of ALL RFP documentation (includes ALL) | x |
| b) | Briefing Session Form D. Bidders must also reflect on the Compulsory Briefing Session Attendance Register (Delete if not applicable) | x |
| c) | Signed Joint Venture, Consortium Agreement or Partnering Agreement (whichever is applicable) |  |
| d) | Proof of CIDB grading. | 3GB / 3EB or higher |

# Table 3: Mandatory Requirements

  - Stage 1B - Other Mandatory Requirements

If you do not submit/meet the following mandatory documents/requirements, PRASA may request the bidder to submit the information within five (5) working days. Should this information not be provided, your bid proposal will be disqualified.

| No. | Description of requirement |  |
| --- | --- | --- |
| a) | Letter of Good Standing: COID | x |
| b) | Valid SARS Pin | x |
| c) | Company Registration Documents | x |
| d) | Copies of Directors ID’s Documents | x |
| e) | CSD supplier registration number | x |
| f) | Professional Qualification | x |
|  | Technician: Building and Construction (National Diploma). Artisan: Building and Construction: N3 or Higher OHS Representative as Safety officer: Health and Safety Certificate or SAMTRAC. | x |
|  | NB: Provide copies of original qualification and certificate. The copies must be certified by commissioner of oath. The date on the stamp shall be three months or less old, before the closing date of the tender. If the qualification has been awarded in other language either than English, please provide translation in English |  |

# Table 4: Other Mandatory Requirements

# 7. Time Frame

The planned time frame for the implementation of this proposed project is 12 months.

8. Evaluation Process

The minimum threshold for the technical/functionality requirements is 70%.  Bidders who score below the minimum requirement shall not be considered for further evaluation.

| ITEM | CRITERIA | WEIGHT |
| --- | --- | --- |
| 1 | Key Staff – Experience | 50 |
| 2 | Previous Company / Organisational Experience: | 50 |
| 3 | TOTAL | 100 |

Summary of the technical/functional requirements are presented in the table below.

Details of the scoring methodology presented above are outlined below:

0 - Not submitted / Irrelevant, 10 - Poor; 20 - Average; 30- Good; 40-Very Good, 50- Excellent.

| CRITERIA | WEIGHT | SCORES |
| --- | --- | --- |
| - Experience of Key Personnel (based on Submitted CVs) Provide CVs of the key personnel staff detailing experience and knowledge related to Construction and Building. Score of key personnel staff will be allocated according to the number of years on Construction and Building. | 5 years’ experience or more. (Equates to a score of 50). Technician in Building and Construction; 35. Artisan in Building and Construction.; 15. 4 years’ experience or more but less than 5 (Equates to a score of 40). Technician in Building and Construction; 30. Artisan in Building and Construction.; 10. 3 years’ experience or more but less than 4 (Equates to a score of 30). Technician in Building and Construction; 23. Artisan in Building and Construction.; 7. 2 years’ experience or more but less than 3 (Equates to a score of 20). Technician in Building and Construction; 16. Artisan in Building and Construction.; 4. 1 year experience and less (Equates to a score of 10). Technician in Building and Construction; 7. Artisan in Building and Construction.; 3. No experience or No CVs submitted. (Equates to a score of 0). Technician in Building and Construction; 0. Artisan in Building and Construction.; 0. | 50 |
| - Organizational Experience The contractor/service provider must provide reference letters of previous experience in the form of completed project related to Construction and Building. The evidence should come in the form of five (5) completed reference letters, where 1 reference letter equates to 1 project completed. The reference letter must be sent to the company/business where services were previously rendered for. The reference letter must be sent to the company/business where services were previously rendered for. Should the bidder be in possession of an existing letter, the letter shall cover all aspects outlined below: - Previous client and completed works/services information - Name of the organisation - Description of works/goods/services that were provided - Year - Duration - Evaluation of Service Provider’s Performance - Project completed within the stipulated timeframes and adhering to lead times. - Quality. | 5 Reference Letters; 50. 4 Reference Letters; 40. 3 Reference Letters; 30. 2 Reference Letters; 20. 1 Reference Letter; 10. No Project or Reference Letter; 0. | 50 |

| QUALITY CRITERIA | INDICATORS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Not submitted / Irrelevant= 0 Points |  | Poor= 10 Points |  | Average 20 Points | Good 30 Points | Very good 40 Points | Excellent = 50 points |
| Experience of Key Personnel (based on submitted CVs) Provide CVs of the key personnel staff detailing experience and knowledge related to Construction and Building. Score of key personnel staff will be allocated according to the number of years on Construction and Building. |  | No experience or No CVs submitted. (Equates to a score of 0). |  | All key staff CV’s was submitted. Technician (B&C) Artisan (B&C) 1 year experience and less (Equates to a score of 10). Range (1-10) | All key staff CV’s was submitted. Technician (B&C) Artisan (B&C) 2 years’ experience or more but less than 3 (Equates to a score of 20). Range (11-20) | All key staff CV’s was submitted. Technician (B&C) Artisan (B&C) 3 years’ experience or more but less than 4 (Equates to a score of 30). Range (21-30) | All key staff CV’s was submitted. Technician (B&C) Artisan (B&C) 4 years’ experience or more but less than 5 (Equates to a score of 40). Range (31-40) | All key staff CV’s was submitted. Technician (B&C) Artisan (B&C) 5 years’ experience or more. (Equates to a score of 50). Range (41-50) |

| QUALITY CRITERIA |  |  | INDICATORS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Not submitted / Irrelevant= 0 Points | Poor= 10 Points | Average20 Points | Good 30 Points | Very good 40 Points | Excellent = 50 points |
| Organizational Experience The contractor/service provider must provide reference letters of previous experience in the form of completed project related to Construction and Building. The evidence should come in the form of five (5) completed reference letters, where 1 reference letter equates to 1 project completed. The reference letters must address the below items. Attached is a reference letter template that can be used. The reference letter must be sent to the company/business where services were previously rendered for. Should the bidder be in possession of an existing letter, the letter shall cover all aspects outlined below: - Previous client and completed works/services information i. Name of the organisation ii. Description of works/goods/services that were provided iii. Year iv. Duration - Evaluation of Service Provider’s Performance - Project completed within the stipulated timeframes and adhering to lead times. - Quality. | No project completed with zero relevant reference letter equates to a score of 0 | One project completed with one relevant reference letter equates to a score of 10 Note: The reference letter must be signed and stamped by the company/business where the services were previously rendered and comply with the two listed criteria. If the reference letter reflects a negative report attributable to the Supplier, that reference letter will not be considered. If the reference letter does not feature all criteria, then the tenderer will be given the lowest score. | Two project successfully completed with two relevant reference letter equates to a score of 20. Note: The reference letter must be signed and stamped by the company/business where the services were previously rendered and comply with the two listed criteria. If the reference letter reflects a negative report attributable to the Supplier, that reference letter will not be considered. If the reference letters do not feature all criteria, then the tenderer will be given the lower score. | Three projects successfully completed with three relevant reference letters equates to a score of 30. Note: The reference letter must be signed and stamped by the company/business where the services were previously rendered and comply with the two listed criteria If the reference letter reflects a negative report attributable to the Supplier, that reference letter will not be considered. If the reference letters do not feature all criteria, then the tenderer will be given the lower score | Four projects successfully completed with four relevant reference letters equates to a score of 40. Note: The reference letter must be signed and stamped by the company/business where the services were previously rendered and comply with the two listed criteria If the reference letter reflects a negative report attributable to the Supplier, that reference letter will not be considered. If the reference letters do not feature all criteria, then the tenderer will be given the lower score. | Five projects successfully completed with five relevant reference letters equates to a score of 50. Note: The reference letter must be signed and stamped by the company/business where the services were previously rendered and comply with the two listed criteria If the reference letter reflects a negative report attributable to the Supplier, that reference letter will not be considered. If the reference letters do not feature all criteria, then the tenderer will be given the lower score. |

METRORAIL WESTERN CAPE

CONTACTABLE REFERENCE FOR TENDERER

TENDER/PROJECT: Removal of the Existing Doors, Supply, Deliver and Install Safety Doors (Steel) and Roller Shutter Doors at Various Substations in the Western Cape

Name of the tenderer: _______________________________________

To:	PRASA – Metrorail – Western Cape

I, the undersigned duly authorized representative, hereby furnish this reference on behalf of the organisation named herein:

- Previous client and completed works/services information

- Name of the organisation 					:______________________________

- Description of works/goods/services that were provided		: ______________________________

______________________________________________________________________________

- Year	 							:________________________________

- Duration							:________________________________

- Value of works/services/goods				: _______________________________

- Evaluation of service provider’s performance

- Were the works/service/ product completed/delivered within stipulated timeframes Yes              No

If no kindly comment __________________________________________________________________

_________________________________________________________________________________

- Were the works/services/product meet the expected and specified quality               Yes              No

Business stamp with date

If no kindly comment _________________________________________________________________________________Signature:				_______________________

Telephone:				_______________________

Date:					______________

9. Bill of Quantity

| Part C2.2 Removal of the Existing Doors, Supply, Deliver and Install Safety Doors (Steel) and Roller Shutter Doors at Various Substations in the Western Cape |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DESCRIPTION | UNIT | QUANTITY | MATERIALS | LABOUR | TOTAL |
|  | Preliminary and General |  |  |  |  |  |
|  | Time Related Items |  |  |  |  |  |
|  | Scaffolding for installation of the doors or roller shutter doors in a double store building. | day | 20 | R | - | R |
| AREA SOUTH | DESCRIPTION | UNIT | QUANTITY | MATERIALS | LABOUR | TOTAL |
|  | Provide weatherproof padlocks as per specification for Roller Shutter Doors and Roller Shutter Doors with wicket gate. | each | 35 | R | - | R |
|  | Diep River 11kV AC |  |  |  |  |  |
| A1 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 1 | R | R | R |
| A2 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 1 | - | R | R |
| A3 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 1 | R | R | R |
| A4 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 1 | R | R | R |
| A5 | Removal of the existing Single Doors with frames and existing roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 4 | - | R | R |
| A6 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 1 | R | R | R |
| A7 | Fully Installation of the new Steel Door with frame. | sum | 1 | - | R | R |
| A8 | Supply, deliver and install the full functioning gear operated Roller Shutter door with frame complete. | sum | 3 | R | R | R |
|  |  |  |  |  |  | R |
|  | Diep River 3kV Sub |  |  |  |  |  |
| A9 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 4 | R | R | R |
| A10 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 4 | - | R | R |
| A11 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 4 | R | R | R |
| A12 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 4 | R | R | R |
| A13 | Removal of the existing Single Doors with frames and existing roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 6 | - | R | R |
| A14 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 4 | R | R | R |
| A15 | Fully Installation of the new Steel Door with frame. | sum | 4 | - | R | R |
| A16 | Supply, deliver and install the full functioning gear operated Roller Shutter door with frame complete. | sum | 2 | R | R | R |
|  |  |  |  |  |  | R |
|  | Fish Hoek 3kV and 11kV sub |  |  |  |  |  |
| A17 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 2 | R | R | R |
| A18 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 2 | - | R | R |
| A19 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 2 | R | R | R |
| A20 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 2 | R | R | R |
| A21 | Removal of the existing Single Doors with frames and existing roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 4 | - | R | R |
| A22 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 2 | R | R | R |
| A23 | Fully Installation of the new Steel Door with frame. | sum | 2 | - | R | R |
| A24 | Supply, deliver and install the full functioning gear operated Roller Shutter door with frame complete. | sum | 2 | R | R | R |
|  |  |  |  |  |  | R |
|  | Wynberg 11kV Sub |  |  |  |  |  |
| A25 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 1 | R | R | R |
| A26 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 1 | - | R | R |
| A27 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 1 | R | R | R |
| A28 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 1 | R | R | R |
| A29 | Removal of the existing Single Doors with frames and deliver them at Prasa Depot in Salt River. | sum | 1 | - | R | R |
| A30 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 1 | R | R | R |
| A31 | Fully Installation of the new Steel Door with frame. | sum | 1 | - | R | R |
|  |  |  |  |  |  | R |
|  | Newlands11kV Sub |  |  |  |  |  |
| A32 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 1 | R | R | R |
| A33 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 1 | - | R | R |
| A34 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 1 | R | R | R |
| A35 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 1 | R | R | R |
| A36 | Removal of the existing Single Doors with frame and deliver them at Prasa Depot in Salt River. | sum | 1 |  | R | R |
| A37 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 1 | R | R | R |
| A38 | Fully Installation of the new Steel Door with frame. | sum | 1 | - | R | R |
|  |  |  |  |  |  | R |
|  | Crawford 3kV Sub |  |  |  |  |  |
| A39 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 5 | R | R | R |
| A40 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 5 | - | R | R |
| A41 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 5 | R | R | R |
| A42 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 5 | R | R | R |
| A43 | Removal of the existing Single Doors with frames and existing roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 6 | - | R | R |
| A44 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 5 | R | R | R |
| A45 | Fully Installation of the new Steel Door with frame. | sum | 5 | - | R | R |
| A46 | Supply, deliver and Install the full functioning gear operated Roller Shutter door with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
|  | Falsebay 33kV Sub |  |  |  |  |  |
| A47 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 1 | R | R | R |
| A48 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 1 | - | R | R |
| A49 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 1 | R | R | R |
| A50 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 1 | R | R | R |
| A51 | Removal of the existing Single Doors with frames and existing wicket gate roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 2 | - | R | R |
| A52 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 1 | R | R | R |
| A53 | Fully Installation of the new Steel Door with frame. | sum | 1 | - | R | R |
| A54 | Supply, deliver and install the full functioning gear operated Roller Shutter door with wicket gate and with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
|  | Falsebay 3kV Sub |  |  |  |  |  |
| A55 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 4 | R | R | R |
| A56 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 4 | - | R | R |
| A57 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 4 | R | R | R |
| A58 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 4 | R | R | R |
| A59 | Removal of the existing Single Doors with frames and existing roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 6 | - | R 1 | R |
| A60 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 4 | R | R | R |
| A61 | Fully Installation of the new Steel Door with frame. | sum | 4 | - | R | R |
| A62 | Supply, deliver and install the full functioning gear operated Roller Shutter door with frame complete. | sum | 2 | R | R | R |
|  |  |  |  |  |  | R |
| AREA NORTH | DESCRIPTION | UNIT | QUANTITY | MATERIALS | LABOUR | TOTAL |
|  | Maitland 11kV |  |  |  |  |  |
| B1 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 1 | R | R | R |
| B2 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 1 | - | R | R |
| B3 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 1 | R | R | R |
| B4 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 1 | R | R | R |
| B5 | Removal of the existing Single Doors with frames and deliver them at Prasa Depot in Salt River. | sum | 1 | - | R | R |
| B6 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 1 | R | R | R |
| B7 | Fully Installation of the new Steel Door with frame. | sum | 1 | - | R | R |
|  |  |  |  |  |  | R |
|  | Maitland 3kV Tie |  |  |  |  |  |
| B8 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 4 | R | R | R |
| B9 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 4 | - | R | R |
| B10 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 4 | R | R | R |
| B11 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 4 | R | R | R |
| B12 | Removal of the existing Single Doors with frames and roller shutter door with frames and deliver them at Prasa Depot in Salt River. | sum | 4 |  | R | R |
| B13 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 4 | R | R | R |
| B14 | Fully Installation of the new Steel Door with frame. | sum | 4 |  | R | R |
|  |  |  |  |  |  | R |
|  | Goodwood 3kV Sub |  |  |  |  |  |
| B15 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 4 | R | R | R |
| B16 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 4 | - | R | R |
| B17 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 4 | R | R | R |
| B18 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 4 | R | R | R |
| B19 | Removal of the existing Single Doors with frames and existing wicket gate roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 5 | - | R | R |
| B20 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 4 | R | R | R |
| B21 | Fully Installation of the new Steel Door with frame. | sum | 4 |  | R | R |
| B22 | Supply, deliver and install the full functioning gear operated wicket gate Roller Shutter door with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
|  | Cape town 3kV Tie |  |  |  |  |  |
| B23 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 3 | R | R | R |
| B24 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 3 | - | R | R |
| B25 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 3 | R | R | R |
| B26 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 3 | R | R | R |
| B27 | Removal of the existing Single Doors with frames and existing roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 4 | - | R | R |
| B28 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 3 | R | R | R |
| B29 | Fully Installation of the new Steel Door with frame. | sum | 3 | - | R | R |
| B30 | Supply, deliver and install the full functioning gear operated Roller Shutter door with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
|  | Cape Town 11kV Sub |  |  |  |  |  |
| B31 | Removal of the double door with frame and wicket gate roller shutter door with frame and deliver them at Salt River Depot. | sum | 2 | - | R | R |
| B32 | Supply, deliver and install the full functioning gear operated Roller Shutter door with frame complete. | sum | 1 | R | R | R |
| B33 | Supply, deliver and install the full functioning Roller Shutter with wicket gate and frame complete. . | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
|  | Salt River 3kV Sub |  |  |  |  |  |
| B34 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 5 | R | R | R |
| B35 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 5 | - | R | R |
| B36 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 5 | R | R | R |
| B37 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 5 | R | R | R |
| B38 | Removal of the existing Single Doors with frames and existing wicket gate roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 6 | - | R | R |
| B39 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 5 | R | R | R |
| B40 | Fully Installation of the new Steel Door with frame. | sum | 5 | - | R | R |
| B41 | Supply, deliver and install the full functioning gear operated wicket gate Roller Shutter door with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
|  | Salt River 11kV AC Sub |  |  |  |  |  |
| B42 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 1 | R | R | R |
| B43 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 1 | - | R | R |
| B44 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 1 | R | R | R |
| B45 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 1 | R | R | R |
| B46 | Removal of the existing Single Doors with frames and existing roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 2 | - | R | R |
| B47 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 1 | R | R | R |
| B48 | Fully Installation of the new Steel Door with frame. | sum | 1 | - | R | R |
| B49 | Supply, deliver and install the full functioning gear operated Roller Shutter door with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
|  | Kensington 3kV Tie |  |  |  |  |  |
| B50 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 2 | R | R | R |
| B51 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 2 | - | R | R |
| B52 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 2 | R | R | R |
| B53 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 2 | R | R | R |
| B54 | Removal of the existing Single Doors with frames and existing roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 3 | - | R | R |
| B55 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 2 | R | R | R |
| B56 | Fully Installation of the new Steel Door with frame. | sum | 2 | - | R | R |
| B57 | Supply, deliver and install the full functioning gear operated Roller Shutter door with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
|  | Kensington 11kV AC |  |  |  |  |  |
| B58 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 1 | R | R | R |
| B59 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 1 | - | R | R |
| B60 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 1 | R | R | R |
| B61 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 1 | R | R | R |
| B62 | Removal of the existing Single Doors with frames and roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 4 | - | R | R |
| B63 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 1 | R | R | R |
| B64 | Fully Installation of the new Steel Door with frame. | sum | 1 | - | R | R |
| B65 | Supply, deliver and install the full functioning gear operated Roller Shutter door with frame complete. | sum | 3 | R | R | R |
|  |  |  |  |  |  | R |
|  | Bellville 11kV AC |  |  |  |  |  |
| B66 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 1 | R | R | R |
| B67 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 1 | - | R | R |
| B68 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 1 | R | R | R |
| B69 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 1 | R | R | R |
| B70 | Removal of the existing Single Doors with frames and double doors with frames and deliver them at Prasa Depot in Salt River. | sum | 4 | - | R | R |
| B71 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 1 | R | R | R |
| B72 | Fully Installation of the new Steel Door with frame. | sum | 1 |  | R | R |
| B73 | Supply, deliver and make necessary renovations of new brick work to accommodate new double steel doors and frames complete. | sum | 3 | R | R | R |
| B74 | Supply, deliver and install the full functioning double door with frame complete. | sum | 3 | R | R | R |
|  |  |  |  |  |  | R |
|  | Bellville 3kV Sub |  |  |  |  |  |
| B76 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 3 | R | R | R |
| B75 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 3 | - | R | R |
| B77 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 3 | R | R | R |
| B78 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 3 | R | R | R |
| B79 | Removal of the existing Single Doors with frames and existing double doors with frames and deliver them at Prasa Depot in Salt River. | sum | 4 | - | R | R |
| B80 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 3 | R | R | R |
| B81 | Fully Installation of the new Steel Door with frame. | sum | 3 | - | R | R |
| B82 | Supply, deliver and make necessary renovations of new brick work to accommodate new double steel doors and frames complete. | sum | 1 | R | R | R |
| B83 | Supply, deliver and install the full functioning double door with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
|  | Modderdam 3kV DC |  |  |  |  |  |
| B84 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 5 | R | R | R |
| B85 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 5 | - | R | R |
| B86 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 5 | R | R | R |
| B87 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 5 | R | R | R |
| B88 | Removal of the existing Single Doors with frames, existing wicket gate roller shutter doors with frames, existing double door with frame and deliver them at Prasa Depot in Salt River. | sum | 7 | - | R | R |
| B89 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 5 | R | R | R |
| B90 | Fully Installation of the new Steel Door with frame. | sum | 5 | - | R | R |
| B91 | Supply, deliver and make necessary renovations of new brick work to accommodate new double steel doors and frames complete. | sum | 1 | R | R | R |
| B92 | Supply, deliver and install the full functioning double door with frame complete. |  | 1 | R | R | R |
| B93 | Supply, deliver and Install the full functioning gear operated wicket gate Roller Shutter door with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
|  | Elsies Rivier 11kV |  |  |  |  |  |
| B94 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 1 | R | R | R |
| B95 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 1 | - | R | R |
| B96 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 1 | R | R | R |
| B97 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 1 | R | R | R |
| B98 | Removal of the existing Single Doors with frames and double doors with frames and deliver them at Prasa Depot in Salt River. | sum | 4 | - | R | R |
| B99 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 1 | R | R | R |
| B100 | Fully Installation of the new Steel Door with frame. | sum | 1 | - | R | R |
| B101 | Supply, deliver and make necessary renovations of new brick work to accommodate new double steel doors and frames complete. | sum | 3 | R | R | R |
| B102 | Supply, deliver and install the full functioning double door with frame complete. | sum | 3 | R | R | R |
|  |  |  |  |  |  | R |
|  | Kasselsvlei 3kV Tie |  |  |  |  |  |
| B103 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 4 | R | R | R |
| B104 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 4 | - | R | R |
| B105 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 4 | R | R | R |
| B106 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 4 | R | R | R |
| B107 | Removal of the existing Single Doors with frames and double doors with frames and deliver them at Prasa Depot in Salt River. | sum | 5 | - | R | R |
| B108 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 4 | R | R | R |
| B109 | Fully Installation of the new Steel Door with frame. | sum | 4 | - | R | R |
| B110 | Supply, deliver and make necessary renovations of new brick work to accommodate new double steel doors and frames complete. | sum | 1 | R | R | R |
| B111 | Supply, deliver and install the full functioning double door with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
|  | Eersterivier 3kV DC |  |  |  |  |  |
| B112 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 4 | R | R | R |
| B113 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 4 | - | R | R |
| B114 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 4 | R | R | R |
| B115 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 4 | R | R | R |
| B116 | Removal of the existing Single Doors with frames and wicket gate existing roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 5 | - | R | R |
| B117 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 4 | R | R | R |
| B118 | Fully Installation of the new Steel Door with frame. | sum | 4 | - | R | R |
| B119 | Supply, deliver and install the full functioning gear operated wicket gate Roller Shutter door with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
|  | Strand 3kV Sub |  |  |  |  |  |
| B120 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 3 | R | R | R |
| B121 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 3 | - | R | R |
| B122 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 3 | R | R | R |
| B123 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 3 | R | R | R |
| B124 | Removal of the existing Single Doors with frames and existing wicket gate roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 4 | - | R | R |
| B125 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 3 | R | R | R |
| B126 | Fully Installation of the new Steel Door with frame. | sum | 3 | - | R | R |
| B127 | Supply, deliver and install the full functioning gear operated wicket gate Roller Shutter door with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
| AREA CENTRAL | DESCRIPTION | UNIT | QUANTITY | MATERIALS | LABOUR | TOTAL |
|  | Chrishani 3kV Tie |  |  |  |  |  |
| C1 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 3 | R | R | R |
| C2 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 3 | - | R | R |
| C3 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 3 | R | R | R |
| C4 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 3 | R | R | R |
| C5 | Removal of the existing Single Doors with frames and double doors with frames and deliver them at Prasa Depot in Salt River. | sum | 4 | - | R | R |
| C6 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 3 | R | R | R |
| C7 | Fully Installation of the new Steel Door with frame. | sum | 3 | - | R | R |
| C8 | Supply, deliver and make necessary renovations of new brick work to accommodate new double steel doors and frames complete. | sum | 1 | R | R | R |
| C9 | Supply, deliver and install the full functioning double door with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
|  | Lentegeur 11kV and 3kV Sub |  |  |  |  |  |
| C10 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 4 | R | R | R |
| C11 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 4 | - | R | R |
| C12 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 4 | R | R | R |
| C13 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 4 | R | R | R |
| C14 | Removal of the existing Single Doors with frames, existing double door with frame and existing roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 8 | - | R | R |
| C15 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 4 | R | R | R |
| C16 | Fully Installation of the new Steel Door with frame. | sum | 4 | - | R | R |
| C17 | Supply, deliver and make necessary renovations of new brick work to accommodate new double steel doors and frames complete. | sum | 1 | R | R | R |
| C18 | Supply, deliver and install the full functioning double door with frame complete. |  | 1 | R | R | R |
| C19 | Supply, deliver and install the full functioning gear operated Roller Shutter door with frame complete. | sum | 3 | R | R | R |
|  |  |  |  |  |  | R |
|  | Langa 11kV Sub |  |  |  |  |  |
| C20 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 2 | R | R | R |
| C20 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 2 | - | R | R |
| C21 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 2 | R | R | R |
| C22 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 2 | R | R | R |
| C23 | Removal of the existing Single Doors with frames and double doors with frames and deliver them at Prasa Depot in Salt River. | sum | 5 | - | R | R |
| C24 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 2 | R | R | R |
| C25 | Fully Installation of the new Steel Door with frame. | sum | 2 | - | R | R |
| C26 | Supply, deliver and make necessary renovations of new brick work to accommodate new double steel doors and frames complete. | sum | 3 | R | R | R |
| C27 | Supply, deliver and install the full functioning double door with frame complete. | sum | 3 | R | R | R |
|  |  |  |  |  |  | R |
|  | Langa 3kV Sub |  |  |  |  |  |
| C28 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 4 | R | R | R |
| C29 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 4 | - | R | R |
| C30 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 4 | R | R | R |
| C31 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 4 | R | R | R |
| C32 | Removal of the existing Single Doors with frames, existing double door and existing wicket gate roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 6 | - | R | R |
| C33 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 4 | R | R | R |
| C34 | Fully Installation of the new Steel Door with frame. | sum | 4 | - | R | R |
| C35 | Supply, deliver and make necessary renovations of new brick work to accommodate new double steel doors and frames complete. | sum | 3 | R | R | R |
| C36 | Supply, deliver and install the full functioning double door with frame complete. | sum | 1 | R | R | R |
| C37 | Supply, deliver and Install the full functioning gear operated wicket gate Roller Shutter door with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
| AREA CAPE FLATS | DESCRIPTION | UNIT | QUANTITY | MATERIALS | LABOUR | TOTAL |
|  | Hazendal 11kV Sub |  |  |  |  |  |
| D1 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 1 | R | R | R |
| D2 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 1 | - | R | R |
| D3 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 1 | R | R | R |
| D4 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 1 | R | R | R |
| D5 | Removal of the existing Single Doors with frames and existing roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 4 | - | R | R |
| D6 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 1 | R | R | R |
| D7 | Fully Installation of the new Steel Door with frame. | sum | 1 | - | R | R |
| D8 | Supply, deliver and install the full functioning roller shutter doors with frame complete. | sum | 3 | R | R | R |
|  |  |  |  |  |  | R |
|  | Rappenburg 3kV DC Tie |  |  |  |  |  |
| D9 | Remove rust on steel door and moving parts by using steel brush, then grind with 150/180 grit metal sandpaper until rust is removed. Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 3 | R | R | R |
| D10 | Transportation of the new Steel Doors from Salt River Depot to Diep River Sub. | sum | 3 | - | R | R |
| D11 | Grind Rust and/or sandblast the single steel door using 150/80 grit metal sandpaper until rust is removed, Wipe the doors using cloth dipped in lacquer thinner or similar/equivalent. Provide material. | sum | 3 | R | R | R |
| D11 | Apply NS4 or similar/equivalent primer, allow it to dry. Then Apply final coat which is an Oil based paint. Provide Material. | sum | 3 | R | R | R |
| D12 | Lubricate all moving parts and locking bolts using Q20 or similar/equivalent. | sum | 3 | R | R | R |
| D13 | Removal of the existing Single Doors with frames and existing roller shutter doors with frames and deliver them at Prasa Depot in Salt River. | sum | 4 | - | R | R |
| D14 | Supply, deliver and make necessary renovations of new brick work to accommodate new single steel doors and frames complete. | sum | 3 | R | R | R |
| D15 | Fully Installation of the new Steel Door with frame. | sum | 3 | - | R | R |
| D16 | Supply, deliver and install the full functioning gear operated Roller Shutter door with frame complete. | sum | 1 | R | R | R |
|  |  |  |  |  |  | R |
| WORK SUB-TOTAL (EXCLUDING VAT) |  |  |  |  |  | R |
| 15% contingency that will be managed and controlled by the PRASA Project Manager and approval will be required for the use of this contingency from the Prasa Regional engineering Manager |  |  |  |  |  | R |
| SUB-TOTAL (INCLUDING CONTINGENCY, EXCLUDING VAT) |  |  |  |  |  | R |
| VAT@15% |  |  |  |  |  | R |
| TOTAL (INCLUDING VAT AND CONTINGENCY) |  |  |  |  |  | R |

10. Specific Goals

Table 1: Specific goals for the tender and points claimed are indicated per the table below. (Note to organs of state: Where the 80/20 preference point system is applicable, corresponding points must also be indicated as such.

Note to tenderers: The tenderer must indicate how they claim points for each preference point system.

| The specific goals allocated points in terms of this tender | Number of points allocated (80/20 system) (To be completed by the organ of state) | Number of points claimed (80/20 system) (To be completed by the tenderer) | Evidence |
| --- | --- | --- | --- |
| B-BBEE Contributor status of at least level 2 | 4 |  | B-BBEE Certificate/Affidavit (in case of JV, a consolidated score card will be accepted) |
| Black youth owned | 4 |  | Certified copy of ID Documents of the Owners |
| Black women owned | 4 |  | Certified copy of ID Documents of the Owners |
| Minimum 51% Black owned | 4 |  | B-BBEE Certificate/Affidavit |
| EME or QSE 51% Black Owned | 4 |  | Audited Annual Financial/ B-BBEE Certificate / Affidavit. |
| Total | 20 |  |  |

11. Recommendation

It is recommended that the above specification be approved Removal of the Existing Doors, Supply, Deliver and Install Safety Doors (Steel) and Roller Shutter Doors at Various Substations in the Western Cape

| Project Description: | (Insert project name) |
| --- | --- |
| Compiled by End User Name & Surname: Zola Mdlankomo Signature: ……….………………………. Date: ………………………………………….. |  |
| Approved/Not Approved (Head of the End-User Department) Name & Surname: Signature: …………………………………. Date: ………………………………………... |  |
| Comments/Remarks: |  |