REQUEST FOR QUOTATION FORM FOR GOODS

AND SERVICES

AGRICULTURAL RESEARCH COUNCIL (ARC)

1.	You are kindly requested to submit a written quotation ‪‪‪RFQ-013486 to Agricultural

Research Council as per below or attached specification.

LABORATORY EQUIPMENT SPECIFICATIONS; Comply to ARC specs; Comment. 1. 1 x CONCRETE TESING MACHINE – AUTOMATIC. Capacity; 2000 kN; Yes / No. Automatic test cycle with standard rapid approach; Yes / No. Graphical – LCD data acquisition and control system; Yes / No. Menu driven software for easy operation; Yes / No. Load vs time plot and instantaneous load rate displayed; Yes / No. Accurate load rate control within ±5 from 1 kN/sec to 20 kN /sec Class 1 starting from % 10 of the full range; Yes / No. Ready stress calculation for the standards of EN 12390-3, EN 12390-5, EN 12390-6, EN 1338, EN 1340, EN 196, SI, Metric units; Yes / No. Data storage up to 256 tests; Yes / No. RS 232 output for printer or PC (OR ANY INCLUDING USB AND HDMI); Yes / No. PC software for test control and advanced report printout. Program must be loaded to laptop. Complete with all consumables & software, (Batteries, Cables, and Ultrasonic Couplant etc.) to make this a fully working instrument unit on delivery.; Yes / No. 2. 1 x FLEXURE TESTING MACHINE – DIGITAL. Automatic test cycle with standard Rapid Approach; Yes / No. Graphical – LCD Data Acquisition and Control System; Yes / No. Menu Driven Software for easy operation; Yes / No. Load vs time plot and instantaneous load rate displayed; Yes / No. Accurate load rate control within ±5 from 1 kN/sec to 20 kN /sec; Yes / No. Class 1 starting from % 10 of the full range; Yes / No. Ready Stress Calculation for the standards of EN 12390-5 , EN 12390-6, EN 1338 ,EN 1340, ASTM C78, C293, C496, SI, Metric Units; Yes / No. Data storage up to 256 tests; Yes / No. Complete with all consumables & software, (Batteries, Cables, and Ultrasonic Couplant etc.) to make this a fully working instrument unit on delivery.; Yes / No. 3. 1 x CONCRETE CUBE MOULDS. Cube moulds to be supplied complete with base plates; Yes / No. Cube Mould (Cast Iron – for Mortar) 50mm; Yes / No. Single Gang Cube Mould (Cast Iron – for Concrete) 100mm; Yes / No. Light Weight Cube Mould (Cast Iron – for Concrete) 100mm; Yes / No. Clamp Type Four-Part Cube Mould (Cast Iron – for Concrete) 150mm; Yes / No. Light Weight Cube Mould (Cast Iron – for Concrete) 150mm; Yes / No. Clamp Type (18 KG) Four-Part; Yes / No. Cube Mould (Cast Iron – for Concrete) 150mm Clamp Type (16 KG) Two-Part; Yes / No. 4. 1 x BEAM MOULDS. Made of Cast Iron / Mild Steel.; Yes / No. Beam moulds are made of 4 plates assembled together.; Yes / No. Faces are machined flat to ± 0.2mm. And finished in size to 0.2mm; Yes / No. 100 mm x 100 mm x 500 mm; Yes / No. 150 mm x 150 mm x 750 mm; Yes / No. 5. 1 x SLUMP TEST APPARATUS. The slump cone has internal dimension 200 mm dia at base, 100 mm top dia and 300 mm height.; Yes / No. The base plate has lifting handle for easy transportation.; Yes / No. One graduated steel rod 16 mm dia x 600 mm long rounded at both ends and graduated in mm.; Yes / No. 1 x Scoop; Yes / No. 1 x Trowel,; Yes / No. 1 x Measuring tape or meter rule; Yes / No. 1 x Funnel; Yes / No. 6. 1 x LABORATORY PAN MIXER. All parts of the mixer are galvanized and painted with non-corrosive paint. The protection class of machine is IP55; Yes / No. Minimum dimensional volume of the mixing pan is 42 litres; Yes / No. Minimum effective mixing capacity is 15 litres; Yes / No. The mixing drum should be tilted to empty the mixture without hassle and it can be removed for easy cleaning on completion of the mixing operation.; Yes / No. Equipped with rubber wheels which provides high mobility.; Yes / No. 7. 1 x LABORATORY DRUM TYPE MIXER. Minimum Yield: 75 litres of concrete; Yes / No. The mixing drum should be tilted to empty the mixture without hassle and it can be removed for easy cleaning on completion of the mixing operation.; Yes / No. Equipped with rubber wheels which provides high mobility.; Yes / No. 8. 1 x CONCRETE PERMEABILITY APPARATUS. Concrete Permeability Apparatus 3 cell model for 100 mm cylinder.; Yes / No. Concrete Permeability Apparatus 3 cell model for 150 mm cube.; Yes / No. Concrete Permeability Apparatus 1 cell model for 150 mm cylinder.; Yes / No. Concrete Permeability Apparatus 1 cell model for 300 mm cylinder; Yes / No. 9. 1 x DIGITAL WEIGHING BALANCE. Backlight display to provide clearer view of results; Yes / No. Simple menu navigation with easy access to advanced features; Yes / No. Weighing range [Max]: 30,000 g / 30 kg; Yes / No. Readability [d]: 1 g; Yes / No. Electricity and/or Battery operation; Yes / No. Calibritable; Yes / No. Complete with all consumables & software, (Batteries, Cables, and Ultrasonic Couplant etc.) to make this a fully working instrument unit on delivery.; Yes / No. 10. 1 x TENSILE STRENGTH TESTER. 0-500 Newtons in 10 Newton division.; Yes / No. Maximum loading capacity 5 kN.; Yes / No. 11. 1 x DIGITAL SIEVE SHAKER. Minimum Capacity 2.5 kg; Yes / No. Test Sieves, manufactured from Stainless Steel .Test Sieves are certified to conform to with SABS ISO 3310 Specification; Yes / No. 50mm WIRE MESH; Yes / No. 37,5mm WIRE MESH; Yes / No. 25mm WIRE MESH; Yes / No. 19mm WIRE MESH; Yes / No. 12,5mm WIRE MESH; Yes / No. 9,5mm WIRE MESH; Yes / No. 4,75 WIRE MESH; Yes / No. 2,36mm WIRE MESH; Yes / No. 1,18mm WIRE MESH; Yes / No. 600micron WIRE MESH; Yes / No. 300micron WIRE MESH; Yes / No. 150micron WIRE MESH; Yes / No. 75micron WIRE MESH; Yes / No. Lid; Yes / No. Receiver; Yes / No. Complete with all consumables & software, (Batteries, Cables, and Ultrasonic Couplant etc.) to make this a fully working instrument unit on delivery.; Yes / No. 12. 1 x FIELD GPS. Simular or equil to Garmin GPSMAP 65s; Yes / No. Must be sunlight readable; Yes / No. Barometric altimeter; Yes / No. Large screen, Minimum size 2.6”; Yes / No. Handling with gloves (Toutch screen functional with gloves); Yes / No. Waterproof outdoor GPS; Yes / No. Wirelessly connects to smartphone; Yes / No. Tough and rugged; Yes / No. Minimum 2.6" sunlight-readable color screen; Yes / No. High-sensitivity GPS and GLONASS quad helix antenna (Global Navigation Satellite System); Yes / No. Minimum 1-year BirdsEye Satellite Imagery subscription; Yes / No. 3-axis compass with barometric altimeter; Yes / No. Plot barometric pressure over time; Yes / No. Wireless connectivity via Bluetooth® technology¹ or ANT+®; Yes / No. 3-axis tilt-compensated electronic compass; Yes / No. dual battery system; Yes / No. Include TOPO Southern Africa PRO maps; Yes / No. Include software to transform paper and electronic maps easily into downloadable maps; Yes / No. Bluetooth connectable; Yes / No. Complete with all consumables & software, (Batteries, Cables, Ultrasonic Couplant etc.) to make this an fully working instrument unit on delivery.; Yes / No. 13. 1 x DYNAMIC CONE PENETROMETER. Supplied with spanners, allen key, weight & cone.; Yes / No. DCP 60° cone (Hardened); Yes / No. 8kg drop weight; Yes / No. 575mm drop height; Yes / No. 1meter bottom Rod; Yes / No. 1m steel ruler; Yes / No. Manual in English; Yes / No. 14. 1 x DIGITAL ULTRASONIC THICKNESS GAUGE METER. Simular or equil to Sauter TN 30-0.1EE Ultrasonic Thickness Gauge. Measuring range: 0.75 to 300mm (in steel); Yes / No. Precision: 0,5 % of [Max] ± 0,04 mm; Yes / No. Accuracy: minimum 0.1mm; Yes / No. Resolution: 0,001; Yes / No. Auto calibration; Yes / No. Accessories: External sensor 2,5 MHz to measure cast iron with rough upper surfaces:. Accessories: External sensor, 7 MHz, ∅ 6 mm, for thin test materials: Measuring range 0,75–80 mm (steel),. Accessories: 60ml Ultrasound contact gel,. Complete with all consumables & software, (Batteries, Cables, and Ultrasonic Couplant etc.) to make this a fully working instrument unit on delivery.; Yes / No. 15. 1 x ULTRASONIC COATING (PAINT) THICKNESS METER. Simular or equil to Major Tech MT155; Yes / No. Measuring Range: 0 -1500 microns; Yes / No. Accuracy: minimum 0.1mm; Yes / No. Resolution: 0,1 micron; Yes / No. Auto calibration; Yes / No. Complete with all consumables & software, (Batteries, Cables, and Ultrasonic Couplant etc.) to make this a fully working instrument unit on delivery.; Yes / No. 16. 1 x CONCRETE TEST HAMMER. Measuring range: 10-60 MPa; Yes / No. Impact energy: 2.207±0.1J（0.225Kgf.m); Yes / No. Length of spring stretch: 75 ±0.3mm; Yes / No. The static friction of pointer slider: 0.65N~0.15N; Yes / No. Radius of spherical tip: 25mm ±1mm; Yes / No. The average rebound values on steel anvil: 80 ±2; Yes / No. Weight: ≈1Kg; Yes / No. 17. 1 x CRACK MEASURING MICROSCOPE. High quality microscope designed for measuring crack widths in concrete members, masonry walls and other structures; Yes / No. Should operate by an adjustable lamp unit and the image is focused by turning a knob; Yes / No. The eyepiece scale can be turned through 360° to align with the direction of the crack or pitch under examination.; Yes / No. The battery operated microscope to have at least 40x magnification; Yes / No. At least 4 mm measuring range with 0.02 mm subdivisions.; Yes / No. Complete with all consumables & software, (Batteries, Cables, and Ultrasonic Couplant etc.) to make this a fully working instrument unit on delivery.; Yes / No. 18. 1 x COMPACTION FACTOR APPARATUS. Consists of two conical hoppers attached on a cylinder; Yes / No. Everything is fixed on a robust steel frame; Yes / No. Each hopper is equipped with a hinge for a Quick Release mechanism; Yes / No. 19. 1 x CYLINDRICAL MOULDS. Cylindrical mould is split vertically into two parts.. Cylindrical mould is split vertically into two parts.. Iinternal diameter is within +/-0.2mm and height is within +/-1mm.; Yes / No. Ends are machined to +/-0.05 mm.; Yes / No. The base plate and top plate are machined flat to +/-0.03mm.; Yes / No. Base plate and top plate are machined flat to +/-0.03mm.; Yes / No. Mould cylinder, cast iron, 150mm dia x 150mm height.; Yes / No. Mould cylinder, cast iron, 150mm dia x 300mm height.; Yes / No. Mould cylinder, cast iron, 160mm dia x 320mm height.; Yes / No. Mould cylinder, cast iron, 100mm dia x 200mm height.; Yes / No. Mould cylinder, cast iron, 300mm dia x 600mm height.; Yes / No. 20. 1 x PROGRAMMABLE ACCELERATED CURING TANK. 24 Hour cycle from time of mixing.; Yes / No. Controlled 35° C or 100° C ± 2° C Curing Temperature for concrete.; Yes / No. Controlled 27° C ± 2° C Curing Temperature for grey cement.; Yes / No. Curing tank to accommodate 150mm / 70.6mm cube moulds and up to 36/72 cube moulds can be cured in one go; Yes / No. The interior is made from stainless steel; Yes / No. The electronic programmer should control up to four test cycles with different thermal gradients and curing time to a defined te; Yes / No. 21. 1 x Semi-Rugged LAPTOP. Dell Latitude 5420 or Equivalent. IP-52 protected; Yes / No. 11th Gen Intel Core i7 processor; Yes / No. 32GB of DDR4 3200 MHz ram; Yes / No. Windows 10 or 11; Battery; Yes / No. Wi-Fi 6E connectivity; Yes / No. 14-inch 1080 Touch Screen, viewable outdoors, 1110 nits of brightness; Yes / No. Win 10; Yes / No. 1 TB SSD; Yes / No. Mobile 4G / 5G Broadband; Yes / No. FHD RGB/IR camera; Yes / No. Stylus slot; Yes / No. Headset and microphone combo port; Yes / No. HDMI 2.0 port,; Yes / No. a RJ45 Ethernet port,; Yes / No. a Serial RS-232; Yes / No. Bluetooth 5.2; Yes / No. Carry bag; Yes / No. Wireless keyboard and mouse; Yes / No. Min 60 mAh battery. Complete with all consumables & software, (Batteries, Cables, Ultrasonic Couplant etc.) to make this a fully working instrument unit on delivery); Yes / No. 22. 1 x LAPTOP SAFE. Numeric operated; Yes / No. Size 200*430*250; Yes / No. Manual override key function; Yes / No. Including 4 Rawl bolts; Yes / No. - All equipment must be in “turnkey” state, i.e. the price must include all fluids, batteries etc. and demonstrated that it work. - A Checklist for all items must be attached to the quotation..

CLOSING DATE : 8 July 2022 16:30

KINDLY ENSURE THAT YOUR COMPANY IS REGISTERED ON CENTRALISED SUPPLIER DATABASE IF NOT ,KINDLY REGISTER ON THIS WEBSITE www.csd.gov.za

2.	The above specified goods/services should be delivered/rendered to:

Address                   : ARC-IAE

Address                : 141 Creswell Road

Weavind Park

Pretoria

3.	The particulars of the guarantee that will apply to the goods quoted for,

with particular regard to the period and extent of the warranty must be

clearly stated. Where services are required, service providers must submit

documentation pertaining the relevant experience.

4.	Your written quotation should be sent to:

Lungile Kubheka

E Mail: KubhekaL@arc.agric.za

5.	All price quotations that have a rand value of R30,000-00 to R50 000 000.00

including VAT, will be evaluated by applying the 80/20 principle as

prescribed by the Preferential Procurement Policy Framework Act 5 of

2000 and its Regulations. The lowest acceptable price will score

80 points, the 20 BBBEE points will be allocated as follows:

| B-BBEE Status Level of Contributor | 80/20 |
| --- | --- |
| 1 | 20 |
| 2 | 18 |
| 3 | 14 |
| 4 | 12 |
| 5 | 8 |
| 6 | 6 |
| 7 | 4 |
| 8 | 2 |
| Non-compliant Contributor | 0 |

6.	Standard conditions:

6.1	The validity of the quotations must be indicated.

6.2	Prices quoted should be in South African Rand and inclusive of VAT costs

such as delivery, insurance, taxes, etc.

6.3	No price adjustments or amendment of the delivery particulars contained

in paragraph 2 will be considered by the ARC.

6.4	The supplier accepts full responsibility for the proper execution and

fulfilment of the goods/services quoted for.

6.5	ARC reserves the right to accept or reject any special terms and

conditions that may qualify the goods/services to be provided.

6.6	Quotes should be submitted on an official letterhead and duly signed

6.7	Goods and services should be supplied/rendered upon receipt of a

purchase order from the ARC

6.8	The General Conditions of Contract issued by National Treasury are

applicable.

6.9	The ARC supply chain management code of conduct is applicable.

6.10	SBD Forms must be signed and returned together with the quotation,, failure to comply will result to disqualification of your quotation.

6.11	Your quotation must indicate the delivery date.

6.12	The ARC reserve the right to do due diligence on the quotations.

6.13	The ARC reserve the right to benchmark prices quoted.

7.   	All equipment must be guarantee for a minimum period of 6 month from the date of final commission at ARC site (see point 8)

Compulsory requirements

- Valid tax PIN number from SARS.

- Central Supplier Database report (Current and updated CSD report) showing a tax compliant status for the duration of the bidding process.

- Complete and sign the Standard Bidding Documents (SBD forms)

Thank you in anticipation

Supply Chain Management: ARC