TECHNICAL SPECIFICATIONS FOR THE EQUIPING OF THE BOREHOLE AND CONSTRUCTION OF THE WATER HOLDING TANKS STAND AT THE MASIA COMMUNITY CENTRE

| # | Item/s | Technical Specifications |
| --- | --- | --- |
| 1 | Automation & electrical wiring | - Electrical cable from the Solar grid to the borehole - 16mm 3 core x 270m (pump protection relay motor scope 2.2 kw - Orange electrical Box 500 x 500 mm and - Issuance of the Certificate of Compliance (COC) |
| 2 | Borehole pump/motor & control box | - 2.2 kW |
| 3 | Miscellaneous/material (piping) | - Supply of water from the borehole to the 40kl elevated steel tank and to the nursery holding tanks - HDP pipe - 32mm x 200m class 10 - Rope - SKI/Nylon 10mmx 200m - Base Plate - 3 x 32mm - Compression male adaptors: 2 x 32mm galvanized elbow male and female - 4 core 200m x 6mm - 2 x Cable joint - 2 x Packets of cable ties - 2x Insulation tapes - 4 x Thread tapes |
| 4 | Borehole cage dimension | - 1,5 m (L) x 1,5 m (W) x 1,5 m (H) - Lockable steel-lid of 4 mm See attached pictures for specification |
| 5 | 40kl elevated steel tank | - Repair tank leaks |
| 6 | Constructing reinforced boxed concrete for water holding tanks (should be able to hold 5 x 10000L) | - Tank stand size: 13M x 3.5M x 1M. - Tank stand reinforced concrete Footing: 0,5M X 0,5 M. - Bricks to use: Clay bricks. - Stand filling: Gravel should be used for compaction purposes in stand compartments. - Stand floor reinforcements: REF 100 - 6 M x 2.4 M: 200 x 200 x 4 mm Welded Mesh. - Cement type: PPC cement: 42.5 N. - Crush runner: 7 mm - 8mm for reinforced stand floor. - Concrete Bonding liquid: Bonding liquid TAL. - Tank stand wall: Double cavity wall. - Brick force: NHBRC BRICKFORCE 2.8MM - double brick wall See attached pictures for specification. |