1.Specification / scope of work / Terms of Reference. 1.1 Composition/information on ingredients Detonator, flash: type: signal: (packing 350 per case) track warning; with shelf life not exceeding two years for detonators with shelf life exceeding two years but not exceeding five years. Chemical entity; Case number; proportion. Potassium chlorate; 3811-01-0; HIGH. Sulphur; 7704-34-9; HIGH. Silica sand; -----; MED. Poly vinyl acetate; 9003-20-7; VLOW. Guar Gum; 9000-30-0; =100%. PROPORTION (% weight per weight): VHIGH>60, HIGH 30-60, MED 10-29, LOW 1-9, VLOW<1 1.2 Technical specification Effect description; Emits an audible report more than 150 decibels. Case dimensions; 50 MM DIA X 10 MM thick; Internal Diameter; NA. Effect height; NA; Effect Width; NA; Burn Time; Instant. Plastic weight; 3 gms; Paper weight; NA; Clay weight; NA; Fuse head; NA. NEQ star; NA; NEQ Inserts; NA; NEQ B/P dr; NA; NEQ COMP; 9 gms. NEQ/Total; 9 gms; Total Gross weight of product:; 13 gms. Sub box size; NA; Cannisters per carton; 50; Carton size CMB; 0.029. Carton dimensions; 31 X 31 X 30 cms; packing; 12\1; Sub box weight Net; NA. Canister weight net; 40 GMS; Carton weight Net; 10 kgs; Carton weight Gross; 11 kgs. Carton explosive content; 5.4 kg. Firing controls; Initiated by the extreme pressure inherent between a locomotive wheel and the track. Place non slip green face on track face. Ensure that the wire assembly part of the Railway Track Signal is facing in the direction of approaching rolling stock, unfurl the aluminium wire attached and wrap it around and into the profile sides of the track.. Set up..