APPOINTMENT OF A SUITABLE AND ACCREDITED SECURITY SERVICE PROVIDER TO INSTALL THE ELECTRONIC ACCESS CONTROL SYSTEM (EACS) AT AGRICULTURAL RESEARCH COUNCIL (ARC) OFFICES – TROPICAL AND SUBTROPICAL CROPS CAMPUS, MBOMBELA, MPUMALANGA PROVINCE

- PURPOSE

The ARC – TSC in Mbombela, Corner Bosch & KaNyamazane, Mpumalanga Province, seeks to appoint a service provider to install a SOFTCON ACCESS CONTROL SYSTEM at the above-mentioned premises.

- COMPULSORY SITE INSPECTION

The bidders are required to attend a compulsory site inspection in order to examine the site and its surroundings (environment) and, the quantities and nature of work, materials necessary for completion of the work and their availability, means of access to the site and in general to obtain all necessary information as to risks, contingencies etc.  and other circumstances which may influence or affect the price.

- DETAILS OF REQUIRED SERVICES

3.1 Product specifications

Supply, installation, testing and commissioning of Electronic Access Control System – Softcon on SoftWin3 Version 2020.

  - System general description

    - The Electronic Access Control System (EACS) shall be a secure, modular, extendable system designed to control and manage the movement of building occupants.

    - The EACS shall include a centralized Access Management Server configured, monitored and operated from web browsers and software clients located on at the control room Building Local Area Network (LAN).

    - The Access Management Server shall be deployed on the Microsoft Windows operating system and capable of being run on a dedicated server or a virtual machine and able to support multiple simultaneous users.

    - Multiple-site management with full site replication using distributed server technology.

    - Graphic user interface in a form of client-customizable drawings depicting specific real site environment.

    - Enhanced system set-up editors which allow a wide variety of site requirements.

    - Fully scalable from small stand-alone system to a large enterprise solution.

    - Ability to design and produce full colour ID labels with photographs using the Card maker function.

  - Drawings

The contractor shall provide complete facility drawings including the following:

    - Architectural floor plans indicating all system device locations.

    - Wiring schematics for all devices including cable types, lengths, routings and termination requirements.

    - Complete block diagram of the EACS.

    - Detailed drawings showing mounting and fastening methods for system components.

    - System commissioning requirements and report format
etc.

  - Product Manuals

Upon completion of the system installation, the contractor shall make available print or digital versions of the following manuals.

    - Hardware manual describing the installation, configuration and operation for hardware deployed as part of the EACS installation.

    - Software manual describing the proper configuration and operation of the Access Control Server.

    - Maintenance manual describing the proper maintenance and repair of the EACS.

- WARRANTY & SOFTWARE MAINTEANCE AGREEMENTS

  - Upon completion of the system installation, the contractor shall make available the manufacturers product warrant, applicable licences and software maintenance agreement.

- ACCESS CONTROL SYSTEM DESIGNED FOR THE ENTRANCES/EXITS AND CORRIDORS OF MAIN BUILDINGS OF TSC CAMPUS AS FOLLOWS:

| Area | No. of Doors | Location | Softcon Controller | HID Proximity Card Reader | Emergency Break Glass Unit | Key Override Switch | Wiring | Door Closer & Magnet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Admin building | 5 | TSC | 5 | 10 | 5 | 5 | Full system | 5 |
| Research building | 3 | TSC | 3 | 6 | 3 | 3 | Full system | 3 |
| Biotech building | 2 | TSC | 2 | 4 | 2 | 2 | Full system | 2 |
| Pathology building | 3 | TSC | 3 | 6 | 3 | 3 | Full system | 3 |
| Library | 2 | TSC | 2 | 4 | 2 | 2 | Full system | 2 |
| Total | 15 |  | 15 | 30 | 15 | 15 |  | 15 |

| Control room equipment | - 01 x Computer set (At least an i5 with 16GB Ram and 23” monitor that will be able to run the solution) - 01 x Printer with compatible colour cartridges/Ribbons SPECIFICATION Standard access control colour card printer with the following features: • Printing Single-sided, • Lifespan of 1-5 years, • Ability to print 1000 cards per year. - Training of two people - Drawings for whole system |
| --- | --- |

NB: ALL SERVICE PROVIDERS ARE REQUESTED TO ATTACH A DETAILED BILL OF QUANTITIES PER ITEM.

PRICING SCHEDULE

## COSTING BREAKDOWN

| Item No | Item description | Quantity | Price per unit (VAT incl.) | Total price (VAT incl.) |
| --- | --- | --- | --- | --- |
| 1 | Card readers | 30 |  |  |
| 2 | Controllers | 15 |  |  |
| 3 | Door Closer and Maglocks | 15 |  |  |
| 4 | Automated booms at 4 meters each | 02 |  |  |
| 5 | Automated boom infrastructure | 02 |  |  |
| 6 | Emergency Break glass Unit | 15 |  |  |
| 7 | Key Override switch | 15 |  |  |
| 8 | Wiring | Full system |  |  |
| 9 | Drawings for whole system | 01 |  |  |
| 10 | Printer with compatible colour cartridges/Ribbons | 01 |  |  |
| 11 | Computer (At least an i5 with 16GB ram) set – i.e., Mouse, keyboard, Monitor (At least 23”) and PC | 01 |  |  |
| 12 | System set up, and reconfiguration eg. create areas, zones and groups etc. and testing. | 01 |  |  |
| 13 | Training of two people | 01 |  |  |
| 14 | Intercom and Access control push button for Receptionist | 01 |  |  |
| 15 | Sundries (provide a clear breakdown) | 01 |  |  |
|  | TOTALS |  |  |  |

- SYSTEM GUARANTEES

- ARC requires performance guarantees for all the systems for 12 months.

- The supplier shall stipulate the guarantee of the System according to ISO and SABS standards or any applicable related regulations or standards.

- The supplier shall in detail document what is covered by of the guarantee as well as the terms and conditions.

The following documents are required:

| Document that must be submitted | Non-submission may result in disqualification. |  |
| --- | --- | --- |
| # PSIRA Business Certificate | YES | Bidders required to submit a certified copy of a PSIRA Business Registration certificate. |
| # Three reference letters on similar projects | Yes | Bidders must submit three reference letters of successfully completed related or similar projects. |
| # Tax Compliance Status # Tax Clearance Certificate | YES | Bidders must ensure compliance with TAX obligations. Bidders required to submit a printed Tax Compliance Status or Tax clearance certificate |

## 5.	SITE INSPECTION

- Before tendering, tenderers shall visit the site for orientation and briefing on the specification.

- All invited suppliers are kindly requested to attend a COMPULSORY SITE BRIEFING – date and venue will be indicated on the RFQ

- Suppliers who failed to attend the compulsory site briefing will not be permitted to submit their bids.

## For further assistance, please call the following persons from 08h00 – 16h00 weekdays ONLY:

## Mr A Nkuna - 072 414 2430

## Mr G Lubisi - 079 763 2809