| REFERNCE | ICT EQUIPMENT PROCUREMENT | DATE | 31 | 08 | 2022 |
| --- | --- | --- | --- | --- | --- |

Terms of reference: Supply Delivery & Installation of Network cabling for Aliwal North’s New Campus

Admin Block (Side A)

- Office 1-4: 2 points each

- Office 5-10: 4 points each

- Main Reception: 8 points

- 2nd Reception: 8 points

Total 48 ports (Side B)

- Office 11-15: 2 points each (office 12 with dual skirting- 6 meters)

- Office 16: 6 points

- Office 17: 2 points

- Boardroom: 10

Total = 28 Server Room

- 47U cabinet (with wire mesh)

- Raised floor: 45-60cm height, must be able to withstand an uniform load of 1,220 kg/m2.

The minimum distance between the upper part of the server rack and the ceiling should be at least 45 cm.

- Connect the cabinet to the UPS

- 48 Port POE switch: 2

Block B

- 2 labs: 164 point

- Classrooms 8 points (Route to the server room)

- 48 port POE Switches: 4

- 27U Cabinet: 2 (Each lab installed at the server room, connected to the UPS in one of the server rooms)

- N6 caps: 6

- 32 mil bosal pipes: 4m

Total Points: 172

# A better future with IKHALA

Administration Centre: 047 873 8800

Aliwal North Campus: 051 634 1035 • Ezibeleni Engineering Campus: 047 873 1293

Workshop

27 U cabinets: 1 Switches: 48 port POE

:

Classroom 1:2 points

Classroom 2:2

Reception 1: 2 points

Reception 2: 3 points

Total: 9 Points

Staff room 1: 6 points

: Skirting (hospital grey, drop, 12 pieces & route around)

:Two internal 90% handy caps Staff room 2: 6 points

: Skirting (hospital grey, drop, 12 pieces & route around)

:Two internal 90% handy caps Office 1: 4 points

Office 2: 4 points

Office 3: 2 points

Office 4: 4 points

Total= 35 points

Guard Room (Entrance)

Points: 6 points

Fiber to the guard house 18U Cabinet

8 POE Switch

Total number of Points: 300

Fiber from the main admin building to block-b & the workshop Certified cabling test documents upon completion.

Service provider must be certified with Krone/ Molex. Fiber trays: ST

Gbix: 10

295	Network Points

295	Fly Leads

295	Patch Cords

295	Flush mount Cat6

1	Raised Flooring Supply patch & brush panels

All cabinets: 5-year limited warranty & UL60950; RoHS; CE Complaint

# A better future with IKHALA

Administration Centre: 047 873 8800

Aliwal North Campus: 051 634 1035 • Ezibeleni Engineering Campus: 047 873 1293

Switch Specification.

Fully managed Gigabit Layer 2 PoE+ switch. Power Consumption Description476 W (maximum) Heat Dissipation236 BTU/hr (248.98 kJ/hr)

Input Voltage100 - 127 / 200 - 240 VAC, rated

External I/O RJ-45 autosensing 10/100/1000 PoE+ ports (4) fixed Gigabit Ethernet SFP ports Latency100 Mb Latency: < 7.4 µs 1000 Mb Latency: < 2.3 µs

Switching Capacity104 Gbps

Stacking Capabilities Virtual 16 switches Throughput up to 77.3 Mpps

Memory and ProcessorARM9E @ 800 MHz 128 MB flash Packet buffer size: 3 MB dynamically allocated 256 MB DDR3 DIMM

PoE Capability382 W

Management Features Aruba Central and Aruba AirWave Network Management IMC— Intelligent Management Center command-line interface Web browser configuration menu out-of-band management (serial RS-232C or Micro USB) IEEE 802.3 Ethernet MIB Repeater MIB Ethernet Interface MIB AirWave Network Management

Warranty Standard Statement Limited Lifetime Warranty

# A better future with IKHALA

Administration Centre: 047 873 8800

Aliwal North Campus: 051 634 1035 • Ezibeleni Engineering Campus: 047 873 1293