# Procurement of Geoinformatics Capex Items 2023

- Zenmuse-L1 Lidar + RGB sensor compatible with Matrice 300 RTK drone

Specifications

- Product Name

- Zenmuse L1

- Dimensions

- Supported Aircraft

  - Matrice 300 RTK

- Operating Temperature Range

- Detection Range

- 450 m @ 80% reflectivity, 0 klx ;
190 m @ 10% reflectivity, 100 klx

- Real-time Point Cloud Coloring Modes

- Reflectivity, Height, Distance, RGB

### Lidar

- Ranging Accuracy (RMS 1σ)2

- 3 cm @ 100 m

- Maximum Returns Supported

- 3

- Scan Modes

- Non-repetitive scanning pattern, Repetitive scanning pattern

- FOV

- Non-repetitive scanning pattern: 70.4° (horizontal) × 77.2° (vertical) ;
Repetitive scanning pattern: 70.4° (horizontal) × 4.5° (vertical)

- Laser Safety

- Class 1 (IEC 60825-1:2014) (Eye Safety)

### Inertial Navigation System

- IMU Update Frequency

- 200 Hz

### Auxiliary Positioning Vision Sensor

- Resolution

- 1280×960

- FOV

- 95°

### RGB Mapping Camera

- Sensor Size

- 1 inch

- Effective Pixels

- 20 MP

- Photo Size

- 5472×3078 (16:9); 4864×3648 (4:3); 5472×3648 (3:2)

- Focal Length

- 8.8 mm / 24 mm (Equivalent)

- Shutter Speed

- Mechanical Shutter Speed: 1/2000 - 8 s
Electronic Shutter Speed: 1/8000 - 8 s

- ISO

- Video: 100 – 3200 (Auto), 100 – 6400 (Manual)
Photo: 100 - 3200 (Auto), 100 - 12800 (Manual)

- Aperture Range

- f/2.8 - f/11

- Supported File System

- FAT (≤32 GB); exFAT (>32 GB)

- Photo Format

- JPEG

- Video Format

- MOV, MP4

- Video Resolution

- H.264, 4K: 3840×2160 30p

### Gimbal

- Stabilized System

- 3-axis (tilt, roll, pan)

- Angular Vibration Range

- 0.01°

- Mount

- Detachable DJI SKYPORT

- Mechanical Range

- Tilt: -120° to +30°; Pan: ±320°

- Operation Modes

- Follow/Free/Re-center

### Data Storage

- Raw Data Storage

- Photo/IMU/Point cloud data storage/GNSS/Calibration files

- Supported microSD Cards

- microSD: Sequential writing speed 50 MB/s or above and UHS-I Speed Grade 3 rating or above; Max capacity: 256 GB

NB: Demonstration/training on the use of sensor is required.

Items should have at least one-year warrantee

Items should be delivered to ARC-SCW, 600 Belvedere Street, Arcadia, Pretoria, 0083