Specifications for Quotation

Ref. No.: MSQ 13 OF 2023

UMsunduzi Museum request quotations from the professional Service Provider to undertake Repairs and Maintenance of the Thatched Roof on the following Areas; Vootrekker House (333 Boom Street), Andries Pretorius House and the Zulu Hut.

- Vootrekker House (333 Boom Street)

1.1. Work to be done

1.1.1. The Service provider to remove the old existing roof thatching and rebuilt from the roof pole structure.

1.1.2. Assess the Roof Poles Thatching Structure to ensure that if it is solid. If not solid, repair the Structure.

1.1.3. And apply fire retardant to the roof.

1.1.4. Roof Size: Approximately 78m²

  - Roof Thatch Application

1.2.150 mm Thick thatch of hand cut grass of an acceptable species in lengths with a minimum of 1200 mm laid in bundles of suitable size.

1.2.2. Thatch Grass to be compacted to a minimum density of 35 kg/m/150 mm thickness.

1.2.3. Thatch Grass to be fixed by means of suitable fixing rods interlaced through and over the bundles at not exceeding 600mm centres.

1.2.4. Thatch Grass secured with an acceptable bitumen impregnated treated sisal rope or SABS approved nylon twine and including eucalyptus battens CCA treated in accordance with SABS 457 varying in diameter from 30 to 40 mm, spaced generally at maximum 250 mm centres and at closer centres nearer to eaves nailed to roof structure and with and including 25 mm thick spread layer to underside

1.3. Roof Ridge

1.3.1. Strip and Replace existing roof ridge of approximately 2.4 L/m.

1.3.2. Ridge capping 1200 mm girth formed of 75 mm thick 25MPa concrete with construction joints at                                    1500mm reinforced with and including type 193 fabric reinforcement on 250 micron "USB" waterproof sheeting.

1.4. Fire Retardant

1.4.1Apply Fire retardant to an entire roof of approximately 78m².

1.4.2. The roof is sprayed inside and outside with special attention to all edges and corners.

- Andries Pretorius House

2.1. Work to be done

2.1.1. The Service provider to repair the old existing roof thatching.

2.1.2. Apply fire retardant to the roof.

2.1.3. Roof Size: Approximately 143, 9 m²

  - .Roof Thatch Application

2.2.1. 50 mm Thick thatch of hand cut grass of an acceptable species in lengths with a minimum of 1200 mm laid in bundles of suitable size.

2.2.2. Thatch Grass to be compacted to a minimum density of 35 kg/m/150 mm thickness.

2.2.3. Thatch Grass to be fixed by means of suitable fixing rods interlaced through and over the bundles at not exceeding 600mm centres.

2.2.4Thatch Grass secured with an acceptable bitumen impregnated treated sisal rope or SABS approved nylon twine and including eucalyptus battens CCA treated in accordance with SABS 457 varying in diameter from 30 to 40 mm, spaced generally at maximum 250 mm centres and at closer centres nearer to eaves nailed to roof structure and with and including 25 mm thick spread layer to underside

- Roof Ridge

3.1. Assess the Roof Ridge; Repair the existing roof ridge if damaged.

  - Ridge capping 1200 mm girth formed of 75 mm thick 25MPa concrete with construction joints at 1500mm reinforced with and including type 193 fabric reinforcement on 250 micron "USB" waterproof sheeting.

- Fire Retardant

4.1. Apply Fire retardant to an entire roof of approximately 143.9m².

  - The roof is sprayed inside and outside with special attention to all edges and corners.

- Zulu Hut

5.1Work to be done

5.1.1. The Service provider to repair and maintain existing thatching, pull and comb the thatch

5.1.2. Treat moisture.

5.1.3. Apply fire retardant to the thatching.

5.2.Roof Size : Approximately 51m²

5.3 Roof Ridge

5.3. Assess, repair or replace the Roof Ridge; if damaged.

5.4. Fire Retardant

5.4.1. Apply Fire retardant to an entire roof of approximately 51.1m².

5.4.2. The roof is sprayed inside and outside with special attention to all edges.

and treat moisture

6. Mandatory Requirements

6.1. Service provider must be registered with TASA (Thatchers Association of South Africa).

6.2. All workmanship and materials to be guaranteed for a period of twenty four (24) months.

6.3. Service provider to come onsite for a quote, and quote per item.

6.4. The Service Provider prior to the awarding of the work will have to sign a Service Level Agreement (SLA) with the Museum.

6.5. Service providers will have to provide us with Work Assurance Certificate after job completion.

6.6. Material to be SABS and TASA Approved standards.

6.7. Service provider to comply with occupational health and safety regulation while, onsite.

6.8. Fill in the declaration form – SBD 4

6.9. Local Content form – SBD 6.1

6.10. BBEEE certificate or sworn affidavit

6.11Three contactable references and written letter as proof of work done.

6.12. Valid Copy of COIDA certificate.

6.13. Valid copy of CSD registration.

6.14. Valid copy of CIDB certificate

7. Delivery Deadlines:

7.1 Date of the Briefing: Thursday 04 December 2023, 11:00

7.2. Quotations By: Thursday, 14 December 2023

7.3. Quotations to: Simphiwe Mbeje, Email to: smbeje@msunduzimuseum.org.za  and CC Msizi Mfeka, Email to: mmfeka@msunduzimuseum.org.za

8. Enquiries:

  - For Quotation processes enquiries: contact Mr. Simphiwe Mbeje, Museum Administration Department. Tel. 033 394 6834

  - For Technical enquiries: Contact Chief Exhibition Officer - Mabo Mdluli or Mr. Reggie Moodley – Exhibition Officer, Exhibitions and Maintenance Department. Tel. 033 394 6834