# Scope of work

The facility will comprise of two 12-meter refurbished shipping containers with isolated walls and an isolated ceiling. The undercover shelter will be between the two containers on an existing slab (size of slab 16.5m x 10.5m).

The complex will be used for Seed sorting and storage as well as processing of other agricultural produce.

- The refurbishment of all the containers was specified as follows:

  - Container 1

    - 2 toilets and a wash basin (2.5 m)

    - Kitchen (2.5 m) with wash basin, cabinet and top space to place kettle and microwave and space for a small refrigerator, 1 plug point

    - Office (4.5m), 1 plug point

    - Germination Laboratory (2.5m), small narrow window for minimum light; 1 plug point

  - Container 2

    - Processing facility to place seed processing machine, big enough windows to allow light – 10m long. 2-3 plug points

    - Storeroom with high narrow window – 2 m long

- The two containers must be insulated and painted and linked to water reticulation, sewer and connections.  Electrical wiring should be to include household plugs and lights.

# Site location

The Ncera farm is situated 25 km South of East London at 33° 7.384'S 27° 36.890'E

# Existing structure

A concrete slab has been laid as a foundation for the seed sorting facility., made of containers. The facility will be erected over the concrete slab, see picture below.

Figure 1: Photos of the site

Figure 2: Drawing of the existing concrete slab

# Design of facilities

## The general layout of the facility

Figure 3: Outlay of the containers (NB: A visible drawing is as per attached pdf document and should be printed on A3 for sharing with potential contractors0

Table 1: Bill of Quantities

| Item No. | Item Description | Units | Quantity |
| --- | --- | --- | --- |
| 1 | 12-meter shipping container | No. | 2 |
| 2 | C4 Windows | No. | 7 |
| 3 | E1 Windows | No. | 3 |
| 4 | E4 Windows | No. | 2 |
| 5 | Standard doors | No. | 7 |
| 6 | Double delivery doors (1359 x 2032) | No. | 1 |
| 7 | Standard toilet | No. | 2 |
| 8 | Standard wash basin | No. | 2 |
| 9 | Double kitchen sink (standard) | No. | 1 |
| 10 | Office Cabinet (standard) | No. | 1 |

- .

## Ramp and steps

- Ramps at the back of the building and in front must be constructed or installed  for trollies to access the building space between the two containers

- Steps next to the ramp on the front of the building must be installed