# PART 3: SCOPE OF WORK

| Document reference |  | Title |  | No of pages |
| --- | --- | --- | --- | --- |
|  |  | This cover page |  | 1 |
|  | C3.1 | Employer’s Service Information |  |  |
|  | C3.2 | Contractor’s Service Information |  |  |
|  |  |  | Total number of pages | 15 |

PART C3: SCOPE OF WORK	1 	C3 TSC3 COVER PAGE

# C3.1: EMPLOYER’S SERVICE INFORMATION

## Contents

Part 3: Scope of Work ..................................................................................................................................... 1

C3.1: Employer’s service Information ........................................................................................................... 2

- Description of the service ...................................................................................................................... 4

  - Executive overview ............................................................................................................................ 4

  - Employer’s requirements for the service ........................................................................................... 4

  - Interpretation and terminology ........................................................................................................... 5

- Management strategy and start up. ...................................................................................................... 5

  - The Contractor’s plan for the service ................................................................................................. 5

  - Management meetings ...................................................................................................................... 5

  - Contractor’s management, supervision and key people ................................................................... 6

  - Provision of bonds and guarantees ................................................... .

  - Documentation control ....................................................................................................................... 6

  - Invoicing and payment ....................................................................................................................... 6

  - Contract change management .......................................................................................................... 7

  - Records of Defined Cost to be kept by the Contractor ...................................................................... 7 2.9 Insurance provided by the Employer ................................................................................................. 7

  - Training workshops and technology transfer ..................................... ..............................................

  - Design and supply of Equipment ....................................................... .

  - Things provided at the end of the service period for the Employer’s use ......................................... 7

    - Equipment .................................................................................... .

    - Information and other things ........................................................................................................ 7

2.13 	Management of work done by Task Order ........................................................................................ 7

- Health and safety, the environment and quality assurance ............................................................... 7

  - Health and safety risk management .................................................................................................. 7

  - Environmental constraints and management .................................................................................... 8

  - Quality assurance requirements ........................................................................................................ 8

- Procurement ............................................................................................................................................ 9

  - People ................................................................................................................................................ 9

    - Minimum requirements of people employed ................................................................................ 9

    - BBBEE and preferencing scheme ............................................................................................... 9

    - Accelerated Shared Growth Initiative – South Africa (ASGI-SA) ................................................. 9

  - Subcontracting ................................................................................................................................... 9

    - Preferred subcontractors ............................................................................................................. 9

    - Subcontract documentation, and assessment of subcontract tenders ........................................ 9

    - Limitations on subcontracting ...................................................................................................... 9

    - Attendance on subcontractors ................................................................................................... 10

  - Plant and Materials .......................................................................................................................... 10

    - Specifications ............................................................................................................................. 10

    - Correction of defects .................................................................................................................. 10

    - Contractor’s procurement of Plant and Materials ...................................................................... 10

    - Tests and inspections before delivery ....................................................................................... 10

    - Plant & Materials provided “free issue” by the Employer ........................................................... 10

    - Cataloguing requirements .......................................................................................................... 10

- Working on the Affected Property....................................................................................................... 10

  - Employer’s site entry and security control, permits, and site regulations ........................................ 11

  - People restrictions, hours of work, conduct and records ................................................................. 11

  - Health and safety facilities on the Affected Property ....................................................................... 11

  - Environmental controls, fauna & flora .............................................................................................. 11

  - Cooperating with and obtaining acceptance of Others .................................................................... 11

  - Records of Contractor’s Equipment ................................................................................................. 11

  - Equipment provided by the Employer .............................................................................................. 12

  - Site services and facilities ................................................................................................................ 12

    - Provided by the Employer .......................................................................................................... 12

    - Provided by the Contractor ........................................................................................................ 12

  - Control of noise, dust, water and waste .......................................................................................... 12

  - Hook ups to existing works .............................................................................................................. 12

  - Tests and inspections ...................................................................................................................... 12

    - Description of tests and inspections .......................................................................................... 12

    - Materials facilities and samples for tests and inspections ......................................................... 13

- List of drawings..................................................................................................................................... 13

  - Drawings issued by the Employer ................................................................................................... 13

## 1 Description of the service

### 1.1 Executive overview

This document defines the technical requirements for the repairs of holes on both Unit 1 tube mills and U2-6 vertical spindle mills.

.

NOTE: The final SOW for holes repairs on all mills will be determined by the condition of the mills during inspections at the time of work and as agreed between Eskom and the Contractor.

The purposes of this Works Information document is to ensure that the repairs on all holes are carried out according to technical specifications and are all repaired mills are reliable once returned to service.

### 1.2 Employer’s requirements for the service

The Contractor provides qualified labour to carry out the service. Minimum qualified labour requirements

(Supervisor; Qualified Artisans; Safety Officer; Quality Controller, etc.)

-The Contractor provide all required tools, consumables, and all necessary Personal Protective

Equipment’s to carry out the work. N.B No tools, consumables or PPE will be provided by the Employer.

-

bottles.

- The Contractor is responsible for the safety of his own personnel by complying with OHS Act No.85 of 1993 and its Regulations.

- The Contractor performs work within the specified period and to the acceptable standard as agreed between Eskom and the Contractor.

- The Contractor perform quality control on own work as per pre-approved control plan before calling the Employer for final inspection.

Scope of work

Repair all damaged holes on the following milling plant components

Mill table holes, mill body holes, Mill bearing cap holes, power cylinder stand holes, mill drum door holes, mill gearbox top flange holes, mill gearbox basement holes, U1 feeder body holes, trunnion arm holes, spring rod pins, mill motor base, U1 screw conveyor stub shaft holes, roller shafts holes

M12, M16,M20 ,M24,M27,M30,M39, M40, M56,M48, 2 1/4” - Drill out the broken bolt and tap the holes.

- Repair the damaged holes by either filling up the damaged hole with welding and re-drill new holes and tap to maintain its original size.

- Blowing out broken bolts

- Cleaning and tapping of holes

- Welding of studs on broken nuts for easy removal of bolts

- Repair the damaged holes by drilling the hole bigger, putting in the insert (N.B the insert minimum thickness is 6mm and minimum length of 50mm and the inserts to be fine threats not corse threats ) and lock the insert with either welding or grab screws to avoid the insert from getting loose.

- N.B All holes to be drilled straight and tapped straight.

- Re-sleeve the feeder table bush and machine to size ID 70.05 mm

Feeder adjusting mechanism

- Repair the feeder plough adjusting mechanism to operate effectively

- Machine new feeder mechanism spindles and handwheels

### 1.4 Additional Requirements

- After assessing the damaged holes, the Contractor must compile a detailed report confirming scope of work and send to the Employer. No work is to be carried out without Employer’s scope approval.

- The Contractor will comply with the OHSA and Regulations

- The Contractor will give the assurance that he has the required skills and tools at his disposal to adequately carry out the Scope of Work as defined in this document.

- The Contractor must declare and keep a record of all spares, tools or equipment that will be brought to site on arrival. Failure to comply may lead to the forfeiting of Contractor’s possessions.

- Each mill is expected to run for duration of at least (4200hrs) working hours without re-work.

1.1.4.1 Key Performance Indicators

The performance of the Contractor shall be measured against the following indicators:

- Employer receives an electronic copy of the Contractor’s reports in terms of this contract within 1 working day of each task order.

- The Contractor is required to provide a program on repairs of holes with a completion date.

A 1% penalty will be levied per task order for the late completion of the repairs of holes per day up to a maximum of 5% after 5 days.

- This means a penalty of up to 5% can be levied on each task order for late completion.

### 1.3 Interpretation and terminology

| Abbreviation | Meaning given to the abbreviation |
| --- | --- |
| SOW | Scope of Work |
| OHSA | Occupational Health and Safety Act |
| ISO | International Organization for Standardization |
| PTW | Permit to work |
| QCP | Quality Control Plan |
| PPE | Personal Protective Equipment |

2 Management strategy and start up.

### 2.1 The Contractor’s plan for the service

The Contractor to provide a detail program for approval on each task per mill after scope being approved by the Employer

### 2 .2 Management meetings

Meetings of a specialist nature may be convened as specified elsewhere in this Service Information or if not so specified by persons and at times and locations to suit the Parties, the nature and the progress of the service.  Records of these meetings shall be submitted to the Service Manager by the person convening the meeting within five days of the meeting.

All meetings shall be recorded using minutes or a register prepared and circulated by the person who convened the meeting.   Such minutes or register shall not be used for the purpose of confirming actions or instructions under the contract as these shall be done separately by the person identified in the conditions of contract to carry out such actions or instructions.

Daily plan and progress feedback meetings shall be held daily in the morning 7:00 am at the designated area by the Employers representative and the Contractor. In addition, the Contractor representative shall attend outage meetings.

Risk assessment and Pre-job brief per task to be discussed daily in the morning meetings .

Workers register to be signed in daily in the morning and sign out in the afternoon before knocking off.

#### 2.3 Contractor’s management, supervision and key people

- All procedures, work instructions, forms and all contractual communications must be controlled for the duration of the contract.

- All contractual communications will be in the form of properly compiled letters or forms attached to emails and not as a message in the email itself.

- Inspections reports together with scope of work to be submitted to the Employer within 8 hours after inspection.

- All communication must be done through Employer’s Representative i.e. Contract Manager

#### 2.4 Documentation control

- Contractor to submit the QCP’s with hold points to the Employer for approval

- All procedures, work instructions, forms and all contractual communications must be controlled for the duration of the contract.

- All contractual communications will be in the form of properly compiled letters or forms attached to emails and not as a message in the email itself.

- All communication must be done through Employer’s Representative i.e. Service Manager

### Use of standard forms

- Early warning forms

- Non-conformance register form

#### 2.5 Invoicing and payment

Within one week of receiving a payment certificate from the Service Manager in terms of core clause 51.1, the Contractor provides the Employer with a tax invoice showing the amount due for payment equal to that stated in the Service Manager’s payment certificate.

The Contractor shall address the tax invoice to

_________________________

and include on each invoice the following information:

- Name and address of the Contractor and the Service Manager;

- The contract number and title;

- Contractor’s VAT registration number;

- The Employer’s VAT registration number 4740101508;

- Description of service provided for each item invoiced based on the Price List;

- Total amount invoiced excluding VAT, the VAT and the invoiced amount including VAT; • 	(add other as required)

Add procedures for invoice submission and payment (e. g. electronic payment instructions)

#### 2.6 Contract change management

- Any changes or requirements will be discussed and agreed by both Employer and Contractor in writing before implementation.

- All compensation events shall be submitted in writing with a full motivation and detailed cost breakdown.

#### 2.7 Records of Defined Cost to be kept by the Contractor

The contractor keeps accurate and complete books of accounts, records and other evidence relating to the actual cost. Records and accounts must reflect all work done on the contract. These are open to audit. All documentation is kept by the contractor for a period of three years following completion of this contract. This information must be kept up to date at all times.

2.8 Insurance provided by the Employer

Insurance issues will be dealt with as per contract conditions

#### 2.9 Things provided at the end of the service period for the Employer’s use

##### 2.9.1 	Information and other things

Quality documents and data packs requested by the Employer will remain the property of the Employer at the end of contract

#### 2.10 Management of work done by Task Order

- 	The Employer shall provide the applicable specifications with the works instruction of each task order.  - 	Contractor shall respond to emergencies whenever required by the Employer.

## 3 Contractor shall comply with safety, environmental and legislative requirementHealth and safety, the environment and quality assurance

### 3.1 Health and safety risk management

Suppliers to comply with the latest version of Occupational Health and Safety Act No. 85 of 1993 and Eskom Health and Safety policy, procedures, specifications and guidelines

## 4 Health and safety, the environment and quality assurance

### 4.1 Health and safety risk management

In addition to the requirements of the laws governing health and safety, Eskom may have some additional requirements particular to the service and the Affected Property for this contract.  The text below provides for these being attached as an Annexure to this Service Information. PLEASE ALSO READ CORE CLAUSE

27.4 TOGETHER WITH Z7 IN THE ADDITIONAL CONDITIONS OF CONTRACT TO MAKE SURE THAT WHATHEVER IS INCLUDED IN THE ANNEXURE FOLLOWS ON FROM THOSE CLAUSES.

The Divisional/Regional Safety Risk Manager or his representative having jurisdiction over the service must provide the relevant safety, health and environmental (SHE) criteria for incorporation into this Service Information. The SHE specification / scope must be signed off by the Divisional/Regional Safety Risk Manager or his representative confirming that the applicable safety criteria have been taken into account.

The Commodity Manager / Buyer must refer the tender to the Divisional/Regional Safety Risk Manager or his representative in order to evaluate against enquiry-specific safety criteria.

The Divisional Safety Risk Managers who will be responsible for the allocation of resources to assist P&SCM with the above processes are as follows:

·         Generation: Roley McIntyre

·         Transmission: Tony Patterson

·         Distribution: Alex Stramrood

·         Enterprises: Jace Naidoo

·         Corporate: Kerseri Pather

The Contractor shall comply with the health and safety requirements contained in Annexure ___________ to this Service Information.

### 4.2 Environmental constraints and management

The Contractor shall comply with the environmental criteria and constraints stated in the Waste management procedure (ASEN  0008) and EH&S Incident management procedure 32-95.

The Contractor shall further comply with  all Arnot procedures and policies and all level  3  documentation and legislations which the station  prescribes to

### 4.3 Quality assurance requirements

-    The returnable in terms of Quality will be based on the Supplier Quality Management Specification 240-

105658000.  The supplier will submit the quality information based on the category that is relevant  to them.  Refer to document 240-105658000.  In case where there are QCP (quality control  plans) required also 3.2 and 5.1  of the document 240-105658000 will be applicable.

Employer has a right to issue an NCR should the Contract fails to meet the agreed target date or should the Contractor be unavailable when needed

## 5 Procurement

### 5.1 People

#### 5.1.1 	Minimum requirements of people employed

Specify any constraints relating to people employed to Provide the Service; for example permits for foreigners, training (other than H & S), use of labour from designated areas and industrial relations.

#### 5.1.2 	BBBEE and preferencing scheme

Specify constraints which Contractor must comply with after contract award in regard to any Broad Based Black Economic Empowerment (B-BBEE) or preferencing scheme measures.

#### 5.1.3 	Accelerated Shared Growth Initiative – South Africa (ASGI-SA)

If the ASGI-SA requirements are to be included in this contract specify constraints which Contractor must comply with after contract award in regard to any ASGI-SA requirements.  The ASGI-SA Compliance Schedule completed in the returnable tender schedules is reproduced here.  If ASGI-SA does not apply, delete this paragraph.

The Contractor complies with and fulfils the Contractor’s obligations in respect of the Accelerated and Shared Growth Initiative - South Africa in accordance with and as provided for in the Contractor’s ASGI-SA Compliance Schedule stated below

.

[Insert the agreed ASGI-SA Compliance Schedule here]

The Contractor shall keep accurate records and provide the Service Manager with reports on the Contractor’s actual delivery against the above stated ASGI-SA criteria.   [Elaborate on access to and format of records and frequency of submission etc.]

The Contractor’s failure to comply with his ASGI-SA obligations constitutes substantial failure on the part of the Contractor to comply with his obligations under this contract.

### 5.2 Subcontracting

5.2.1 	Preferred subcontractors N/A

5.2.2 	Subcontract documentation, and assessment of subcontract tenders

N/A

5.2.3 	Limitations on subcontracting N/A

5.2.4 	Attendance on subcontractors N/A

### 5.3 Plant and Materials

#### 5.3.1 	Specifications

The contractor will be working on both Vertical spindle mills and Tube stein mills. Vertical spindle mill bodies are made of cast steel material and Tube mills components material is hard steel. The mild steel insert to be used on the damaged holes to be at lease 6mm thick.

#### 5.3.2 	Correction of defects

- In case there is a defect due to Contractor poor workmanship that may cause delays during assembling of the mill or mill feeder, Contractor will be informed and must be available within 12 hours. The defect to be repaired at no extra costs charged by the Contractor. All labour, material and travel expenses will be for the Contractor’s account.

N.B Each mill holes that has been repaired should run 1400hrs and feeder mechanism to operate 6000hrs without any defect. Should there be a defect on the repaired holes and repaired feeder mechanism while still under warranty, the Contractor will be notified and repairs to be done accordingly.

N.B Therefore, the Contractor will have report to Arnot Power station within 12 hours including non-working days (weekends and holidays)

#### 5.3.3 Contractor’s procurement of Plant and Materials

Specify any constraints on how the Contractor is to order, codify, expedite, freight, import, transport to the Affected Property and any other requirements for delivery and storage before installation.  The Employer may require warranties from suppliers to be in favour of the Employer and not just to the Contractor.   The Employer may also need schedules of vendor data for his own use after the end of the service period.

#### 5.3.4 	Tests and inspections

Approved Quality control plan and check sheets- to be used as a guideline for inspection and to ensure the quality of work. Only holes that will be repaired according to standard will be accepted.

#### 5.3.5 	Plant & Materials provided “free issue” by the Employer

| Item | Date by which it will be provided |
| --- | --- |
| Induction | As and when the need arise |
| Permit to Work | Ongoing |
| Compressed air | Ongoing |
| Scaffolding | As and when the need arise |
| Relevant technical information, specifications and drawings where available. | Ongoing |
| Electrical supply | Ongoing |

5.3.6 	Cataloguing requirements by the Contractor

N/A

## 6 Working on the Affected Property

The Power Station is situated approximately 50km from Middleburg

Altitude  (Elevation  above sea level) 	1636  m

Mean annual barometer pressure 	84, 0 kpa

Maximum ambient temperature 	38 degrees Celsius

Minimum ambient temperature 	6 degrees Celsius

Maximum relative  humidity 	79%

Minimum relative humidity 	10%

Arnot  Power  Station  is  situated  in  a summer  rainfall  area with  an average annual  precipitation  of about 650mm  falling  almost  entirely  during  the  months  of  October  to  April.    The  average  rainfall  per  month generally exceeds 40 mm during this period, although drought periods do occur which can last for 20 days or longer.  Drought  periods occur most frequently  during the  months  of October/November  and March/April. December is  statistically the highest rainfall month with  an average monthly rainfall  of about 125mm.   July has the  lowest rainfall with  an average monthly  rainfall of about 3  mm.  approximately  85% of the annual rainfall occurs in the months between October and February.   January is normally the hottest month with an average daily maximum temperature of 38c with a   minimum daily temperature in winter being about 6°c.

### 6.1 Employer’s site entry and security control, permits, and site regulations

- No taking of pictures allowed on Employer's premises without prior approval or consent

- Contractor must attend induction before entering premises

- Contractor must make pre-arrangements before coming to Employer’s site to allow Employer to make proper arrangements  with  Security.   If no arrangements made,  no  access  will  be  granted  to  the Contractor

### 6.2 People restrictions, hours of work, conduct and records

- Monday – Thursday working time: 07H00 – 16H15

- Fridays working time: 07H00 – 12H00

- Any visits outside above timelines, arrangements must be made with an Employer in advance

- N.B Holes repairs must be done on a 24hour cycle and Contractor must provide enough resources to cover 24 hours to complete the activity per mill.

### 6.3 Health and safety facilities on the Affected Property

Section 3 deals with contractual H & S requirements in addition to those of the OHSA Act.  This section allows the Employer to state what measures are to be taken on the Affected Property by describing where First Aid facilities provided by the Employer are located and any other emergency arrangements.  Do not use if already addressed in 2.3.

6.4 Environmental controls, fauna & flora

N/A

### 6.5 Cooperating with and obtaining acceptance of Others

Other contractors and Eskom Employees working on the milling plant. Communication and Prearrangements to be done.

### 6.6 Records of Contractor’s Equipment

The Contractor must declare and keep a record of all spares,  tools or equipment that must be brought to site on arrival. Failure to do so may lead to forfeiting some of your belongings.

### 6.7 Equipment provided by the Employer

N.B No equipment to be provided by the Employer. The Contractor shall provide all required equipment, hand tools and power tools to execute the work.

### 6.8 Site services and facilities

6.8.1 	Provided by the Employer Portable Water Supply

- Potable water is available at the existing points

Electrical  Power Supply

- Power is available at the existing points

Toilet Facilities

The employer provides the contractor access to existing toilet facilities. The contractor is to provide this facility should the existing facilities not be within reasonable distance from the working areas.

N.B The Employer will not assist the Contractor with arrangements of any of required tools, inserts or drilling machines.

#### 6.8.2 	Provided by the Contractor

- The Contractor must provide transportation, full protective equipment, accommodation, tools.

- The Contractor must provide daily timesheet and copy must be submitted to the Employer after every shift

- PPE to be supplied by the Contractor and must have contractor emblem for easy identification.

- The contractor to provide hand tools, power tools and consumables.

- The Contractor to provide heavy duty magnetic drill that can take up to 75mm drill bit.

- The Contractor to provide drill bits, taps, inserts, cutting fluid and all special tools that will be required to execute the work.

- The Contractor to provide electrical extension cords, lead lights and mounting plates.

- The Contractor to provide grinders, welding machine and cutting torch including oxygen and acetylene gas.

### 6.9 Control of noise, dust, water and waste

Contractor shall provide proper personal protective equipment's. No employee will be allowed on site without proper protective equipment (overalls, gloves, hard hat, safety boots/shoes, dust masks, ear protection, heat shields (if necessary), eye protection etc.)

6.10 Hook ups to existing works

-    All hook ups to existing structures in the plant shall be onto approved lifting beams and lifting points.

### 6.11 Tests and inspections

#### 6.11.1 Description of tests and inspections

Approved Quality  control  plan and check sheets- to be used as a guideline for inspection  and to ensure the quality of work

The contractor to do quality check on their own for the holes repaired before informing the Employer for verification. Should obvious defects be identified due to Contractor incompetence and poor workmanship, the Contractor will be issued with Non-Conformance Register and necessary processes to be followed.

6.11.2 Materials facilities and samples for tests and inspections  N/A

## 7 List of drawings

### 7.1 Drawings issued by the Employer

This is the list of drawings issued by the Employer at or before the Contract Date and which apply to this contract- N/A

| Drawing number | Revision | Title |
| --- | --- | --- |