1 x Horizontal Laminar Flow Cabinet with Support stand (Caster wheels)

- External Dimensions (Width x Depth x Height)

Without base – 1340 x 795 x 1118mm

With 711mm base stand – 1340 x 795x 1829mm

- Internal Dimensions (Width x Depth x Height) 1203 x 631 x 573mm

- Antimicrobial Powder Coating

- ISO 14644-1 Class 3 Work zone

- ULPA H14 filter with 99,99% efficiency at 0,3µm

- Low noise emission of 55.8 dBA

- UV lamp(253,7nm), front cover

- Fluorescent lamp intensity: 1139 Lux

- Sentinel Microprocessor Controller, audible and visual alarms

- Electrical requirement: 220-240V, AC 50/60Hz

- Easy to clean work surface, Abrasion and Corrosion resistant