Specifications for Roof waterproofing at Pot House Building

Specification for Roof repair and painting at the following buildings

Background

This specification is for the repairing and painting of the existing roof. This exercise is meant to improve and maintain the ageing buildings.

Repair leaking roof and paint (Charcoal)

| Building | Work description | Square meter (m2) |
| --- | --- | --- |
| First part | Waterproof and paint | 192 |
|  | Remove the existing sheets Replacing the damaged timber wood of the same size SABS untreated 38x38, 11 lines Reinstated the sink sheets back as they still in good condition | 130 |
| Second part | Waterproof and paint | 192 |

Interior building

| Building | Work description | Square meter(m2) |
| --- | --- | --- |
| Ablution | -Replace ceiling and paint in white two coat | 34m2 |
| Storeroom for Soil samples | Replace ceiling and paint ceiling with white colour two coat | 50m2 |
| Passage | Replace ceiling paint ceiling with white colour |  |

Scope of work

- Sand down rusted areas, replace loose nails

- Prepare the roof surface thoroughly with mechanical brushes to remove loose paint, existing waterproofing as much as possible, dirt and rust.

- Apply Daracryl DC4 primer onto total roof surface to eliminate existing rust/ prevent future rust.

- Replace loose and lost screws with possidrives.

- Supply and fit pop rivets in between roof screws.

- Waterproof all joints, screws, flashing and ridges with acrylic waterproofing emulsion in bedded with polyester membrane (one coat under and one coat on top of membrane)

- Supply and install a new PVC gutters and fascia body

- Supply 6x3m downpipe

- Apply two coats super-roof coating paint onto total roof surface

- NOTE: Corrugated Iron roof sheets must be replaced if it has corroded beyond repair

Construction Industry Development Board

1GB /1SN grading

Guarantee

10 years guarantee on painting and workmanship from the date of completion

References

At least three proof of reference letters that indicate you have successful completed similar work. The letter should be on a letterhead of the company where services were provided with the value of work completed.

Site Briefing

Compulsory site briefing so that the buildings can be assessed

Safety

Contractor will provide all PPE, ladder and scaffolding for their workers as per the required when a person works in an elevated position. Work will be stopped in case proper protection equipment are not find with the workers and lapse of the time will at the part of the contractor.

-Contractor will make sure that all the arrangements for the work are in working conditions are safe in all respect, under no circumstance ARC will be held responsible for any incident happen on the site due to negligence of the contractor/ worker.