CCTV SPECIFICATIONS AND REPAIR SPECIFICATIONS

Agricultural Research Council at KwaMhlanga / Moloto Road R573, Roodeplaat, Pretoria has installed Dahua CCTV and wishes to do maintenance on the existing system. Prior to quoting all service providers are requested to attend a site briefing and get finer detail of the systems and related environment.

DETAILS OF REQUIREMENTS

CAMERA & HOUSING

For the system’s camera and housing, verify the following:

Camera/lens focus and auto iris is adjusted properly.

Camera field of view is adjusted to customer’s requirements.

Camera/housing viewing window is clean, inside and out.

Camera lens is dust free.

Interior of camera enclosure is clean and dry.

Check operation of pan tilt and zoom focus. Use controller in control room to check all these operations.

WIRING & CABLES

For the system’s wiring and cables, verify the following:

Check wiring and cable harnesses for wear and fray.

Check to make sure cable is dressed properly.

Check connectors and cable entry points for loose wiring.

Check that the coaxial cable is transmitting an adequate video signal to control room. Signal should be free of distortion

Make sure all coaxial connectors are insulated from conduit and pull boxes.

CONTROL EQUIPMENT

For the system’s control equipment, verify the following:

Monitors are free from picture burn-in and distortion.

Monitors have proper contrast and brightness.

NVR is are functioning properly and providing distortion free recording.

Check that all control equipment is operational. Clean all monitor screens, control panels, and keyboards with a diluted cleaning solution.

Check all power connections to ensure AC plugs are not loose.

PLANT HEALTH PROTECTION

| AREA | ITEM DESCRIPTION | QUANTITY |
| --- | --- | --- |
| VOP Control Room Network Video Recorder | - Check NVR,Clean Monitors - Label NVR | 4 |
| VOP Control Room Wiring cables | - Repair and reconnect network cables - Clean Network cabinet | 1 |
| PHP Camera | - Clean,adjust,check operation of camera - Label cameras - Replace faulty camera (Biosystematic Wing) | 89 |
| PHP Camera Wiring & Cables | - Check connectors, loose wiring - Supply and install (500m Cat 6 Copper cable) | 01 |
| VOP Control room Power | - Check all power connections to ensure AC plugs are not loose - Link NVR to small UPS |  |

GUARANTEES

- The supplier shall stipulate the guarantee of the CCTV System according to ISO and SANS standards or any South African CCTV System related regulations or standards.

- The supplier shall in detail document what is covered by the guarantee as well as the terms and conditions.

- The supplier shall in detail document on what will constitutes a “system failure”. In addition, what will be Response plan in the event of system failure after implementation (i.e. while the system is still on guarantee).