Specifications for ARC / AE  -  Audit & Commissioning of AV Systems

(RFQ-012884)

Specifications for ARC / AE  -  Audit & Commissioning of AV Systems

- Repairing of the audio-visual equipment in the Indaba conference room, AE Boardroom and Kopano Hall:

Projectors are not connected and the lights in the Kopano hall are not working by switching in the  three phases front, middle and back, etc.

The services of a specialized audio-visual service provider are required to repair the technology before anybody will be able to use the venues.

KOPANO HALL

| Quantity | Description |
| --- | --- |
|  | Projection System |
| 1 | Install (Existing) Epson EB-1980 projector - Ceiling bracket & Cable interfacing & Control - RS232, POE LAN, 3.5 Audio, USB, HDMI, VGA, Mains - Control software programming (Software principle) |
|  | Sound & Speaker System |
| 6 | CS-8220 Stereo coaxial ceiling speaker, 20W, 8” Two-way, Full range - White metal grill |
| 2 | Passive stereo wall mount cabinet speaker – Cable interfacing |
| 1 | Commissioning & Reset of 4 microphone sound system & PC Sound - Wireless system UHF handheld microphone - Goose neck podium microphone - AA batteries - Control software programming (Software principle) |
|  | AV Control System (Principal – User interface license) |
| 1 | Projector, Sound, Light control gateway (RS232/IR/GPIO/Ethernet) - Projector control protocol: On / Off / Source (RS232) - User control interface & Android tablet/phone - Rackmount kit & Port extenders - Venue lights control box - 4 Channel Light control (Control NET) |
| 1 | Relay Box - 4 Channel Light control – 3 Rows Fluorescents (Software principle) |
| 1 | Electrical installation executed by qualified Electrical Contractor - Issue COC with project invoicing |
| 3 | Florescent lumina light bank - 3 Setting: venue clear, projection, house - Surfix cable mains distribution – 3 circuits - Contactor switching & relays - Manual venue “Cleaner” switch - Power supply |
| 1 | Reconnect Wi-Fi Wireless presenter system (VGA & HDMI) |

MAIN BOARDROOM

| Quantity | Description |
| --- | --- |
|  | Projection System |
| 1 | Supply: LCD Laser data projector, 3500 Lumens, WXGA (1280x800) - 5 Year guarantee - RS 232 Control - 2 000 000:1 Contrast Ratio - 1x HDMI, 1x VGA, Audio In/Out (Variable), RS232 - Lamp - 20 000 Hours Rated - 3.5kg |
| 1 | Ceiling bracket & Cable interfacing, RS232, 3.5 Audio, USB, HDMI, VGA |
| 1 | Projector (AV System) control gateway (RS232/IR/GPIO/Ethernet) - Projector control protocol: On / Off / Source (RS232) - User control interface - Rackmount kit & Port extenders |
| 1 | AV Control user interface license and software |
| 1 | Installation of Projector AV Control, Bracket & Cabling |
|  | AV Connection |
| 1 | Digital Interactive Board connection, USB with Active extension |
| 1 | M-HDMI (4K v2.4) Flexible Hi-speed Male-Male molded cable, 10m |
| 2 | M-HDMI (4K v2.4) Flexible Hi-speed Male-Male molded cable, 5m |
| 2 | M-HDMI (4K v2.4) Flexible Hi-speed Male-Male molded cable, 1,8m |
| 1 | HDMI Computer Graphics automatic video switcher, 4K, 2 Way |
| 1 | HDMI Computer Graphics automatic video distribution amplifier, 4K, 2 Way |
|  | DSTV Front-end |
| 20 | RF Antenna RG-6 Coaxial cable (per m) |
| 1 | HDMI HD De-modulator, digital RF to Analogue RF network distribution |
| 1 | Installation cabling & Commissioning of system - Decoder, Dish, Codec & Cabling (F-Connectors) also to the KOPANO HALL |

INDABA BOARDROOM

| Quantity | Description | UNIT |
| --- | --- | --- |
|  | Projection System |  |
| 1 | Relocate: Installation of Projector, Bracket, Cabling (RS232 Control) - NEC NP-M260WG projector, LCD, 2600 Lumen, HDMI, VGA | (Existing) |
| 1 | Ceiling bracket & Cable interfacing, RS232, 3.5 Audio, USB, HDMI, VGA |  |
| 1 | M-HDMI (4K v2.4) Flexible Hi-speed Male-Male molded cable, 10m |  |
| 1 | Installation: 2,1m Pull down projection screen | (Existing) |
| 1 | Installation of Projector, Bracket & Cabling |  |
|  | AV Control |  |
| 1 | Projector control gateway (RS232/IR/GPIO/Ethernet) - Projector control: On / Off / Source (RS232) - User control interface - Rackmount kit & Port extenders |  |
| 1 | AV Control user interface license and software |  |
| 1 | Mylar foiled coaxial 0,5mm 1pr cable |  |
| 1 | Installation of AV Control gateway system |  |
|  | Sound system |  |
| 2 | 30W Cabinet wall mount speakers, 30-Watt, 6" Full range |  |
| 1 | Stereo amplifier, 3 Line input switching, 120W, 8Ώ, Rack mountable |  |
| 1 | 1,5mm 1pr Speaker cable |  |
| 1 | Installation cabling & Commissioning of system |  |

2.	Compulsory requirements

- Compulsory Site meeting -17 March 2022 at 11:00

- Workmanship and supply equipment warranty of one year

3.	Additional Information

- The above-specified services should be rendered at:

Name of Institute     : ARC-IAE

Address                   : 141 Creswell Road

: Weavind Park

: Pretoria

- The particulars of the guarantee that will apply to the goods quoted for,

with particular regard to the period and extent of the warranty must be

clearly stated. Where services are required, service providers must submit

documentation pertaining the relevant experience.

- Your written quotation should be sent to:

Lungile Kubheka

Tel: 012 842 4078

Fax: 012 8040863

E Mail: KubhekaL@arc.agric.za

- All price quotations that have a rand value of R30,000-00 to R50 000 000.00

including VAT, will be evaluated by applying the 80/20 principle as

prescribed by the Preferential Procurement Policy Framework Act 5 of

2000 and its Regulations. The lowest acceptable price will score

80 points, the 20 BBBEE points will be allocated as follows:

| B-BBEE Status Level of Contributor | 80/20 |
| --- | --- |
| 1 | 20 |
| 2 | 18 |
| 3 | 14 |
| 4 | 12 |
| 5 | 8 |
| 6 | 6 |
| 7 | 4 |
| 8 | 2 |
| Non-compliant Contributor | 0 |

- Standard conditions:

- The validity of the quotations must be indicated.

- Prices quoted should be in South African Rand and inclusive of VAT costs

such as delivery, insurance, taxes, etc.

- No price adjustments or amendment of the delivery particulars contained

in paragraph 2 will be considered by the ARC.

- The supplier accepts full responsibility for the proper execution and

fulfilment of the goods/services quoted for.

- ARC reserves the right to accept or reject any special terms and

conditions that may qualify the goods/services to be provided.

- Quotes should be submitted on an official letterhead and duly signed

- Goods and services should be supplied/rendered upon receipt of a

purchase order from the ARC

- The General Conditions of Contract issued by National Treasury are

applicable.

- The ARC supply chain management code of conduct is applicable.

- SBD Forms must be signed and returned together with thefailure to    comply will result to disqualification of your quotation.

- Your quotation must indicate the delivery date.

- The ARC reserve the right to do due diligence on the quotations.

- The ARC reserve the right to benchmark prices quoted.

Compulsory requirements

- Valid tax PIN number from SARS.

- Central Supplier Database report (Current and updated CSD report) showing a tax compliant status for the duration of the bidding process.

- Complete and sign the Standard Bidding Documents (SBD forms)

Thank you in anticipation

Supply Chain Management: ARC