Scope of work

The provision of hiring mobile cranes in the North West Operating unit.

- Description of the service

1 Legal requirement - Refer to Driven Machinery Regulations No. 18.

2 DMR 18(5) Lifting machines shall be tested at intervals not exceeding 12 months by a competent person. The test shall include mechanical test and material fatigue (cracks)test.

3 DMR 18 (6) The user shall cause all ropes, chains, hooks or other attaching devices,

sheaves, brakes and safety devices, forming an integral part of a lifting machine to be

thoroughly examined by a competent person at intervals not exceeding six months.

4 DMR 18 (e) No user shall use or allow the use of any lifting tackle unless the following

conditions are complied with, namely -such lifting tackle is examined at intervals not

exceeding three months by a competent person who shall enter and sign the result of

each such inspection in a book kept for his purpose.

5 DMR 18 (II) The user shall ensure that every lifting machine is operated by an operator

specifically trained for a particular type of lifting machine; provided that in the case of lift

trucks with a lifting capacity of 5 000 kg or more a minimum jib-radius, the user shall not require or permit any person to operate such a lifting machine unless the operator is in possession of a certificate of training, issued by a person or organisation approved for the purpose by the Chief

Inspector.

6 Valid certificates of accredited training for operators of  cranes

7 Users of any other lifting machines shall be adequately trained in the use of those

machines.

8 All registers and test certificates shall be filed under this element.

9 Lifting machines shall be tested as prescribed by the standard to which the machine was

manufactured, or 110 % of the rated mass load. The safe working load shall be displayed

on all lifting tackle and lifting machines.

10 An updated list of all lifting equipment shall be kept on this file for ease of reference.

11 All current test certificates shall be filed under this element. Where possible, these shall

be filed with the inspection record sheet.

12 list of mobile crane to which form part of hiring of mobile cranes and earth moving plant in the north west operating unit are

- 20 tone

- 30 tone

- 50 tone

- 80 tone

- 150 tone

- 220 tone

- 440 tone

- 550 tone

- 750 tone

- Included in the hour rate: (eg. 20  Ton crane description –include the consumables and any other necessities that go with the required activity/ service) .

- Rigging team:(NWOU authorised person in the team Outcome 3).

- Site establishment according to this request? (Site establishment to be paid for a project that is longer the 5 days).

- Insurance  cost per hour

- Crane Operator cost per hour

The contractor will work according to Eskom maintenance, operating, safety and environmental standards, business rules, standards and specifications and will take full responsibility for all work.

Eskom will regularly evaluate the quality of work and compliance to standards and regulations, non-conformances must be rectified within an within 24hrs  or as per instruction or as agreed with project manager .  Repeat findings will not be tolerated and possible annulling of the contract might occur.

Contractor operators to be in possession of a valid operating authorisation for the n Equipment they will be operating. This is essential in order to meet set KPI’s as a result of vast distances.

Response Times

The Contractor must acknowledgement request within 15 minutes to work orders dispatched by Eskom.

Ensure that restoration KPI`s of the CNC are met, if this could not be achieved, the contractor should provide acceptable reasons for non-achievement.

Must have an emergency plan for instances where they do not have enough resources to comply with the required response. This plan must be approved by the contract  Manager or his delegate on an annual basis.

COMMUNICATION

The personal protective equipment and vehicles of the contractor must be marked in a way that he is identifiable as a lawful operator on Eskom`s behalf. Identification cards of the contractor and all contractor employees. Ensure to inform Eskom customer, house or land owner of intention to enter premises for work or inspections. Ensure necessary respect and communication to enhance Eskom`s image.