TO: WHOM IT MAY CONCERN

SCOPE OF WORKS: SUPPLY AND INSTALLATION OF HANDRAILS AT C1 CONSTANTIABERG MAST STRUCTURE

- SCOPE OF WORKS

  - Remove existing worn out hand rails

  - Install new hand rails.

  - Handrails must be galvanised and Paint - The recommended paint coating system is the 3 coat 100% Epoxy coating system consisting of:

- PRIMER COAT

- INTERMEDIATE COAT

- FINAL COAT (Signal Red or White)

- WORK SPECIFICATION AND METHOD STATEMENT

2.1 REMOVE EXISTING HANDRAILS

- Loosen and remove existing worn out handrails. Discard everything from the site to a designated site to be supplied by Sentech representative.

2.2	INSTALL NEW HANDRAILS

- The handrails on the main platform and on the other levels that need to be changed, must be done per level so that the safety of staff is not compromised.

- The traditional round bar handrail will be changed to angle iron.

Main platform handrail

- The handrail on the main platform is (-+) approximately 6.050m x 6.050m square in length.

- The height will be 1.250m from the attachment point and 500mm top horizontal to horizontal.

- 16 vertical poles (3 / 5 /3 / 5)

Handrails on levels

- Handrails at levels must be one-piece handrails (welded), not being bolted together.

- The height must be 1150mm from anchor point, 2170mm in length, vertical anchor points 1560mm apart and the two horizontals 460mm apart.

- 24 handrails must be manufactured for these levels.

- All bolts, washers and nuts must be stainless steel 304.

- The thickness of the angle iron must not be less than 5mm x 50mm x 50mm.