The scope of works entails the renovation of the Rietkuil Single Quarters kitchen to meet compliance requirements and attend to general maintenance issues. The required repairs include repair of damaged/leaking roofs, replacement of damaged entrance/exit doors, repairs of cold room doors, repair of ventilation/extraction unit and kitchen and relocation of gas tank with cage.

All the repair works and installations of the new components shall include all fittings and all necessary ancillary items.

# Description of the works

  - Dining hall

    - Design and install/replace the new roof structure including drainage systems on the roof :

      - Dismantle existing roof structure by removal of the following components;

        - Top roof tiles - Marley Double Roman concrete type tiles– coverage 91m2

        - Membrane/coating waterproofing, 30m perimeter

        - Old  fascia boards, 40m perimeter

        - Old Steel galvanized gutters, 30m perimeter

        - Old cast iron downpipe, 110mm dia, 3,5m long

        - Remove and terminate/reroute electrical cables inside the roof

        - Dismantle and remove existing temporary support scaffolding structure; 6 off frames;

          - 3m long standards; 6 off

          - 2m long ledgers; 9 off

          - 1.5m short ledgers; 12 off

          - 3m long ladder beams; 8 off

      - Remove standard dry walling of 6mm board that enclose the dining hall; 10m perimeter, 3m high

      - Install temporary covering during construction period to cover the opening

      - Design new roof structure

        - Assess the existing roof/wall layout and design a Monoplanar prefabricated timber roof structure as per SANS 1900 and SANS 2001-CT2 standard to match existing layout.

        - Wall layout of 15m length by 8m width

      - Reconstruct roof structure

        - Fabricate and install new roof structure including roof trusses, waterproofing, roof tiles, flashing, valleys, gutters, downpipes and fascia.

        - Roof tile, gutter and downpipe to match existing

        - New roof to cover 91m2

        - Reinstate water supply pipes in the roof as existing

        - Reinstate electrical cables in the roof as existing

        - Paint gutters, downpipes, fascia and make good. Match existing. With acrylic paint

      - Supply and install new ceiling (91m2)

        - Supply and install 4,6mm fibre-cement plain ceiling boards

        - Paint the ceiling and make good the area.

          - Painted with Acrylic PVA white paint

        - Install standard Polystyrene cornices on the ceiling edge perimeter (39m).

          - Upper Edge Z60 Polystyrene Cornice

        - Supply and install new LBP-24-S-60W-LED lights

on the dining hall ceiling (x7)

        - Remove old light switches and install new ones

    - All interior walls in the dining hall shall be painted (115m2)

      - Grecian white paint colour

      - Fill and repair all cracks on walls before painting

    - Floor tiling

      - Remove existing ceramic tiles (91m2)

        - Scrape residual adhesive off the floor, clean and prepare the floor for new floor tile installation

      - New floor tile installation (39m2)

        - Supply and install ceramic floor tiles

          - Light grey in colour, 330 mm x 330 mm

      - Reinstate all fixed tables and benches in the dining hall

      - Install new timber skirting after tiling to match existing (39m)

    - Replace wall mounted heaters in dining hall

      - Remove existing old heaters from wall. Close wall with bricks and plaster and paint. Install wall-mounted heater and provide power source if not existing already.

  - Dining patio

    - Reinstate the outdoor open roof

      - Dismantle existing roof structure (32m2)

        - Remove all roof tiles, waterproofing, flashing, fascia, gutters and downpipes of affected roof area

        - Make good the roof drainage

          - Reinstate all gutters and downpipes

      - Replace the missing drain cover with new square dish grid inlet of size 600x600 to fit

  - Kitchen area roof and ceiling

    - Roof leaks repairs

      - Identify all leaking areas

      - Identify all damaged gutters and downpipes and replace with galvanised steel gutters and downpipes.

      - Install waterproofing at identified leaking areas

    - Replace affected ceiling

      - Replace water damaged ceiling

      - Paint the ceiling with Acrylic PVA and make good the area.

    - Replace asbestos ceiling in the kitchen (131m2)

      - Replace with flush plaster Rhino-board ceiling and plaster with Rhino-lite plaster

        - Paint the ceiling and make good the area.

        - Painted with Acrylic PVA white paint

      - Supply and install new LBP-24-S-60W-LED lights on the kitchen hall ceiling (x12)

      - Remove old light switches and install new ones

      - Install standard Polystyrene cornices on the ceiling edge perimeter (66m).

        - Upper Edge Z60 Polystyrene Cornice

  - Swirl room

    - Re-design and install the new roof structure

      - Dismantle existing roof structure (28m2)

        - Remove all roof tiles, waterproofing, flashing, fascia, gutters and downpipes of affected roof area

      - Design new roof structure

        - Assess roof/wall layout and design appropriate roof structure as per SANS 1900 and SANS 2001-CT2 standard to match existing.

      - Reconstruct roof structure

        - Install new roof structure including roof trusses, waterproofing, roof tiles, flashing, valleys, gutters, downpipes and fascia. Match existing.

        - Roof tile, gutter and downpipe to match existing (galvanised)

        - New roof to cover 28m2

      - Supply and install new ceiling (28m2)

        - Supply and install 4,6mm fibre-cement plain ceiling boards

        - Paint the ceiling and make good the area.

          - Painted with Acrylic PVA white paint

        - Supply and install new LBP-24-S-60W-LED lights on the swirl room ceiling (x2)

        - Remove old light switches and install new ones

        - Install standard Polystyrene cornices on the ceiling edge perimeter (21m).

          - Upper Edge Z60 Polystyrene Cornice

    - Replace the existing gates – entrance & exit

      - Remove old entrance and exit gates

        - Single gate Clear view lockable hinged gate for entrance

        - Double gate Clear view lockable hinged gate for exit

  - Cold room freezer

    - House the cold room unit with brickwork structure

      - Repair existing foundation brickwork on the foundation

      - Build a double wall masonry enclosing the cold room freezer up to a height of 2.1m

        - Cold room freezer enclosure dimensions

          - Length 4.6m

          - Width 3.40m

      - Supply and install service door Clear view on the eastern side of the enclosure

      - New sloped IBR roof with gutters and downpipe

        - Design appropriate IBR roof covering for cold room as per concept sketch below

        - Supply and install IBR roof sheet (0.8mm)

        - Supply and install isotherm insulation below the IBR roof sheet

          - Thickness 40mm

          - Density 8.5kg/m3

          - Size 16m2

      - Supply and install gutters in the front of the enclosure (3.4m). Paint to match existing with acrylic paint

      - Supply and install 2x downpipes in the front of the enclosure, one on each corner to drain into the drain inlet. Paint to match existing with acrylic paint

Slope direction

Slope direction

    - Install new heavy-duty door gasket (x 2, for exterior and interior doors)

      - Supply and install cold room large door heavy duty gasket around all sides of the door

        - Gasket spec: Height 25mm – Width 35mm, to suit door of 92mm thickness

        - Door height 2.1m; Door width 1m

    - Repair door hinges

      - Repair or replace loose cold room door hinges. If replacing, replace with matching existing spec or similar.

  - Vegetables cold room

    - Install new heavy-duty door gasket

      - Supply and install cold room large door heavy duty gasket around all sides of the door

        - Gasket spec: Height 25mm – Width 35mm, to suit door of 92mm thickness

        - Door height 2.1m; Door width 1m

    - Repair door hinges

      - Repair or replace loose cold room door hinges. If replacing, replace with matching existing spec or similar.

  - Kitchen equipment

    - Relocating the gas cage (as per SANS 10087-1)

      - Construct and install a lockable gas cylinder storage cage to house 2x 19kg gas tanks on the eastern side of the kitchen as per SANS 10087-1

      - Reroute gas supply pipe from gas tank on the eastern side of the kitchen. Sleeve tube in stainless steel conduit as per SANS 10087-1

    - Fat trap

      - Supply and install an underground grease trap with phase separation and a solid strainer basket.

      - Stainless Steel 304 material frame and lid to ensure acid and stain resistance.

      - Assess, size and install new fat trap for the kitchen drains. Retrofit into existing.

      - Replace rusted cast iron drain pipes with 60mm diameter PVC pipes

    - Window screen

      - Install adjustable magnetic mesh window fly screen on all kitchen windows sized 2mx1.2m

  - Kitchen floor drains

    - Replace rusted floor drain gratings with stainless steel drain gratings to match existing size and design

      - Length 9m

      - Width 0.25m

  - General maintenance

    - Entrance/exit door frame and doors

      - Replace rusted door frames and door with aluminium frame and glass doors

        - Double glass door with aluminium frame

        - Length 2.4m; Width 1.8m

        - Supply and install automatic door closer

    - Repair floor area where tiling has been repaired with plaster. Remove plaster and install floor tiling (2m2) to match existing tile

    - Repair shower walls by finishing off with cladding and installing wall tiles (1m2)  to match existing wall tiles