# Part 3: Scope of Work

| Document reference | Title | No of pages |
| --- | --- | --- |
|  | This cover page | 1 |
| C3.1 | Purchaser’s Goods Information | 8 |
| C3.2 | Supplier’s Goods Information (insert at award stage or delete if not applicable) | 0 |
|  | Total number of pages |  |

# C3.1: Purchaser’s Goods Information

Table of Contents

Part 3: Scope of Work	1

C3.1: Purchaser’s Goods Information	2

1	Overview and purpose of the goods and services	3

2	Specification and description of the goods	3 - 5

2.1	Work to be performed by the contractor for the works	.6

2.2	Contract Deliverables	6

2.3	Procedure for submission and acceptance of Supplier’s design	6

3	Supply Requirements	6

4	Constraints on how the Supplier Provides the Goods	6

4.1	Programming constraints	6

4.2	Work to be done by the Delivery Date	6

4.3	Marking the goods	6

4.4	Constraints at the delivery place and place of use	6

4.5	Services & other things to be provided by the Purchaser or Supplier	6

4.6	Management meetings	7

4.7	Documentation control	7

4.8	Health and safety risk management	7

4.9	Environmental constraints and management	8

4.10	Quality	8

4.11	Invoicing and payment	8

4.12	Insurance provided by the Purchaser	9

4.13	Records of Defined Cost, payments & assessments of compensation events to be kept by the Supplier	9

5	Procurement	9

5.1	Subcontracting	9

5.1.1	Preferred subcontractors	9

# Overview and purpose of the goods and services

The supply and delivery of various Turbine Turbovisory equipment and field Instrumentation as per scope of work at Grootvlei Power Station for as an when required for a period of 5 years.

# Specification and description of the goods

Detailed specifications of the goods are as follows:

| Material | UoM | Description |
| --- | --- | --- |
| 0234191 | EA | SENSOR: TYPE: SPEED; RATING: 5 VDC; SUPPL P/N: INT-SSR-250-750BW; PULSE OUTPUT, FOR USE ON COAL PLANT; RANGE: 0-6 MILLISECOND |
| 0240306 | EA | TRANSMITTER, TEMPERATURE: REFERENCE NO: TMT 121-A1KCB-5; FOR DIN RAIL TURBINE PLANT; SUPPLY: 24 VOLT DIRECT CURRENT; OUTPUT: 4-20 MILLIAMPERE |
| 0240307 | EA | PUSH BUTTON: TYPE: EMERGENCY STOP; REFERENCE NO: SZT-42-92130-00023; MATERIAL PLASTIC, FOR USE ON TURBINE PLANT; COLOR: RED |
| 0240309 | EA | AMPLIFIER: TYPE: ISOLATING; REFERENCE NO: 2864723; 3 WAY; TYPE - MINI MCR-SL-1-1-SP, INSTRUMENTATION/FIELD EQUIPMENT, REQUIRED FOR TURBINE PLANT |
| 0242880 | EA | AMPLIFIER: TYPE: SIGNAL; INPUT: 15 KHZ; OUTPUT: 24 V; POWER: 1.2 W; POTENTIAL: 24 V; CURRENT: 50 MA; SUPPL P/N: ISV301/01-36-018;FOR C AND I; PULSE 3 CHANNELS |
| 0244132 | EA | NO OBJECT: TRANSMITTER, PRESSURE; RANGE 0-600 KPA, OUTPUT 4-20 MA, SUPPLY 24 VDC, CONNECTION BSP 1/2 IN, REQUIRED FOR INSTRUMENTATION FIELD EQU IPMENT, TYPE CEREBAR S PMC71, SERIAL NO: PMC71-AAA1PBGAABA-1~TRANSMITTER,PRESS;0-600 KPA,4-20 MA |
| 0244167 | EA | RELAY: TYPE: DOUBLE CONTACT; COIL VOLTAGE: 24 VDC; CONTACT RATING: 24 VDC; TERMINAL: 8 PIN; MOUNT: DIN RAIL; ENCLOSURE RATING:PLASTIC; CONTACT MATERIAL: SILVER PLTD; MODEL NO: EMG10-REL/KSR-G 24/1-LC AU; REFERENCE NO: 2940087; REQUIRED FOR USE ON TURBINE PLANT |
| 0244168 | EA | TRANSDUCER: TYPE: INDUCTIVE DISPLACEMENT; INPUT: 4-20 MA; POWER SOURCE: 24 VDC; APPLICATION: TURBINE PLANT; REFERENCE NO:PR9350/02; TYPE K20315/03; IN PROTECTIVE HOUSING |
| 0244169 | EA | TRANSDUCER: TYPE: ELECTRODYNAMIC VELOCITY SENSOR; INPUT: 4-1000 HZ; OUTPUT: 28.5 MV/MIL; POWER SOURCE: 24 VDC; APPLICATION: TURBINE PLANT; SPECIFICATION: EN50014; OEM P/N: PR9268/200-000; REFERENCE NO: 9200-00071; ELECTRO DYNAMIC ABSOLUTE VIBRATION |
| 0244170 | EA | SWITCH, PROXIMITY: TYPE: DISPLACEMENT; SENSING RANGE: 50-500 UM; POTENTIAL: 24 VDC; APPLICATION: TURBINE PLANT; MODEL NO:PR6423/000-100; REFERENCE NO: 9210-00077; EDDY CURRENT |
| 0244194 | EA | TRANSMITTER: OUTPUT: 4-20 MA; SUPPLY: 24 VDC; APPLICATION: PROCESSING OF LOAD CELLS; MODEL NO: PR5210/01; REFERENCE NO: SZ50013923; RANGE: 0-36 MICRO METER |
| 0244241 | EA | TRANSMITTER, PRESSURE: RANGE: 0-350 KPA; OUTPUT: 4-20 MA; SUPPLY: 24 VDC; CONNECTION: G1/2 IN; DRAWING NO: PMC71 REV 1; FOR C AND I, CEREBARS |
| 0244242 | EA | TRANSMITTER, PRESSURE: RANGE: 0-160/250 KPA; OUTPUT: 4-20 MA; SUPPLY: 24 VAC/DC; CONNECTION: G1/2 IN; APPLICATION: C/I CEREBARS;DRAWING NO: PM C71-AAASMBGAAPA-1 REV 1 |
| 0244245 | EA | SWITCH: TYPE: POTENTIAL; POTENTIAL: 24 VDC; CONTACT ARRANGEMENT: 2NO; MOUNT: SURFACE; APPLICATION: C/I BARRING GEAR POSITION;DRAWING NO: 3SE2100-1E REV 1 |
| 0244759 | EA | CONVERTER, SIGNAL: TYPE: EDDY CURRENT; INPUT: -4 TO -20 V; OUTPUT: -18 TO -2 V; APPLICATION: VIBRATION MONITORING; SPECIFICATION:PR6423; SUPPL P/N: 9200-00001; REFERENCE NO: CON 011; MATERIAL 28CRMOV4-9; STANDARD FACTORY CALIBRATED |
| 0245173 | EA | MODULE: TYPE: DUAL CHANNEL KEY MONITOR; INPUT: 0-27.3 VDC; OUTPUT: 0-5 V; POWER SOURCE: 24 VDC; APPLICATION: TURBINE PLANT;STANDARD: DIN414494; SUPPL P/N: 9100-00004; REFERENCE NO: MMS 6310; DIMENSIONS: WD 30 X LG 160 X HT 128.4 MILLIMETER |
| 0245174 | EA | MONITOR: TYPE: SHAFT RELATIVE VIBRATION; RANGE: 0 TO -22 VDC; POWER SOURCE: 24 VDC; SIZE: WD 30 X LG 160 X HT 128.4 MM;SPECIFICATION: DIN414494; REFERENCE NO: A6110; REQUIRED FOR USE ON TURBINE PLANT, SHAFT, MMS 6110 |
| 0245175 | EA | MONITOR: TYPE: CASE SEISMIC VIBRATION; RANGE: -5 TO 15 VDC; POWER SOURCE: 24 VDC; SIZE: WD 30 X LG 160 X HT 128.4 MM;SPECIFICATION: DIN41494; REFERENCE NO: A6120; REQUIRED FOR TURBINE PLANT, BEARING, MMS 6120 |
| 0245176 | EA | MONITOR: TYPE: DIFFERENTIAL EXPANSION MONITOR; RANGE: 0 TO -22 VDC; POWER SOURCE: 24 VDC; SIZE: WD 30 X LG 160 X HT 128.4 MM;SPECIFICATION: DIN414494; SUPPL P/N: A6210; REQUIRED FOR TURBINE PLANT, MMS 6210 |
| 0245177 | EA | AMPLIFIER: TYPE: CASE EXPANSION; INPUT: 3.6 VDC; OUTPUT: 4-20 MA; POWER: 6 W; POTENTIAL: 24 VDC; CURRENT: 250 MA; REFERENCE NO:A6410; REQUIRED FOR USE ON TURBINE PLANT, MEASURING INDUCTIVE DISPLACEMENT SENSORS, MMS 6410 |
| 0251928 | EA | THERMOCOUPLE: SHEATH DIAMETER: 6 MM; DESIGN LENGTH: 375 MM; SHEATH MATERIAL: SS GR 316; HEAD: NONE; SUPPL P/N: 61-A1K-375;REFERENCE NO: SZ50003858; REQUIRED FOR USE ON L.P EXHAUST STEAM, ME ASURING INSERT, SPRING LOADED |
| 0251929 | EA | THERMOCOUPLE: SHEATH DIAMETER: 6 MM; DESIGN LENGTH: 405 MM; SHEATH MATERIAL: SS GR 316; HEAD: NONE; SUPPL P/N: 61-A1K-405;REFERENCE NO: SZ50008496; REQUIRED FOR HP TURBINE CASING BOLT SHANK , MEASURING INSERT, SPRING LOADED |
| 0251930 | EA | THERMOCOUPLE: SHEATH DIAMETER: 6 MM; DESIGN LENGTH: 435 MM; SHEATH MATERIAL: SS GR 316; HEAD: NONE; SUPPL P/N: 61-A1K-435;REFERENCE NO: SZ50013958; REQUIRED FOR LP STEAM CHEST INNER WALL, MEASURING INSERT, SPRING LOADED |
| 0251931 | EA | THERMOCOUPLE: SHEATH DIAMETER: 6 MM; DESIGN LENGTH: 525 MM; SHEATH MATERIAL: SS GR 316; HEAD: NONE; SUPPL P/N: 61-A1K-525;REFERENCE NO: SZ50003636; REQUIRED FOR HP TURBINE OUTER CASING FLAN GE INNER TEMPERATURE, MEASURING INSERT, SPRING LOADED |
| 0251932 | EA | THERMOCOUPLE: SHEATH DIAMETER: 6 MM; DESIGN LENGTH: 405 MM; SHEATH MATERIAL: SS GR 316; HEAD: NONE; SUPPL P/N: 61-A2K-405;REFERENCE NO: SZ50013976; REQUIRED FOR HP TURBINE CASING INNER WALL, MEASURING INSERT, SPRING LOADED |
| 0251933 | EA | THERMOCOUPLE: SHEATH MATERIAL: SS GR 316; HEAD: YES; MODEL NO: SZ50013933; REQUIRED FOR HP TURBINE INNER CASING FLANGE MIDDLE AND FLANGE INNER, SPRING LOADED |
| 0256641 | EA | THERMOCOUPLE: DESIGN LENGTH: 8 M; TEMPERATURE RANGE: 0-150 DEG C; SHEATH MATERIAL: STAINLESS STEEL; JUNCTION: 2; HEAD: NONE; WIRES:2; REFERENCE NO: SZ50000255; FOR USE ON MAIN TURBINE/STEAM FEED PUMP THRUST BEARINGS, 2 X NICR-NI |
| 0256642 | EA | THERMOCOUPLE: DESIGN LENGTH: 7 M; TEMPERATURE RANGE: 0-150 DEG C; SHEATH MATERIAL: STAINLESS STEEL; JUNCTION: 2; HEAD: NONE; WIRES:2; REFERENCE NO: SZ50014000-R001; REQUIRED FOR USE ON GENERATOR FRONT BEARING |
| 0256643 | EA | THERMOCOUPLE: DESIGN LENGTH: 7 M; TEMPERATURE RANGE: 0-150 DEG C; SHEATH MATERIAL: STAINLESS STEEL; JUNCTION: 2; HEAD: NONE; WIRES:2; REFERENCE NO: SZ50014000-R002; REQUIRED FOR USE ON GENERATOR REAR BEARING |
| 0256644 | EA | THERMOCOUPLE: DESIGN LENGTH: 8 M; TEMPERATURE RANGE: 0-150 DEG C; SHEATH MATERIAL: STAINLESS STEEL; JUNCTION: 2; HEAD: NONE; WIRES:2; REFERENCE NO: SZ50013934; REQUIRED FOR USE ON STEAM FEED PUMP JOURNAL BEARING |
| 0501225 | EA | ASSEMBLY: TYPE: POSITIONER; APPLICATION: TURBINE SHAFT; MATERIAL: STAINLESS STEEL; REFERENCE NO: 9200-00001; 9200-00077; FOR TURBINE SHAFT POSITIONER; EDDY CURRENT SIGNAL CONVERTER 011 COMBINED WITH DISPLACEMENT TYPE SENSOR; RATING 24VDC; PR6424/000 100 |
| 501240 | EA | ASSEMBLY: TYPE: DISPLACEMENT; APPLICATION: DIFF EXPANSION; MATERIAL: STAINLESS STEEL; REFERENCE NO: 9610-00009; 9210-00074; FOR H.P TURBINE DIFFERENTIAL EXPANSION; EDDY CURRENT SIGNAL CONVERTER SER: 011/916-160 COMBINED WITH DISPLACEMENT TYPE SENSOR; RATING 24VDC; PR6426/000-100; FOR EXTENDED MEASURING RANGE |
| 0501242 | EA | ASSEMBLY: TYPE: DISPLACEMENT; APPLICATION: DIFF EXPANSION; MATERIAL: STAINLESS STEEL; REFERENCE NO: 9610-00011; 9210-00074; FOR USE ON L.P. TURBINE DIFFERENTIAL EXPANSION; EDDY CURRENT SIGNAL CONVERTER COMBINED WITH DISPLACEMENT TYPE SENSOR; RATING 24VDC; PR64261000 100; 100 1000 UM |
|  | EA | HIRSHMANN POWER SUPPLY - MICE SWITCH POWER MSP30 |
|  | EA | HIRSHMANN MM3 - 4FXM2 MEDIA MODULE |

## Work to be performed by the contractor for the works.

## Transporting the Turbine Turbovisory equipment and field Instrumentation at Grootvlei Power Station.

## Supply and delivery on Turbine Turbovisory equipment and field Instrumentation

## Contractor shall always adhere to the delivery agreement period as per Eskom standard (24-56361435)

## Contractor shall comply to Eskom’s policies and procedures

## Contract Deliverables

## The following deliverables are required as per the Configuration Management requirements, during the Contract period:

## Turbine Turbovisory equipment and field Instrumentation and Testing/ Calibration Certification

## Procedure for submission and acceptance of Supplier’s design

Supplier should adhere to the specified goods information during sourcing of the goods to ensure compliance during quality inspections or checks after delivery has been made.

# Supply Requirements

The Supply Requirements for this contract are in an Annexure A to the Contract Data provided by the Purchaser.

# Constraints on how the Supplier Provides the Goods

## Programming constraints

The contract is as and when required, the supplier to adhere to the conditions stipulated on the contract data.

## Work to be done by the Delivery Date

Goods are to be delivered at the delivery place, in good condition and as per their detailed specifications.

## Marking the goods

All goods will be paid once they are delivered at the delivery place as well as after goods inspection or checks is completed.

## Constraints at the delivery place and place of use

Supplier to arrange with the Purchaser prior to delivery of goods, for security access to be arranged. No deliveries will be allowed during weekends and public holidays. Deliveries should be made to the delivery place, during the following times:

Monday – Thursday             :         08H00 - 16H00

Friday                                   :         08H00 - 11H30

## Services & other things to be provided by the Purchaser or Supplier

Both parties will assist during offloading and where material handling equipment such as forklift, overhead crane is required the purchaser will provide for the purpose of offloading the goods.

## Management meetings

Quarterly meetings to be held once the contract is up and running to discuss the progress and anticipated next delivery. The meeting to be attended by the Supply Manager or his/her representative, Service Provider representative and the End User. The meeting will be held at Grootvlei Power Station.

The following text could be used as a model for this section:

| Title and purpose | Approximate time & interval | Location | Attendance by: |
| --- | --- | --- | --- |
| Kick off meeting | Once off after contract placement | Grootvlei Power Station | Purchaser and Supplier |
| Quarterly contract meeting | As an when required | Grootvlei Power Station | Purchaser and Supplier |

Meetings of a specialist nature may be convened as specified elsewhere in this Goods Information or if not so specified by persons and at times and locations to suit the Parties, the nature and the progress of the manufacture of the goods.  Records of these meetings shall be submitted to the Supply Manager by the person convening the meeting within five days of the meeting.

All meetings shall be recorded using minutes or a register prepared and circulated by the person who convened the meeting.   Such minutes or register shall not be used for the purpose of confirming actions or instructions under the contract as these shall be done separately by the person identified in the conditions of contract to carry out such actions or instructions.

## Documentation control

All communication will be in the form of meeting minutes as well as emails and both parties must have contract file to ensure the details of the contract are stored accordingly.

## Health and safety risk management

The Supplier shall always comply with OHS Act 95 of 1993 or ISO 45001, Eskom’s SHE rules and requirements. The Supplier must identify, assess and manage Safety, Health and Environmental risks related to the scope of work. The methodology used for the risk assessment must be provided together with the BRA.

The Contractor shall adhere to all OHS Legal requirements, OHS corporate policies, standards and procedures to which Eskom subscribes and as indicated on the issued SHE specification.

The Contractor shall, when coming on site (Grootvlei Power Station), abide by the Lifesaving Rules.  These will be provided by the Employer on the start of the contract.

The Contractor shall also abide by the Grootvlei High risk Safety, Health and Environmental Specifications 240-73418055, which will also be provided by the Employer.

The Contractor shall, when coming on site (Grootvlei Power Station), make use of approved personal protective clothing such as overalls, safety shoes, safety hat, safety goggles, dust mask and gloves when necessary.

The Employer follows an Incident management procedure (32-95) that includes the investigation of all accidents involving personnel and property.  This is done with the intention of introducing control measures to prevent a recurrence of the same incidents.  The Contractor is expected to fully co-operate to achieve this objective.  The Contractor will report any incident and accidents to Grootvlei Power Station within 24 hours or before end of shift.  This report does not relieve the Consultant of his legal obligation to report certain incidents to the Department of Labour, or to keep records in terms of the Occupational Health and Safety Act, and Compensation for Occupational Injuries and Diseases Act.

The Contractor implements a safety plan and maintains the safety system until the completion of the whole of the works.  The plan, will as a minimum, contain PPE information, written safe work procedures, job specific risk assessments, safety meetings, etc.  The plan will be to the Employer’s satisfaction and will be accepted prior to the commencement of any work.

The Contractor will be subject to periodic audits by the Employer to ensure compliance with the plan.  Any deviations will be corrected to the Employer’s satisfaction.

The Service Manager has the right to stop the Contractor's work activities which, in the opinion of Service Manager, is un-safe.  The Contractor may only continue with work activities when all safety deficiencies have been corrected to the Service Manager’s satisfaction.  The Contractor shall have no claim against the Employer in respect of delay due to the above.

The Contractor shall comply with the health and safety requirements contained in the contract tender package

The Contractor shall comply with the health and safety requirements contained in the contract tender package

## Environmental constraints and management

The contractor and or supplier shall have a documented and implemented environmental management system e.g. environmental policy, operational procedures relating to their activities, aspects/impacts register etc.

The contractor shall comply with all Eskom Grootvlei Power Station environmental requirements such as policies, standards and procedures (work instructions). Non-conformance, incident reporting and investigations shall be done by the contractor. Polluter pays principles shall apply to all Contractors. It is the responsibility of the polluter to clean all spillages and for the rehabilitation of the polluted land and the cost associated with that.

Eskom Grootvlei Power Station shall issue non-conformances where there are deviations from Grootvlei Power Station Procedures and any other environmental requirements. Adherence to the ‘Duty of Care’ as stipulated in section 28 of the National Environmental Management Act 107 of 2008.

All incidents shall be managed according to Eskom Environmental incident management procedure-240-133087117. Station Waste management procedure and colour coding shall be always adhered to.

## Quality

The Contractor shall implement a quality system and maintains the quality system until the completion of the whole of the Works.  The system will comply with the provisions of the ISO9001 and the Eskom Supplier Contract Quality Requirements Specification (240-105658000). The system will be to the Employer’s satisfaction and will be accepted prior to the commencement of any work as per services.

The Contractor will be subject to periodic audits by the Employer to ensure compliance with the system. Any deviations will be corrected to the Employer’s satisfaction.

The Supply Manager has the right to stop the Contractor's work activities which, in the opinion of Supply Manager, does not meet the requirements of the system and will have a detrimental effect on plant performance.

## Invoicing and payment

Within one week of receiving a payment certificate from the Supply Manager in terms of core clause 51.1, the Supplier provides the Purchaser with a tax invoice showing the amount due for payment equal to that stated in the Supply Manager’s certificate.

The Supplier shall address the tax invoice to Purchaser and include on each invoice the following information:

Name and address of the Supplier and the Supply Manager;

The contract number and title;

Supplier’s VAT registration number.

The Purchaser’s VAT registration number.

Description of goods and services provided for each item invoiced based on the Price Schedule.

Total amount invoiced excluding VAT, the VAT and the invoiced amount including VAT;

(add other as required)

## Insurance provided by the Purchaser

No additional insurance apart from the stated in the contract data.

## Records of Defined Cost, payments & assessments of compensation events to be kept by the Supplier

Assessments signed by all parties and notification of compensation events should be kept by the Supplier as well as the Purchaser for future reference, should there be any.

# Procurement

## Subcontracting

### Preferred subcontractors

Tenderers shall subcontract a minimum of 5% of the contract value to the following designated groups:

• an EME or QSE which is 51% owned by black people living in rural or underdeveloped area or townships.

NOTE 1: Tenderers shall submit the following mandatory returnable for Subcontracting:

• Letter of intent to subcontract 5% from Main contractor.

Potential scope to be subcontracted and/or outsourced

• Transportation

Tenderers are required to submit their proposals in the table below

| Local Procurement Content | Eskom target | Tenderer Proposal |
| --- | --- | --- |
|  | 100% |  |