GENERAL ARCHITECTURAL SPECIFICATIONS

1 - FLOOR FINISHES

To be laid on properly levelled screed. Floor finishes to be laid in strict accordance with manufacturers guidelines, preference must be given to manufacturers recommended installers to retain required insurances and warranties on the product and installation methodology.

CARPET TILES –

Areas: Executive Offices 1-4, Meeting Room 1, and Executive Boardroom

Heavy Commercial graded carpet with Axi or Nexbac Backing; +/- 8,4mm thickness; 4m width format or similar, with a fire rating index of 2 and/ or 3, as per SANS 10177-4 respectively.

- Tiles to be laid in accordance with SANS 10186 fitting code of practice.

- Carpet tiles to be manufactured in South Africa, with min 5year installation guarantee and a 5year product warranty to have a green tag level; a gold rated certification.

* *Tiles to be laid with adhesives in accordance with the manufacturers’ recommendations.

Areas: PA’s Space, and Passage 1, General Offices 1 & 2, Open Plan Offices, Meeting Room 2

Heavy Commercial graded carpet with Axi or Nexbac Backing; +/- 5,5mm thickness; 1000 x 250mm format or similar, with a fire index 2 and 3 as per SANS 10177-4 respectively.

- Tiles to be laid in accordance with SANS 10186 fitting code of practice.

- Carpet tiles to be manufactured in South Africa, with min 5year installation guarantee and a 5year product warranty to have a green tag level; a gold rated certification.

- Tiles to be laid with adhesives in accordance with the manufacturers’ recommendations.

**Tiles to be laid with adhesives in accordance with the manufacturers’ recommendations.

Areas: Multi-purpose Space

Heavy Commercial graded carpet with Bitumen Backing (Nexbac); +/- 6mm thickness; 500 x 500mm format or similar, with a fire index 2 and 3 as per SANS 10177-4 respectively.

- Tiles to be laid in accordance with SANS 10186 fitting code of practice.

- Carpet tiles to be manufactured in South Africa, with min 5year installation guarantee and a 5year product warranty to have a green tag level; a gold rated certification.

- Tiles to be laid with adhesives in accordance with the manufacturers’ recommendations.

**Tiles to be laid with adhesives in accordance with the manufacturers’ recommendations.

*Underlay required for wall-to-wall carpets, Specifications as below noted:

Extended SPECIFICATIONS:

- 100% recyclable Fibre / Yarn staple blend (PP/PA6)

- Total Thickness: +/- 8,5mm; Product Width 4m

ADDITIONAL PROPERTIES

- Extremely durable; improved sound insulation; Improved Thermal Insulation

- Odour free; Dust free; free of lumps

- High resistance to mould and mildew, Anti-fatigue construction

VINYL TILES –

Areas: IT/Server Room

Permanently Static Dissipative, Heavy Commercial graded flooring; 2mm thickness; 2m x 23m length Sheet format or similar, with a fire rating index of 2 and/ or 3, as per SANS 10177-4 respectively.

- to be laid in accordance with SANS 10070 fitting code of practice.

- to be manufactured in South Africa, with min 5year installation guarantee and a 5year product warranty to have a green tag level; a gold rated certification.

* *Vinyl to be laid with adhesives in accordance with the manufacturers’ recommendations.

Areas: Passage 2, Storeroom, Print room, Exhibition Area, Reception, Dining

Heavy Commercial graded flooring; +/-2,5mm thickness; stick down, LVT, dryback or similar, with a 0,55mm wear layer and a fire rating index of 2 and/ or 3, as per SANS 10177-4 respectively.

- Tiles to be laid in accordance with SANS 10070 fitting code of practice.

- Tiles to be manufactured in South Africa, with min 5year installation guarantee and a 5year product warranty to have a green tag level; a gold rated certification.

* *Vinyl to be laid with adhesives in accordance with the manufacturers’ recommendations.

Vinyl Tiles/Sheeting: Vinyl tiles (LVT) and sheet vinyl installation guidelines require strict moisture management, precise adhesive scheduling, and rolling protocols to prevent seam splitting or bubbling.

- Environmental Controls Acclimatization: Condition all vinyl materials and adhesives in the installation space for at least 48 hours. Temperature: Maintain a constant room and subfloor temperature between 18°C and 29°C. Climate Stability: Maintain these temperature boundaries for 48 hours after completion to allow adhesive cross-linking.

- Phase 1: Substrate Preparation Smoothness: Subfloor must be completely level; deviations must not exceed 3mm over a 3-metre radius. Remediation: Apply a high-compressive-strength, polymer-modified self-levelling underlayment to smooth out dips and rough concrete texture. Cleaning: Vacuum twice to remove all dust, drywall compound, grit, and structural debris. Moisture Limits: Concrete must register below 75% RH (ASTM F2170) or under 2.27kg / 1000 sq ft / 24 hours (Calcium Chloride ASTM F1869).

- Phase 2: Layout & Dry Fitting Tiles (LVT): Snap perpendicular chalk lines to find the centre of the room, dry-laying tiles to ensure border cuts are at least half-tile width. Sheet Vinyl: Unroll and rough-cut sheets 50mm to 100mm longer than the room dimensions, letting the material relax flat before final trimming. Seam Planning: Position sheet seams away from high-traffic walkways and main doorway entry points.

- Phase 3: Adhesive ApplicationTrowel Selection: Use the manufacturer-specified notched trowel (typically 1.6mm x 1.6mm x 1.6mm V-notch) to control spread rate. Open Time: Allow the pressure-sensitive or wet-set adhesive to flash off according to ambient humidity (typically 10 to 30 minutes). Transfer Check: For wet-set adhesives, ensure immediate 100% glue transfer to the vinyl backing without oozing through joints.

- Phase 4: Placement & Rolling Air Elimination: Lay the vinyl down smoothly from the centre outward to push out trapped air pockets. Three-Section Rolling: Roll the entire floor with a heavy 45kg to 68kg three-section roller. Cross-Rolling: Roll first along the width, then crosswise along the length to thoroughly flatten adhesive ridges. Detail Rolling: Use a hand-roller along wall edges, seams, and tight corners to guarantee bond integrity.

- Phase 5: Seam Sealing (Sheet Vinyl Only) Chemical Welding: For commercial sheet vinyl, clean the seam channel and apply a cold-weld liquid sealant using a fine applicator nozzle. Heat Welding: For heavy-duty institutional sheet vinyl, rout the seam joint and use a hot-air welding gun with a matching vinyl welding rod, then trim flush.

- Phase 6: Curing & Post-Installation Protection Foot Traffic: Restrict all foot traffic for a minimum of 24 hours. Machinery Moving: Wait 48 to 72 hours before rolling heavy equipment or light machinery onto the floor. Point Loads: Place protective plywood paths over the vinyl when moving machinery to prevent gouging or track indentations.

PORCELAIN/ CERAMIC TILES –

Areas: Executive Ablutions

Large Format, with rectified edges for narrow grout lines, slip-resistant, soft matte finish.

Size: 750mm x 1500mm format

Areas: General Ablutions, Kitchen and Kitchenette

Large Format, with rectified edges for narrow grout lines, slip-resistant, soft matte finish.

Size: 600 x 600mm format.

Ceramic/Porcelain Tiles: Ceramic and porcelain tile installations require strict adherence to substrate deflection limits, precise mortar coverage, and intentional movement joints to prevent cracking and tenting.

- Environmental Controls & Substrate Limits Temperature: Maintain ambient and substrate temperatures between 10°C and 35°C during installation and for 48 hours after. Deflection: For porcelain or large-format tiles, the subfloor deflection must not exceed L/360 under live and dead loads. Cure Time: New concrete substrates must be cured for a minimum of 28 days before tile installation begins.

- Phase 1: Substrate Preparation Flatness: Subfloor deviations must not exceed 6mm in 3 metres for standard tiles, or 3mm in 3 metres for large-format tiles (any edge >38cm). Isolation: Apply a liquid or sheet crack isolation membrane over concrete control joints or shrinkage cracks to prevent stress transfer. Cleaning: Remove all dust, laitance, sealers, and curing compounds to ensure an open, porous surface for mortar adhesion.

- Phase 2: Mortar Selection & Mixing Mortar Type: Use a polymer-modified thin-set mortar meeting ANSI A118.4 or ISO C2S1 standards. Porcelain requires highly modified mortars to bond to its dense, non-porous back. Large-Format: Use a Medium Bed Mortar (LFT mortar) for heavy or large tiles to prevent slumping and lippage. Slake Time: Mix the mortar thoroughly, let it sit (slake) for 5 to 10 minutes to activate chemicals, then remix without adding more water.

- Phase 3: Layout & Expansion Joints Chalk Lines: Snap perpendicular layout lines from the centre of the room, adjusting to avoid narrow sliver cuts at the perimeter walls. Movement Joints: Install EJ171-compliant expansion joints every 6 to 8 metres in interior spaces, and at all perimeters where the tile meets a vertical surface. Joint Width: Maintain a minimum grout joint width of 1.5mm for rectified tiles and 3mm for non-rectified tiles; never butt tiles tightly together.

- Phase 4: Mortar Application & Tiling Directional Trowelling: Comb the mortar in straight, parallel lines using a square-notched trowel. Do not use swirl patterns, which trap air pockets. Back-Buttering: Flat-trowel a thin layer of mortar onto the back of every large-format tile to guarantee 95% mortar coverage (required for commercial/heavy traffic zones). Setting & Levelling: Bed the tile into the mortar, pushing perpendicular to the ridges to collapse them. Use a tile levelling clip system to eliminate edge lippage.

- Phase 5: Grouting Preparation: Remove all levelling clips and clean excess, dried thin set out of the grout joints before it cures completely. Application: Pack the grout (cementitious or chemical-resistant epoxy) firmly into the joints using a hard rubber float held at a 45-degree angle. Clean-up: Wait for the grout to firm up slightly, then wipe the tile faces diagonally with a damp, tightly wrung hydro-sponge. Use a microfiber cloth to buff off remaining grout haze after 2 to 4 hours.

- Phase 6: Curing & Traffic Protection Grout Cure: Avoid walking on the newly grouted floor for at least 24 hours. Heavy Loads: Keep machinery, rolling carts, and heavy training equipment off the tiled surface for a minimum of 7 days.

All floor finishes and colours to be advised by the Architect.

2 - INTERNAL WALLS:

DRYWALL - Soundproof –

Areas: Executive Offices, Meeting Rooms

High-performance acoustic drywall system: non-load bearing, soundproof, single frame drywall that uses dense gypsum, steel cores or constrained layer-damping to reduce sound.

DRYWALL – Moisture Resistant Wall System –

Areas: Ablutions and Kitchens

Lightweight, non-load bearing, which has water repellant additives in the core and treated facing (or similar approved). Board thickness 12,5mm.

DRYWALL – Moisture Resistant Wall System –

Areas: All other walls

Lightweight, non-load bearing high-performance fire-resistant system, providing a fire rating of up-to 60minutes.

DRYWALL FINISHES

All Paint applications to be primed with 1 coat Professional Plaster Primer, then 2 coats of paint (smooth water based Low sheen). All Paint to be Professional Hygiene Low Sheen or similar. Colours: to Architectural Spec.

INTERNAL WALL TILES

To be Matt White 200 x 200mm A Grade.

CURTAIN WALL SYSTEM (Internal):

Internal curtain walls and acoustic-rated space, Aluminium Systems utilizes a mix of specialized heavy duty architectural systems. Glazed Shopfront, with profiles that focus tightly on acoustic dampening, flush visual aesthetics, and simplified indoor assembly. The required system must sustain:

- Glazing Flexibility: Built to support double glazing configurations natively. Standard internal pockets easily secure insulated glass units (IGUs) from 20 mm up to 32 mm thick.

- Acoustic Performance: The system features deep structural internal pockets that isolate glass edges using high-grade EPDM / SEBS gaskets, preventing structural flanking noise between corporate meeting rooms.

- Slim Sightlines: Engineered as a "slim-line" alternative to traditional stick curtain framing, reducing visible interior vertical lines while maintaining structural rigidity up to standard ceiling heights.

- Speed of Install: Fabricated in up to half the time of standard commercial framing due to clip-together profiles, eliminating messy on-site wet silicone work inside carpeted office environments.

Summary of Internal Double-Glazing Engineering Specs

i.	Aluminium Grade: Architectural extruded 6063-T6 alloy providing sharp, clean line definitions across indoor viewing angles.

ii.	Glazing Gaskets: Employs co-extruded SEBS/EPDM rubber dry-glazing gaskets, which remain stable under indoor HVAC environments and avoid shrinkage gaps over time.

iii.	Thermal Breaks: Optional integrated thermal breaks are available for internal walls that divide temperature-controlled data centers or cold chambers from standard office temperatures.

iv.	Integrated System Inserts

v.	The curtain wall framing must seamlessly accept and integrate the following internal opening elements:

- Standard Windows.

- Hinged Doors.

- Sliding Doors.

- Tilt & Turn Units.

3 - MATERIAL & FINISHES:

SURFACE TREATMENT

All aluminum extrusions must be finished in strict accordance with Qualicoat architectural powder-coating standards or processed via an approved anodizing specification to resist UV degradation and atmospheric corrosion.

WALLPAPER

Majestic range: Fiberglass (or similar approved) - paint it with any of the suggested paint colours or

Majestic range:  Caro-Line (or similar approved)

BLINDS AND CURTAINS:

BLINDS: - Smart Black-out Roller – smart controlled, on a silent motor and a premium, silent curtain track motorization. With a sun sensor or timer. Colour: to Architect’s Spec.

Areas: Executive Offices, PA Area, Meeting Room 1 and Executive Boardroom

BLINDS: - Shear weave – remote controlled, Linen or Velvet fabric on a silent motor and a premium, silent curtain track motorization. Colour: to Architect’s Spec.

Areas: Open Plan Office, Office 1 & 2, Dining Room, Kitchen, Meeting Room 2, Exhibition Space, & Multi-purpose Space

4 - DOORS:

Internal Doors & Internal Door frames: Primed & Painted with Plascon Velvaglo VLO10 Tequila. External doors and door frames: timber hardwood to be sanded & sealed with Plascon Exterior Preservative Colour: Teak FBR19. Refer to Door schedule.

DOORS:

- WOOD - 813 x 2100 High, Solid core timber veneer doors, sleek modern flush. Coil spring hinges, stainless steel handles with stainless steel self-closer and floor mounted door stopper, Lockset fit for door & room occupancy by manufacturer/supplier.

- FULL GLASS - Frameless or slim-frame 1800 x 2100 High, glass doors. Lockset fit for door & room occupancy by manufacturer/supplier with or without 300x813mm stainless kickplate.

- ALUMINIUME FRAMED - 1610 x 2032high, with safety glass and supplier's ironmongery. Natural Anodised Finish. Colour to Client or Architect's Spec.

- Single Commercial Veneer Flush Hollowcore Door 760x 2032 High: Semi Solid Door leaf to fit frame, to be sanded & primed with 1 coat primer and painted with 2 coats paint. Ironmongery to be brushed aluminium finish. Colours to Client or Architect's Spec. Lockset fit for door & room occupancy by manufacturer/supplier with or without 300x760mm stainless kickplate.

- Single Commercial Veneer Flush Hollowcore Door 900x 2032 High: Semi Solid Door leaf to fit frame, to be sanded & primed with 1 coat primer and painted with 2 coats paint. Ironmongery to be brushed aluminium finish. Colours to Client or Architect's Spec

- FULL GLASS - Custom Frameless or slim-frame 3000 x 2100 High, sliding double glass door. With Supplier's ironmongery

- ALUMINIUME FRAMED - 813 x 2100high, with safety glass and supplier's ironmongery. Natural Anodised Finish. Colour to Client or Architect's Spec.

- WOOD - 1800 x 2100 High, Solid core timber veneer doors, sleek modern flush.

- WOOD - Service Duct Doors, Fire Escape Doors, with supplier's ironmongery

- FIRE DOOR - 1630mm Wide x 2032mm Height with 44~60mm thickness - Double Leaf, fully cladded Class A Fire door with Class A fire rated frame, Fire rated ironmongery with surface mounted EN 1125 Horizontal panic push bar, with 3-locking vertical points with mild steel and electroplated door keeps and adjustable shoots, ironmongery coating finish to later specification, actuator units to be linked to monitoring/alarm system, Internal or external use

Ironmongery & Hardware Specifications:

All hardware and ironmongery must be heavy-duty, marine-grade where applicable, and fully compatible with the chosen aluminum profile series to prevent galvanic corrosion.

- Hinged Doors: Equipped with minimum Grade 13 stainless steel ball-bearing hinges or heavy-duty pivot mechanisms. Locking mechanisms must utilize a secure 3-point or 5-point hook-bolt system [Insert Lock Brand/Model] with euro-profile cylinders master-keyed to the building layout. Overhead heavy-duty door closers [Insert Closer Code] must be fitted to high-traffic routes.

- Sliding Doors: Fitted with adjustable dual tandem stainless-steel rollers engineered to support the specific double-glazed panel weights. Locking mechanisms must feature multi-point hook locks with internal flush pulls or D-handles [Insert Handle Style].

- Finish: All visible hardware handles and plates must match the architectural finish specification or be finished in Matt Black Powder.

Warranties & Maintenance for Aluminium Finishes:

- Finish Warranty: The applicator shall provide a written product warranty guaranteeing the finish against peeling, cracking, blistering, or significant color fading for a minimum period of 10 / 15 or 20 years.

- Structural Integrity Warranty: The manufacturer shall provide a 10 / 15- or 20-year warranty on the structural integrity of the system components and thermal breaks.

5 - FIRE:

Fire Hydrant and Fire Extinguishers to Fire Consultant's spec. and approval, with SABS & NBR certification.

6 - CEILINGS:

Flush Plastered ceilings:

to be primed with 1 coat Professional Plaster Primer & 2 coats Professional Contractors Matt White.

Lay in Ceilings:

- Lay in 600 x 600mm or 600 x 1200mm ceiling tile fissured style exposed grid suspended ceilings in commercial buildings.

- Lay in ceiling, light diffuser panels 600 x 600mm or 600 x 1200mm.

- Lay in Ceiling Baffles sound barrier in various sizes, to be installed.

- Contractor to establish fixing positions prior to installation ensuring structural integrity and not to negatively impede new proposed work, any discrepancies to be raised with the Architectural Professional and Engineer.

*The appropriate main tee and cross tee suspended ceiling grid system to be selected that has an exposed designed ''face'' in [colour selection to later specification] and that is according to part t of sans for this occupancy class.

BULKHEADS:

To be Gypsum Board, non-combustible – general, various sizes refer to architect’s drawings.

CEILING TRAP DOORS:

Steel Ceiling Trapdoor with Hinged Steel Lid (610mm x 610mm).

7 – ABLUTIONS and SANITARY FITTINGS

TAPS:

Whb to Male & Female WC & Staff WC: Demand (1 for hot & 1 for cold). Wheel-chair Accessible WHB. Medical Pillar Mixer Tap (hot & cold). Kitchen Sink taps: Star Tap Pillar Mixer.

WC PANS:

to be Elite Closed Coupled White Suite, or Sanitaryware Wall Hung White, with bolt through wall bracket fixing set for 220mm wall and floor bracket fixing set for use in 220mm wall (also adjustable for paraplegic applications). Wheel Chair Accessible WC: Paraplegic Low-Level Suite

WHBS:

Vanity Basin, Whb for Accessible: Franke (Washroom Systems) Oval A Basin wall mounted 420 x 340 x 135mm stainless steel.

URINALS:

Wall Hung Bowl Urinal supplied with domical grating, c.p top inlet spreader) and two hangar brackets with exposed top inlet flush valve.

* To be as per Mechanical Engineer specifications and to comply with SANS 10400 regulations.

GRABRAILS:

Grabrails for Accessible WC: 3 x Cistern Rail 750x206mm, Dogleg Rail with 2 flanges 300x300 x300mm, 2 x RH & 2 x LH. Exact quantities to be assessed on site.

CORNICE:

75mm Polystyrene Cornice, painted with Wall & All Colour: White or Cornice to be a shadow Line Style: 45mm x 3600mm x 32.5mm or 25mmx25mm Cornice also known as plaster-trim sizing that works in unison with the suspended grid ceiling system and tiles.

TOILET PAPER HOLDER:

Sanicare Toilet Roll Holder - Holds 3 rolls (stainless steel) to be fitted to each WC.

DOORSTOPS:

Design Hardware Rubber Door Stop Size 24mm, Colour Black to be fitted to all doors.

8 - INTERNAL SIGNAGE:

To be from Design Hardware: Male Code: B3344 x 2, Female: Code: B3345 x 2, Physically Impaired Code: B3346 x1.

9 - FIRST AID EQUIPMENT:

To be specified by SHEQ specialist.

10 - LIGHTING PANELS:

To Electrical Engineer’s Specification.

Specifications Policy (ESK PB AAQ 3) and the official Corporate Identity (CI) Manual to maintain corporate branding.

Corporate Paint & Finishes Colour Codes

Eskom applies its primary and secondary corporate color palettes to interior architecture, partitions, and custom furniture elements:

| Element | Material/Brand Ref | Colour Code / Specification |
| --- | --- | --- |
| Primary Corporate Blue | Doors, cubicles & accents | Pantone 287 C (100C, 70M, 0Y, 10K) |
| Secondary Corporate Green | Door frames & architraves | Pantone 582 C (30C, 0M, 100Y, 45K) |
| Corporate Gold | Pot plants & special details | Pantone 8004 C (40C, 40M, 55Y, 5K) |
| Reception Desktop | Laminate Face | Formica – Sierra Granite (534) |
| Powder Coating | Desk/table legs & metal handles | Akzo Nobel Interpon 600 Matt (ANP 9054 / Cowrie Gold) |
| General Interior Walls | Wall Paint | Dulux 'Crushed Satin' |

11 - PLANTERS & FRAMING ACCESSORIES:

Decorative office pot plant holders must be finished in metallic CI colours. Corporate frames for notices or artwork must remain slimline and avoid wide, heavy structural profiles.

PAINT:

GENERAL NOTE:

It is advised that the appointed contractor should subcontract all specialized installations to the manufacturers preferred or recommended merchants, to maintain mandatory guarantees or warranties and insurances for the installations and supplied products.

All wall coverings, including paint colours should be verified once the contractor is on site as the colour codes may change due to amendments in industry norms/standards and compliances or manufacturer amendments.

All floor finishes should be verified once the contractor is on site as specification codes may change due to amendments in industry norms/standards and compliances or manufacturer amendments.

Eskom Corporate Identity must take precedence when considering all interior finishes, the contractor must obtain written approval from the designated professional before installation of any finishes.

All indicated sanitaryware, including taps, mixer taps, wash hand basins, sinks, WC’s, should be verified once the contractor is on site as specification codes may change due to amendments in industry norms/standards and compliances or manufacturer amendments.

QUALITY ASSURANCE:

Under official Eskom Procurement Frameworks, the principal contractor is held contractually, financially, and legally liable for quality through the enforcement of the [Eskom Supplier Quality Management Specification paired with standard NEC3 contract conditions.

A contractor cannot be awarded an Eskom tender without completing and signing Form A (Tender & Contract Quality Requirements). By signing this document, the contractor legally commits to satisfying both ISO 9001 standards and Eskom's exact material policies. If the contractor fails to fix the defect within the contractual window, Eskom reserves the right to assess the cost of having a third party rectify the defect and deduct those costs directly from the contractor.