Bid 00384/26

Supply and Installation of Storage Water Tank at Joburg Theatre

ADDENDUM TO TENDER DOCUMENT

Addendum#1

The tenderer is to note that the full content of this Addendum to Tender Document for Bid No. 00384/26 as described above, will form part of the tender documentation as originally compiled.

In terms of the Bid, issued by JCT on Tuesday,10th February 2026, herewith please find enclosed the addendum for the subject project, which shall be included as part of the tender document submission when the tender closes

Except to the extent modified by this Addendum, the original terms of the tender shall remain unchanged and binding on all the interested parties. The tenderer shall submit a copy of the addendum, including all tender deliverables specified in the original Tender Document.

1.THE FOLLOWING CONSTITUTE A CHANGE TO THE TENDER DOCUMENT

| ITEMS | Current on the tender document | Corrections / additional on the tender document |
| --- | --- | --- |
| Preventive Maintenance Requirements | It was not specified | Preventive Maintenance Requirements The Contractor shall provide a recommended maintenance schedule including: Routine Inspection - Weekly visual inspection for leaks - Monthly inspection of valves, joints, and pipe supports - Quarterly inspection of structural components - Annual full internal inspection |
| Future Repairs and Rehabilitation | It was not specified | Future Repairs and Rehabilitation The tank system shall be designed to allow future repair without full dismantling. Structural Repairs Provision shall be made for: - Crack repair - Internal lining repair or recoating - Corrosion protection touch-up Mechanical Component Replacement The following components shall be replaceable without major civil works: - Float valves - Inlet/outlet valves - Level sensors - Gaskets and seals - Pump seals and bearings - Drain valves |
| Future Maintenance and Repairs | It was not specified | Bidders must Include a 2-year maintenance and repair plan post-handover, extending beyond the standard 12-month warranty period: Annual Maintenance Contract (AMC) shall include: - Scheduled inspections and maintenance - Preventative repairs - Annual cleaning - Minor repairs - Water quality testing - Emergency call-out support |

I/We the undersigned hereby confirm that we have received addendum no 1.

Company Name …………………………………………………………………………………………..……………….

Name & Surname ……………………………………………………….      Signature …………………………………

Designation ………………………………………….    		Date ………………………………………….