# Part 4: Site Information

| Document reference | Title | No of pages |
| --- | --- | --- |
|  | This cover page | 1 |
| C4 | Site Information |  |
|  | Total number of pages |  |

# Part 4: Site Information

# General description

## Rigi SS

Rigi Substation is located in the Gauteng province, in Vanderbiljpark; Figure 1 below displays the locality map. The refurbishment of the access road at Rigi Substation is designed for which this report includes the standard civil works as part of the detail design.

Site coordinates

Latitude 26°40'52"S Longitude 27°51'22"E

Figure 1: Locality map and site of Rigi Substation

Figure 2: Road layout

# Existing buildings, structures, and plant & machinery on the Site

# The site is located on the road that leads to the access of the Rigi substation/s therefore the existing infrastructure would be the Rigi Substation and the boundary fencing

# Subsoil information

N/A

# Hidden services

N/A

# Other reports and publicly available information

The drainage along the road from the gate must be cleaned and where required should be repaired. The drainage outside the substation adjacent to the gate must be cleared of debris and a screen should be installed to both drainage headwalls to prevent debris blocking the drains. After the completion of cleaning and repairing the drains, vegetation should be add/planted in the area adjacent to the gate, this will reduce the soil from washing into the drains as the drains are the lowest point. A cut of drain will be constructed on one side alongside the road at the gate as detailed on RIG20P08-SE-D42 . The drainage and concrete gutter along the road inside the substation should be cleared and repaired where damaged.