| PART B RESPONSE SHEET IN TERMS OF A REQUEST FOR INFORMATION To be completed by the supplier |  |  |  |  |
| --- | --- | --- | --- | --- |
| To | Eskom Holdings SOC Ltd | Date |  |  |
| Attention | Njabulo Masango |  |  |  |
| Tel no |  | Fax no and /or e-mail address |  |  |
| From |  | Address |  |  |
| Address |  |  |  |  |
| Sender |  |  |  |  |
| Description of the works/goods/services | Expression of interest (EOI) to supply, deliver, install, commission and provide training on the operation of an Automated Asphaltene analyser and/or an Automated S-value analyser for fuel oil samples. |  |  |  |

Scope of work

Supply, deliver, install, commission and calibrate instrumentation as listed below. Provide training on the operation and maintenance of the equipment:

1) System 1: To determine the concentration of asphaltenes in fuel oil, a fully automatic asphaltene analyser, model APD-600A

a) Fully Automatic Asphaltene analyser, model APD-600A correlating with ASTM D6560 and D3279, complete with

b) water bath with temperature control of 80 ˚C to 100 ˚C;

c) constant temperature chamber with temperature control of 60 ˚C to 80 ˚C;

d) analytical balance with the minimum reading of 0,0001g,

e) printer (IDP-100),

f) data management system with

g) Soft-CapE installed,

h) UPS and

i) standard accessories as listed in brochure.

2) System 2: Automated S-value analyser, model SV10

a) Automated S-value analyser, model SV10 according to ASTM D7157 complete with

b) USB graphic printer,

c) suitable balance,

d) UPS and

e) data management system and

f) required software.

| No. | Questions | Response |
| --- | --- | --- |
| 1 | What is the supplier’s name? |  |
| 2 | What equipment are you competent to supply, deliver, install, commission and provide training on with reference to the options stated above? |  |
| 3 | Provide the brand name and model of such equipment. |  |
| 4 | Provide a description of the unique properties of the equipment. |  |
| 5 | How many samples could be loaded at a time? |  |
| 6 | What is the expected time to analyse a sample from opening the sample container to completion of the test. |  |
| 7 | What peripheral/auxiliary components will be required for optimum performance (PC, water bath, printer etc.) of the system? |  |
| 8 | Are there any of these systems installed in South Africa? |  |
| 9 | What will be the expected delivery time of the system from receipt of the purchase order? |  |

Yours faithfully

| Name | Designation | Signature | Date |
| --- | --- | --- | --- |
| Telephone number |  | Fax and/or e-mail address |  |