Specifications for tractor services required in Marobala Village, Capricon District, Limpopo

The following specification is for planting an experimental trial for scientific purposes.

- Ploughing or ripping 1ha of land.

- Discing 1ha of ploughed/ ripped land.

- Planting two cultivars of sorghum on the 1ha.

- Service provider must bring own tractor & implements, driver for the tractor and the appropriate sorghum seed plates for the planter.

Operations are to be carried out as follows:

Location: Marobala Village, Capricon District, Limpopo.

Date: 4 – 6 December 2024 and 20 – 21 January 2025

- Service provider must select the appropriate tractor depending on the size of the implements (plough, ripper, disc or planter) they will use. Here are some details for the planter:

- Planting is to be done on an experimental trial that has 72 plots which fit in 1 hector. Each plot is +/- 54 m² (12 x 4.5 m) and has 7 rows (the length and width of the plots can be adjusted according to the size of the planter and tractor used)

- A 6-row or 4-row sorghum/maize planter is preferred to minimize the number of tractor rotations in the field. However, small planters are not discouraged.

- The interrow spacing is 75 / 76 cm and intra-row spacing is 30 cm.

- The design of the experimental trail can be supplied upon request to the service provider if further clarification is required.

- Two cultivars of sorghum will be planted on designated plots. 36 plots for each cultivar

- The plots are at located at random.

- There will be two planting dates, half of the trial (36 plots) will be planted in December 2024 and the remaining 36 plots will be planted in January 2025

N.B: Service provider must bring sorghum/maize planter with appropriate plates for sorghum (sorghum planting plates or similar).

Explanations:

- The experimental trial is the sum total of the 72 plots that are within the 1 ha of land.

- The plots are the 72 planting bays /areas on which the two cultivars of sorghum will be planted.